3GPP TSG RAN WG1 #96bis                                 	       R1-1903902

Xi’an, China, April 8th – 12th, 2019

Agenda item:		7.2.6.4

Source: 		Idaho National Laboratory

Title:   	Non-Periodic Scheduling Request Transmitter for NR Release 16

Document for:    	Discussion/Decision

# Introduction

During the study phase of NR Release 16, a Non-Periodic Scheduling Request (NP-SR) enhancement scheme was studied [1]. With respect to the periodic SR scheme used in NR, this scheme has the advantages of eliminating the wait time, or SR alignment time, and the bandwidth overhead associated with Rel-15 SR transmissions, both of which make it attractive for URLLC applications requiring very low latency and high reliability [2], [3]. In this contribution, we examine some aspects of the transmitter structure for this NP-SR transmission scheme and related aspects to UE design.

The contribution is organized as follows:

- Section 2 presents the NP-SR transmitter structure.

- Section 3 presents a brief comparison between the NP-SR and Rel-15 SR transmitter structures.

- Section 4 presents some conclusions.

# Non-Periodic SR (NP-SR) Transmitter Structure

The details of our proposed NP-SR transmission scheme have been summarized in [2] and [3]. This alternative method to transmit SRs has the following advantages over the current periodic Rel-15 SR method:

- Even for the lowest periodicity of 2 OFDM symbols used in Rel-15 SR, the NP-SR method offers lower latency because with NP-SR, the scheduling request can be transmitted in the OFDM symbol immediately following packet arrival at the UE, thus eliminating the wait time to transmit an SR (see Table 1 in Section 2 of [2] for a URLLC latency comparison under various conditions).

- NP-SR is transmitted at a sufficiently low power density over spectrum that is already used by other resources causing negligible interference, thus eliminating the bandwidth overhead that needs to be allocated for Rel-15 SR (see Figure 3 in [3]). This also increases the spectrum available for data transmission, thus improving spectral efficiency.

In [3], we have described the NP-SR transmission scheme as a spread spectrum method using Direct Sequence – Spread Spectrum (DS-SS). It is also possible to describe the same method using a description that is similar to OFDM transmission to spread the SR bit transmission over a wide bandwidth at a low power density so that it can be transmitted reliably without causing undue interference to other signals occupying the same spectrum.

Using Zadoff-Chu (ZC) sequences derived from a single root sequence, z0, of length NZC = NFFT (the FFT size used in the OFDM transmitter), it is possible to construct a signal by assigning a single ZC sequence with a unique shift ∆i  to each UE which is assigned to the ith-UE by the network to spread it over the entire carrier bandwidth W to yield sufficiently low power density. To ensure a sufficiently large number of shifts are available to accommodate a given number of UEs, and that sequences assigned to different UEs are mutually orthogonal over a single OFDM symbol, the shift spacing between sequences, ∆, is chosen so that ∆ = int(NFFT/Nu), where NFFT is the OFDM FFT size corresponding to bandwidth W for a given subcarrier spacing ∆f and Nu is the number of URLLC users to be accommodated. As a result, the individual sequence shifts are evenly spaced by ∆. For example, with NFFT = 2048 and Nu = 20 users, a shift spacing ∆ = 102 can be chosen to accommodate 20 users.  If more users need to be accommodated, the spacing can be reduced, which would increase mutual interference slightly in some channels with long delay spread.

Based on the above, a NP-SR signal at the UE is generated by applying a FFT to the Δi-samples right circularly shifted z0 sequence. This shifted ZC-sequence is then fed into the NR OFDM transmitter. Notice the NP-SR is time synchronized with the system time and the CP insertion is performed as part of the OFDM transmission. If desired a windowing step could be performed to obtain a band-limited transmission confined to a given BWP. One further simplification is after receiving the assigned shift parameter value (∆i) from the network, the ith-UE could perform the above steps once and store the FFT version of the Δi-samples right circularly shifted z0 sequence for subsequent NP-SR transmissions.

Table 1 below lists the key parameters in the NP-SR signal generation procedure. These parameters can be applied to a fixed numerology and to a mixed numerology separately.

Table 1 NP-SR Signal Parameters

| Parameter | Value |
| --- | --- |
| Root ZC sequence z0 length NZC | NZC= NFFT=WΔf=W2μ*15 [kHz] |
| Number of users | Nu |
| Right circularly shift ∆i with i=1,2,…., Nu | ∆i=i-0.5*intNZCNu |

To effectively detect the transmitted shifted ZC sequences in a noise plus interference environment, a variety of signal processing algorithms can be designed. One approach may be to correlate the received signal samples with stored samples for each possible sequence shift location and see if the result exceeds a certain threshold. Other algorithms can be designed to improve the performance of NP-SR significantly.

# NP-SR vs. NR SR Comparison

Generating the NP-SR signal using the commonly known ZC sequences provides a simple method that could be used by UEs that is compatible with OFDM signal generation used in LTE/NR. Table 2 below gives a brief comparison of the two methods.

Table 2 NR SR vs NP-SR feature comparison

| Category | NR SR (Periodic) | NP-SR (Non-Periodic) |
| --- | --- | --- |
| Wait Time | Average = P/2 symbols (where P is the SR periodicity in OFDM symbols) Worst case = P symbols | Average = One Half OFDM symbol Worst case = One OFDM symbol |
| Resource Overhead | Significant resources need to be allocated for SR transmission via PUCCH (see Figure 3 in [3]) | No overhead. |
| Multiplexing SR with ACK/NACK | Possible depending on PUCCH format used | NP-SR is independent of ACK/NACK transmissions. |
| Multiplexing eMBB with URLLC traffic | Requires pre-emption or prioritization of URLLC traffic with additional signalling overhead | Could be used with priority URLLC traffic to take advantage of lower latency and reduced overhead |

# Conclusion

In this contribution we have shown one possible realization of the Non-Periodic Scheduling Request (NP-SR) transmission method previously described in [2] and [3]. This SR transmission method is a more efficient match for URLLC applications because it is event-driven and does not depend on any SR periodicity, which has an inherent wait time and suffers from increased overhead as periodicity is decreased to reduce latency.

This method has the following advantages over the periodic method adopted in Rel-15:

- It eliminates the wait time incurred by the periodic SR method, which becomes a non-negligible latency component in the stringent overall latency target for some applications, such as Factory Automation.

- It eliminates spectrum overhead, as the NP-SR signal is transmitted over spectrum already occupied by other resources and is spread at a sufficiently low spectral density to cause negligible interference.

Proposal: The NP-SR design described above should be considered for NR Release 16.

# References

# [1] TR 38.824, V 1.2.0, see section 6.4.3.

# [2] R1-1901589, "Additional Results for Underlay Scheduling Request for NR Rel. 16," Idaho National Laboratory, RAN1#96 meeting, Feb-Mar, 2019, Athens, Greece.

# [3] R1-1900015, “Underlay Scheduling Request for NR Rel. 16 and Latest Results,” Idaho National Laboratory, RAN1 AH January 2019 meeting, Taipei, Taiwan.

# [4] 3GPP TS 38.211, see section 4.2.

The FFT size is assumed to be consistent with LTE/NR numerology as implied by [4] and accounts for nulling of desired subcarriers.