3GPP TSG RAN WG1 #96bis Meeting		R1-1904072

Xi’an, China, 8th – 12th April 2019

Source:	vivo

Title:	Physical layer structure for NR sidelink

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

- Introduction

In RAN plenary meeting #83, the work item of NR based V2X was agreed [1]. In general, NR V2X would support advanced V2X services defined by SA1 and the consolidated requirements for each use case group are captured in TR 22.886 [2]. RAN WGs have conducted the feasibility study on NR V2X, and it was concluded in TR 38.885 [3] that it is feasible to support advanced V2X services using the technical solutions identified during the study.

The sidelink physical layer structure should be specified in Rel-16 to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. The following aspects of physical layer structure are listed in the WID under one of the objectives [1].

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

In this contribution, we discuss physical layer structure related aspects that are essential for NR sidelink, including the following topics:

- Multiplexing between PSCCH and PSSCH

- PSCCH/PSSCH resource allocation

- DMRS design for PSCCH and PSSCH

- BWP operation

- Multiplexing between PSCCH and PSSCH

During the study, the following options for multiplexing of a PSCCH and associated PSSCH are studied and a working assumption was achieved [3].

Option 1: PSCCH and the associated PSSCH are transmitted using non-overlapping time resources.

-	Option 1A: The frequency resources used by the two channels are the same.

-	Option 1B: The frequency resources used by the two channels can be different.

Option 2: PSCCH and the associated PSSCH are transmitted using non-overlapping frequency resources in the all the time resources used for transmission. The time resources used by the two channels are the same.

Option 3: Part of PSCCH and the associated PSSCH are transmitted using overlapping time resources in non-overlapping frequency resources, but another part of the associated PSSCH and/or another part of the PSCCH are transmitted using non-overlapping time resources.

Working assumption:

- Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM.

  - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3.

  - FFS how to determine the starting symbol of PSCCH and the associated PSSCH

  - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

For option 3, high spectrum efficiency can be achieved. However, different channels may collide if dynamic sharing between PSCCH and PSSCH is supported, especially for UEs operate in the idle state or out of coverage scenario. At least when these UEs transmit in broadcast mode, the starting symbol of PSCCH should be fixed, which could reduce the blind detection complexity of the receiving UEs. Furthermore, the first symbol may be used for AGC adjustment. PSCCH should be transmitted after the AGC symbols (e.g. the second symbol) to achieve better performance, especially considering that no HARQ combining is possible for PSCCH. The duration of PSCCH can be two or three symbols if it is based on the design of PDCCH. With regard to the associated PSSCH, its starting symbol can be the same as PSCCH or even start from the first symbol used for AGC.

Proposal 1: For option 3 of multiplexing between PSCCH and PSSCH, PSCCH should start from the second symbol of the slot. The associated PSSCH could start from the same starting symbol of PSCCH or from the first symbol.

If only option 3 is allowed, it means that the resources used for PSCCH and the associated PSSCH always overlap in time, then, it cannot support cross-slot scheduling/reservation, because there is no associated data transmitted in the same slot. In our view, at least for mode-2a, cross-slot scheduling/reservation enables the sensing UE to acquire the resource allocation information in advance based on PSCCH decoding. Therefore, it is beneficial to support one-shot transmission or small scale sensing for autonomous resource allocation for aperiodic traffic, which would reduce the probability of collision as evaluated in [4]. Therefore, it is beneficial to allow PSCCH and the associated PSSCH transmitting in non-overlapping time resources, which is deemed useful at least for cross-slot scheduling/reservation.

Furthermore, option 1b is more aligned with open-loop power control mechanism. The PSCCH is likely to be transmitted in a broadcast manner for higher reliability and for the sake of sensing from other UEs. Therefore, it may be transmitted with higher power, regardless of the pathloss between the Tx and Rx UEs. At the same time, according to the unicast OLPC, the PSSCH would be transmitted in lower power considering the pathloss between the Tx and Rx UEs, in order to decrease the interference in NR V2X system. As a result, the Tx power of PSCCH and PSSCH could be different. More details are discussed in [8]. Compared with option 3, where the power of PSCCH and that of the associated PSSCH are assumed to be the same, for option 1b, as the PSSCH and PSCCH located in non-overlapped symbols, it supports independently power control for PSCCH and PSSCH.

According to the reply LS from RAN4 [5], when the RB configuration or total transmit power changes, transient period is needed. For option 1b, it could result in a different RB configuration/power spectral density for PSCCH and PSSCH. Therefore, transient period is required. Based on TS38.101, 10us is required for the transient period. Anyway, such transient period is not an issue for cross-slot scheduling/reservation. On the other hand, even for option 3, transient period is required for more than one rank transmission. Moreover, as discussed above, due to the OLPC scheme, inevitably different Tx power are applied to PSCCH and PSSCH, which means that the transient period is required for option 3 either.

Observation 1: Transient period between PSCCH and PSSCH is required for option 3 for rank-2 transmission, or when different Tx power are applied to PSCCH and PSSCH according to the OPLC.

Proposal 2: For multiplexing between PSCCH and PSSCH, option 1b should be supported.

- PSCCH/PSSCH resource allocation

During the study, the following agreements regarding resource pool and resource allocation were achieved [3].

Agreements:

- For time domain resources of a resource pool for PSSCH,

  - Support the case where the resource pool consists of non-contiguous time resources

Agreements:

- Sub-channel based resource allocation is supported for PSSCH

Regarding PRB allocation in the frequency domain for PSSCH resource pool, we believe the same principle for time domain should apply to frequency domain as well. Thus, there should be no restriction in terms of continuous frequency domain resources to allow flexible and efficient resource pool management. Such flexibility is important for V2X operation in licensed spectrum, enabling better coexistence with Uu interface. Therefore,

Proposal 3: For frequency domain resources of a resource pool for PSSCH, the resource pool can consist of non-contiguous PRBs.

In above section, we discuss the multiplexing between PSCCH and the associated PSSCH. In our opinion, both option 3 and option 1b should be supported in NR V2X. In order to support flexible resource assignment for PSCCH and PSSCH, it is beneficial to design separate resource pools, one dedicated for PSCCH while the other dedicated for PSSCH, as illustrated in Figure 1. A separate resource pool for PSCCH may also be beneficial to reduce the complexity of blind decoding for PSCCH.

Figure 1 Resource pool structure

Proposal 4: Dedicated resource pools for control and data respectively should be supported for NR sidelink.

Within a control resource pool, when the associated PSSCH is transmitted in the same slot as PSCCH, the PSSCH can be transmitted in the following symbols of PSCCH if TDM manner is adopted. The PSCCH located in the first few symbols of this slot as illustrated in Figure 2 is beneficial for scheduling extremely low latency service via PSCCH early decoding.

Figure 2 An example of PSCCH and the associated PSSCH in the same slot

If NR V2X supports resource reservation for the initial transmission of a TB, or supports pre-emption and resource unbooking or CSI report trigger, the system should support “orphan PSCCH”. For an “orphan PSCCH”, there is no scheduled PSSCH from a UE in the same slot. As illustrated in Figure 3, there should be a GP symbol following the last PSCCH symbol. A UE sending PSCCH can still receive data in the rest symbols of that slot that is scheduled by other UEs in the previous slots.

Figure 3 An example of “orphan PSCCH”

Proposal 5: GP is required between “orphan PSCCH” and the rest symbols if those are used for PSSCH transmission scheduled by other UEs.

For PSCCH, sub-channel can be considered to reduce the overhead of resource allocation. It can also reduce the blind decoding complexity of PSCCH. Particularly, in the TDM resource pool, it is better that PSCCH and PSSCH follow a single sensing procedure. So we propose

Proposal 6: Sub-channel based resource allocation is supported for PSCCH.

- DMRS design for PSCCH and PSSCH

During the study, the following agreements regarding reference signals were achieved [3].

Agreements:

- Multiple DMRS patterns in time domain are supported for PSSCH

  - FFS: Whether a DMRS pattern is selected based on the subcarrier spacing

  - FFS: Single or multiple DMRS pattern(s) per a resource pool

  - FFS: How TX UE and RX UE can be aligned in terms of the DMRS pattern used for PSSCH

  - FFS: RE mapping, sequence generation

- Continue to study DMRS pattern in frequency domain for PSSCH

  - E.g. whether multiple patterns are supported, whether PDSCH/PUSCH DMRS configuration 1 or 2 is reused.

In this section, we evaluate the performance of Rel-15 DMRS design when it is applied in NR V2X and propose some improvements to make it more suitable for NR sidelink.

  - DMRS for PSCCH

It is desirable to reuse as much as possible the existing design of NR Uu interface. Therefore, DMRS design in PDCCH can be a starting point for NR V2X PSCCH. The pattern is illustrated in Figure 4.

Figure 4 DMRS pattern for PSCCH of 2 OFDM symbols

The performance of PSCCH is evaluated in the ITS band. In this simulation, the non-interleaved mapping is assumed. Various aggregation levels, configurations of the antenna and SCS are evaluated under a relative speed of 500km/h. The payload size of PSCCH is 39 bits (not including CRC). Polar coding is applied for PSCCH transmission. The detailed simulation parameters are listed in Table 4 in the Appendix.

- PSCCH 5.9GHz QPSK-39bits

| (a) 15 KHz, 2 symbols | (b) 30 KHz, 2 symbols |
| --- | --- |
| Figure 5 Performance evaluation for PSCCH QPSK of 2 OFDM symbols |  |

The results show that the configuration of the higher aggregation level is beneficial. If the required BLER is 1%, and the target SNR is -6 dB, higher aggregation level and multiple antenna configurations are required.

Observation 2: The following are beneficial for the performance of PSCCH:

- High aggregation level

- Multiple antenna configurations.

  - DMRS for PSSCH

In NR system, the DMRS pattern for PDSCH is flexible, which contains front-loaded DMRS and additional DMRS. In order to achieve robust channel estimation, up to three additional DMRS are supported. However, the maximum relative speed for NR V2X is 500 km/h, which will result in severe Doppler Effect. In the following simulation, we evaluate the performance of PSSCH taking into account AGC and GP symbols, with different relative UE speed and different numerologies. The DMRS patterns for evaluation are illustrated in Figure 6 with different number of OFDM symbols in time domain (left) and different comb structures in frequency domain (right).

Figure 6 Evaluated DMRS pattern for PSSCH in time domain (left) and frequency domain (right)

In the simulation, a 1Tx-2Rx CP-OFDM system is assumed, with subcarrier spacing of 15 KHz, 30 KHz and 60 KHz with normal CP at 5.9 GHz. The modulation scheme is QPSK and the code rate is 1/2. LDPC coding is applied for PSSCH transmission. The detailed simulation assumptions are listed in Table 4 shown in the Appendix.

- PSSCH 5.9GHz 15k QPSK-0.5

| (a) 140km/h, 300ns | (b) 500km/h, 100ns |
| --- | --- |
| Figure 7 Performance evaluation for PSSCH QPSK-0.5 with 15KHz SCS |  |

- PSSCH 5.9GHz 30k QPSK-0.5

| (a) 140km/h, 300ns | (b) 500km/h, 100ns |
| --- | --- |
| Figure 8 Performance evaluation for PSSCH QPSK-0.5 with 30KHz SCS |  |

- PSSCH 5.9GHz 60k QPSK-0.5

| (a) 140km/h, 300ns | (b) 500km/h, 100ns |
| --- | --- |
| Figure 9 Performance evaluation for PSSCH QPSK-0.5 with 60KHz SCS |  |

It is observed that the performance of PSSCH increases as DMRS density (i.e., the number of DMRS) in frequency domain increases for the same subcarrier spacing. Note that although the performance of comb 6 is worse than that of comb 2 and comb 4 for a given SCS, it may still satisfy the performance requirement in some scenarios. For example, the performance difference of comb 6 compared to that of comb 2 and/or comb 4 is small at least for 15 KHz and 30 KHz SCS configurations. On the other hand, comb 6 is beneficial to reduce the overhead, which is especially important for sidelink considering the overhead of AGC and GP symbols. Therefore, reduction of DMRS density in frequency domain can be taken into consideration at least for small SCS configurations.

Observation 3: The performance of DMRS comb 6 is acceptable at least for 15 KHz and 30 KHz SCS configurations.

Proposal 7: Multiple DMRS patterns in frequency domain are supported for PSSCH.

Similar observations can be made on DMRS density in time domain where the performance of PSSCH increases as DMRS density (i.e., the number of DMRS) in time domain increases for the same subcarrier spacing. In particular, it is observed that high DMRS density is required for high speed scenario. For example, it is observed from Figure 7(b) that, in the high-speed scenario, the performance of 15 KHz with 4 DMRS in time domain may not meet the requirement. Figure 10 illustrates some DMRS pattern enhancement compared to Rel-15 DMRS configuration type 1 and type 2. Case 1-1 is DMRS pattern with 4 DMRS for 15/30 KHz with NCP and case 1-2 is DMRS pattern with 4 DMRS for 60 KHz with ECP. Both of them are Rel-15 NR DMRS pattern as the baseline. Case 1-3 is DMRS pattern with 6 DMRS for 15 KHz with NCP, case 1-4 is DMRS pattern with 3 DMRS for 60 KHz with ECP and case 1-5 is DMRS pattern with 2 DMRS for 60 KHz with ECP.

Figure 10 DMRS pattern enhancement examples, configuration type1 (left) and configuration type 2 (right)

It has been agreed to support different numerologies for NR V2X.  The following Table 1 and Table 2 summarize their corresponding CP and DMRS overhead assuming DMRS configuration type 1 and type 2 respectively. 1 ms slot in 20 MHz bandwidth is assumed in the calculation. Note that both 15 KHz with NCP and 60 KHz with ECP can meet the maximum communication range requirement. However, the existing baseline NR DMRS pattern for 15 KHz with NCP cannot meet the performance requirement in high-speed scenario. On the other hand, the usage of 60 KHz with ECP results in high overhead even though the performance and communication range requirement can be met.

Table 1 CP and DMRS overhead of different numerologies for DMRS configuration type 1

|  | CP length (us) | Meet the maximum communication range requirement (1000 m) | CP+DMRS overhead |  |
| --- | --- | --- | --- | --- |
|  |  |  | NR baseline | Enhancement |
| 15 KHz w/ NCP | 4.69 | Y | 20% (case 1-1) | 26.7% (case 1-3) |
| 30 KHz w/ NCP | 2.35 | N | 20% (case 1-1) |  |
| 60 KHz w/ ECP | 4.17 | Y | 33.3% (case 1-2) | 30% (case 1-4) 26.7% (case 1-5) |

Table 2 CP and DMRS overhead of different numerologies for DMRS configuration type 2

|  | CP length (us) | Meet the maximum communication range requirement (1000 m) | CP+DMRS overhead |  |
| --- | --- | --- | --- | --- |
|  |  |  | NR baseline | Enhancement |
| 15 KHz w/ NCP | 4.69 | Y | 14.6% (case 2-1) | 19.9% (case 2-3) |
| 30 KHz w/ NCP | 2.35 | N | 14.6% (case 2-1) |  |
| 60 KHz w/ ECP | 4.17 | Y | 28.9% (case 2-2) | 26.7% (case 2-4) 24.4% (case 2-5) |

During the study, there was discussion whether a DMRS pattern is selected based on the subcarrier spacing and whether single or multiple DMRS pattern(s) per resource pool is used. In our view, although subcarrier spacing is an important factor to determine a DMRS pattern, it is not the only factor. Other factors such as UE speed should also be considered when select DMRS pattern. Looking at the results of Figure 7(a), Figure 8(a) and Figure 9(a), a lower density of DMRS in time domain may meet the BLER requirements with lower overhead, at least for low speed scenario. For example, 2 DMRS symbols in time domain may provide sufficient performance for low speed scenario while 4 DMRS symbols is required when the relative velocity is 350 km/h as shown in Figure 11. Therefore, for a specific subcarrier spacing, multiple DMRS patterns should be defined to provide flexibility and to allow efficient V2X operation. Consequently, for a resource pool, multiple DMRS patterns are defined.

| (a) 30km/h | (b) 350km/h |
| --- | --- |

Figure 11 Performance evaluations for PSSCH 15 KHz, QPSK

In general, multiple DMRS patterns with variable DMRS density in both time and frequency domain are beneficial for NR V2X due to various application scenarios and the tradeoff between performance and overhead. Furthermore, such patterns should be configurable for NR sidelink depends on different factors such as UE speeds, SCS etc.

Proposal 8: Multiple DMRS patterns are defined for a subcarrier spacing and multiple DMRS patterns are configured for a resource pool.

- BWP operation

During the study, the following agreements regarding SL BWP were achieved [3].

Agreements:

- BWP is defined for NR sidelink.

  - In a licensed carrier, SL BWP is defined separately from BWP for Uu from the specification perspective.

    - FFS the relation with Uu BWP.

  - The same SL BWP is used for both Tx and Rx.

  - Each resource pool is (pre)configured within a SL BWP.

  - Only one SL BWP is (pre)configured for RRC idle or out of coverage NR V2X UEs in a carrier.

  - For RRC connected UEs, only one SL BWP is active in a carrier. No signalling is exchanged in sidelink for activation and deactivation of SL BWP.

    - only one SL BWP is configured in a carrier for a NR V2X UE

  - Numerology is a part of SL BWP configuration.

Agreements:

- Configuration for SL BWP is separated from Uu BWP configuration signalling.

  - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

    - FFS the time scale

    - FFS relation to DL BWP including initial Uu BWP

    - FFS relation in terms of frequency location and bandwidth

- The configuration of SL BWP

Some UEs might not be capable of supporting the entire carrier bandwidth, thus a narrow band can be used to serve such UEs. Furthermore, UE can be configured with different bandwidths so that it can achieve power saving which may be especially beneficial for pedestrian UE (PUE).

In previous RAN1 meetings, it was agreed that only one SL BWP is configured in a carrier. In order to support broadcast transmission in SL, a common SL resource pool is needed for all the NR V2X UEs. In order to support broadcast transmission, UEs should be configured with the same numerology. Consequently, SL BWPs for UEs should be overlapped with the same numerology within an area to support broadcast transmission. Figure 12 illustrates the common SL BWP configuration in idle and connected mode.

Figure 12 Common SL BWP in idle mode (left) and dedicated SL BWP in connected mode (right)

The SL BWP may be pre-configured, or provided by SIB or dedicated RRC signaling, as described in Table 3.

For in-coverage UE, the common SL BWP can be configured by SIB, where the bandwidth is restricted to the minimum UE capability. It is applicable also for RRC connected mode UE, unless reconfigured by dedicated RRC signaling. Such reconfiguration is beneficial in order to improve the spectrum efficiency and to avoid collision and congestion in SL. For example, the high capability UE can be configured with the non-overlapping resource pool in a wider BWP, for specific unicast and groupcast transmissions. By this way, load balancing can be achieved, and congestion in the common resource pool [7] can be avoided.

Table 3 Applicable configuration methods for SL BWP

|  | Pre-configuration | SIB | RRC signaling | Configuration method |
| --- | --- | --- | --- | --- |
| Out of coverage | Yes | - | - | Pre-configuration |
| In coverage, RRC idle mode | Yes | No | - | Pre-configuration |
|  | Yes | Yes | - | SIB |
| In coverage, RRC connected mode | Yes | No | No | Pre-configuration |
|  | Yes | Yes | No | SIB |
|  | Yes | Yes/No | Yes | Dedicated RRC signal |

Observation 4: In connected mode, UE-specific SL BWP configured by dedicated RRC signaling is beneficial for performance improvement (e.g., reliability, throughput, resource utilization).

- The relationship between SL BWP and Uu BWP

It was agreed that configuration for SL BWP is separated from Uu BWP configuration signaling. Therefore, the SL BWP and Uu BWP can be configured independently. However, the relationship between SL BWP and Uu BWP should be discussed for limited UE capability.

If UE supports only one Tx RF and one Rx RF, SL BWP and DL BWP should be confined in the UE Rx RF bandwidth in the same carrier. Similarly, SL BWP and UL BWP are confined in the UE Tx RF bandwidth in the carrier as illustrated in Figure 13. The gNB is aware of the UL and DL bandwidth capability from the UE capability reports. Therefore, it should be up to the gNB to ensure the SL BWP and UL/DL BWP are located such that their aggregated bandwidth does not exceed the RF bandwidth of a UE.

Figure 13 Relationship between SL BWP and Uu BWP in the same carrier

For the relationship with UL BWP, only when the following conditions are supported, UE could support UL BWP and SL BWP at the same time in the same carrier.

- The numerologies of SL BWP and UL BWP are the same

- The SL BWP and UL BWP are confined in the UE Tx RF bandwidth.

For the relationship with DL BWP, in NR Rel-15, UE only supports the same numerology for UL (except SUL) and DL in the same carrier according to the definition of UE capability. Therefore, if the numerology for SL BWP were the same as UL BWP, it would be the same as the DL BWP.

- Deactivation/activation for SL BWP

According the analysis above, if a UE receives a DCI indicating the UL BWP switching, and the above conditions cannot be satisfied after the new UL BWP activated, the UE cannot operate in the SL BWP. As discussed above, the gNB is responsible for proper provision of SL, UL and DL BWP. Therefore, if the gNB triggers UL BWP switching that results in a UE cannot support SL BWP and UL BWP simultaneously, the UE automatically deactivate the SL BWP. Figure 14 illustrates SL BWP activation/deactivation with UL BWP switching.

Figure 14 SL BWP activation/deactivation

On the other hand, if a BWP change occurs for a UE due to expire of BWP inactivity timer, the UE should fall back to the default BWP. It is the gNB’s responsible to ensure the SL BWP is compatible with the default BWP in order to maintain the QoS requirement of the V2X service.

Proposal 9: Configuration of SL BWP is up to the gNB implementation.

Proposal 10: If SL BWP and UL BWP are not configured with the following conditions, UE should deactivate SL BWP.

- The numerologies of SL BWP and UL BWP are the same.

- The SL BWP and UL BWP are confined in the UE Tx RF bandwidth.

In NR Rel-15, the activation timing is aligned with subframe in FR1 and is aligned with half-subframe in FR2. For NR V2X, the activation timing can be reused.

Proposal 11: The activation timing of SL BWP is the same as that of indicated UL BWP.

- Conclusion

In this contribution, we have provided our considerations on the design of physical layer structure for NR sidelink. Some observations are as follows:

Observation 1: Transient period between PSCCH and PSSCH is required for option 3 for rank-2 transmission, or when different Tx power are applied to PSCCH and PSSCH according to the OPLC.

Observation 2: The following are beneficial for the performance of PSCCH:

- High aggregation level

- Multiple antenna configurations.

Observation 3: The performance of DMRS comb 6 is acceptable at least for 15 KHz and 30 KHz SCS configurations.

Observation 4: In connected mode, UE-specific SL BWP configured by dedicated RRC signaling is beneficial for performance improvement (e.g., reliability, throughput, resource utilization).

Based on the discussion, we have the following proposals:

Proposal 1: For option 3 of multiplexing between PSCCH and PSSCH, PSCCH should start from the second symbol of the slot. The associated PSSCH could start from the same starting symbol of PSCCH or from the first symbol.

Proposal 2: For multiplexing between PSCCH and PSSCH, option 1b should be supported.

Proposal 3: For frequency domain resources of a resource pool for PSSCH, the resource pool can consist of non-contiguous PRBs.

Proposal 4: Dedicated resource pools for control and data respectively should be supported for NR sidelink.

Proposal 5: GP is required between “orphan PSCCH” and the rest symbols if those are used for PSSCH transmission scheduled by other UEs.

Proposal 6: Sub-channel based resource allocation is supported for PSCCH.

Proposal 7: Multiple DMRS patterns in frequency domain are supported for PSSCH.

Proposal 8: Multiple DMRS patterns are defined for a subcarrier spacing and multiple DMRS patterns are configured for a resource pool.

Proposal 9: Configuration of SL BWP is up to the gNB implementation.

Proposal 10: If SL BWP and UL BWP are not configured with the following conditions, UE should deactivate SL BWP.

- The numerologies of SL BWP and UL BWP are the same.

- The SL BWP and UL BWP are confined in the UE Tx RF bandwidth.

Proposal 11: The activation timing of SL BWP is the same as that of indicated UL BWP.

References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- TR 22.886, “Study on enhancement of 3GPP support for 5G V2X Services”, V16.1.0, 2018-09.

- TR 38.885, “Study on Vehicle-to-Everything”, V2.0.0, 2019-03

- R1-1904074, “Discussion on mode 2 resource allocation mechanism”, vivo, Xi’an, 8th – 12th April, 2019.

- R4-1902514 Reply LS on NR V2X UE transient period for sidelink channels, LG, 25th February – 1st March, 2019.

- RAN1 #96 Chairman’s notes, Athens, March 2019.

- R1-1901688, “Enhancements of Uu link to control sidelink”, vivo, Athens, 25th February – 1st March, 2019.

- R1-1904077, “Physical layer procedure for NR sidelink”, vivo, Xi’an, 8th – 12th April, 2019.

Appendix

Table 4 Link level evaluation assumption

| Assumption | Value |
| --- | --- |
| Carrier frequency | 5.9 GHz |
| Waveform | CP-OFDM |
| Subcarrier spacing | 15 KHz, 30 KHz, 60 KHz |
| Transmission layers for PSSCH | 1 layer |
| Modulation order | QPSK |
| Chanel model | TDL-A |
| Delay spread | 100ns, 300ns |
| Relative velocity | 500km/h, 140km/h |
| Channel estimation method | MMSE, non-ideal |
| Punctured symbol | AGC, GP, DMRS |
| PSSCH |  |
| TBS | 190bytes, 300bytes |
| Channel coding | LDPC |
| Antenna configuration | 1Tx - 2Rx |
| DMRS pattern in the time domain | 2 DMRS: <2, 11> 3 DMRS: <2, 7, 11> 4 DMRS: <2, 5, 8, 11> 5 DMRS: <2, 4, 6, 9, 11> 6 DMRS: <1, 3, 5, 7, 9, 11> |
| DMRS pattern in the frequency domain | Comb 2: <0 2 4 6 8 10> Comb 4: <0 4 8> Comb 6: <0 6> |
| PSCCH |  |
| Payload (without CRC) | 39bits |
| Channel coding | Polar |
| Antenna configuration | 1Tx - 2Rx, 1Tx - 4Rx, 2Tx - 4Rx |
| Aggregation level | 2, 4, 6, 8, 16 |
| Time domain resource | 2 symbols |
| Frequency domain resource | 48 PRBs |