3GPP TSG RAN WG1 meeting #96bis	 R1-1904455

Xi’an, China, 8th – 12th April, 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: Rank 3-4 parameter setting and subset selection

Document for:	Discussion and Decision

# Introduction

The following agreement was made during the email discussion [96-NR-08] [1].

Agreement 1: On the L/p parameter setting for RI=3-4, consider/compare only the alternatives given in sections 2.1 to 2.6 (Alt1; 2A, 2B; 3A, 3B, 3C; 4A, 4B, 4C, 4D, 4E, 4F; 5A, 5B, 5C; 6A, 6B, 6C, 6D, 6E, 6F, 6G, 6H) – see [1] for details - No other alternative will be considered Agreement 2: On selecting the scheme for the L/p parameter setting for RI=3-4, the following aspects need to be considered, and decided together or after the scheme selection: - SD and/or FD basis subset selection for RI=3 and 4: layer-common vs. layer/layer-group-specific - Fixed/pre-defined vs. configurable setting, whenever applicable Restriction on parameter setting and/or value range for L and/or p to control overhead

This contribution provides simulation results for (a) SD basis subset selection and (b) parameter setting in support of proposals made in the companion contribution [2].

# Simulation results for rank 3-4

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in low (20% target RU) traffic loading scenario, and SU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. As reference scheme, Rel. 15 Type II for rank 1-2 and Rel. 15 Type I for rank 3-4 is considered in this evaluation. For comparison, simple extension of Rel. 15 Type II codebook up to rank 4 is also considered.

In first evaluation, the performance-overhead trade-offs of the following SD basis subset selection alternatives are compared.

- Layer-common: the L SD beams {bi:i=0,1,..,L-1} are common for layers;

- Layer-pair-specific: the first set of L SD beams {bi0:i=0,1,..,L-1} are common for layers (0, 1), and a second set of L SD beams {bi1:i=0,1,..,L-1} are common for layers (2, 3).

Regarding FD basis subset, the rank 2 design is simply extended, i.e., FD basis subset selection is layer-specific. The results are provided in Figure 1 for the following parameters.

- Spatial compression: L = 2, 4

- Frequency compression: M = 7 (i.e., p=12)

- Parameter setting: Alt6D (as explained later)

- β∈{14,12,34}.

We can observe the following.

Observation 1: For SD basis subset selection,

- Up to 2% gain can be achieved with layer-pair-specific SD basis when compared with layer-common SD basis

- There is no gain (even some loss) with β=34 over β=12 when L=4

No gain

1%

Figure 1: Performance-overhead trade-off for layer-common and layer-pair-specific SD basis

In second evaluation, the performance-overhead trade-offs of the following parameter setting alternatives are compared.

- Alt1: parameters L and p are layer-common (hence same as RI=1-2), and the β value for RI=3-4 is layer-common and equals 2β/RI.

- Alt6D (as explained in [2]): unequal number of SD basis vectors

  - i=0RI-1Xi=4L, which is the total number of SD basis subsets for RI=2

  - Xi≤2L, i.e., for each layer, the number of SD basis vectors is at most equal to that for RI=2

  - Xi is non-increasing (Xi≥Xj for i<j).

  - M and p are layer-common, and equal to the configured values (same as RI=1-2).

- Alt6E (as explained in [2]): unequal number of FD basis vectors

  - i=0RI-1Mi=2M, which is the total number of FD basis subsets for RI=2

  - Mi≤M, i.e., for each layer, the number of FD basis vectors is at most equal to that for RI=2

  - Mi is non-increasing (Mi≥Mj for i<j).

  - L and p are layer-common, and equal to the configured values (same as RI=1-2).

The results are provided in Figure 1 for the following parameters.

- Spatial compression: L = 4

- Frequency compression: M = 7 (i.e., p=12)

- Layer-pair-specific SD basis subset and layer-specific FD basis subset

- β∈{14,12,34}.

We can observe the following.

Observation 2: For parameter setting,

- Alt1 is the worst; this is due to large overhead to report bitmap

- Up to 2.5% gain can be achieved with Alt6D when compared with Alt6E

- There is no or marginal gain with β=34 but the overhead is large

Large overhead, no or tiny gain

2.5%

Figure 2: Performance-overhead trade-off for parameter setting (Alt1, Alt6D, and Alt6E)

# Conclusions

In this contribution, simulation results are provided for the rank 3-4 extension of DFT-compression based Type II CSI codebook. The observations made are summarized as follows.

- For SD basis subset selection,

  - Up to 2% gain can be achieved with layer-pair-specific SD basis when compared with layer-common SD basis

  - There is no gain (even some loss) with β=34 over β=12 when L=4

- For parameter setting,

  - Alt1 is the worst; this is due to large overhead to report bitmap

  - Up to 2.5% gain can be achieved with Alt6D when compared with Alt6E

  - There is no or marginal gain with β=34 but the overhead is large

# References

- R1-1903822, “Summary of email discussion [96-NR-08] on parameter setting for RI=3-4 extension,” Samsung

- R1-1904447, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz with 13 SBs, 10 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 20% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |