3GPP TSG RAN WG1 meeting #96bis						 	R1-1904501

Xi’an, China, April 4 - 8, 2019

Agenda Item: 7.2.10.2

Source: MediaTek

Title: On measurement aspect for NR positioning

Document for:	Discussion and decision

# Introduction

In RAN1 AH-1901 meeting [1], it is agreed that DL-AoD based positioning techniques should be supported in NR:

Agreement:

- NR should support angle (DL-AoD) based DL-only positioning techniques with at least beam sweeping at least at the gNB in FR1 and FR2

  - Note: This does not necessarily need different reference signals from the timing based DL-only positioning techniques

This agreement has been also captured in TR 38.855 [2].

The main benefits of using DL-AoD positioning are:

- It  can help to position an UE when GNSS signal is not available to that UE

- It doesn’t require gNBs to be highly synchronized as UE does not need to measure TDOAs

Therefore, considering practical UE environment and deployment/configuration of base stations, DL-AoD positioning can be a useful technique.

In DL-AoD positioning, an UE measures multiple RS beams transmitted from TPs and report the measurement results to the network. The network estimates the UE’s position based on the measurement results reported by the UE.  In this contribution, we share our views on how UE performs measurement, and how UE may reduce the overhead for reporting measurement results. We consider the impact on positioning precision caused by non-omni-directional UE Rx antenna elements. Finally, we share our views on applicable scenarios for DL-AoD positioning.

# DL-AoD positioning framework and UE RSRP reporting overhead

The DL-AoD positioning may work as follows (see Figure 1 for an illustration):

Step 1: The network configures an UE to measure reference signal (RS) power for TPs

Step 2: Each TP transmits RS with multiple beams

Step 3: The UE measures RS beams transmitted from TPs and reports measurement results to the network

Step 4: The network estimates the AoDs based on the UE’s RSRP report.

Step 5: The location server estimates the UE’s position by using the estimated AoDs

Note that in Step 4, the AoD estimation can be done in a gNB or in the location server (in Figure 1, it is done in the location server). For Step 3, we have following observations:

Observation 1: An UE may perform Rx beamsweeping to search for a coarse direction between the UE and a TP. After the coarse direction search is done, the UE may fixes its Rx beam and then measures the RS beams transmitted from the TP

Observation 2: For limited resolution use cases (i.e., the requirement of positioning precision is relatively low), it may be sufficient that the UE only reports the index of beam with strongest measured RSRP.

Figure 1: an illustration of DL-AoD positioning framework

Observation 3: For high resolution use cases, it is necessary that UE reports its RSRP measurement results to the network.  As such, the data size of UE RSRP report should be made as small as possible, while achieving an acceptable positioning performance

Observation 4: Since the RSRP measurements results can be quite different for UEs at different locations, the RSRP measurement results should be normalized in some sense and the reporting format should be unified

For the problems mentioned above, we propose that

- From all TPs that a UE measured, UE may select a portion of TPs, and the UE only reports RSRPs measured from the selected TPs to the network

- Suppose UE has decided to report RSRPs measured from a certain TP. From all beams that a UE measured from that TP, UE may select a portion of beams, and UE only reports RSRP measured from the selected beams to the network

- Suppose UE has decided to report RSRP measurements results of selected beams of a selected TP, UE may report RSRP ratios (or differential RSRPs) derived from the RSRP measurements results

In the following 3 sections, we provide examples and simulation results for these approaches. The simulation results show that UE reporting overhead can be saved without degrading the positioning performance.

# UE selects a portion of all measured TPs for RSRP reporting

Consider the UMi FR1 scenario. In our simulation, UEs are configured to measure 7 TPs (cells) for which UEs are in their 1200 cell coverage. See Figure 2 for an illustration, in which UEs dropped in cell 0 are configured to measure signals transmitted from cell 0, 3, 8, 11, 13, 16, 18.

Figure 2: an illustration for the setting that UEs are configured to measure 7 TPs (cells) for which UEs are in their 1200 cell coverage

For an UE, some of the 7 TPs are with LOS channel condition and the other TPs are not. Table 1 shows the number of LOS TPs an UE has and the corresponding UE percentage.

| Number of LOS TPs | UE percentage |
| --- | --- |
| 0 | 35% |
| 1 | 40% |
| 2 | 20% |
| 3 | 4% |
| 4 | 1% |

Table 1: number of LOS TPs and the corresponding UE percentage for UMi FR1

From Table 1 we can see that no UEs have more than 4 LOS TPs. Since NLOS channel condition is a major factor impacting positioning performance, it is natural not to use more than 4 TPs for positioning a UE.

An UE may select a portion of the 7 TPs, and only reports RSRPs measured from the selected TPs. Suppose all RSRP measurement results reported by an UE are used for estimating the UE position. In Figure 3, we compare the performance obtained by assuming two different UE behaviour (detailed simulation settings and positioning techniques are given in the appendix):

- UE selects all 7 TPs.

- UE selects best 4 TPs (TPs with maximum measured beam RSRP)

Figure 3: for UMI FR1, performance comparison between UE selecting 4 TPs or 7 TPs

From Figure 3 we see that UE selecting 4 best TPs for RSRP reporting dose not degrade positioning performance.

Now consider the office FR2 scenario. Again in our simulation, UEs are configured to measure 7 nearest TPs (cells) for which UEs are in their 1200 cell coverage. Table 2 shows the number of LOS TPs an UE has and the corresponding UE percentage:

| Number of LOS TPs | UE percentage |
| --- | --- |
| 2 | 0.6% |
| 3 | 1.8% |
| 4 | 13.2% |
| 5 | 34.7% |
| 6 | 31.7% |
| 7 | 20% |

Table 2: number of LOS TPs and the corresponding UE percentage for office FR2

From Table 2 we see that the average number of LOS TPs an UE has is higher than that in UMi scenario. In the Figure 4, we compare the performance by assuming two different UE behaviour as mentioned above:

Figure 4: for office FR2, performance comparison between UE selecting 4 TPs or 7 TPs

From Table 2 and Figure 4 we see that although UE may have more than 4 LOS TPs, UE selecting 4 best TPs for RSRP reporting achieves much better performance. This is because by UE selection, the TPs that are more close to UEs are selected for positioning with higher probability.

Based on above, we have the following observation and proposal:

Observation 5: The simulation results show that by UE’s selection of TPs for RSRP reporting:

- For UMi, the performance is not degraded

- For indoor office, the positioning performance is improved such that DL-AoD can be used for commercial purpose

Proposal 1: For high resolution DL-AoD based positioning, an UE may select a portion of all measured TPs (cells) and report RSRP measurement results only for the selected TPs. A method for UE to select TPs is given as follows:

- The network configures the UE to report RSRPs measured from k TPs

- The UE selects k TPs such that the maximum RSRPs measured from these k TPs are larger than that measured from the other TPs

The detailed specification is FFS.

# UE selects a portion of all measured beams of a TP for RSRP reporting

An UE may be configured to measure a lot number of beams of a TP. Suppose the UE selects the TP and is going to report RSRP of beams measured from the TP. It is natural to analyse if it is necessary for an UE to report RSRP measured from every beam of a TP.

Consider UMi FR1 scenario. Suppose TPs transmit RS with 8 beams. An UE selects 4 best TPs for RSRP reporting as described in Section 3. Furthermore, an UE may select a portion of beams of a TP, and only reports RSRPs measured from the selected beams. In Figure 5, we compare the performance obtained by assuming the following two different UE behaviour:

- UE select all beams a TP.

- UE select a portion of measured beams of a TP.

Figure 5: for UMI FR1, performance comparison between UE selecting 8 beams and 6 beams

From Figure 5, we see that by UE selecting 6 beams, the performance is degraded but still meet the regulatory requirement.

Now consider office FR2 scenario. The simulation result is shown in Figure 6:

Figure 6: for office FR2, performance comparison between UE selecting 8 beams and 4 beams

From Figure 6, we see that by UE selecting 4 from all 8 beams, the performance is degraded but still meet the commercial requirement.

Based on above, we have the following observation and proposal:

Observation 6: The simulation results show that by UE selecting a portion of beams measured from a TP for RSRP reporting:

- For UMi, the regulatory performance requirement can be met

- For indoor office, the commercial performance requirement can be met

Proposal 2: For high resolution DL-AoD based positioning, an UE may select a portion of beams measured from a TP and only report RSRP measured from the selected beams. A method for UE to select beams is given as follows:

- The network configures the UE to report RSRPs measured from k beams of a TP

- For the TP, the UE selects k beams such that the maximum RSRPs measured from these k beams are larger than that measured from the other beams

The detailed specification is FFS.

# UE reports RSRP ratios or differential RSRPs of measured beams of a TP

To reduce the UE RSRP reporting overhead and unify the reporting format, we have the following proposal:

Proposal 3: For high resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured beams of a TP. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

- Suppose pk is the RSRP measured from beam k, k = 1,2,…,N

- Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

- Then UE reports the beam index m, but UE does not report pm

- For each of the N beams (excluding m), say it is beam j, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

The detailed specification are FFS.

In the following we give examples on how an UE reports RSRP ratios and differential RSRPs. Consider the scenario in Figure 7. Suppose the TP transmits RS with beam 1,2,3,4 (in a beamsweeping manner) with beamforming angle θ=-450,-150,150,450, respectively. UEs are dropped on the x axis, from x = -50 to x = 50. Suppose the RSRPs of beams measured by UEs are as shown in Figure 8.

Figure 7                                                                       Figure 8

For the UE located at x = -31, the measured RSRPs for beam 1,2,3,4 are p1=60.59, p2=33.99,  p3=12.44,  p4=41.06, respectively.

Example for RSRP ratio report: The UE reports the max power beam index to the network and the UE does not report the max power RSRP, i.e., UE reports beam index 1 and does not report  p1 to the network. For the other beams, the UE reports their beam indices and RSRP ratios w.r.t. beam 1 to the network, i.e., the UE reports (2,p2/p1), (3,p3/p1), (4,p4/p1) to the network.

Example for differential RSRP ratio report: The UE reports the max power beam index to the network and the UE does not report the max power RSRP, i.e., UE reports beam index 1 and does not report  p1 to the network. For the other beams, the UE reports their beam indices and differential RSRP w.r.t. beam 1 to the network, i.e., UE reports (2,10logp1-10logp2), (3,10logp1-10logp3), (4,10logp1-10logp4), to the network. Taking logarithm is to change power scale from linear to dB.

# Impact of non-omni-directional UE Rx antenna elements

If an UE is equipped with one omni-directional Rx antenna element (AE), then the antenna gain is the same for all AoAs for the UE. On the other hand, if the UE is equipped with one non-omni-directional Rx AE, then there may be a large range of AoAs along which signal power received by the UE is relatively low. See Figure 9 for an illustration. Figure 9 shows the horizontal antenna gain versus various AoAs for one AE (using the model in Table A.2.1-8 in TR 38.802). Here we assume the vertical antenna gain is 0 dB, and assume the maximum directional gain is 0 dB.

Figure 9: Antenna gain of one non-omni-directional antenna element

To see the impact of such an impairment on the DL-AoD positioning performance, consider the following scenario as example:

Figure 10: An UE is dropped at the center of cell 0

The scenario is UMi with 7 sites. An UE is dropped at the center of cell 0. Assume LOS channel for every UE-cell link. In Table 3, we compare the AoD estimation errors obtained by using the following 3 different UE Rx assumptions (the other simulation settings are given in Appendix):

- A1: UE with 1 omni-directional Rx AE

- A2: UE with 1 non-omni-directional Rx AE as modelled in Table A.2.1-8 in TR 38.802, and the maximum antenna gain direction is the –y direction (see Figure 11)

- A3: UE with 1 non-omni-directional Rx AE, and the maximum antenna gain direction is the +y direction (see Figure 11)

|  | Cell 0 | Cell 3 | Cell 8 | Cell 11 | Cell 13 | Cell 16 | Cell 18 | Loc est error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 0 | 0.8304 | 1.6e-4 | 1e-4 | 0 | 0.8e-4 | 0.2779 | 3.16m |
| A2 | 44.126 | 11.6716 | 46.6692 | 1e-4 | 0 | 1.8435 | 22.0903 | 46.3m |
| A3 | 0 | 0.2779 | 3.5647 | 15.6084 | 10.8935 | 0.8e-4 | 0.1679 | 20.8m |

Table 3: AoD and location estimation error caused by non-omni-direction antenna elements

For example, with assumption A1, the AoD estimation error of cell 3 is 0.8304 degrees, and the location estimation error of the UE is 3.16m. From Table 3 we see that non-omni-direction AEs have great impact on the positioning precision of the UE.

Now suppose that the UE is equipped with 2 non-omni-directional Rx AEs, one AE’s maximum antenna gain direction is the -y direction, and the other AE’s maximum antenna gain direction is the +y direction (see Figure 11 below for an illustration)

Figure 11: UE with 2 opposite non-omni-directional Rx AEs

Suppose for each cell, the UE chooses to report RSRP measured by the Rx AE with maximum measured RSRP (e.g., if the maximum RSRP measured by Rx AE 1 is larger than that of Rx AE 2, then the UE chooses to report RSRP measured by Rx AE 1), then the AoD and location estimation error becomes:

| Cell 0 | Cell 3 | Cell 8 | Cell 11 | Cell 13 | Cell 16 | Cell 18 | Loc est error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.2779 | 3.5647 | 1e-4 | 0 | 0.8e-4 | 0.1679 | 0.9m |

Table 4: AoD and location estimation error obtained by combining 2 antenna elements’ measurement results

It can be seen that the AoD errors are greatly improved, and the positioning precision is improved accordingly.

Now we consider the indoor office FR 2 scenario. In Figure 12, we compare the positioning performance obtained by using the following 3 different UE Rx antenna settings:

- S1: Every UE is equipped with one omni-directional Rx AE

- S2: Every UE is equipped with one non-omni-directional Rx AE

- S3: Every UE is equipped with two opposite non-omni-directional Rx AEs (the concept is the same as that shown in Figure 11). The RSRPs measured by the Rx AE with maximum measured RSRP will be used for UE’s TP selection later

Each UE is configured to measure 7 nearest TPs. Each UE selects 4 (maximum power) TPs and reports RSRP measured from these 4 TPs. For a selected TP, an UE selects all beams and reports RSRPs measured from the selected beams to the network.

Figure 12: office FR2, performance for the different UE Rx antenna settings

It can be seen from Figure 12 that S1, S3 can meet the commercial requirement, but S2 cannot.

Base on above, we have the following observation:

Observation 7:

- For high resolution DL-AoD positioning, UE Rx antenna configuration should be arranged such that the AoA range with low antenna gain is as small as possible

- Suppose an UE equipped with multiple Rx antenna panels (or antenna port groups). For a TP, the UE may select a best panel and report RSRP measured from that panel to the network

# Applicable scenarios for DL-AoD positioning

For UMa scenario, our simulation shows that the performance of DL-AoD positioning is:

| Percentile | 50 | 67 | 87 | 90 |
| --- | --- | --- | --- | --- |
| UMa | 67.5m | 91.6m | 127.4m | 161.3m |

Table 5: performance of DL-AoD positioning for UMa

In our simulation, UEs are configured to measure 7 cells for which UEs are in their 1200 cell coverage. An UE selects all 7 cells and reports RSRPs measured from these 7 cells. For a cell selected by an UE, the UE further selects all beams and report RSRPs measured from these beams. Other simulation settings are given in Appendix.

From the Table 5, Figures 5, and Figure 12, we have the following observation:

Observation 8: Scenarios for using DL-AoD:

- UMa: non-applicable

- UMi: applicable, but only for regulatory requirements, not for commercial requirements

- Office: applicable for commercial requirements

# Conclusions

The observations are summarized below:

Observation 1: An UE may perform Rx beamsweeping to search for a coarse direction between the UE and a TP. After the coarse direction search is done, the UE may fixes its Rx beam and then measures the RS beams transmitted from the TP

Observation 2: For limited resolution use cases (i.e., the requirement of positioning precision is relatively low), it may be sufficient that the UE only reports the index of beam with strongest measured RSRP.

Observation 3: For high resolution use cases, it is necessary that UE reports its RSRP measurement results to the network.  As such, the data size of UE RSRP report should be made as small as possible, while achieving an acceptable positioning performance

Observation 4: Since the RSRP measurements results can be quite different for UEs at different locations, the RSRP measurement results should be normalized in some sense and the reporting format should be unified

Observation 5: The simulation results show that by UE’s selection of TPs for RSRP reporting:

- For UMi, the performance is not degraded

- For indoor office, the positioning performance is improved such that DL-AoD can be used for commercial purpose

Observation 6: The simulation results show that by UE selecting a portion of beams measured from a TP for RSRP reporting:

- For UMi, the regulatory performance requirement can be met

- For indoor office, the commercial performance requirement can be met

Observation 7:

- For high resolution DL-AoD positioning, UE Rx antenna configuration should be arranged such that the AoA range with low antenna gain is as small as possible

- Suppose an UE equipped with multiple Rx antenna panels (or antenna port groups). For a TP, the UE may select a best panel and report RSRP measured from that panel to the network

Observation 8: Scenarios for using DL-AoD:

- UMa: non-applicable

- UMi: applicable, but only for regulatory requirements, not for commercial requirements

- Office: applicable for commercial requirements

The proposals are summarized below:

Proposal 1: For high resolution DL-AoD based positioning, an UE may select a portion of all measured TPs (cells) and report RSRP measurement results only for the selected TPs. A method for UE to select TPs is given as follows:

- The network configures the UE to report RSRPs measured from k TPs

- The UE selects k TPs such that the maximum RSRPs measured from these k TPs are larger than that measured from the other TPs

The detailed specification is FFS.

Proposal 2: For high resolution DL-AoD based positioning, an UE may select a portion of beams measured from a TP and only report RSRP measured from the selected beams. A method for UE to select beams is given as follows:

- The network configures the UE to report RSRPs measured from k beams of a TP

- For the TP, the UE selects k beams such that the maximum RSRPs measured from these k beams are larger than that measured from the other beams

The detailed specification is FFS.

Proposal 3: For high resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured beams of a TP. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

- Suppose pk is the RSRP measured from beam k, k = 1,2,…,N

- Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

- Then UE reports the beam index m, but UE does not report pm

- For each of the N beams (excluding m), say it is beam j, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

The detailed specification are FFS.

# Reference

- Chairman's Notes RAN1 AdHoc 1901

- R1-1903824 TR 38.855

# Appendix

Simulation settings for UMa, UMi and office scenario are given below:

| Parameter | UMi & UMa, FR1 | Office, FR2 |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |
| Carrier frequency | 4Gz | 30GHz |
| Subcarrier spacing | 15kHz | 60kHz |
| Reference Signal Transmission Bandwidth | 20MHz | 36MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 |
| Reference signal (type of sequence, number of ports, …) | PN sequence, 1 port | PN sequence, 1 port |
| Number of sites | 7 | 12 |
| Number of symbols used per occasion | 2 | 2 |
| number of occasions used per positioning estimate | 10 | 10 |
| Power-boosting level | 6.02 dB | 6.02 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | RSRP = average power per RE | RSRP = average power per RE |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured RSRP ratios of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured RSRP ratios of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator |
| Network synchronization assumptions | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A |
| Additional notes, if any | UE is equipped with one omni-directional antenna element | Unless otherwise specified, UE is equipped with one omni-directional antenna element |