3GPP TSG RAN WG1 #96bis	R1-1904585

Xi’an, China, 8th – 12th April, 2019

Agenda Item:	7.2.2.1.1

Source: 	Fujitsu

Title: 	Consideration on PRACH in NR-U

Document for:	Discussion/Decision

- Introduction

This contribution is revised from [1]. In this contribution, we would like to discuss some issues related to PRACH formats, RO configuration and multiplexing of UL channels for NR-U. The following are some related agreements in previous meetings.

RAN1#93 [2]

Agreement:

- Support for Rel-15 NR PRACH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that interlaced based PRACH can be beneficial.

- The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

  - Interlacing based on PRB or REs

  - Targeted cell sizes

  - Targeted PRACH capacity

  - Targeted false alarm and detection rates

  - Targeted timing estimation accuracy

  - Number of formats

  - Multiplexing with other channels such as block interlaced PUCCH and PUSCH

RAN1#94 [3]

Agreement:

- For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

  - Note: This is only from a user-multiplexing perspective. Other aspects of PRACH design need to be considered, i.e., timing estimation accuracy, miss detection rate, PAPR, RACH capacity, transmission power

- For scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

- FFS: Potential LBT blocking due to TA difference between FDM’d PUSCH, PUCCH, and PRACH.

RAN1#96 [4]

Agreement:

LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy

Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths.

- Discussion

PRACH format

According to agreements in RAN1#93 meeting, support for Rel-15 NR PRACH formats can be considered and exclusion of the support of certain formats is to be identified. NR Rel-15 supports both long preamble formats with sequence length L=839 and short preamble formats with sequence length L=139. The long preamble formats are designed for the use cases e.g. LTE refarming, large cell and high speed etc. And short preamble formats are for use cases e.g. small cell and normal cell. Some short preamble formats (e.g. A2/A3/B4/C0/C2) can support a maximum cell radius more than 2 km with 15 kHz SCS. So support short preamble formats only would be sufficient for NR-U scenario considering the limited cell range. And down selection could be further made e.g. considering PRACH SCS and corresponding targeted cell sizes.

Proposal 1: NR-U only supports preamble formats with sequence length L=139.

RO configuration

For short preamble formats, time-domain RACH occasions (ROs) are configured continuously within a PRACH slot in NR Rel-15. One example is as illustrated in Figure 1. In addition, as shown in Table 1, assume the LBT for PRACH takes at least 25 us, for most of the short preamble formats, there would be no enough extra space in the GT period for LBT between two neighboring time-domain ROs. Therefore, the transmission on the prior RO would block the later one, resulting in wasting resources, reducing PRACH capacity and increasing latency. RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

Figure 1. Example of time-domain ROs within a PRACH slot.

Table 1. Guard Time of short preamble formats

| Format |  |  |  | PRACH duration | Guard Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 15 kHz (us) | 30 kHz (us) | 60 kHz (us) |
| A1 |  |  |  | 2 | 0 | 0 | 0 | 0 |
| A2 |  |  |  | 4 | 0 | 0 | 0 | 0 |
| A3 |  |  |  | 6 | 0 | 0 | 0 | 0 |
| B1 |  |  |  | 2 | 72k∙2-μ | 2.34 | 1.17 | 0.59 |
| B2 |  |  |  | 4 | 216k∙2-μ | 7.03 | 3.52 | 1.76 |
| B3 |  |  |  | 6 | 360k∙2-μ | 11.72 | 5.86 | 2.93 |
| B4 |  |  |  | 12 | 792k∙2-μ | 25.78 | 12.89 | 6.45 |
| C0 |  |  |  | 2 | 1096k∙2-μ | 35.68 | 17.84 | 8.92 |
| C2 |  |  |  | 6 | 2912k∙2-μ | 94.79 | 47.40 | 23.70 |

Observation 1：Based on RO configuration in NR Rel-15, there would be no enough blank period for LBT between neighboring time-domain ROs.

Proposal 2: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

- Conclusions

Proposals in this contribution are summarized as following:

Proposal 1: NR-U only supports preamble formats with sequence length L=139.

Observation 1：Based on RO configuration in NR Rel-15, there would be no enough blank period for LBT between neighboring time-domain ROs.

Proposal 2: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

References

- R1-1901938, Consideration on PRACH in NR-U, Fujitsu

- Chairman's Notes RAN1_93_final, 3GPP TSG RAN WG1 Meeting #93, May 2018.

- Chairman's Notes RAN1_94_final, 3GPP TSG RAN WG1 Meeting #94, Aug. 2018.

- Chairman's Notes RAN1_96_final, 3GPP TSG RAN WG1 Meeting #96, Feb. 2019.