3GPP TSG RAN WG1 #96bis		R1-1904591

Xi’an, China, 8th – 12th April 2019

Source:	Panasonic

Title: 	Issues on Timing Advance and RACH for NTN

Agenda Item:		7.2.5.3 (Uplink timing advance/RACH procedure)

Document for:	Discussion

Introduction

A study item on Non Terrestrial Network (NTN) has been started. According to the SID[1], the following is to be studied in RAN1.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In this document, timing advance and RACH related issues for NTN are discussed.

Issues on Timing Advance

In NTN scenarios, RTD (round trip delay) and differential delay (variance of RTD) within a cell/beam impact on Timing Advance (TA). Differential delay is difference between RTD at near side of cell/beam and RTD at far side of cell/beam from the satellite (corresponding to d3 in Figure 1). As shown in clause 4.2 of TR 38.821 v0.3.0 [3] of the current SI, the maximum RTD in NTN is 541.14 ms for GEO (at 35786 km altitude) and 25.76/41.75 ms for LEO (at 600/1200 km altitude). Maximum differential delay within a cell/beam is 1.6ms for GEO and 0.65ms for LEO. These values are much larger than that in terrestrial network.

Figure 1  differential delay within a cell/beam

In the current NR specification, Timing Advance (TA) is carried out for uplink transmissions based on TA command so that gNB can receive uplink signals from different UEs at an aligned timing at gNB reception point. TA command is transmitted by gNB during RACH procedure and for TA maintenance in RRC_CONNECTED. For PRACH transmission, UE does not use TA and transmits only based on reception timing of SSB/CSI-RS.

To support a long delay in NTN it is proposed (e.g.[4]) to introduce common TA and individual TA. Common TA is used to compensate the RTT at a reference point within the cell/beam, e.g. at most near point of the cell/beam. Individual TA is used to control timing per UE. In Figure 1, delay corresponding to d1 could be compensated by common TA and differential delay corresponding to d3 could be compensated by individual TA.

Common TA would be known to gNB from the satellite location. Common TA could be informed to UE via MIB/SIB and UE could adjust the transmission timing based on both common TA and individual TA. On the other hand, because common delay can be compensated within gNB as shown in Figure 2, common TA indication to UE would not be needed. In this case, UE adjusts the transmission timing only based on individual TA.

Figure 2 timing relation between gNB tx/rx and UE tx

Proposal 1: Common TA can be compensated within gNB and UE use only individual TA.

Issues on RACH design/procedure

PRACH format

As mentioned in section 2, because of the large variance of RTD (i.e. differential delay) within a cell/beam, PRACH reception timing at gNB would vary up to 1.6 ms depending on UE position within a cell/beam (Figure 3). This variance of RTD within a cell needs to be taken into account for the PRACH format design. In addition, a robust transmission method would be necessary to support long range transmission especially in case of GEO, e.g. use of long sequence (length 839) for both FR1 and FR2, more repetition or multiple sequence transmission. Use of longer root sequence than currently specified would also be one possibility, but this would not be preferable because it would require large effort on new sequence design and significant impact on the receiver implementation.

Figure 3 PRACH reception timing at gNB

PRACH is defined as ZC sequence (root sequence) and its cyclic shift (CS). PRACH transmitted from different UEs are distinguished by the ZC sequence and CS. Different sets of ZC sequences are assigned to different cells/beams to mitigate intercell interference. Within a cell, CS multiplexing is additionally used to increase the number of PRACH sequences. However, for a NTN scenario, availability of CS multiplexing would be limited (i.e. no or only a few CS can be used) because of the large variation of the RTT within a cell. Typically, 64 PRACH sequences, i.e. combination of ZC sequence and CS, would be assigned in a cell although total number of sequences is configurable by RRC. If cyclic shift is not available or very limited, a lot of ZC sequences need to be assigned per cell. In this case, cell reuse factor of ZC sequence number becomes small. Then, intercell interference would become more severe. Therefore, it would be beneficial to study interference mitigation method with a limited number of ZC sequences per cell.

On the other hand, if UE transmits PRACH with TA, CS multiplexing works well and PRACH capacity is improved. PRACH is transmitted by not only initial access UE (i.e. idle UE) but also RRC connected UEs e.g. for UL scheduling request. It would be worth to consider PRACH transmission with TA for UE which has valid TA. In this case, separate PRACH occasions for with TA and without TA could be configured.

Proposal 2: Robust PRACH format should be defined for long distance transmission, e.g. use of long sequence (length 839) for both FR1 and FR2, more repetition or multiple sequence transmission.

Proposal 3: Study intercell interference mitigation method with a limited number of ZC sequence per cell.

Proposal 4: Consider PRACH transmission with TA for UE which has valid TA to allow CS multiplexing between UEs.

RACH procedure

After reception of msg1 (PRACH) gNB transmit PDCCH scrambled with RA-RNTI to indicate PDSCH resource containing msg2. RA-RNTI is linked to time frequency resource of PRACH. Actual calculation is defined in TS38.321 section 5.1.3 as follows.

TS38.321V15.3.0 section 5.1.3

Due to the large delay variance between UEs within a cell/beam, gNB may not be possible to distinguish the PRACH transmission timing (i.e. slot number or symbol number). As the result, it may happen that RA-RNTI calculated by UE and RA-RNTI gNB actually transmitted are different. In order to avoid a confusion, gNB should configure RACH transmission timing (i.e. RACH Occasion) such that adjacent PRACH transmission timing is separated larger than the delay variance in a cell/beam (up to 1.6ms). This would be gNB implementation issue.

Regarding 2-step RACH procedure, although we also think 2 step RACH procedure is quite useful for NTN as mentioned in [5], the necessary modification of 2 step RACH for NTN should be discussed after more stabilization of the current 2-step RACH WI.

Conclusion

We discussed on RACH related issues for NTN. We propose the following.

Proposal 1: Common TA can be compensated within gNB and UE use only individual TA.

Proposal 2: Robust PRACH format should be defined for long distance transmission, e.g. use of long sequence (length 839) for both FR1 and FR2, more repetition or multiple sequence transmission.

Proposal 3: Study intercell interference mitigation method with a limited number of ZC sequence per cell.

Proposal 4: Consider PRACH transmission with TA for UE which has valid TA to allow CS multiplexing between UEs.

- Reference

- RP-190710, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.811 V15.0.0, “Study on New Radio (NR) to support non terrestrial networks”

- TR38.821 V0.3.0, “Solutions for NR to support non-terrestrial networks (NTN)”

- R1-1901722 “Overview for NR supporting NTN”, Nokia, Nokia Shanghai Bell

- RP-190131 " Proposed revision of WID “2-step RACH for NR”, Thales