3GPP TSG-RAN WG1 Meeting #96bis	R1-1904863

# Xi’an, China, April 8 – April 12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | -CRNum | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE procedure for determining subframes for transmitting PSSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SHARP |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#86, it was agreed that V2V pool is defined by a repeating bitmap and the bitmap defines which subframes are allowed for transmission and/or reception of SA/data, which means any SA/data transmission subframe belongs to the V2V pool. In current TS36.213, e.g. UE procedure for determining subframes for transmitting PSSCH in mode 3 SPS, when UE determines a transmission in subframe tmSL, the same sub-channels in subframes tm+j×PSPS'SLare also determined for PSSCH transmissions. While there is no description ensuring subframes tm+j×PSPS'SLbelong to PSSCH subframe pool, as no correlation between SPS periodicity and bitmap length. UE may determine PSSCH transmission subframes not belonged to the pool, which somehow does not coincide with definition of the resource pool. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE expects subframes tm+j×PSPS'SLin sidelink mode 3 and subframes tm+j×Prsvp_TX'SLin sidelink mode 4belong to the subframe pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE determines the PSSCH transmission subframes based on specified procedure, while the transmission subframes may not belong to the subframe pool which is allowed for transmission/reception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.4A, 14.1.1.4B, 14.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

14.1.1.4A	UE procedure for determining subframes and resource blocks for transmitting PSSCH for sidelink transmission mode 3

If the UE has a configured sidelink grant (described in [8]) in subframe  with the corresponding PSCCH resource m (described in Subclause 14.2.4), the resource blocks and subframes of the corresponding PSSCH transmissions are determined according to 14.1.1.4C.

If the UE has a configured sidelink grant (described in [8]) for an SL SPS configuration activated by Subclause 14.2.1 and if a set of sub-channels in subframe  is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [8]) of the SL SPS configuration, the same set of sub-channels in subframes  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, , and  is determined by Subclause 14.1.5. Here,  is the sidelink SPS interval of the corresponding SL SPS configuration. The UE is not expected that subframes  do not belong to the subframe set (r0SL, r1SL,r2SL,…), and (r0SL, r1SL,r2SL,…) is determined by Subclause 14.1.5.

14.1.1.4B	UE procedure for determining subframes and resource blocks for transmitting PSSCH and reserving resources for sidelink transmission mode 4

< Unchanged parts are omitted >

If a set of sub-channels in subframe  is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [8]), the same set of sub-channels in subframes  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, ,  , and  is determined by Subclause 14.1.5. Here,  is the resource reservation interval indicated by higher layers. The UE is not expected that subframes  do not belong to the subframe set (r0SL, r1SL,r2SL,…), and (r0SL, r1SL,r2SL,…) is determined by Subclause 14.1.5.

< Unchanged parts are omitted >

14.1.5	UE procedure for determining resource block pool and subframe pool for sidelink transmission mode 3 and 4

< Unchanged parts are omitted >

The UE determines the set of subframes assigned to a PSSCH resource pool as follows:

-	A bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where  the length of the bitmap is configured by higher layers.

-	A subframe  belongs to the subframe pool if  where  .

The subframes determined in the subframe pool are denoted by (r0SL, r1SL,r2SL,…) arranged in increasing order of subframe index.

< Unchanged parts are omitted >