3GPP TSG RAN WG1 #96bis			R1- 1905613

Xi’an, China, 8th – 12th April, 2019

Agenda item:	7.1.3

Source: 	Qualcomm Incorporated

Title: 	Summary of CRs on UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, a summary of CRs on UCI multiplexing on PUSCH is provided in Section 2.

# Summary of CRs

| TDOC | Source | Spec | CR title | Summary of CR | Comments |  |
| --- | --- | --- | --- | --- | --- | --- |
| R1-1904470 | MediaTek | 38.214 | Draft CR on collision handling of PUCCH, PUSCH and SRS | Reason for change: UE behaviour is not clear in case SRS overlapping with (more than one) PUCCH/PUSCH on the same CC. Summary of change: Add text to sub-clause 6.2.1 of TS 38.214 to clarify that the UE does not evaluate overlap between SRS and non-SRS channels until overlap across all non-SRS channels has been resolved. | Recommendation: Conclude the CR is not needed. No Spec update is needed. Reason: The scenario described in the CR is already implicitly treated as an error case per current spec. Furthermore, one should notice that for intra-band CA, spec states that overlapping between SRS and PUCCH/PUSCH is an error case. Lastly, the CR in current form only addresses PUCCH/PUCCH collision but not PUCCH/PUSCH collision, before handling SRS. Companies are welcome to add your views in this column. |  |
| R1-1904538 | CATT | 38.213 | Correction on PUSCH selection for overlapping of PUCCH and PUSCH | Reason for change: In UL CA with different numerologies, one PUCCH (on CC with smaller SCS) can overlap with multiple PUSCHs (on CCs with larger SCS). The UE behaviour in current spec, which is handling PUCCH/PUSCH collision from PUSCH slot perspective, is not aligned with previous agreements in certain scenarios. Summary of change: On high level, the proposed change is to remove “in a slot” in spec in texts related to above mentioned scenarios. By removing “in a slot”, the PUCCH/PUSCH collision is handled from slot with smallest SCS point of view. Then UE behaviour is aligned with previous agreements. | Recommendation: Conclude the CR is not needed. No Spec update is needed. Reason: although not aligned with previous agreements, current spec can work without any issue in these scenarios mentioned in the CR. The change is an NBC, which changes UE implementation, at a late stage of Rel-15. Companies are welcome to add your views in this column. |