3GPP TSG RAN WG1 Meeting #96bis	R1-1905682

# Xi’an, China, April 8-12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0012 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mapping from virtual to physical resource blocks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE, Spreadtrum Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The BWP size used for resource mapping when CORESET 0 is configured and not configured is not consistent with that determined according to the specifications of TS 38.214 and TS 38.212. In section 5.1.2.2.2 of TS 38.214 there is following: In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size PRBs except for the case when DCI format 1_0 is decoded in any common search space in which case the size of CORESET 0 shall be used if CORESET 0 is configured for the cell and the size of initial DL bandwidth part shall be used if CORESET 0 is not configured for the cell. The following issues exist in the current specification: - The RBs for the interleaved VRB-to-PRB mapping for DCI 1_0 with the CRC scrambled by SI-RNTI in Type0-PDCCH CSS in CORESET 0 should be the RBs in CORESET 0 as CORESET is provided. However, the description in current TS 38.211 indicates that initial active downlink bandwidth part is used. - For the case of any other common search space with or without CORESET 0 configured, it is not clear what is the size of NBWP,initsize used for interleaving. It will cause different understanding between UE and gNB on the size used for resource mapping if without the above corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “initial active downlink bandwidth part” to “CORESET 0”. Clarify the size of initial active DL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification with TS 38.214 and TS 38.212 with respect to interleaved VRB to PRB mapping for PDSCH scheduled by DCI format 1_0 in CSS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact. This CR impacts interleaved mapping from virtual to physical resource blocks for PDSCH scheduled by DCI format 1_0 in common search space. It is expected that no change on UE and gNB implemention is required due to this CR, because other specifications have specified that corresponding UE behavior proposed in this CR. Inter-operability analysis: if the UE is implemented in accordance to this CR and the gNB is not, or the gNB is implemented in accordance to this CR and the UE is not, there is an issue that gNB and UE may perform VRB to PRB mapping referring to different sets of RBs. |  |  |  |  |  |  |  |  |

#### 7.3.1.6	Mapping from virtual to physical resource blocks

The UE shall assume the virtual resource blocks are mapped to physical resource blocks according to the indicated mapping scheme, non-interleaved or interleaved mapping. If no mapping scheme is indicated, the UE shall assume non-interleaved mapping.

For non-interleaved VRB-to-PRB mapping, virtual resource block  is mapped to physical resource block , except for PDSCH transmissions scheduled with DCI format 1_0 in a common search space in which case virtual resource block n is mapped to physical resource block n+NstartCORESET where NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received.

For interleaved VRB-to-PRB mapping, the mapping process is defined by:

-	Resource block bundles are defined as

-	for PDSCH transmissions scheduled with DCI format 1_0 with the CRC scrambled by SI-RNTI in Type0-PDCCH common search space in CORESET 0, the set of NBWP,initsize resource blocks in CORESET 0 are divided into Nbundle=NBWP,initsizeL resource-block bundles in increasing order of the resource-block number and bundle number where L=2 is the bundle size and NBWP,initsize is the size of CORESET 0.

-	resource block bundle Nbundle-1 consists of NBWP,initsize mod L resource blocks if NBWP,initsize mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for PDSCH transmissions scheduled with DCI format 1_0 in any common search space other than Type0-PDCCH common search space in bandwidth part i with starting position NBWP,istart, the set of NBWP,initsize virtual resource blocks 0,1,…,NBWP,initsize-1, where NBWP,initsize is the size of CORESET 0 if CORESET 0 is configured for the cell and the size of initial downlink bandwidth part if CORESET 0 is not configured for the cell, are divided into Nbundle virtual resource-block bundles in increasing order of the virtual resource-block number and virtual bundle number and the set of NBWP,initsize physical resource blocks NstartCORESET, NstartCORESET+1,…,NstartCORESET+NBWP,initsize-1 are divided into Nbundle physical resource-block bundles in increasing order of the physical resource-block number and physical bundle number, where Nbundle=NBWP,initsize+NBWP,istart+NstartCORESET mod LL, L=2 is the bundle size, and NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received.

-	resource block bundle 0 consists of L-NBWP,istart+NstartCORESET mod L resource blocks,

-	resource block bundle Nbundle-1 consists of NBWP,initsize+NBWP,istart+NstartCORESET mod L resource blocks if NBWP,initsize+NBWP,istart+NstartCORESET mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for all other PDSCH transmissions, the set of NBWP,isize resource blocks in bandwidth part  with starting position NBWP,istart are divided into Nbundle=NBWP,isize+NBWP,istart mod LiLi resource-block bundles in increasing order of the resource-block number and bundle number where  is the bundle size for bandwidth part  provided by the higher-layer parameter vrb-ToPRB-Interleaver and

-	resource block bundle 0 consists of  resource blocks,

-	resource block bundle  consists of  resource blocks if  and  resource blocks otherwise,

-	all other resource block bundles consists of  resource blocks.

-	Virtual resource blocks in the interval  are mapped to physical resource blocks according to

-	virtual resource block bundle  is mapped to physical resource block bundle

-	virtual resource block bundle  is mapped to physical resource block bundle  where

-	The UE is not expected to be configured with Li=2 simultaneously with a PRG size of 4 as defined in [6, TS 38.214]

-	If no bundle size is configured, the UE shall assume Li=2.

The UE may assume that the same precoding in the frequency domain is used within a PRB bundle and the bundle size is determined by clause 5.1.2.3 in [6, TS 38.214]. The UE shall not make any assumption that the same precoding is used for different bundles of common resource blocks.