3GPP TSG RAN WG1 #96bis	R1-1905793

Xi’an, China, April 8 –12, 2019

Agenda Item:	7.2.1.1

Source:	ZTE

Title:	Updated summary of 7.2.1.1 Channel Structure for Two-step RACH

Document for:	Discussion and decision

# Introduction

This document summarizes the key issues discussed under agenda 7.2.1.1 Channel Structure for Two-step RACH based on the views expressed in the contributions submitted to this agenda.

# Channel Structure

Agreements:

- PUSCH occasion for 2-step RACH is defined as

  - The time-frequency resource for payload transmission.

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP,  reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Configuration

1) Opt 1 - PUSCH occasion(s) are separately configured

Opt 1 was discussed in [1][2][3][6][7][11][14][18][21].

Similar to resource allocation for NR configured grant, the periodicity, offset, resource size, PUSCH mapping type, enabling/disabling frequency hopping, etc. for a PUSCH occasion can be flexibly configured by broadcasting [2][6][7][18].

Basically, for the time domain resource allocation, POs are periodically configured in each PRACH periodicity. A bitmap can also be applied to indicate the time location of POs [2][11].  Another alternative is to configure PUSCH occasion using the similar mechanism as PRACH configuration [3][11][16]. For the frequency domain resource allocation, some default frequency-domain location can be defined [2], i.e. allow to allocate PUSCH occasion within associated PRACH occasion bandwidth, which may help save signaling overhead [6][13].

For separate configuration, each RO can be associated with one or multiple POs [1][2][15][18][20], wherein each PUSCH occasion occupies contiguous subcarriers and symbols, and the size of PUSCH occasions can be different.

Opt 1 can provide flexible configuration of PUSCH resource and can be forward compatible [3]. However, it may also complicate 2-step RACH operation and gNB receiver [7], and the association role between PRACH and PUSCH is complicated and shall be adapted to the change of PUSCH ROs [21].

PO group or a set of resources containing multiple PUSCH occasions can be defined to reduce signaling overhead by (partly) sharing same configurable parameter in a group [2][15][18].

2) Opt 2 - PUSCH occasion(s) configuration based on relative location

Opt 2 was discussed in [1][2][5][6][7][8][9][11][14][17][18][21].

For Opt 2, each RO in a RO period can be associated with one or multiple POs as well [1][5][8][9][11][12][17][18].

Compared with opt1, opt 2 can avoid additional signaling between UE and gNB related to PUSCH MsgA allocation because the association is included in the configuration [5][7][17][21], but at the cost of less flexibility, and may need more specification effort to introduce such configuration mechanism in SIB [2][3].

Among different alternatives under Opt 2, alt.3 or 4 are proposed by multiple companies [1][8][11][18] at least to enable flexible configuration of the time gap between PRACH and PUSCH. Compared with alt.3, alt. 4 provides more flexibility for the mapping between RO(s) preambles and PO(s), at the cost of more signalling overhead required.

Similarly, PO group or a set of resources containing multiple PUSCH occasions can be defined [1][8][11][13][15], so that only the time/frequency offset between the starting point of the PO group and the associated RO(s) needs to be semi-statically configured, and the location of each PO within the PO group can be derived from the RO/preamble index based on a predefined mapping rule [1][8].

Considering that there are pros and cons for both options, we may postpone the down-selection between Opt 1 and different alternatives of Opt 2, depending on further comparisons/evaluations, based on the detailed mapping designs between PRACH and PUSCH.

A common idea for both options is that, in order to reduce the signalling overhead for the configuration, the POs can be configured in a group manner.

Updated Proposal 1:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

Updated proposal 2:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

Updated proposal 6:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission,.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

- The following options of waveform for msgA payload has been discussed in RAN1

  - Opt 1: follow the same waveform of msg3 as msg3-Transformprecoder

  - Opt 2: separate configured, which may or may not be the same as msg3

Any comments?

| Company | View |
| --- | --- |
| Qualcomm | We think additional discussion is necessary for a unified definition of PUSCH occasion or PUSCH resource unit, to clarify whether DMRS ports are included. Regarding Opt1 and Opt2 for PUSCH occasion configuration, we think both can be supported, depending on the RRC state of two-step RACH operation. |

## Detailed mapping between PRACH preamble and PUSCH + DMRS

For both opt 1 and opt 2, apart from the association between RO and PO, the mapping between preamble and DMRS port for each associated PO should be further defined.

1) Mapping ratio

The following mapping ratios are mentioned in the submitted TDocs, taking into account the trade-off between the reserved resource overhead and DMRS collision probability:.

1) One-to-one mapping between preambles in each RO and DMRS ports in the associated PUSCH occasion(s) [1][2][3][8][9][10][12][14][18][20].

2) Multiple-to-one mapping between preambles in each RO and DMRS ports in the associated PUSCH occasion(s) [1][4][9][14][18].

3) One-to-multiple mapping between preambles in each RO and DMRS ports in the associated PUSCH occasion(s) [10][18].

1) Mapping order

The mapping order between preambles and POs within a RACH periodicity is also proposed in [3] and [20], e.g. the order of preamble index in RO(s) increase in code domain, frequency domain and then time domain. The order of PO index increase in code domain, frequency domain, and then time domain [20].

Possible proposal 3:

- Preambles and PUSCH resources are mapped with each other with the following mapping rule:

- PUSCH resources are mapped to preambles in the following order:

- First, in increasing order of preamble indexes within a single PRACH occasion

- Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

- Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

- Fourth, in increasing order of indexes for PRACH slots

- Preambles are mapped to PUSCH resources in the following order:

- First, in increasing order of PUSCH DMRS ports index within a single PUSCH occasion

- Second, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

- Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions

## Guard time/Guard period

Majority companies propose to reserve a time gap between preamble and payload [5][13][18][31][32][34][35][36],  in order to facilitate the UE implementation in terms of the switching of numerology, sampling frequency, and transmission power etc as well as the receiver implementation like preamble decoding. It is noticed that for NR-U case, there should be a configurable time gap in order to avoid the need of multiple LBT. For large cell case, when the round-trip delay exceeds the CP, the time gap can also be used for handling the issue of cross-slot interference. Similarly, a guard period in PUSCH resource is also proposed [6][18][34][35], e.g. to leave the last symbol blank to avoid the cross-symbol interference due to the timing offset. The detailed value depends on the cell size.

In addition, several companies also consider to have the PRACH and PUSCH configured in the same slot [4][13][14][15]. It should be noticed that in NR, a UE is not allowed to transmit PRACH and PUSCH within the same slot. It should be further checked whether this constrain should be also applied for the msgA transmission.

Possible proposal 4:

- A configurable time gap between preamble and payload should be supported

  - FFS the range or candidate values

- The PUSCH resource configured with a guard period can be supported, regarded as implementation issue

- FFS whether PRACH and PUSCH can be configured in the same slot

Any comments?

| Company | View |
| --- | --- |
| Qualcomm | We think the duration of the time gap between msgA preamble and payload should be configurable. For two-step RACH operation on licensed spectrum, a few candidate values (e.g. {0, x, y} symbols) can be specified; for two-step RACH on unlicensed spectrum, the candidate values should depend on the LBT mechanisms adopted by NR and are up to the decisions of NR-U. |

# msgA payload

## Payload size(s), MCS(s) and time-frequency resource size(s) of PUSCH in msgA

Most of the companies propose to have configurable MCS and resource size(s) for payload transmission [1] [2] [5][7][9][15][18]. Several companies propose to support 56/72 bits payload size to accommodate the CCCH message[1][2][15]. Larger payload size have been proposed in[18] and evaluations have been done for larger payload size in [6]. The MCS and resource size are also related to the payload size to be carried [6]. And the combinations of the payload size, MCS and resource size should be limited [1][2][4] [7][9][12][15][18], e.g. limited as QPSK with a single[12] or up to 2 code rates[4], the number of PRBs used for payload transmission may be limited [11][12]. In addition, it is proposed in [6][15] that repetition of payload can be supported for the coverage enhancement of payload transmission according to the simulation results. The indication of MCS, payload size ,resource size could be done via preamble group[1][2][4][7][15] [18] or UCI[7][8][11][18] if a set is configured to the UEs. If the combination of  {MCS, time-frequency resource size} is reduced to a single value, it could be broadcast with system information.

Possible proposal 5:

- A limited number X of {payload size, MCS} configurations should be supported.

  - FFS the value of X and the MCS/resource size values, taking into account the msgA content and RRC state.

  - FFS the indication of configuration, e.g. implicitly indicated (associated with preamble index/group), explicitly indicated (carried in UCI transmitted together with PUSCH payload), or semi-statically configured (in the system information).

  - FFS whether to support slot repetition for payload transmission.

Any comments?

| Company | View |
| --- | --- |

## Configuration of payload data transmission

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2:  same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Numerology and waveform

Regarding the 2 options of msgA PUSCH numerology determination, majority companies [2][8][9][15][20] preferred to re-use that of UL BWP to avoid the potential conflict caused by following the PRACH numerology.

During the RAN1# 96 meeting, it has been agreed to further study the indication of waveform. The indication can be done in the system information. A parameter similar to the msg3-transformPrecoder IE used for msg3 in 4-step RACH could be defined [8][9][12][15]. [9] proposed to include the IE in SIB1.

Possible proposal 6:

- Down-select from the following alternatives, for the numerology of msgA PUSCH:

  - Alt 1: The numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission, if the time gap between PRACH preamble and msgA PUSCH is larger than or equal to X

    - FFS the value of X, X>=1 PUSCH slot.

    - FFS the numerology for msgA PUSCH if the gap between PRACH preamble and msgA PUSCH is less than X

  - Alt 2: The numerology for msgA PUSCH should always follow the numerology configured for the UL BWP for msgA transmission

    - It can be left to implementation if the same numerology is desired for msgA preamble and PUSCH.

- The following options of waveform for msgA payload has been discussed in RAN1

  - Opt 1: follow the same waveform of msg3 as msg3-Transformprecoder

  - Opt 2: separate configured, which may or may not be the same as msg3

Any comments?

| Company | View |
| --- | --- |

### Scrambling ID for payload data

For the msg3 data transmission in legacy 4-step RACH, TC-RNTI/C-RNTI is needed for the initialization of scrambling sequence. For the 2-step RACH, especially for the msgA transmission from RRC_IDLE/RRC_inactive, TC-RNTI/C-RNTI is not available. So it is necessary to discuss which initiation ID to be used for the generation of scrambling sequences. Since there exist cases where multiple UEs selected the same RO are multiplexed on the same PUSCH resource, more inputs can be considered to make the scrambling code distinguishable, such as UE-ID, preamble index, DMRS port index [1][2][6][8][18]. [18] proposed a formula for the RNTI calculation to generate the msgA scrambling code. Moreover, configured scrambling ID could also be considered for the PUSCH as well as the DMRS scrambling [2][15].

Possible proposal 7:

- Consider the following scrambling ID for the payload transmission in msgA, for potential down-selection

  - Modified from Rel.15 RA-RNTI, e.g. incorporate with UE-ID, preamble index or DMRS port index.

  - New RNTI values, e.g. for monitoring the PDCCH scheduling the msgB transmissions, if defined

  - Configured scrambling ID by gNB

Any comments?

| Company | View |
| --- | --- |
| Qualcomm | We think the new RNTI related to CRC scrambling of msgB/PDCCH should be discussed separately, since proposal 6 is about bit level scrambling of msgA payload transmission. |

## Others

Any missing issue in this summary?

| Company | View |
| --- | --- |

# Evaluation

Several companies provided evaluation results on the payload size or collision probability [6][12] [16][18][23][24][25][26][27][28][30]. Details can be found in the Appendix C. Those evaluation results can be useful for the determination of payload size, mapping designs, etc.

Moreover, different system level assumptions may have influence on the distribution of coupling loss, and thus impact the observations on the payload size evaluations [29]. It would be helpful to draw meaningful results if the SLS assumptions are aligned. The assumptions used in the recent evaluations done in NOMA study can be referred, with the ISD/carrier frequency revised according to the LLS assumptions.

Possible proposal 8:

- Adopt the following SLS assumptions for the evaluations of feasible payload size.

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |
| Inter-BS distance | 1732m | 500m | 200m | 25km |
| BS antenna height | 35m | 10m | 10m | 35m |
| Carrier frequency | 700MHz | 4GHz | 4GHz | 700MHz |
| Bandwidth | 10MHz |  |  |  |
| Channel model | UMa in TR 38.901 |  |  | Rural deployment in ITU-R P.1546-5 |
| BS Tx power | 46 dBm | 46 dBm | 41 dBm | [46] dBm |
| BS antenna configurations | (M, N, P, Mg, Ng) = (10, [1,2,4], 2, 1, 1), +-45 Polarization; (dH, dV) = (0.5, 0.5)λ; |  |  |  |
| BS antenna downtilt | 92 | 98 | 102 | [92] |
| BS antenna height | 25m |  |  | [35]m |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |
| UE antenna configuration | 1 (vertical polarization) |  |  |  |
| UT array orientation | uniformly distributed on [0,360] degree |  |  |  |
| UE antenna gain | 0dBi |  |  |  |
| UE receiver noise figure | 9dB |  |  |  |
| UE distribution and antenna height | Follow the modelling of TR 38.901 |  |  |  |
| UE power control | Open loop PC, P0 = [-80, -90, -100] dBm, alpha = 1. |  |  |  |
| HARQ/repetition | 1 |  |  |  |
| UE attachment | Refer to 36.873 |  |  |  |

# Summary

The following proposals to be presented online

Updated proposal 2:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

Updated Proposal 1:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

Updated proposal 6:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission,.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

The following proposals have not been fully discussed during the offline session.

Possible proposal 3:

- Preambles and PUSCH resources are mapped with each other with the following mapping rule:

- PUSCH resources are mapped to preambles in the following order:

- First, in increasing order of preamble indexes within a single PRACH occasion

- Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

- Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

- Fourth, in increasing order of indexes for PRACH slots

- Preambles are mapped to PUSCH resources in the following order:

- First, in increasing order of PUSCH DMRS ports index within a single PUSCH occasion

- Second, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

- Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions

Possible proposal 4:

- A configurable time gap between preamble and payload should be supported

  - FFS the range or candidate values

- The PUSCH resource configured with a guard period can be supported, regarded as implementation issue

  - FFS whether PRACH and PUSCH can be configured in the same slot

Possible proposal 5:

- A limited number X of {payload size, MCS} configurations should be supported.

  - FFS the value of X and the MCS/resource size values, taking into account the msgA content and RRC state.

  - FFS the indication of configuration, e.g. implicitly indicated (associated with preamble index/group), explicitly indicated (carried in UCI transmitted together with PUSCH payload), or semi-statically configured (in the system information).

  - FFS whether to support slot repetition for payload transmission.

Possible proposal 8:

- Adopt the following SLS assumptions for the evaluations of feasible payload size.

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |
| Inter-BS distance | 1732m | 500m | 200m | 25km |
| BS antenna height | 35m | 10m | 10m | 35m |
| Carrier frequency | 700MHz | 4GHz | 4GHz | 700MHz |
| Bandwidth | 10MHz |  |  |  |
| Channel model | UMa in TR 38.901 |  |  | Rural deployment in ITU-R P.1546-5 |
| BS Tx power | 46 dBm | 46 dBm | 41 dBm | [46] dBm |
| BS antenna configurations | (M, N, P, Mg, Ng) = (10, [1,2,4], 2, 1, 1), +-45 Polarization; (dH, dV) = (0.5, 0.5)λ; |  |  |  |
| BS antenna downtilt | 92 | 98 | 102 | [92] |
| BS antenna height | 25m |  |  | [35]m |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |
| UE antenna configuration | 1 (vertical polarization) |  |  |  |
| UT array orientation | uniformly distributed on [0,360] degree |  |  |  |
| UE antenna gain | 0dBi |  |  |  |
| UE receiver noise figure | 9dB |  |  |  |
| UE distribution and antenna height | Follow the modelling of TR 38.901 |  |  |  |
| UE power control | Open loop PC, P0 = [-80, -90, -100] dBm, alpha = 1. |  |  |  |
| HARQ/repetition | 1 |  |  |  |
| UE attachment | Refer to 36.873 |  |  |  |

# Reference

- R1-1903878	On the channel structure of msgA	ZTE, Sanechips

- R1-1903923	Further discussion on channel structure for 2-step RACH	Huawei, HiSilicon

- R1-1904059	Discussion on channel structure for 2-step RACH	vivo

- R1-1904196	Channel Structure for 2-Step RACH	LG Electronics

- R1-1904246	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1904280	Channel structure for 2-step RACH	Intel Corporation

- R1-1904357	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1904392	Channel Structure for Two-Step RACH	Samsung

- R1-1904542	Discussion on channel structure for 2-step RACH	CATT

- R1-1904584	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1904715	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1904724	Discussion on channel structure for two-step RACH	CMCC

- R1-1904778	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1904819	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1904842	Channel Structure for Two-step RACH	Ericsson

- R1-1904848	On PUSCH Transmission in msgA 	InterDigital, Inc.

- R1-1904944	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1904992	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1905043	Discussion on channel structure for 2-Step RACH	ITRI, NTPU

- R1-1905051	On Channel Structure for 2-step RACH	OPPO

- R1-1905125	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1905205	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1903880	Analysis on the collision probability of msgA	ZTE, Sanechips

- R1-1903881	Preliminary evaluation results for 2-step RACH	ZTE, Sanechips

- R1-1904061	Evaluation for 2-step RACH	vivo

- R1-1904684	Preliminary results for 2-step RACH evaluations	Huawei, HiSilicon

- R1-1904685	Other issues for 2-step RACH	Huawei, HiSilicon

- R1-1904717	2-step RACH latency and resource reservation	Nokia, Nokia Shanghai Bell

- R1-1904844	Evaluation methodology and metrics for Two-Step RACH	Ericsson

- R1-1904845	Initial results on coverage and msgA configurations	Ericsson

- R1-1901626	Considerations on the channel structure of msgA	ZTE

- R1-1901669	Discussion on channel structure for 2-step RACH	vivo

- R1-1902133	Channel Structure for Two-Step RACH Considerations 	Sierra Wireless, S.A.

- R1-1902135	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1902241	Channel Structure for Two-Step RACH	Samsung

- R1-1902747	On Channel Structure for 2-step RACH	OPPO

# Appendix A: Objective in 2-step RACH WID

The work item aims to specify 2-step RACH as general MAC procedure covering both physical layer and higher layer aspects. Only Contention based RACH procedures are specified for 2-step RACH. All the objectives are applicable for both licensed and unlicensed spectrum except for the bullet under “For unlicensed operation” where additional work dedicated to unlicensed spectrum will be done after completion of unlicensed spectrum aspects of 4-step RACH in NR-U WI.

2-step RACH [RAN1, RAN2]

- 2-step RACH shall be able operate regardless of whether the UE has valid TA or not.

- 2-step RACH is applicable to any cell size supported in Rel-15 NR;

- 2-step RACH is applied for RRC_INACTIVE , RRC_CONNECTED and RRC_IDLE state

- Specify contention-based 2-step RACH procedure (RAN2)

- Channel structure of msgA is Preamble and PUSCH carrying payload (RAN1)

  - Only reuse the Rel-15 NR PRACH Preambles design.

  - Only reuse the Rel-15 NR PUSCH including Rel-15 DMRS for transmission of payload of msgA

    - No new CP length and no sub-PRB guard subcarrier(s)

Note 1: The above sub-bullet is to ensure that signal structure optimizations for any specific cell size (e.g. cells with RTT larger than Rel-15 PUSCH CP duration) are not pursued.

  - Specify the mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+ DMRS

    - PRACH Preamble and PUSCH in a msgA is TDMed

  - Specify the supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA

    - Consider the msgA payload contents determined by RAN2

  - Specify power control of PUSCH of msgA

- Specify msgA’s content: to include the equivalent contents of msg3 of 4-step RACH (RAN2/RAN1)

  - Inclusion of UCI in msgA is not precluded

- Specify msgB’s content: to include the equivalent contents of msg2 and msg4 of 4-step RACH (RAN1/RAN2)

- Contention resolution for 2-step RACH (RAN2)

- Design of RNTI for msgB of 2-step RACH (RAN2)

- Specify the fall back procedure from 2-step RACH to 4-step RACH (RAN2/RAN1)

- All triggers for Rel-15 NR 4-step RACH are applied for 2-step RACH except for SI Request and BFR which are up to RAN2 discussion

  - No new triggers for 2 step RACH

For unlicensed operation:

- After PRACH and PUSCH design enhancements are completed for NR-U in the Rel-16 NR-U WI, identify and specify the necessary modification of 2-step RACH design for its application in NR-U(RAN1/RAN2)

Note 2: UP data transmission in RRC_IDLE and RRC_INACTIVE state is not in the scope. UP data transmission in RRC_CONNECTED mode as in Rel-15 NR is supported.

# Appendix B: Agreements in RAN1#96

Agreements:

- PUSCH occasion for 2-step RACH is defined as

  - The time-frequency resource for payload transmission.

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP,  reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2:  same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Agreements:

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

| Parameters | Values |
| --- | --- |
| Scenario | - For evaluation of schemes: 200m, UMi, 4 GHz. FFS: 500m, UMa, 4 GHz. - For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz; or 25km, RMa, 700 MHz. Other values can be reported if applicable. Note: this does not restrict preamble format selection. |
| Preamble format | Company report |
| Waveform (data part) | CP-OFDM, or DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30/60kHz at 4 GHz, 120kHz at 30GHz |
| TBS | - 56 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |
| MCS and Resource size | Company report the MCS, time/frequency resource size, and DMRS overhead. Strive to agree to some common values in RAN1#96bis. |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or interference from the adjacent PUSCH resource, including how to model relative SINR, timing, etc. |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx. FFS: 2 Tx |
| gNB antenna configuration | 2Rx or 4Rx, 8Rx as optional |
| Propagation channel & UE velocity | TDL/CDL-A 30ns, or TDL/CDL-C 300ns, 3km/h or 30km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 as starting point, other values are not precluded and company should report the details of HARQ |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. Ideal can be considered for calibration, if needed. |
| Target BLER | [10%, 1%] for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

# Appendix C: Evaluation results

ZTE [R1-1903881] (a) Preamble Format 0 (b) Preamble Format B1 (c) Comparison between 2Rx and 4Rx Figure 1 BLER vs. SNR for 1 PRB payload (a) Preamble Format 0 (b) Preamble Format B1 (c) Comparison between 2Rx and 4Rx Figure 2 BLER vs. SNR for 6 PRB payload Table 1 Link budget calculation with 4G+200m Format B1 10bytes; 40bytes; 80bytes; 120bytes; 160bytes. Transmitter. (1) Tx Power (dBm); 23; 23; 23; 23; 23. Receiver. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 2160000; 2160000; 2160000; 2160000; 2160000. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -105.66; -105.66; -105.66; -105.66; -105.66. (7) Required SINR (dB) (Target BLER=0.1); -5; -0.5; 2.3; 4.8; 7. (8) Receiver sensitivity = (6)+(7) (dBm); -110.66; -106.16; -103.36; -100.86; -98.66. (9) Receiver processing gain; 0; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 133.66; 129.16; 126.36; 123.86; 121.66. Figure 3 Coupling loss distribution in different scenarios - 700MHz + 1732m, 2Rx Table 2 Supported percentage with 6 PRB, Preamble Format 0 Supported percentage; 10bytes; 40bytes; 80bytes; 120bytes; 160bytes. MCL (dB); 138.37; 132.67; 129.67; 126.87; 124.67. 700MHz + 1732m; 95%; 82%; 72%; 63%; 56%. Table 3 Supported percentage with 1 PRB, Preamble Format 0 Supported percentage; 10bytes; 20bytes; 30bytes. MCL (dB); 137.65; 134.25; 130.45. 700MHz + 1732m; 93%; 86%; 75%. - 4GHz + 500m, 4Rx Table 4 Supported percentage with 6 PRB Format B1 Supported percentage; 10bytes; 40bytes; 80bytes; 120bytes; 160bytes. MCL (dB); 136.36; 133.96; 131.16; 129.16; 126.66. 4GHz + 500m; 92%; 87%; 81%; 76%; 69%. Table 5 Supported percentage with 1 PRB Format B1 Supported percentage; 10bytes; 20bytes; 30bytes. MCL (dB); 138.14; 135.24; 131.74. 4GHz + 500m; 95%; 90%; 82%. - 4GHz + 200m, 2Rx Table 6 Supported percentage with 6 PRB Format B1 Supported percentage; 10bytes; 40bytes; 80bytes; 120bytes; 160bytes. MCL (dB); 133.66; 129.16; 126.36; 123.86; 121.66. 4GHz + 200m; 100%; 99%; 99%; 99%; 98%. Table 7 Supported percentage with 1 PRB Format B1 Supported percentage; 10bytes; 20bytes; 30bytes. MCL (dB); 133.64; 130.44; 126.74. 4GHz + 200m; 100%; 99%; 99%. Observation 1: For a minimum resource size with 1 PRB and 1 slot, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m). Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas. - All 64 Preambles are mapped to 1 PUSCH resource (6 PRB), the DRMS port for each preamble index is mapped as DMRS_port_index = mod(preamble_index, 12), and thus there is a probability of DMRS collision 1/12, as illustrated in Figure 4-(1). - All 64 Preambles are divided into 2 groups, each group of Preambles are mapped to 1 PUSCH resource (3 PRB), then there is a probability of DMRS collision of 1/24, as illustrated in Figure 4-(2). - All 64 Preambles are divided into 4 groups, 2 groups of Preambles are mapped to 2 PUSCH resource (1 PRB), and the other 2 groups of Preambles are mapped to 2 PUSCH resource (2 PRB), then there is a probability of DMRS collision (1/48) , as illustrated in Figure 4-(3). - All 64 Preambles are divided into 6 groups, each group of Preambles are mapped to 1 PUSCH resource (1 PRB), then there is no DMRS collision, as illustrated in Figure 4-(4). Figure 4. Possible mapping between preambles and PUSCH occasions. - 1T2R (b) 1T4R Figure 5 BLER vs. SNR for 2UEs with Preamble Format 0 - 1T2R (b) 1T4R Figure 6 BLER vs. SNR for 2UEs with Preamble Format B1 (a) 1T2R (b) 1T4R Figure 7 BLER vs. SNR for 2UEs with Preamble Format B3 Observation 2: Given the same PUSCH resource overhead, it would be preferred to have PRACH preambles mapped to up to 2 PUSCH occasions. Figure 8 BLER vs. SNR for 2UEs with DMRS collision Observation 3: As long as the timing offset (non-zero) can be accurately estimated by preamble and compensated at the receiver, the performance impact of DMRS collision can be alleviated.

Ericsson [R1-1904844] Table 1: Parameters for minimum payload size evaluations Parameters; Values. The number of PUSCH symbols & PUSCH scheduling type; 14, Type A. PUSCH DMRS configuration; Type 1, with [2 or 3] DMRS symbols. Number of PUSCH PRBs; [1]. Repetition or retransmission; [2], for cases targeting low BLER such as 1%. Table 2: System level parameters for coverage evaluations Parameters; Values. Layout; Single layer - Macro layer: Hex. Grid, 57 cells. Inter-BS distance; 200m. Carrier frequency; 4 GHz. Channel model; UMi in TR 38.901. UE Tx power; Max 23 dBm. BS antenna configurations; 2 ports: (M, N, P, Mg, Ng) = (10, [1,2,4], 2, 1, 1), +-45 Polarization dH = dV = 0.5λ;. BS antenna downtilt; 102. BS antenna height; 25m. BS antenna element gain + connector loss; 8 dBi, 0dB cable loss. BS receiver noise figure; 5dB. UE antenna configuration; 1. UE antenna height; Follow the modelling of TR 38.901. UE antenna gain; 0dBi. UE distribution; Follow the modelling of TR 38.901. UE attachment; Refer to 36.873. Observations: - The capacity benefit for 2-step RACH used with small data applications is not likely to come from optimizing the capacity of msgA PUSCH - Capacity of PUSCH that can be scheduled after msgB can be substantially better than msgA due to better link adaptation, scheduling flexibility, etc. - However, for e.g. small cells, the SNR is likely to be quite high over a substantial part of the cell, so some ability to support different msgA sizes has potential. Proposals: - Link level evaluations simulate a single UE as a starting point - The need for evaluations of 2-step RACH collision among UEs should be further studied - Any such evaluations take into account all relevant ‘system’ behaviors - Study the benefit of variable msgA payload size - 2-step RACH link level evaluations include the parameters and values in Table 1 - 2-step RACH coverage evaluations use the parameters and values in Table 2 for the 200m ISD UMi scenario Ericsson [R1-1904845] Figure 1: Single User SNR CDF in 200m ISD UMi Observations: - The SNR can be quite high over much of a UMi cell: median SNRs are on the order of 13 dB over a wide range of power control settings. - A UMi cell can have relatively low cell edge SNRs: about -2.0 and -1.5 dB at the 5% and 10% CDF points, respectively. Figure 2: Operating SNR and Bandwidth Normalized Operating SNR vs. Number of PRBs, 7 symbol slot Figure 3: Operating SNR and Bandwidth Normalized Operating SNR vs. Number of PRBs, 14 symbol slot Observations In a 200m ISD UMi scenario with gNB 4 Rx antennas, - 2 PRBs seems a reasonable size for 56 bit payloads with 7 symbols - The reduction in required SNR for a fixed transmit power drops fairly quickly (by about 1 dB) from 1-2 PRBs for both 1% and 10% BLER, and flattens quickly thereafter. - 1 PRB should be sufficient to carry 56bits when 4 Rx antennas are used with 14 symbols - The reduction in required SNR vs. the number of PRBs is quite flat, only varying a few tenths dB up to 4 PRBs. - Operating SNRs for 2 PRBs with 56 bit payloads for 7 symbols are on the order of: - -7.5 dB for 10% BLER - -4.4 dB for 1% BLER - Operating SNRs for 1 PRB with 56 bit payloads for 14 symbols are on the order of: - -8.3 to for 10% BLER - -4.9 to for 1% BLER In Table 1 below, the margin over the outage at a given outage rate is calculated as the (SNR at the outage rate) – (margin for impairments) – (the normalized operating SNR). Table 1: Coverage of 56 bit msgA vs. outage and BLER 5% CDF point (dB); -2. 10% CDF point (dB); 1.5. Impairments (dB); 2. Normalized Operating SNR; Margin over 5% Outage; Margin over 10% Outage. 7 symbol (2 PRBs); 14 symbol (1 PRB); 7 symbol (2 PRBs); 14 symbol (1 PRB); 7 symbol (2 PRBs); 14 symbol (1 PRB). 1% BLER; 2.2; -1.8; -6.2; -2.2; -2.7; 1.3. 10% BLER; -1.2; -5.0; -2.8; 1.0; 0.7; 4.5. Observations: In a 200m ISD UMi scenario with gNB 4 Rx antennas, - A 14 symbol slot seems needed to reach 5% outage at 10% BLER with 56 bit payloads - Repetition or retransmission seems needed to reach 5% outage for 1% BLER with 56 bit payloads Proposals: - 2-Step RACH simulations of minimum payload size include configurations with a 14 symbol slot - Further study repetition and retransmission for low coverage and/or low BLER targets.

Huawei [R1-1904684] ### Miss-detection rate and false-alarm rate - the required SNR values to meet the given performance requirements are summarized as Table 1 if the target Pm=1%; - the link budget for different preamble formats is further analyzed in Table 2, showing that the MCL for preamble format 0, A1, A2, A3 are 143.8dB, 130.3dB, 133.3dB, and 134.9dB, respectively with Pm=1%, Pf=0.1%. (a). Format 0, SCS=1.25 kHz; (b). Format A1, SCS=30 kHz. (c). Format A2, SCS=30 kHz; (d). Format A3, SCS=30 kHz. Figure 1. Miss-detection rate and false-alarm rate of preamble detection Table 1. Minimum SNR values to meet the requirements of Pm and Pf Preamble format; (Pm,Pf); Minimum SNR value (dB). 0; (1%, 1%); -13.2. (1%, 0.1%); -12.0. A1; (1%, 1%); -6.7. (1%, 0.1%); -4.5. A2; (1%, 1%); -9.7. (1%, 0.1%); -7.5. A3; (1%, 1%); -11.5. (1%, 0.1%); -9.1. Table 2. Link budget calculation of different preamble formats Preamble Format; 0; A1; A2; A3. (1) Tx Power (dBm); 23; 23; 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 1.04875e6; 4.17e6; 4.17e6; 4.17e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -108.8; -102.8; -102.8; -102.8. (7) Required SINR (dB) (Pm=1%, Pf=0.1%); -12.0; -4.5; -7.5; -9.1. (8) Receiver sensitivity = (6)+(7) (dBm); -120.8; -107.3; -110.3; -111.9. (9) Receiver processing gain; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 143.8; 130.3; 133.3; 134.9. ### Timing and frequency offset estimation (a). Performance of TA estimation for ISD=200 m; (b). Performance of FO estimation for ISD=200 m. (c). Performance of TA estimation for ISD=1732 m; (d). Performance of FO estimation for ISD=1732 m. Figure 2. Estimation error of frequency offset for different preamble formats and ISDs Observation 1: The estimation of timing offset can be comparably accurate irrespective ISD. Observation 2: The estimation error of frequency offset is higher than or close to the average frequency offset for minimum SNR values meeting the requirements of miss-detection and false-alarm. ## Payload Size In the following evaluation, the target BLER of initial transmission is set to be 1%. Table 3. Simulation cases for payload size Case; # of PRBs; Modulation; TBS. 0; 1; QPSK; 72. 1; 2; QPSK; 72. 2; 6; QPSK; 72. 3; 3; 16QAM; 1000. 4; 6; QPSK; 1000. 5; 12; QPSK; 1000. (a). TBS=72 bits; (b). TBS=1000 bits. Figure 3. BLER performance of different cases for single UE case Table 4. Link budget calculation of different payload sizes Case; 0; 1; 2; 4; 5; 5. SCS (kHz); 30; 30; 30; 60; 30; 60. TBS; 72; 72; 72; 1000; 1000; 1000. (1) Tx Power (dBm); 23; 23; 23; 23; 23; 23. (2) Thermal noise density (dBm/Hz); -174; -174; -174; -174; -174; -174. (3) eNB receiver noise figure (dB); 5; 5; 5; 5; 5; 5. (4) Interference margin (dB); 0; 0; 0; 0; 0; 0. (5) Occupied channel bandwidth (Hz); 3.6e5; 7.2e5; 2.16e6; 4.32e6; 4.32e6; 8.64e6. (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm); -113.4; -110.4; -105.6; -102.6; -102.6; -99.6. (7) Required SINR (dB); 6.5; 2.8; -2.5; 8.6; 5.3; 2.9. (8) Receiver sensitivity = (6)+(7) (dBm); -106.9; -107.6; -108.1; -94.0; -97.3; -96.7. (9) Receiver processing gain; 0; 0; 0; 0; 0; 0. (10) MCL = (1)-(8)+(9) (dB); 129.9; 130.6; 131.1; 117.0; 120.3; 119.7. Observation 3: When payload size is 72 bits, the MCL is close to that of short preambles (format A1, A2, A3), and more than 10dB worse than that of long preamble (format 0). Observation 4: When payload size is 1000 bits, the MCL is more than 10dB worse than that of short preambles (format A1, A2, A3), and more than 23dB worse than that of long preamble (format 0), which potentially require more repetitions for payload transmission thus likely larger latency compared to Rel-15 4-step RACH. (a). TBS=72bits, 6PRB, SCS=30 kHz; (b). TBS=1000bits, 12PRB, SCS=30 kHz. Figure 4. BLER performance for multiple UE cases Observation 5: With small payload size, more UEs can be multiplexed in the same PO, and higher resource utilization can be achieved. Proposal: At least 56 or 72 bits are supported for the MsgA payload, FFS other payload size. Huawei [R1-1904685] (a). N=2; (a). N=4. Figure 1. Collision probability for multiple preamble groups Observation 3: For the case with multiple preamble groups, the collision probability increases if the traffic load with respect to the available PRACH resource is not matched. Observation 4: The number of UEs performing contention-based 2-step RACH on the same RO needs to be small to avoid large collision probability and error floor in BLER performance.

Nokia [R1-1904717] Latency and resource reservation comparison Figure 6: Average latency in slots for 4-step RACH and 2-step RACH. For 2-step RACH M is the number of PUSCH occasions associated with a PRACH occasion. Observation 1: The availability of multiple antenna ports increases the 2-step RACH performance. Proposal 1: The use of multiple antenna ports in the PUSCH resources for MsgA should be considered in 2-step RACH. Figure 7: PUSCH resource reservation and usage for four-step RACH and 2-step RACH. For 2-step RACH M is the number of PUSCH occasions associated with a PRACH occasion. Observation 2: The benefit of reduced latency with 2-step RACH is accompanied by an increase in PUSCH resource reservation and usage.

VIVO [R1-1904061] Payload sizes Figure 1: BLER vs. SNR of different payload size with 2/4/6 PRBs Table 1: Required SNR and MCL with PUSCH target BLER = 1% (2, 4, or 6 PRBs) 2 PRBs; 4 PRBs; 6 PRBs. Payload size (bits); Required SNR (dB); MCL (dB); Required SNR (dB); MCL (dB); Required SNR (dB); MCL (dB). TBS=56; 1.9; 124.5; -2.1; 125.5; -3.9; 125.5. TBS=96; 3.6; 122.8; -0.3; 123.7; -2.5; 124.1. TBS=224; 7.2; 119.2; 2.3; 121.1; 0.11; 121.5. TBS=320; 9.4; 117.0; 4.2; 119.2; 1.4; 120.2. TBS=456; 13.0; 113.4; 5.6; 117.8; 2.8; 118.8. Observation 1: From MCL perspective, PRB size 4 and PRB size 6 provides better performance than smaller PRB size.

InterDigital [R1-1904848] Figure 1 BLER vs SNR in 700MHz TDL-A 3 kph 2 Rx Figure 2 Required SNR vs payload size Figure 3 Coupling loss distribution in RMa 700MHz 1732 ISD Figure 4 MCL vs payload Observation 1: Assuming 10% target BLER, up to 160 bits payload can be supported in 1732m RMa 700MHz deployment scenario with a transmission bandwidth of 6 PRBs.

Intel [R1-1904280] MsgA PRACH. MsgA PUSCH. Figure 3. Link level simulation results for MsgA PRACH and PUSCH Table 2. MCL difference between MsgA PRACH and PUSCH TDL-A 30ns; 2 Rx antennas; 4 Rx antennas. Format 0; Format A1; Format 0; Format A1. PUSCH, 56 bits; 5 dB; -3 dB; 6.5 dB; -3 dB. PUSCH, 480 bits; 13 dB; 5 dB; 14.5 dB; 6 dB. PUSCH, 1032 bits; 17 dB; 9 dB; 18.5 dB; 10 dB. TDL-C 300ns; 2 Rx antennas; 4 Rx antennas. Format 0; Format A1; Format 0; Format A1. PUSCH, 56 bits; 6 dB; -1.5 dB; 7 dB; -1 dB. PUSCH, 480 bits; 14 dB; 6.5 dB; 14.5 dB; 6 dB. PUSCH, 1032 bits; 16 dB; 8.5 dB; 17.5 dB; 9 dB. Observation 1 - Relatively large range of MCL difference between MsgA PUSCH and PRACH is observed, which depends on payload size carried by MsgA PUSCH, associated PRACH format, MCS, allocated resource and subcarrier spacing. - For short PRACH format, when same subcarrier spacing is employed for the transmission of PRACH and PUSCH in MsgA, the coverage difference between PRACH and PUSCH can be relatively small. This may indicate that a large payload size, e.g., > 480 bits, can be carried by MsgA PUSCH for cell center UEs with good channel condition. - For certain combinations of MsgA PRACH format and PUSCH, appropriate power offset may be applied to match overage status in MsgA.

CMCC [R1-1904724] Figure 1 BLER performance of PUSCH transmission The granularity of PUSCH resource can be symbol-level in the time domain and RB-level in the frequency domain. Proposal 1: The modulation scheme of msgA PUSCH is configured as one modulation scheme, such as QPSK, at least for the idle or inactive state, when the payload size of PUSCH takes 56 bits or 72bits. Proposal 2: Introduce a power offset when UE sending msgA PUSCH with the larger payload size such as 72bits compared with 56 bits.

Qualcomm [R1-1904992] - For a small payload size of 9 bytes (plus 16-bit CRC), 2 or 3 PRBs are sufficient for spectral efficiency of 0.2~ 0.3 bit/symbol. - For larger payload size of 125 bytes (plus 16-bit CRC), 15 PRBs are needed for spectral efficiency of 0.5 bit/symbol, and 8 PRBs are sufficient for higher spectral efficiency of 1 bit/symbol. Figure 7: Resource Size Considerations for Different Payload Size Observation 3: Two-step RACH can operate in any RRC state. Different payload size, waveform and MCS can be specified for different RRC state and mapped to different PO. Proposal 2: For msgA transmission, the following designs should be supported: - a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; maximal payload sizes up to a few hundred bytes can be supported by higher MCS; - for msgA transmission in any RRC state, modified RNTI can be used to initialize the bit level scrambling sequence of PUSCH, which can be defined as a function of RA-RNTI and the sequence index of preamble/DMRS; - initial UL BWP can be used for msgA transmission in RRC_IDLE/RRC_INACTIVE state, or RRC re-establishment in RRC_CONNECTED state; - active UL BWP can be configured for msgA transmission in RRC_CONNECTED state except for RRC re-establishment; - feasible choices of MCS and payload size for msgA payload should be evaluated and specified in NR Rel-16; - multiple POs can be configured per RO period, to accommodate different MCS, payload size and waveform; - multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar MCS/waveform/payload size; - resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain; - the association between preamble sequences and DMRS/PUSCH resources can be based on one-to-one, one-to-many or many-to-one mapping, and the mapping rule can be indicated by SI or RRC signalling; - the configurations and resource allocations of RO and PO need to be balanced to achieve a good trade-off in resource utilization efficiency and collision probability reduction. Figure 8: Preamble Pool (S) to DMRS Pool (D) Mapping Figure 9: Collision Probability of Preamble or DMRS Random Selection for msgA (CBRA)