3GPP TSG RAN WG1 Meeting #96bis							R1-1905830

Xi’an, China, April 8 - 12, 2019

Title:	[DRAFT] Reply LS on clarification about CSI-RS measurement

Response to:	R1-1903856 (R2-1902730)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Intel [RAN1]

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name:	Daewon Lee

E-mail Address:	daewon.lee (at) intel (dot) com

Attachments:

1. Overall Description:

RAN1 has discussed the signalling framework for CSI-RS configuration for RRM measurements based on LS provided by RAN2 [1] and have concluded that CSI-RS configuration signalling framework is acceptable to RAN1.

As for associated restrictions on maximum number of CSI-RS resources, RAN1 has previously agreed to put maximum number of CSI-RS resource restriction per frequency layer and has captured the restriction in TS38.214 section 5.1.6.1.3 (See below).

5.1.6.1.3 CSI-RS for mobility --- omitted --- A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured - with no more than 96 CSI-RS resources when all CSI-RS resources per frequency layer have been configured with associatedSSB, or, - with no more than 64 CSI-RS resources per frequency layer when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB - For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resources per frequency layer. --- omitted ---

Therefore, even though CSI-RS configuration signalling framework allows configuration of 9216 CSI-RS resources, the RAN1 specification does not allow such configuration to be done per frequency layer. This was done to ensure UE complexity for performing CSI-RS measurements is not over-burdened.

However, RAN1 does recognize that terminology of ‘frequency layer’ is not used in RAN2 specification and also not clearly defined in the current RAN1 specification.

To resolve this issue, RAN1 has agreed that ‘frequency layer’ used in RAN1 specification should be a measurement object.

Additionally, RAN1 has agreed that CSI-RS resources in the same MO shall have the same startPRB, and would like to recommend to RAN2 capture the restriction in appropriate RAN2 specifications.

Additionally, RAN1 has agreed that CSI-RS resources in the same MO shall have the same (startPRB + floor(nrofPRBs/2)) value and would like to recommend to RAN2 capture the restriction in appropriate RAN2 specifications.

Additionally, RAN1 has agreed that the difference between the maximum of (startPRB + nrofPRBs) of all configured CSI-RS resources in a MO and minimum of startPRB of all configured CSI-RS resources in a MO shall be equal or smaller than 264 PRBs.

The following are RAN1 responses to the rest of the questions in the LS from RAN2.

Figure 1. Figure from LS from RAN2 [1]

- Whether the CSI-RS resources of serving cell and CSI-RS resources of cell1 configured in MO1 are part of the same ‘frequency layer’?

=> Yes. All CSI-RS resources from the same MO should be considered as same frequency layer.

- Whether the CSI-RS resources of serving cell, CSI-RS resources of cell1 and CSI-RS resources of cell2 configured in MO1 are part of the same ‘frequency layer’?

=> CSI-RS resources that have different startPRB in a MO should not be supported.

=> CSI-RS resource that have different center frequency (i.e. startPRB + floor(nrofPRBs/2)) in a MO should not be supported.

=> As long as difference between the maximum of (startPRB + nrofPRBs) of all configured CSI-RS resources in a MO and minimum of startPRB of all configured CSI-RS resources in a MO is be equal or smaller than 264 PRBs, they can be part of the same ‘frequency layer’.

- Whether the CSI-RS resources configured in MO1 and MO2 are part of the same ‘frequency layer’?

=> No. CSI-RS resources from different MOs are considered to be in different frequency layers.

- If only CSI-RS resource of cell2 is configured in MO1 and if only CSI-RS resource of cell5 is configured in MO2, are they part of the same ‘frequency layer’?

=> No. CSI-RS resources from different MOs are considered to be in different frequency layers.

- Whether the CSI-RS resources belonging to the same “frequency layer” must have the same subcarrier spacing?

=> Yes. This is part of RAN1 agreement in RAN1 #91. It was agreed in RAN1 that “A common value for CSI-RS resource numerology is assigned across all the resources configured per frequency carrier”. RAN1 would like to recommend to RAN2 capture the restriction in appropriate RAN2 specifications as well.

For information purposes, R1-190xxxx has been attached to the LS, which is the endorsed CR for TS38.214 that updates the “per frequency layer” text with “per higher layer parameter measObjectNR”.

RAN1 would like to recommend to RAN2 to implement the above suggestions.

2. Actions:

To RAN2: 	RAN1 would like to recommend to RAN2 to implement the above suggestions.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #97 	13 – 17 May 2019			Reno, U.S.A.

TSG-RAN WG1 Meeting #98 	26 – 30 Aug 2019			Prague, Czech Republic

4. References

[1] R1-1903856 (R2-1902730), “LS on clarification about CSI-RS measurement,” RAN WG2