3GPP TSG RAN WG1 Meeting #98bis	R1-1909972

Chongqing, China, 14th – 20th October, 2019

Title:	Considerations on DL reference signals and channels design for NR-U

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2.1.2

Document for:	Discussion and Decision

Introduction

In the RAN #82 meeting, the new WID on NR-U (NR-based Access to Unlicensed Spectrum) [1] aims to identify solutions and techniques for next generation wireless systems operating on unlicensed bands. In the WID, one of the objectives is to specify DL physical layer signals and channels. In last several RAN1 meetings, the following agreements were reached on DL reference signals and channels design for NR-U [2-4].

Agreement:

- The following is agreed for DM-RS positions for PDSCH mapping Type B of length 9 and 10:

| in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | ,4,7 | ,4,7 |
| 10 |  | , 7 | ,4,7 | ,4,7 |

- If the PDSCH duration  is 10 OFDM symbols for normal cyclic prefix, and if at least one of the DM-RS symbols corresponding to the PDSCH allocation collides with resources reserved for at least one of the symbols of a LTE CRS transmission, shall be incremented by 1

  - FFS: Whether the increment is applicable to all slots or a subset of slots

Conclusion:

- PDSCH type B durations 9,10 will not be optimised for double symbol DM-RS for avoiding LTE-CRS collisions

Agreement:

Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

Agreement:

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE.

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

- Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI

- Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

Agreement:

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

- Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

- FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

In RAN #84 plenary meeting, the views on essential/optimization items on each WGs for on time completion of NR-U WI was presented in [5]. Wherein, the essential/optimization items in “7.2.2.1.2 DL signals and channels” are as follows:

Essential

- Channel occupancy time and frequency domain structure indication for LBE

- UE COT detection for FBE

- gNB control for the Cat 4 UL transmission (including CG) switching to cat 2 when fall in gNB COT

- Behavior when P/SP-CSI-RS fail to transmit due to LBT

Optimization

- CSI-RS enhancement outside of DRS

  - More opportunities for CSI-RS to compensate the possible LBT failure

  - A-TRS directly QCL with SSB

In this contribution, we discuss potential DL reference signals and physical layer channels design for NR-U, including RS for DL burst detection, dynamic PDCCH monitoring for LBE, COT detection for FBE, COT structure indication, PDSCH transmission, CSI-RS, and DMRS sequence generation and so on.

DL Reference Signals and Physical Channels Design

Dynamic PDCCH Monitoring for LBE

In NR, the PDCCH monitoring information per search space is configured via RRC, such as PDCCH monitoring periodicity, offset and symbols for PDCCH monitoring in the slots. In Phase A (outside of gNB’s COT), in order to improve spectrum utilization and reduce access delay, UE can monitor PDCCH and corresponding DMRS for DL burst detection with a shorter monitoring periodicity. In implementation, UE in RRC_CONNECTED state can decode PDCCH after DMRS is detected so as to saving power consumption. For the RRC_IDLE UE, it will check whether there is paging information of its own by detecting the PDCCH scrambled by P-RNTI at a given paging occasion. If PDCCH scrambled by P-RNTI is detected (whether there is paging information that belongs to it or not), it will stop the monitoring of the additional PDCCH monitoring occasions at the given paging occasion for power saving. For the first slot of the COT (i.e. Phase B: beginning of gNB’s COT), if shorter monitoring periodicity is configured, better performance may be achieved for NR unlicensed systems, due to the increased chance of PDCCH transmission. From the beginning of the second slot of the COT (i.e. Phase C: remainder of gNB’s COT), the PDCCH monitoring granularity can be longer, e.g., slot-level PDCCH monitoring, for the sake of UE complexity and power consumption reduction. One simple example is shown in Figure 1.

UE determines the transition from Phase A to Phase B based on DL burst detection. Once it detects an PDCCH+DMRS, the UE starts to detect other RSs/channels including PDCCH according to the above monitoring strategy in Phase B. After the first slot of the COT, UE would transit from Phase B to Phase C. The transition from Phase C to Phase A can depends on the length or end position of the COT, which can be indicated by DCI format 2_0.

Figure 1: PDCCH monitoring pattern in a COT

Observation 1: For RRC_IDLE UEs, PDCCH scrambled by P-RNTI at a given paging occasion can be used for UEs to judge DL burst detection and/or decide to stop the monitoring of the additional PDCCH monitoring occasions for power saving.

Proposal 1: Different PDCCH monitoring patterns can be performed for Phase A (outside of gNB’s COT), Phase B (beginning of gNB’s COT) and Phase C (remainder of gNB’s COT). The PDCCH monitoring occasion granularity in the first slot in Phase A/B could be smaller than the remaining slot(s) in Phase C. The PDCCH monitoring patterns can be semi-statically configured.

Proposal 2: UE can determine the transitions between different phases associated with different PDCCH monitoring efforts implicitly according to:

- From Phase A (outside of gNB’s COT) to Phase B (beginning of gNB’s COT) based on DL burst detection.

- From Phase B to Phase C (remainder of gNB’s COT) based on the end boundary of the first partial slot, or the start of the first full slot.

- From Phase C to Phase A based on the length or end position of the COT.

In the current Rel-15 specification, UE needs to perform the same PDCCH monitoring pattern configured by RRC until it is re-configured. In order to have different PDCCH monitoring patterns for a COT, some enhancements for PDCCH monitoring configuration should be considered. Pre-defined or semi-static configuration method can be considered to achieve the two PDCCH detection patterns above. However, for semi-static configuration, as the slot detection PDCCH is an absolute slot index configured through search space configuration in monitoringSlotPeriodicityAndOffset, it may not be the first slot of the COT. Therefore, the slot parameter defined in monitoringSlotPeriodicityAndOffset should be changed to the slot position in the COT, or the slot index should be started from the first slot of the COT.

Proposal 3: In NR-U, the slot index indicated in parameter monitoringSlotPeriodicityAndOffset of the search space should be defined relative to the starting slot of the COT.

UE COT detection for FBE

Frame Based Equipment (FBE) is specified in ETSI  EN 301 893 [6] as equipment where the transmit/receive structure has a periodic timing with a periodicity equal to the Fixed Frame Period, as shown in Figure 2. A single Observation Slot in Idle Period shall have a duration no less than 9 us.

Figure 2: Timing for Frame Based Equipment

The Fixed Frame Periods supported by the equipment shall be declared by the manufacturer. This shall be within the range of 1 ms to 10 ms. Transmissions can start only at the beginning of a Fixed Frame Period. An equipment may change its Fixed Frame Period but it shall not do more than once every 200 ms. Immediately before starting transmissions on an operating channel at the start of a Fixed Frame Period, the initiating device shall perform a CCA check during a single Observation Slot. The COT shall not be greater than 95 % of the Fixed Frame Period and shall be followed by an Idle Period until the start of the next Fixed Frame Period such that the Idle Period is at least 5 % of the Channel Occupancy Time, with a minimum of 100 us.

In some scenarios, such as shopping malls, factories, office buildings, FBE (frame based equipment) is a better type of equipment if no other type of equipment exists, as it is beneficial for frequency reuse. From above definition and requirements of FBE, we can see that the channel access mechanism of FBE is based on a fixed frame structure. Therefore, compared with LBE, UE COT detection for FBE is relatively simple.

- In Phase A (outside of gNB’s COT), UE can do no COT detection (including DMRS and PDCCH) to save the power until Fixed Frame Period starts. UE only needs to start detecting DMRS and GC-PDCCH at the beginning of Fixed Frame Period. UE determines the transition from Phase A to Phase B (beginning of the COT) based on DL burst detection (i.e. DMRS). Once it detects an DMRS, UE starts to monitor PDCCH and other RSs/channels. In order to increase spectrum efficiency and reduce impacts on PDSCH SLIV, the beginning of Fixed Frame Period can be aligned with slot boundary. With this solution, UE can perform same PDCCH monitoring efforts in Phase B and in Phase C (remainder of the COT). The transition from Phase C to Phase A can depends on the length or end position of the COT, which can be indicated by DCI format 2_0 to be discussed in Section 2.4.

Proposal 4: For FBE, the beginning of Fixed Frame Period can be aligned with slot boundary to increase spectrum efficiency and reduce impacts on PDSCH mapping.

Proposal 5: For FBE, UE can make no DMRS and PDCCH detection efforts in Phase A and perform same PDCCH monitoring granularity in Phase B and Phase C.

COT Structure Indication

In licensed band, Rel-15 NR supports dynamic and semi-static slot configuration with the period of {0.5, 0.625, 1, 1.25, 2, 2.5, 3, 4, 5, 10} ms.  The semi-static DL/UL assignment is done via cell-specific and UE-specific RRC configuration. With cell-specific configuration, a UE is allocated a DL-F-UL pattern. The "F" resources are flexible resources that can be assigned DL or UL direction using UE-specific configuration or dynamic signaling. The UE-specific RRC signaling or dynamic signaling includes the indication as per slot basis that can override the "F" allocation in the cell-specific configuration. Resources in the slot configuration that are not indicated as DL/UL remain flexible resources. For NR-U, dynamic and semi-statically configured UL/DL configuration as in licensed band should also be considered. For example, some semi-static UL/DL assignments can be used to indicate DL slots for UE to receive SSB, and some other slots can be assigned for UE to transmit PRACH or configured grant transmission.

Rel-15 NR PDCCH can be configured to start from any symbol, and the gNB can start downlink transmission from the nearest symbol after a successful LBT, such as in the middle of the slot transmission through the search space configuration. Therefore, once the gNB performs LBT successfully on the semi-statically configured DL slot and symbols, it can dynamically indicate the COT structure via DCI format 2_0. Besides, DCI format 2_0 can indicate multiple slots and the partial slot structure. In addition to the slot structure, DCI format 2_0 can also include the following information: duration or end time of COT, COT sharing information. The information of duration or end time of the COT can be beneficial for UE to determine the transitions between different phases associated with different PDCCH monitoring efforts. COT sharing information is used to indicate whether UL transmission (including CG) can share the current COT when fall in gNB COT and LBT type (e.g. Cat 1 or Cat 2) if COT sharing is allowed.

Proposal 6: In addition to the slot structure, DCI format 2_0 can also include the following information: duration or end time of COT, COT sharing information.

PDSCH Transmission

For PDSCH transmission, additional durations from 2 to 13 symbols (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B has been supported in RAN1 #97 meeting, and which duration are supported depend on UE’s capability. Capability signaling will be defined for UEs to indicate which specific subset of durations is supported by the UE. Meanwhile, below four FFSs still needs to be discussed.

- FFS: A duration of 14

- FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

For the first FFS of PDSCH duration of 14 symbols, considering a gap is needed for performing CCA and type A PDSCH has already supported 14 symbols, we do not think that a duration of 14 symbols for PDSCH mapping type B is necessary to be supported.

In RAN1 #98 meeting, RAN1 has defined DMRS positions for PDSCH mapping Type B of length 9 and 10. For the other additional durations other than length 9/10, reusing Rel-15 existing DMRS patterns may be sufficient, such as to reuse the PUSCH mapping type B DMRS pattern with the same duration, or reuse the PDSCH mapping type A DMRS pattern with the same duration.

According to the analysis in our companion contribution [7], at least {S=1, L=6}, {S=2, L=4}, {S=2, L=5}, {S=2, L=12} with PDSCH mapping type A in the existing default PDSCH SLIV table can be used for scheduling RMSI-PDSCH associated with the first SSB in a slot, and at least {S=8, L=4} with PDSCH mapping type B in the existing SLIV table can be used for scheduling RMSI-PDSCH associated with the second SSB in the slot, if Alt.1 pattern for SSB and type-0 PDCCH is adopted. Therefore, it seems that there is no need to support other mandatory durations for initial access.

For processing times of PDSCH mapping type B with the additional durations, it can reuse the values of durations of 2/4/7 symbols as specified in chapter 5.3 in TS 38.214, which can have little specification impact. For example, the new durations with greater than 7 symbols can use the same d1,1 in chapter 5.3 as 7, the durations of 5/6  symbols can use the same d1,1 as 4, and the duration of 3 symbols can use the same d1,1 as 2 or 4. Based on this, we don’t think that introducing new durations will lead to an increased requirement in PDSCH processing and decoding capabilities, and thus recommend reusing the values for durations of 2/4/7.

Proposal 7: The following points for additional durations for PDSCH  mapping type B can be considered:

- A duration of 14 symbols for PDSCH mapping type B is not necessary to be supported.

- For the other additional durations other than length 9/10, DMRS positions for PDSCH mapping type B can reuse DMRS patterns of PUSCH mapping type B or PDSCH mapping type A with the same duration in Rel-15.

- No need to support other mandatory durations for initial access.

- Processing times of PDSCH mapping type B with the additional durations can reuse the values of durations of 2/4/7 symbols as specified in chapter 5.3 in TS 38.214.

CSI-RS

Rel-15 NR supports a CSI-RS with 1, 2 and 4 OFDM symbols and up to 32 antenna ports. The functionalities of CSI-RS include mobility support, CSI acquisition, L1 RSRP computation and tracking. CSI-RS can be sent as a NR-U DRS component together with SS/PBCH block, type-0 PDCCH and RMSI PDSCH. How CSI-RS and SS/PBCH block multiplex in time/frequency domain please refer to companion contribution [7].

Similar to LTE, Rel-15 NR also supports periodic, aperiodic and semi-persistent CSI-RS transmission. For NR-U, due to the requirement of LBT, aperiodic CSI-RS can well match the characteristics of opportunistic transmission in unlicensed spectrum. It can be triggered to be sent after gNB obtains a COT. Periodic/semi-persistent CSI-RS may fail to be sent at a time point according to semi-static configuration due to the failure of LBT. However, considering periodic/semi-persistent CSI-RS is conductive to signaling overhead reduction and channel state information maintenance, periodic/semi-persistent CSI-RS should also be supported even if their transmission can not be always guaranteed due to LBT. RAN2 has agreed that RAN2 can assume that missing measurements due to LBT failures do not impact the RAN2 specification of L3 filtering and the subsequent steps. We don’t think there is anything that needs to be specially defined for behavior when P/SP-CSI-RS fail to transmit due to LBT.

Proposal 8: There is nothing that needs to be specially defined for behavior when P/SP-CSI-RS fail to transmit due to LBT.

DMRS sequence generation

The sequence generation of PDSCH/PDCCH DMRS in Rel-15 NR depends on the absolute slot/symbol index. The pseudo-random sequence generator for PDSCH/PDCCH DMRS shall be initialized with

(for PDSCH DMRS)

cinit=217Nsymbslotns,fμ+l+12NID+1+2NIDmod231                   (for PDCCH DMRS)

where  is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame.

In NR-U, the gNB needs to perform LBT first before PDSCH/PDCCH transmission. If LBT succeeds, the gNB can start PDCCH/PDSCH transmission. But if LBT fails, the gNB should continue performing LBT for the subsequent slot/mini-slot transmissions and regenerate absolute slot/symbol index dependent PDSCH/PDCCH DMRS sequence in advance, or regenerate it in a short time after LBT is successful. Some companies believed that the implementation complexity can increase if the existing DMRS generation mechanism is reused for NR-U and a potential solution is that the slot/symbol number for generating DMRS in the first slot of the COT can be set to 0, and the slot number in the remaining slots of the COT are relative to the first slot. However, in our view, the increased complexity would be very minor, and the existing implementation of the gNB can support the generation of one or more absolute slot/symbol-index dependent PDSCH/PDCCH DMRS sequences in advance. Moreover, there may be some problems if the solution mentioned above is adopted. If a UE misses detection of wideband DMRS as the initial signal at the start of DL burst, it would miss the entire gNB initiated COT. To solve that, wideband DMRS may be supported and has to be transmitted multiple times in a COT. Further, if above DMRS sequence generation option is applied, the first DMRS for indicating DL burst uses slot index 0 to generate its sequence, and the following DMRSs will use the relative slot/symbol index. If UE loses the first DMRS, it will  leads to a chain effect, e.g., losing all the following DMRSs and other remaining transmissions from the gNB. Based on this, even if sending DMRS multiple times in a COT is supported , it cannot also improve robustness of DL burst detection. Therefore, no change to DMRS sequence generation is needed.

Proposal 9: No change to DMRS sequence generation is needed.

Conclusion

In this contribution, we discuss some potential solutions and techniques for NR-U DL reference signal design and physical channel design, and have the following observation and proposals:

Observation 1: For RRC_IDLE UEs, PDCCH scrambled by P-RNTI at a given paging occasion can be used for UEs to judge DL burst detection and/or decide to stop the monitoring of the additional PDCCH monitoring occasions for power saving.

Proposal 1: Different PDCCH monitoring patterns can be performed for Phase A (outside of gNB’s COT), Phase B (beginning of gNB’s COT) and Phase C (remainder of gNB’s COT). The PDCCH monitoring occasion granularity in the first slot in Phase A/B could be smaller than the remaining slot(s) in Phase C. The PDCCH monitoring patterns can be semi-statically configured.

Proposal 2: UE can determine the transitions between different phases associated with different PDCCH monitoring efforts implicitly according to:

- From Phase A (outside of gNB’s COT) to Phase B (beginning of gNB’s COT) based on DL burst detection.

- From Phase B to Phase C (remainder of gNB’s COT) based on the end boundary of the first partial slot, or the start of the first full slot.

- From Phase C to Phase A based on the length or end position of the COT.

Proposal 3: In NR-U, the slot index indicated in parameter monitoringSlotPeriodicityAndOffset of the search space should be defined relative to the starting slot of the COT.

Proposal 4: For FBE, the beginning of Fixed Frame Period can be aligned with slot boundary to increase spectrum efficiency and reduce impacts on PDSCH mapping.

Proposal 5: For FBE, UE can make no DMRS and PDCCH detection efforts in Phase A and perform same PDCCH monitoring granularity in Phase B and Phase C.

Proposal 6: In addition to the slot structure, DCI format 2_0 can also include the following information: duration or end time of COT, COT sharing information

Proposal 7: The following points for additional durations for PDSCH  mapping type B can be considered:

- A duration of 14 symbols for PDSCH mapping type B is not necessary to be supported.

- For the other additional durations other than length 9/10, DMRS positions for PDSCH mapping type B can reuse DMRS patterns of PUSCH mapping type B or PDSCH mapping type A with the same duration in Rel-15.

- No need to support other mandatory durations for initial access.

- Processing times of PDSCH mapping type B with the additional durations can reuse the values of durations of 2/4/7 symbols as specified in chapter 5.3 in TS 38.214.

Proposal 8: There is nothing that needs to be specially defined for behavior when P/SP-CSI-RS fail to transmit due to LBT.

Proposal 9: No change to DMRS sequence generation is needed.

References

- 3GPP, RP-182878, New WID on NR-based access to unlicensed spectrum, Qualcomm, RAN#82, December, 2018.

- 3GPP, Draft_Minutes_report_RAN1 AH_1901_v1.0, January, 2019

- 3GPP, Draft_Minutes_report_RAN1 #97_v1.0, May, 2019

- 3GPP, Draft_Minutes_report_RAN1 #98_v1.0, August, 2019

- 3GPP, RP-191581, Guidance on essential functionality for NR-U, Qualcomm, RAN#84, June 2019

- 3GPP, ETSI EN 301 893 V2.1.1, Harmonized European Standard, 5GHz RLAN, May, 2017

- 3GPP, R1-1909971, Considerations on initial access signals and channels for NR-U, ZTE, October 2019