3GPP TSG RAN WG1 meeting #98bis						 	R1-1909981

Chongqing, China, October 14 - 20, 2019

Agenda Item: 7.2.5.1

Source: MediaTek

Title: System level performance for NR-NTN phase 1 calibration

Document for:	Discussion and decision

# Introduction

For NR NTN, potential impacts have been identified in TR 38.811 “Study on NR to support non-terrestrial networks”. Solutions are currently being investigated in RAN1 meeting and captured in TR 38.821 “Solutions on NR to support non-terrestrial networks”. For NTN, RAN1 TUs have been allocated starting from April 2019 to November 2019.

In RAN1#97 meeting, agreements on the following items are given in [1]

- Satellite parameters for system level simulations

- UE characteristics for system level simulations

- Beam layout definition

In RAN1#98, the parameters regarding above items have been updated [2] and captured in TR 38.821 [3]. In addition, RAN1#98 has identified the following as phase 1 calibration cases

| Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency reuse factor |
| --- | --- | --- | --- | --- | --- |
| GEO | Set 1 | 45 deg | VSAT | Ka-band | 1 |
| GEO | Set 1 | 45 deg | VSAT | Ka-band | 3 |
| LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | 1 |
| LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | 3 |
| LEO-600 | Set 1 | 90 deg | Handheld | S-band | 1 |
| LEO-600 | Set 1 | 90 deg | Handheld | S-band | 3 |
| LEO-1200 | Set 1 | 90 deg | Handheld | S-band | 1 |
| LEO-1200 | Set 1 | 90 deg | Handheld | S-band | 3 |

In this contribution, we show our simulation results on SINR and DL throughput distribution for [LEO-600, S-band] and [LEO-1200, S-band]. Detail CDF numbers on SINR, SIR and coupling loss are given in the companion excel sheet. Based on the simulation results, in order to improve system spectral efficiency, we propose that the concept of bandwidth part (BWP) can be used in frequency domain resource allocation of beams.

# Simulation Results

For calibration, the following settings have been agreed for SLS simulation:

- Ionospheric scintillation loss = 0 (i.e., the UEs are located between 20 and 60 degrees of latitude)

- All UE-satellite links are in LOS channel condition

- For frequency reuse factor (FRF) = 1, two additional tiers beams are considered in the simulation for wrapping around the 19-beam layout

- For FRF > 1, four additional tiers beams are considered in the simulation for wrapping around the 19-beam layout

- Only the UEs in the inner-19 beams are considered

As such, for scenarios with frequency reuse factor (FRF) = 1, total 61 beams are formed. The beam layout and UE dropping for [LEO-600, S-band] and [LEO-1200, S-band] with FRF = 1 are shown below:

Figure 1: Beam layout and UE dropping for [LEO-600, S-band] and [LEO-1200, S-band] with FRF = 1

For scenarios with FRF = 3, total 127 beams are formed. The beam layout and UE dropping for [LEO-600, S-band] and [LEO-1200, S-band] with FRF = 3 are shown below:

Figure 2: Beam layout and UE dropping for [LEO-600, S-band] and [LEO-1200, S-band] with FRF = 3

Note that we show beam layout in 2-D dimension for simplicity. Also note that we drop 20 UEs per beam for LEO-600 and 30 UEs per beam for LEO-1200.

From SLS we obtain SINR distribution. For DL throughput distribution, we perform SLS + LLS simulation to obtain the results. Specifically, for each UE, SLS generates SNR and channel information for serving and neighbour beams, which in turn are used as input to LLS for modelling data reception with interference. LLS settings are given in Appendix.

In the following, we show SINR and DL throughput distribution for [LEO-600, S-band] and [LEO-1200, S-band]:

Figure 3: SINR and DL throughput distribution for [LEO-600, S-band]

Figure 4: SINR and DL throughput distribution for [LEO-1200, S-band]

See attached excel sheet for detailed CDF numbers on SINR, SIR and coupling loss.

We have the following statistics:

| LEO-600 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| FRF 1 | 80%UE: -1.087 Avg: -1.989 | 80%UE: 10.52 Avg: 8.24 |
| FRF 3 | 80%UE: 7.513 Avg: 5.794 | 80%UE: 10.88 Avg: 8.15 |

Table 1: SINR and throughput statistics for [LEO-600, S-band]

| LEO-1200 | SINR (dB) | Throughput (Mbps) |
| --- | --- | --- |
| FRF 1 | 80%UE: -1.581 Avg: -2.42 | 80%UE: 9.096 Avg: 7.49 |
| FRF 3 | 80%UE: 6.658 Avg: 5.19 | 80%UE: 9.712 Avg: 7.52 |

Table 2: SINR and throughput statistics for [LEO-1200, S-band]

Observation 2: For [LEO-1200, S-band], for 80% percentile UE, we have 8.24 dB SINR gain by using FRF = 3 instead of FRF = 1 deployment. In average, we have 7.61 dB SINR gain by using FRF = 3 in comparison with FRF = 1.  Again, there is no significant difference between throughput distributions of FRF = 3 and FRF = 1

Observation 1: For [LEO-600, S-band], for 80% percentile UE, we have 8.6 dB SINR gain by using FRF = 3 instead of FRF = 1 deployment. In average, we have 7.78 dB SINR gain by using FRF = 3 in comparison with FRF = 1.  However, there is no significant difference between throughput distributions of FRF = 3 and FRF = 1

From the simulation results, we have the following observations:

# BWP for Frequency Domain Resource Allocation of Beams

Observation 3: For [LEO-1200, S-band], with FRF 1, approximately 68% of UEs are with SINR ≤-2 dB. These UEs are unlikely to get much throughput due to poor synchronization, AFC, and channel estimation. In addition, such UEs may suffer high latency if many HARQ re-transmissions are required

For NTN beam deployment with FRF 1, the available bandwidth allocated to each beam is large, however a UE in a beam coverage may suffer severe interference due to co-channel signals transmitted to nearby beams. From simulation results, we have the following observation

On the other hand, for NTN beam deployment with FRF 3, the interference problem is mitigated. However, the available bandwidth allocated to each beam is generally reduced.

Proposal 1: The concept of BWP can be used for frequency resource allocation among NTN beams. Network may configure a specific active BWP for UEs in a beam (all UEs in a beam are associated with the same active BWP). Different beams may be associated with different active BWPs

In practice, the number of UEs in each beam is not a constant. Intuitively, the network should allocate larger bandwidth for beams with larger number of UEs, and allocate smaller bandwidth for the other beams. Considering that the number of UEs in a beam may vary with time, a dynamic/scalable frequency resource allocation scheme is expected to improve the system spectral efficiency. Thus, we have the following proposal:

See Figure 5 for an illustration:

- In this example, there are 7 beams, and the network may configure BWP 0,1, or 2, as an active BWP for a beam

- In bandwidth, we have BWP 0 > BWP 1 > BWP 2

- Suppose at a time instance, the number of UEs in each beam is as shown in the table, then the network may configure BWPs to beams according the number of UEs in beams as shown in the right column of the table

- Note that BWP 0 is allocated for beam 4 since beam 4 has maximum number of UEs

- Note also that the interference is mitigated as nearby beams are using non-overlapping BWPs

Figure 5: Example of BWP Allocation for Beams

In general, the active BWPs of beams may or may not overlap with each other in frequency domain.

Furthermore, for a beam, the bandwidth and the frequency domain location of the active BWP may not be fixed at all time. The network may adjust the settings of the active BWP based on factors such as number of UEs and amount of data transmission in the beam.

# Conclusions

The observations and proposals are summarized below:

Observation 1: For [LEO-600, S-band], for 80% percentile UE, we have 8.6 dB SINR gain by using FRF = 3 instead of FRF = 1 deployment. In average, we have 7.78 dB SINR gain by using FRF = 3 in comparison with FRF = 1.  However, there is no significant difference between throughput distributions of FRF = 3 and FRF = 1

Observation 2: For [LEO-1200, S-band], for 80% percentile UE, we have 8.24 dB SINR gain by using FRF = 3 instead of FRF = 1 deployment. In average, we have 7.62 dB SINR gain by using FRF = 3 in comparison with FRF = 1.  Again, there is no significant difference between throughput distributions of FRF = 3 and FRF = 1

Observation 3: For [LEO-1200, S-band], with FRF 1, approximately 68% of UEs are with SINR ≤-2 dB. These UEs are unlikely to get much throughput due to poor synchronization, AFC, and channel estimation. In addition, such UEs may suffer high latency if many HARQ re-transmissions are required

Proposal 1: The concept of BWP can be used for frequency resource allocation among NTN beams. Network may configure a specific active BWP for UEs in a beam (all UEs in a beam are associated with the same active BWP). Different beams may be associated with different active BWPs

# Reference

- Chairman's Notes RAN1#97

- Chairman's Notes RAN1#98

- R3-194796, TR 38.821, “Solutions for NR to support non-terrestrial networks”

# Appendix

LLS simulation settings

| Carrier frequency | 2GHz |
| --- | --- |
| Subcarrier spacing | 15kHz |
| DL bandwidth for each beam | 30 MHz for FRF = 1 10 MHz for FRF = 3 |
| UE speed | 3 km/hr |
| UE antenna configuration | 1 Rx |
| UE channel estimation | Realistic MMSE |