3GPP TSG RAN WG1 Meeting #98bis	R1-1910046

Chongqing, China, October 14 – 20, 2019

Agenda Item:	7.2.2.2.2

Source:	Huawei, HiSilicon

Title:	Initial access procedure in NR unlicensed

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss remaining issues on QCL acquisition from SSB/DRS in section 2 based on the agreement in RAN1#98. The enhancements on RACH procedure were proposed in section 3. The question on additional PDCCH monitoring occasions for paging received from RAN2 was discussed in section 4. Finally in section 5, the mechanisms for UE to exclude missing RS due to LBT failure in RRM/RLM procedures were studied.

Remaining issues on NR-U DRS transmission procedures

QCL determination

In RAN1#98[1], the following agreements related to QCL determination were achieved.

Agreement:

For purposes of SSB QCL derivation, the following values of Q are supported: {1, 2, 4, 8}.

- FFS: Further down-selection of allowed values.

Conclusion:

The maximum number of PBCH DMRS sequences used in a cell is unchanged from Rel-15 (=8).

- The number of PBCH DMRS sequences used in a cell is independent of Q.

Agreement:

The 3 LSB bits of the SSB candidate position index are represented by the PBCH DMRS sequence index.

Agreement:

For a cell (either serving or a neighbour cell), UE may assume a QCL relation between SS/PBCH blocks within or across DRS transmission or measurement windows that have the same value of modulo(A, Q), once Q is known to the UE

- A is the PBCH DMRS sequence index.

- Note: This agreement extends a prior agreement for serving cells on QCL relation between SS/PBCH blocks to neighbour cells

Agreement:

If Q is known, candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present

- Note: Q may be always known depending on where Q is signalled. This aspect is to be discussed further.

According to the agreement, the value Q is the minimum difference between PBCH DMRS sequence indices of 2 SSB with same QCL assumption. In RAN1#98, there was discussion on the necessity of flexible configuration of Q. For example, one option is that UE always assumes Q=8 which can help simplify UE implementation when doing PSS combination across DRS transmission window.  From our point of view, Q should be configured according to the number of actual transmitted SSB/beam. It is more efficient than fixing Q=8 when gNB transmits DRS with small number of SSB/beam. For example as illustrated in Figure 1, gNB expects to transmit a DRS with 2 beams (corresponding to blue and green SSB). If gNB cannot transmit on the SSB candidate position 0/0 due to LBT failure, there will be a 3ms gap between 2 transmitted SSBs when both gNB and UE always assume Q=8 (Figure 1 (a)). It is not efficient for gNB to either transmit reservation signal to maintain the channel occupancy or perform another LBT before candidate position 0/1. On the other side, if there is mis-match of Q between gNB and UE, the accuracy of RRM/RLM measurement on SSB will be compromised due to less samples detected (Figure 1(b)). Depending on the payload size and MCS of RMSI/OSI/Paging, gNB could transmit one SSB or two SSB within a slot. Q should be same as the number of transmitted SSB if gNB is going to transmit SSB on both of SSB candidate positions in a slot. Q is two times of the number of transmitted SSB if at most one SSB is transmitted in a slot.

Observation 1: Q is the minimum difference between PBCH DMRS sequence indices of 2 SSB with QCL assumption. Q should be integer times of the number of transmitted SSB depending on the number of SSB transmitted per slot. There is no need to further down select the allowed Q values.

Figure 1 DRS with 2 beams and UE always assume Q=8

Q value will be used by UE to acquire QCL among SSB across DRS transmission windows. When Q value is given in MIB, it is possible for initial accessing UE to just buffer Type0-PDCCH and RMSI PDSCH QCLed with the selected SSB for soft combining as illustrated in Figure 2(a). The buffer size can be reduced significantly compared with receiving signals from whole DRS transmission window as illustrated in Figure 2(b). Considering only four Q values allowed in NR-U, at most 2 bit is necessary. It can reuse MIB bits saved from other fields, e.g. pdcch-ConfigSIB1. As for beam-based RRM measurement for neighboring cell, Q value reflecting actual transmitted SSB in the neighboring can help improve measurement accuracy. For connected UE, it can be carried in the RRC signaling.

Proposal 1: gNB could indicate Q value to UE through MIB for serving cell UE. gNB could indicate Q value through RRC signaling for neighboring cell measurement.

Figure 2 Different soft buffer requirement depending on where Q is indicated in MIB (Q=4)

SSB and CORESET0 in DRS

The motivation of introducing DRS in NR-U is to design channels and signals as compact as possible. The gap among signals and channels in a DRS burst should be minimized which allows gNB to finish DRS burst transmission with single LBT. The signals and channels with same QCL assumption should be located close together in order to avoid beam switching back and forth. It is also facilitate gNB to shift SSB in time domain in case of LBT failure.

Proposal 2: gNB should transmit SSB and its associated CORESET0 in the same slot.

According to NR Rel-15, the monitoring occasion of Type0-PDCCH CSS set is over two consecutive slots starting from slot n0 in SFNC.. For SSB in SSB candidate position with index i, the UE determines an index of slot n0 as  located in a frame with SFN satisfying  if  or in a frame with SFN satisfying  if . Table 13-11 in TS38.213 [2] provides all combinations of parameter M and O corresponding to SSB and CORESET0 multiplexing pattern 1， also illustrated in Figure 3.

Figure 3  SFN and slot mapping of Type-0 PDCCH monitoring occasion

It is observed that only two entries with index=1(O=0, M=1/2) and index=5 (O=5, M=1/2) can meet the requirement if DRS burst is transmitted in the 1st or 2nd half frame in a frame with even SFN. The DRS transmission periodicity should be also in the unit of 20ms. Either introducing additional offset values of O=10 and O=15 or defining PDSCH monitoring occasions explicitly with the SFN and slot index of the associated SSB can be considered.

According to the current agreement for NR-U, gNB could only transmit on SSB candidate positions with even PBCH DMRS sequence indices when the associated Type0-PDCCHs and RMSI PDSCHs are occupying full slot length and are in the same slot as the SSB. It is impossible for gNB to transmit 8-beam DRS because only 4 different PBCH DMRS sequence indices are available, as illustrated in Figure 4(a). If at most 8 beams same as NR Rel-15 is expected in NR-U, gNB could transmit SSB on the SSB candidate positions with even PBCH DMRS sequence indices in the 1st ~ 4th and 9th ~ 10th slot in the DRS transmission window. Otherwise, SSB candidate positions with odd PBCH DMRS sequence indices can be used in the 5th to 8th slot. It is illustrated in Figure 4(b). Thus, it is necessary to support one search space set per slot when M=1/2. And association between odd SSB candidate position index and first symbol index equal to 0 should be allowed. The new entries to support receiving SSB and its associating CORESET0 in the same slot are summarized in Table 1 by means of (a) M/O parameters or (b) explicit SFNC and nC (originally used for pattern 3). It was also discussed in our companion contribution [3].

Proposal 3: In order to support receiving SSB and associated CORESET0 in the same slot, the entries in Table 1 should be considered for pdcch-ConfigSIB1in MIB.

Figure 4 SSB candidate transmission positions

Table 1  PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 in unlicensed band

- New/updated entries by M/O parameters

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| New | 0 | 1 | 1/2 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} , or {0, if is even}, {6, if is odd} if Alt 1 , or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 5 | 1 | 1/2 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} , or {0, if is even}, {6, if is odd} if Alt 1 , or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 10 | 1 | 1/2 | 0 |
| New | 10 | 2 | 1/2 | {0, if is even}, {, if is odd} , or {0, if is even}, {6, if is odd} if Alt 1 , or {0, if is even}, {7, if is odd} if Alt 2 |
| New | 15 | 1 | 1/2 | 0 |
| New | 15 | 2 | 1/2 | {0, if is even}, {, if is odd} , or {0, if is even}, {6, if is odd} if Alt 1 , or {0, if is even}, {7, if is odd} if Alt 2 |

- New entries with by SFNC and nC.

| Index | PDCCH monitoring occasions (SFN and slot number) | Number of search space sets per slot | First symbol index |
| --- | --- | --- | --- |
| new |  | 1 | 0 |
| new |  | 2 | {0, if is even}, {, if is odd} , or {0, if is even}, {6, if is odd} if Alt 1 , or {0, if is even}, {7, if is odd} if Alt 2 |

Length of DRS transmission

According to ETSI regulation [4], short control signaling could be transmitted without CAT 4 LBT if the number of transmission is less than 50 times and the total transmission duration is less than 2500 µs within an observation period of 50 ms.

As discussed in TR 38.889 [5], CAT 2 LBT could be used for DRS transmission only when the DRS duty cycle ≤ 1/20, and the total duration is up to 1 ms. Hence at maximum 4 SSB multiplexed with RMSI could be transmitted within 2 slots (1 ms).

Observation 2: When CAT 2 LBT is used before DRS transmission, the maximum allowed transmission duration of DRS is 1ms.

gNB could transmit SSBs with different QCL assumption on multiple of SSB candidate positions in a DRS window. gNB might also shift the SSB transmission among the SSB candidate positions in case of LBT failure. The longer the DRS transmission window, the higher probability gNB can successfully transmit DRS. The resource on the SSB candidate positions not transmitted can be rescheduled to other downlink channel in order to improve resource utilization. The fix length of DRS transmission window could simplify signaling design

Proposal 4: gNB should fix DRS transmission window as Y=20 for 30 kHz SCS and Y=10 for 15k Hz SCS.

Indication of RMSI/OSI/paging in DRS

At SI stage, the transmission of OSI and paging within the NR-U DRS was agreed. It is beneficial to reduce the required number of channel access attempts and better satisfy the OCB requirement in NR-U. From another perspective, it is also not desirable to always consist of all these channels/signals in the NR-U DRS due to limited resources available for the DRS transmission.

In NR, PBCH within SS/PBCH block can indicate the multiplexing pattern between SS/PBCH block and RMSI CORESET. When additional signal (i.e. OSI, paging) may also be included in DRS, it is natural to extend the configuration information in PBCH to carry the indication of inclusion for additional signal and the multiplexing pattern between SS/PBCH block and those included signals in DRS.

Proposal 5: PBCH within SS/PBCH block can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

PDSCH rate-match around transmitted SSB

In NR Rel-15, a same bitmap (8 bit for FR1 above 3GHz) of ssb-PositionsInBurst is carried in SIB1 and ServingCellConfigCommon which indicates the time domain positions of the transmitted SS-blocks in a half frame. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. When UE receiving the PDSCH NOT scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources.

In NR-U, there are Y=20 candidate positions for 30 kHz SSB and Y=10 candidate positions for 15 kHz SCS and gNB can transmit at most on 8 of the candidate positions. Simply extend bitwidth of ssb-PositionsInBurst similar as FR2 cannot work because gNB cannot update ssb-PositionsInBurst as frequently as LBT outcome varies. UE could acquire the actual transmit SSB positions base on both semi-static high layer signalling and dynamic physical signalling. For example, each bit in the field of ssb-PositionsInBurst could correspond to a set of SSB candidate positions with same QCL assumption. The system information indicator (1bit) can be carried in DCI scheduling PDSCH. UE determine the scheduled PDSCH is rate-matched by transmitted SSBs if both the system information indicator in scheduling DCI and the bit in ssb-PositionsInBurst corresponding to SSB candidate positions overlapped with the PDSCH are “1”.

Proposal 6: UE determines whether the scheduled PDSCH is rate-matched by transmitted SSB based on both higher layer signaling ssb-PositionsInBurst and system information indicator in DCI.

NR-U Physical RACH (PRACH)

Resource allocation of PRACH

In NR Rel-15, gNB can semi-statically configure one or multiple PRACH occasions by RMSI. The configuration parameters include periodicity, preamble format, available SFN number, slot number and PRACH occasion number within a PRACH slot etc. In NR-U, the transmission of PRACH may be blocked by uncertain LBT failure, semi-statically configured SFN number, slot number may become un-available. Thereby, a dynamic PRACH resource allocation scheme should be complementally considered. In TR38.889 [5], several dynamic resource allocation alternatives were given as following:

-	Alt.1: For connected mode UE, scheduling of PRACH resources via DCI.

-	Alt.2: For idle mode UE, scheduling of PRACH resources via paging

-	Alt.3: New RACH resources are used immediately following detection of DRS transmission

In Alt.1, scheduling of PRACH is triggered by DCI, e.g. a group common PDCCH. The scheduling information at least includes time and frequency resource allocation for PRACH. This trigger method is mainly applicable for connected mode UE. For idle mode UE, Alt.2 and Alt.3 can both be considered. While for Alt.2, a single paging resource may belong to a tracking area consisting of multiple cells. Thereby, potential collision among PRACH resources of UEs that belong to different cells will occur. If this issue can’t be addressed, Alt.2 will not be an attractive scheme for idle mode UE. In Alt.3, additional RACH resource can be immediately followed within DRS duration after of detection of DRS transmission. It can avoid new LBT procedure for additional PRACH resource, thereby more efficient PRACH   transmission opportunities can be clearly foreseen.

Based on the above analysis, we propose that Alt.1 should be supported at least for connected mode UE. And Alt.3 can be considered as a supplementary scheme for idle mode UE.  Furthermore, for Alt.1, to reduce signaling overhead, only parameters subject to LBT results need to be configured in DCI, e.g. starting OFDM symbol and frequency resources etc.  And the other parameters including PRACH periodicity, format and RO number per PRACH slot can still be acquired by RMSI.

Proposal 7: Semi-static resource allocation of PRACH supported in NR could be the baseline of resource allocation design of PRACH in NR-U.

Proposal 8: Dynamic scheduling of PRACH occasion via DCI (i.e. Alt.1) and via DRS (i.e. Alt.3) shall be supported in NR-U.

In NR Rel-15, an association between RACH resource(s) and SS/PBCH block(s) is established. In NR-U, actual transmission of one SSB is associated with multiple SSB candidate positions to overcome uncertain LBT results. For Alt.2 (i.e. cyclical wrapping’s method), it is natural to map multiple candidate SSB positions in different cyclically wrapped periodicity to same RO. For example, SSB(s) which satisfy SSB candidate position(s) % X = k will be all mapped to the k-th RO in RO pool.

In Rel-15, open loop power parameters for PRACH are semi-statically configured by RMSI. Different power parameters are needed for ROs associated with different SS/PBCH blocks (i.e. different large scale fading).While in NR-U, an extra RO can be dynamically scheduled via DCI. Thereby, an extra power parameter should be accordingly updated in the new scheme. To achieve this, a more flexible power control scheme for RACH except for semi-static manner should be studied in NR-U.

Proposal 9: Dynamic power allocation for PRACH should be supported in NR-U.

In NR Rel-15, multiple Msg.1 transmission before RAR reception had been extensively discussed and agreed for CFRA. While due to limited time budget for detailed design, it was not be captured in specification.

In TR38.889 [5], multiple Msg1 transmissions before RAR reception and group wise SSB-to-RO mapping in time domain had been captured as alternative of PRACH enhancement schemes. In NR-U, when RACH retransmission occurs, the access delay of the RACH procedure may be seriously deteriorated. As shown in Figure 5, reduced access delay can be achieved by multiple Msg.1 transmissions, wherein duplicated Msg.1 transmissions plus corresponding response window can be constrained into a COT. Thereby, it is beneficial to support multiple Msg.1 transmission in NR-U.

In addition, multiple beam sweeping is not necessary and lower transmission power (i.e.18dBm) is assumed in unlicensed band, thereby, the interference issue due to multiple RACH transmissions in licensed band will be significantly alleviated.

Proposal 10: Multiple Msg1 transmissions for both CBRA and CFRA should be supported in NR-U.

Figure 5 Illustration of multiple Msg1 transmission

In licensed band, a UE attempt to detect a DCI with CRC scrambled by RA-RNTI within a RAR window corresponding to the RO where a PRACH is transmitted. Considering that multiple ROs within a group are actually associated with the same UE/preamble index. When multiple Msg1 transmissions before RAR reception were supported, a common RNTI associated with these Msg1 is beneficial to reduce UE’s blind detection complexity and the probability of false alarm. Alternatively, this common RNTI can be determined by a reference RO in this group and the group information of ROs can be configured by the RMSI. For example, this reference RO can be the first RO or the last RO in the group.

Proposal 11: When multiple Msg1 transmissions before Msg2 reception is configured, a common RA-RNTI is used for all the ROs where msg1 are duplicated transmitted.

In licensed band, power ramping is performed according to the configured value (herein termed as delta) in the time occasion that is out of the RAR window, while in unlicensed band, as discussed above, it is possible that a UE could also send Msg1 for multiple times within the RAR window. Considering that the access chance is precious due to the requirement of being LBT successful, while the gNB might suffer from the unsuccessful detection of Msg1, it should be careful to set a suitable power for the transmission of Msg1 within the RAR window to ensure the successful detection. Therefore, it is suggested that within the RAR window, power ramping could be performed for Msg1. Since this is additional chance for Msg1 transmission, it is proposed to use delta1 as power ramping value within the RAR window and use delta2 as power ramping value while outside of the RAR window, wherein delta1 and delta2 could be separately configured by the gNB.

Proposal 12: Power ramping could be performed within the RAR window, and power ramping could be set separately for Msg1 transmission within and outside the RAR window.

RACH related procedure

Random access response

In NR, the UE expects to receive random access response (RAR) within a RAR time window after successful PRACH preamble transmission, which is determined by higher layer parameter. A RAR carries scheduling information of a PUSCH transmission from the UE (Msg3 PUSCH). If a UE does not detect the PDCCH with a corresponding RA-RNTI and a corresponding DL-SCH transport block within the time window, it retransmits the PRACH preamble. In unlicensed operation, the RAR transmission could be blocked due to LBT failure. In last meeting, it was identified beneficial to extend the maximum RAR time window size which provide chances for the gNB to transmit RAR when LBT failure occurs.

However, in NR unlicensed system, always setting a large value of RAR window may bring the accessing latency in case that the retransmission of Msg1 is due to detection failure of Msg.1 at the gNB. Consequently, suitable setting of RAR window is preferred by taking into account two factors, i.e., successful detection of Msg1 and potential LBT failure to send RAR by the gNB. Therefore, it would be beneficial to dynamically configure the RAR window size. The RAR window size should be increased with the number of Msg1 retransmission. The more times Msg1 is retransmitted, the higher probability RAR reception failure is due to LBT failure at the gNB side.

Proposal 13: It is beneficial to reduce RACH latency that RAR window could be configured with multiple values and the RAR window size could be increased with the number of Msg1 retransmission.

Msg3 PUSCH

As discussed above, a UE will transmit Msg3 PUSCH according to the scheduling information indicated by the RAR. In RAN1#97 meeting, it was agreed that LBT category for Msg3 initial transmission is provided to the UE in the RAR. Note that there is a minimum time requirement between the last symbol of a PDSCH reception conveying a RAR and the first symbol of the corresponding Msg3 PUSCH transmission of the UE. Due to COT duration limitation and the required time gap between RAR and the corresponding Msg3, they may not be allowed to be transmitted within the same COT. At end of current COT, the gNB will stop transmission and is required perform LBT in order to obtain a new COT. Due to uncertainty of LBT, Msg3 transmission delay may become large. To reduce access latency and increase Msg3’s transmission opportunity, it is beneficial to allow multiple Msg3 transmission occasions outside of COT. For instance, a UE can do LBT before each transmission occasion outside of COT after RAR grant is received, and Msg3 is allowed to transmit with successful LBT. For example, the gNB can configure multiple Msg3 transmission occasions (e.g. number) to the UE, in which Msg3 transmission could be outside of COT.  An illustrative example can be found in Figure 6. When Msg2 is transmitted at the end of COT N and the remaining symbols are all downlink symbols in current COT, it is foreseeable that the transmission of Msg3 will be outside of COT N. If the transmission of Msg3 can be allocated in next COT N+1, the UE sends Msg3 with a single transmission occasion within COT, and default LBT type (e.g. CAT 2 LBT) is used. Otherwise, the UE can perform multiple LBT until the maximum number of allowed attempts is achieved, which depends on the configured number of Msg3’s transmission occasions outside of COT and LBT type carried by RAR.

Figure 6 Multiple transmission occasions for Msg3 outside of COT

Furthermore, to support this cross-COT transmission scheme, concept of 2-step triggered scheduling mechanism in LTE eLAA could also be considered, i.e., Msg3 is triggered to be transmitted in the subsequent COT and the scheduling information of triggering DCI is indicated by RAR in the previous COT.

Proposal 14: NR-U should support following mechanisms to reduce transmission latency of Msg3 in 4-step RACH:

- Allocating multiple Msg3’s transmission occasions

- 2-step trigger mechanism

Proposal 15: When Msg3 transmission falls into the new COT initiated by gNB, the default LBT type (e.g. LBT cat2) is used; otherwise, LBT type indicated by RAR should be used.

PDSCH with UE contention resolution identity

In response to an Msg3 PUSCH transmission, the UE attempts to detect a PDCCH with a corresponding TC-RNTI or C-RNTI scheduling a PDSCH that includes a UE contention resolution identity, and it transmits HARQ-ACK information in a PUCCH. Similar to RAR transmission discussed above, Msg4 could also be enhanced in the same manner in NR-U.

Paging enhancement

In RAN2#107, it is agreed to allow S×X PDCCH monitoring occasions in a PO. X is signaled in paging configuration. RAN2 also think that UE may stop monitoring the additional PDCCH monitoring occasions at paging occasions (PO) if it detects that gNB had access to the channel at the PO and the detection is reliable. RAN2 has agreed to use transmission addressed to P-RNTI for this purpose. An LS was sent to RAN1 [6] ask whether other DL signals can be used for the same purpose. The earlier UE can terminate PDCCH monitoring for paging, the more power saving UE can achieve. If gNB is mandated to transmit PDCCH addressed to P-RNTI at the earliest PDCCH monitoring occasion once acquired channel occupancy, the DL channels for COT identification discussed in [7] can also be used as stopping criterion even if there is no P-RNTI received. For example, if UE detect GC-PDCCH indicating the DL COT from serving cell during the period of conventional PO as plotted in Figure 7, UE can stop monitoring PDCCH in extended PO even if there is no P-RNTI received.

Proposal 16: UE could stop PDCCH monitoring in the extended PO when received GC-PDCCH indicating DL COT in the conventional PO.

Figure 7 PDCCH monitoring occasions when Ns=1, S=2 and X=2

Enhancement on RRM/RLM

RSSI measurement in NR-U

For RRM measurement in NR-U, to detect hidden node, UE reporting of RSSI and channel occupancy same as in LTE LAA had been agreed in last meeting. Wherein, channel occupancy percentage had already reflected the condition of channel occupancy and medium contention.

In LTE-LAA, RSSI measurement for the interested frequency or carrier is limited to the configured RMTC (RSSI Measurement Timing Configuration) which lasts for measDuration symbols and channel occupancy measurements. In the TS 36.331, the UE shall setup the RMTC according to the received rmtc-Period and rmtc-SubframeOffset. The subsequent parameter, i.e., rmtc-SubframeOffset, is the first symbol of each RMTC occasion occurs at first symbol of an SFN. For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDuration between 0 and rmtc-Period. Meanwhile, RMTC appears periodically in the unlicensed band, i.e., the “rmtc-Period” just mentioned, has the following values: 40ms, 80 ms, 160ms, 320ms and 640ms. “measDuration” is defined as the number of consecutive symbols for which the L1 reports samples of RSSI. So in the IE MeasObjectEUTRA:

RMTC-Config-r13 ::=	CHOICE {

release								NULL,

setup								SEQUENCE {

rmtc-Period-r13					ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r13			INTEGER(0..639)					OPTIONAL,		-- Need ON

measDuration-r13				ENUMERATED {sym1, sym14, sym28, sym42, sym70},

...

}

}

For the event of reporting of the measured RSSI, it has be introduced in the IE ReportConfigEUTRA:

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

Where parameter “measRSSI-ReportConfig” represents the RSSI reporting configuring for the appointed UE. The frequency or carrier which is measured by UE is in the parameter “measObject”.

In TR38.215, there is no definition on the RSSI measurement yet. Similar definition in TS36.214 as copied in Table 2 can be introduced for NR-U. Some details need further considered, such as whether need to redesign the rmtc-Period or measDuration considering different numerologies. In LTE-LAA, the RSSI is measured over N RB across the whole carrier. When wideband carrier is configured with multiple LBT bandwidth, the RSSI could be configured on each LBT bandwidth. The configuration in the frequency domain should be given with higher layer signaling.

Proposal 17：Definition of RSSI in TS36.214 can be introduced to TS38.215 for NR-U.

Table 2 Definition of RSSI in TS36.214

| Definition | E-UTRA Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) observed only in the configured OFDM symbol and in the measurement bandwidth over N number of resource blocks, by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. Higher layers indicate the measurement duration and which OFDM symbol(s) should be measured by the UE. The reference point for the RSSI shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding RSSI of any of the individual diversity branches |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

RLM

In NR Rel-15, the downlink radio link quality of the serving cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE could be configured with a set of periodic CSI-RS resources within the active DL BWP. The physical layer in UE indicates, in the frame where the radio link quality is assessed, out-of-sync to higher layer when the radio link quality is worse that the configured threshold for all resources in the set of resource for radio link monitoring (RLM). When the radio link quality is better than the configured threshold for any resource in the configured resource set, the physical layer in UE indicates in-sync to higher layers.

In NR-U, periodic CSI-RS resources configured for RLM, which are transmitted at the scheduled occasions, cannot always be guaranteed due to uncertainty of LBT. In case of CSI-RS transmission fails in the frames that UE is configured for radio link quality assessment, it may declare the radio link failure even if the radio link quality is still good. UE may also perform re-connection by a PRACH signal transmission, which could increase its power consumption and decrease spectral utilization efficiency.

Based on the above discussion, enhancement to CSI-RS based measurements shall be investigated, i.e., robust CSI-RS transmission scheme by introducing more CSI-RS transmission opportunities and/or CSI-RS transmission state indication. As illustrated in Figure 8, assuming each radio link quality assessment window contains three CSI-RS transmission occasions (TO). In case A, the UE will indicate out-of-sync to higher layers since all CSI-RS transmission fails due to LBT. In case B, the gNB may transmit CSI-RS replacement triggered by PDCCH until the beginning of the next pre-configured CSI-RS TO when the transmission fails at the current TO. Once the UE receives the CSI-RS replacement, it knows that the previous CSI-RS is missing due to LBT.

Figure 8   NR-U CSI-RS transmission schemes

Proposal 18: gNB could indicate UE to perform RLM measurement based on DCI triggered aperiodic CSI-RS if periodic CSI-RS is not transmitted due to LBT failure.

As shown in Figure 9, if a UE detects n310 consecutive out-of-sync indications, it starts timer t310. If the timer expires, the UE claims link failure. Otherwise, if the UE detects consecutive n311 in-sync indications prior to timer t310 expiration, the timer is reset and the link is claimed to be recovered.

Figure 9   NR-U in-sync and out-of-sync indications

In TR 38.889 [5], a new metric for identification of unsuccessful RLM-RS detection was proposed. However, UE is difficult to distinguish the unsuccessful reception is due to LBT failure at gNB side or serious channel degradation. Two mechanisms can be considered.

Alt 1: based on gNB feedback

UE sends report to the gNB as timer T310 starts, as illustrated in Fig 9. gNB send actual transmission state of one or multiple RLM RS(s) to the UE. UE could reset timer T310 and update counter N310 after receiving feedback from the gNB. To this end, LBT impact on NR-U RLM is mitigated.

Figure 10  Handshaking between gNB and UE for RLM

Alt 2: based on measurement on assisting RS

NR system supports interference measurement in the CSI-IM which is configured with ZP-CSI-RS. The REs which configured with CSI-IM, only have the signal power from other access devices (including intra-operator cells). For the RLM-RS, in the REs which configured with RLM-RS, the useful signal power from the serving cell also exists. After quantization, the signal power in the RLM-RS occupied REs can be treated as the plus of the interference signal power from other RATs and useful signal power from the serving cell. Therefore, UE can compare the energy detectedon the targeted frequency resource (REs) which configured with ZP-CSI-RS and RLM-RS and then judge whether the RLM-RS is successfully transmitted or not. If the measured results from ZP-CSI-RS and RLM-RS are almost the same, or less than a pre-defined threshold, UE may assume that the configured RLM-RS is not transmitted successfully

Proposal 19: the following two mechanisms can be considered to mitigate LBT impact on NR-U RLM

- Alt 1: UE could update its timer based on the gNB’s indication by reporting non-detected RLM RS, which can also mitigate LBT impact on NR-U RLM.

- Alt 2: UE would excludes measurement on the RLM-RS if the energies received on the RLM-RS and assisting RS are similar.

When UE is configured with BWP larger than 20MHz, gNB could also configure RLM-RS with bandwidth larger than 20MHz. Since LBT is performed on a 20 MHz sub-band basis, it is possible that only part of sub-band(s) succeed LBT while other(s) cannot be used for transmission. If UE can get the bandwidth indication through GC-PDCCH at beginning of COT, UE can obtain measurements from one or multiple sub-band(s) where RLM-RS is/are transmitted and make decision based on information collected from monitoring sub-band(s).

Proposal 20: In NR-U, RLM shall be performed based on RLM-RS on the available LBT subband(s) according to the bandwidth indication carried in GC-PDCCH.

Conclusions

In this contribution, we further discuss remaining issues on initial access including DRS, PRACH. Considerations on paging and RRM/RLM are also discussed. Based on the discussion, we have made following observations and proposals:

DRS:

Observation 1: Q is the minimum difference between PBCH DMRS sequence indices of 2 SSB with QCL assumption. Q should be integer times of the number of transmitted SSB depending on the number of SSB transmitted per slot. There is no need to further down select the allowed Q values.

Proposal 1: gNB could indicate Q value to UE through MIB for serving cell UE. gNB could indicate Q value through RRC signaling for neighboring cell measurement.

Proposal 2: gNB should transmit SSB and its associated CORESET0 in the same slot.

Proposal 3: In order to support receiving SSB and associated CORESET0 in the same slot, the entries in Table 1 should be considered for pdcch-ConfigSIB1in MIB.

Observation 2: When CAT 2 LBT is used before DRS transmission, the maximum allowed transmission duration of DRS is 1ms.

Proposal 4: gNB should fix DRS transmission window as Y=20 for 30 kHz SCS and Y=10 for 15k Hz SCS.

Proposal 5: PBCH within SS/PBCH block can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

Proposal 6: UE determines whether the scheduled PDSCH is rate-matched by transmitted SSB based on both higher layer signaling ssb-PositionsInBurst and system information indicator in DCI.

RACH:

Proposal 7: Semi-static resource allocation of PRACH supported in NR could be the baseline of resource allocation design of PRACH in NR-U.

Proposal 8: Dynamic scheduling of PRACH occasion via DCI (i.e. Alt.1) and via DRS (i.e. Alt.3) shall be supported in NR-U.

Proposal 9: Dynamic power allocation for PRACH should be supported in NR-U.

Proposal 10: Multiple Msg1 transmissions for both CBRA and CFRA should be supported in NR-U.

Proposal 11: When multiple Msg1 transmissions before Msg2 reception is configured, a common RA-RNTI is used for all the ROs where msg1 are duplicated transmitted.

Proposal 12: Power ramping could be performed within the RAR window, and power ramping could be set separately for Msg1 transmission within and outside the RAR window.

Proposal 13: It is beneficial to reduce RACH latency that RAR window could be configured with multiple values and the RAR window size could be increased with the number of Msg1 retransmission.

Proposal 14: NR-U should support following mechanisms to reduce transmission latency of Msg3 in 4-step RACH:

- Allocating multiple Msg3’s transmission occasions

- 2-step trigger mechanism

Proposal 15: When Msg3 transmission falls into the new COT initiated by gNB, the default LBT type (e.g. LBT cat2) is used; otherwise, LBT type indicated by RAR should be used.

Paging:

Proposal 16: UE could stop PDCCH monitoring in the extended PO when received GC-PDCCH indicating DL COT in the conventional PO.

RLM/RRM:

Proposal 17：Definition of RSSI in TS36.214 can be introduced to TS38.215 for NR-U.

Proposal 18: gNB could indicate UE to perform RLM measurement based on DCI triggered aperiodic CSI-RS if periodic CSI-RS is not transmitted due to LBT failure.

Proposal 19: the following two mechanisms can be considered to mitigate LBT impact on NR-U RLM

- Alt 1: UE could update its timer based on the gNB’s indication by reporting non-detected RLM RS, which can also mitigate LBT impact on NR-U RLM.

- Alt 2: UE would excludes measurement on the RLM-RS if the energies received on the RLM-RS and assisting RS are similar.

Proposal 20: In NR-U, RLM shall be performed based on RLM-RS on the available LBT subband(s) according to the bandwidth indication carried in GC-PDCCH.

References

RAN1#98 Chairman’s Notes, 3GPP TSG RAN1 Meeting #98 meeting, Reno, USA, May, 2019

TS38.213 v 15.6.0

R1-1910042, “Initial access signal and channels in NR unlicensed band”, Huawei, HiSilicon

ETSI EN 301 893 (2.1.1),“Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

3GPP TR 38.889 (v16.0.0): “Study on NR-based access to unlicensed spectrum ".

R2-1911861, “LS on additional PDCCH monitoring occasions for paging for NR-U”, ZTE

R1-1910039, “DL channels and signals in NR unlicensed band”, Huawei, HiSilicon