3GPP TSG RAN WG1 Meeting #98bis                                                                  R1-1910303

Chongqing, China, October 14th – 20th, 2019

Title:	[Draft] Reply LS on single PDCCH-based multi-TRP operation

Response to:	R2-1911746

Release:	Release 16

Work Item:	NR_newRAT-Core

Source:	ZTE [RAN1]

To:	RAN2

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-1911746 on single PDCCH-based multi-TRP operation. For RAN2’s questions as shown in the boxes, RAN1 gives the answers accordingly.

RAN2 would like to ask the following questions which will help to design the MAC CE for the single PDCCH-based multi-TRP operation. Each TCI codepoint can correspond to 1 or 2 TCI states, then RAN2 think there are two options to indicate the activated TCI states: - Option 1: Network configures the set of TCI trigger points which include one or two TCI states each for the Rel-16 NR-eMIMO UE, and MAC CE selects the candidate TCI trigger points among the configurable set of TCI trigger points. - Option 2: Network configures TCI-states for multi-TRP using existing TCI-states field and MAC CE selects one or two TCI states for each DCI code point dynamically. Question 1: Should the pairing of the TCI states be flexible such that each TCI state can be dynamically paired with each TCI state or is there an upper limit of pairs by configuration?

For above Question 1, based on RAN1’s understandings, there is no RRC impact for TCI state framework to support single PDCCH-based multi-TRP operation. The only enhancement is on MAC CE which is used to select one or two TCI states from RRC configured TCI state list for each DCI code point. In other words, the above Option 2 recited from R2-1911746 is RAN1’s agreements. Therefore, pairing of the TCI states should be flexible such that each TCI state can be dynamically paired with each TCI state by using MAC CE.

The number of maximum activated TCI states should be determined in order to design MAC CE. Since the TCI field is 3-bit, there are up to 8 TCI codepoints in DCI. If each TCI codepoint has to correspond to two different TCI states, at most 16 different TCI states should be activated. Noted that the maximum number of activated TCI states is 8 in Rel-15 TCI States Activation / Deactivation for UE-specific PDSCH MAC CE. Question 2: What is the total number RAN1 have in mind for the activated TCI states in mTRP operation? Is up to 16 correct understanding?

For above Question 2, RAN1 would like to confirm that RAN2’s understanding is correct i.e. up to 16 activated TCI states is supported in mTRP operation.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #99	18 – 22 November 2019, Reno, US

TSG RAN WG1 Meeting #100	24 – 28 February 2020, Athens, Greece