3GPP TSG RAN WG1 Meeting #98-Bis	R1-1911207

Chongqing, China, 14th – 20th October, 2019

Agenda Item:	7.2.10.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Remaining Details on Uplink PRS Signal Design

Document for:	Discussion & Decision

# Introduction

In RAN1#98, the following agreements were reached regarding the uplink positioning signal design for the NR.

Agreement: - The RE pattern of an SRS resource for positioning is configured with a comb offset for the first symbol in an SRS resource. - The relative RE offsets of following symbols are defined relative to the comb offset of the first symbol in the SRS resource. - A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource. - FFS: The use of the following for deriving the relative offset in addition to the above - A configurable number of consecutive symbols with the same offset - Repetition factor - No additional parameters will be considered Conclusion: No additional comb sizes for SRS for positioning purposes will be introduced in Rel-16 beyond those already agreed. Conclusion: No additional number of symbols per SRS resource other than the values already agreed is supported

In this contribution, we present our views on the remaining details on SRS enhancements for positioning purposes, part of which were left FFS.

# Proposals for SRS enhancements

## Defining staggering pattern

### Transmission comb size of 2

The staggering pattern for KTC = 2 is straightforward. There are two possible patterns and the effect of staggering can be achieved by configuring two SRS resources with same transmission comb factor, and different offset.

Figure 1: Staggering pattern for transmission comb size 2 to obtain transmission comb size 1

### Transmission comb size of 4

There are two variants for staggering pattern possible with transmission comb size of 4. The first provides a regular pattern, with a constant shift and simpler to construct which is easily extensible to (any) larger comb sizes.

Figure 2: Regular diagonal pattern for KTC = 4, Offset = 0

The second variant, which constructs for a zig-zag pattern, allows a pattern of transmission comb size 2 to be synthesized from two adjacent OFDM symbols. Two combinations are possible: 0 and 1, 2 and 3 – which allows step-wise processing. Nevertheless, combinations of 1 and 2, for example, cannot be used to the same effect. Nevertheless, OFDM symbol 0 and 2 could be used in the regular pattern to the same effect, albeit with 1 symbol delay.

Figure 3: Alternative zig-zag pattern for KTC = 4, Offset = 0

### Transmission comb size of 8

The transmission comb size of 8 requires an even number of resource blocks to be allocated, and does not end at the resource boundary. Restrictions arise and need to be considered that this comb size is only defined when the SRS resource blocks are even numbers

Figure 4: A possible zig-zag pattern for comb size of 8.

Observation 1: The zig-zag configuration allows for using few symbols to arrive at an effective lower transmission comb factor when destaggered, with a complexity in defining the symbol pattern.

Proposal 1: Allow for both regular and zig-zag pattern to be defined, signaling via DCI which pattern the UE is to choose.

At RAN1#98, the following was agreed:

- The RE pattern of an SRS resource for positioning is configured with a comb offset for the first symbol in an SRS resource.

- The relative RE offsets of following symbols are defined relative to the comb offset of the first symbol in the SRS resource.

- A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource.

- FFS: The use of the following for deriving the relative offset in addition to the above

  - A configurable number of consecutive symbols with the same offset

  - Repetition factor

- No additional parameters will be considered

At RAN1#98, the following was agreed:

- The RE pattern of an SRS resource for positioning is configured with a comb offset for the first symbol in an SRS resource.

- The relative RE offsets of following symbols are defined relative to the comb offset of the first symbol in the SRS resource.

- A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource.

- FFS: The use of the following for deriving the relative offset in addition to the above

  - A configurable number of consecutive symbols with the same offset

  - Repetition factor

- No additional parameters will be considered

Proposal of mathematical representation for determining staggering pattern

In order to determine the method for signaling the staggering pattern, we perform the following analysis:

Provided that an initial comb offset is 0, the following patterns, which allow obtaining channel impulse response over the bandwidth, are possible:

Comb factor 2  only one pattern possible (0, 1)

Comb factor 4  one regular (0, 1, 2, 3) and two patterns for faster decorrelation {(0, 2, 1, 3), (0, 2, 3, 1)}

Comb factor 8  one regular (0, 1, 2…, 7) and a total of 48 permutations, of which one possible permutation is (0, 4, 2, 6, 1, 5, 3, 7). Table 1 lists all of the combinations.

Instead of specifying one fixed permutation of possible relative RE offsets, it would be beneficial to signal the pattern. The pattern may be set collectively for each gNB, in order to allow randomization between UEs served by different gNBs, so that the SRS in different OFDM symbols are affected by different level of interference from UE served by other gNBs.

Table 1: Possible patterns for staggered pattern with transmission comb 8

| Regular | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pattern 1 | 0 | 4 | 2 | 6 | 7 | 5 | 3 | 1 |
|  | 0 | 4 | 2 | 6 | 7 | 5 | 1 | 3 |
|  | 0 | 4 | 2 | 6 | 7 | 3 | 5 | 1 |
|  | 0 | 4 | 2 | 6 | 7 | 3 | 1 | 5 |
|  | 0 | 4 | 2 | 6 | 7 | 1 | 5 | 3 |
|  | 0 | 4 | 2 | 6 | 7 | 1 | 3 | 5 |
|  | 0 | 4 | 2 | 6 | 5 | 7 | 3 | 1 |
|  | 0 | 4 | 2 | 6 | 5 | 7 | 1 | 3 |
|  | 0 | 4 | 2 | 6 | 5 | 3 | 7 | 1 |
|  | 0 | 4 | 2 | 6 | 5 | 3 | 1 | 7 |
|  | 0 | 4 | 2 | 6 | 5 | 1 | 7 | 3 |
|  | 0 | 4 | 2 | 6 | 5 | 1 | 3 | 7 |
|  | 0 | 4 | 2 | 6 | 3 | 7 | 5 | 1 |
|  | 0 | 4 | 2 | 6 | 3 | 7 | 1 | 5 |
|  | 0 | 4 | 2 | 6 | 3 | 5 | 7 | 1 |
|  | 0 | 4 | 2 | 6 | 3 | 5 | 1 | 7 |
|  | 0 | 4 | 2 | 6 | 3 | 1 | 7 | 5 |
|  | 0 | 4 | 2 | 6 | 3 | 1 | 5 | 7 |
|  | 0 | 4 | 2 | 6 | 1 | 7 | 5 | 3 |
|  | 0 | 4 | 2 | 6 | 1 | 7 | 3 | 5 |
|  | 0 | 4 | 2 | 6 | 1 | 5 | 7 | 3 |
|  | 0 | 4 | 2 | 6 | 1 | 5 | 3 | 7 |
|  | 0 | 4 | 2 | 6 | 1 | 3 | 7 | 5 |
|  | 0 | 4 | 2 | 6 | 1 | 3 | 5 | 7 |
| Pattern 2 | 0 | 4 | 6 | 2 | 7 | 5 | 3 | 1 |
|  | 0 | 4 | 6 | 2 | 7 | 5 | 1 | 3 |
|  | 0 | 4 | 6 | 2 | 7 | 3 | 5 | 1 |
|  | 0 | 4 | 6 | 2 | 7 | 3 | 1 | 5 |
|  | 0 | 4 | 6 | 2 | 7 | 1 | 5 | 3 |
|  | 0 | 4 | 6 | 2 | 7 | 1 | 3 | 5 |
|  | 0 | 4 | 6 | 2 | 5 | 7 | 3 | 1 |
|  | 0 | 4 | 6 | 2 | 5 | 7 | 1 | 3 |
|  | 0 | 4 | 6 | 2 | 5 | 3 | 7 | 1 |
|  | 0 | 4 | 6 | 2 | 5 | 3 | 1 | 7 |
|  | 0 | 4 | 6 | 2 | 5 | 1 | 7 | 3 |
|  | 0 | 4 | 6 | 2 | 5 | 1 | 3 | 7 |
|  | 0 | 4 | 6 | 2 | 3 | 7 | 5 | 1 |
|  | 0 | 4 | 6 | 2 | 3 | 7 | 1 | 5 |
|  | 0 | 4 | 6 | 2 | 3 | 5 | 7 | 1 |
|  | 0 | 4 | 6 | 2 | 3 | 5 | 1 | 7 |
|  | 0 | 4 | 6 | 2 | 3 | 1 | 7 | 5 |
|  | 0 | 4 | 6 | 2 | 3 | 1 | 5 | 7 |
|  | 0 | 4 | 6 | 2 | 1 | 7 | 5 | 3 |
|  | 0 | 4 | 6 | 2 | 1 | 7 | 3 | 5 |
|  | 0 | 4 | 6 | 2 | 1 | 5 | 7 | 3 |
|  | 0 | 4 | 6 | 2 | 1 | 5 | 3 | 7 |
|  | 0 | 4 | 6 | 2 | 1 | 3 | 7 | 5 |
|  | 0 | 4 | 6 | 2 | 1 | 3 | 5 | 7 |

For comb factor, there are 49 different sequences for each initial comb offset, and there are eight comb offsets, which can be signaled using a bitmap of 9 bits. Alternatively, the pattern can be separately specified by using a bitmap of 6 bits independent of the initial comb offset.

The offset used at each OFDM symbol can be computed by taking the offset (from table above) and

relOffset = mod(combOffset + patternOffset (k), KTC);

floor(l/Nrep),

where

patternOffset is obtained using the row index signaled. For a regular diagonal pattern, this goes from 0 to 7 sequentially.

l is the OFDM symbol index,

Nrep is the number of repetitions or the number of OFDM symbols having the same pattern Offset. To keep patterns simple, the following is needed:

Nrep <= Number of OFDM symbols per resource

Observation 2: Interference can be randomized if different UEs use different staggering patterns, the staggering pattern may be coordinated between gNBs and signaled to the UE by its s-gNB.

Proposal 2: Allow the staggering pattern to be signaled to an UE using an index, per UE or per cell basis, for example using DCI.

## Multiplexing multiple users

### Multiplexing using Cyclic Shifts

The auto-correlation of two Zadoff-Chu sequences is 0, except for shift α, where α is the integer number of cyclic shifts with respect to the root sequence. The maximum value of cyclic shifts allowed depends on the transmission comb factor used, as follows:

| Transmission Comb Number (KTC) | Maximum Cyclic Shift |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 (Rel. 16, agreed at RAN1#98) | ? (6, 8 or higher values) |

(a)

(b)

Figure 5: Depiction of cross correlation for 8 different cyclic shifts using comb factor of 4 and a maximum cyclic shift of 12 in (a) (Rel. 15) and a maximum cyclic shift of 64 in (b)

(c)

(d) Figure 6: Depiction of cross correlation for 8 different cyclic shifts using comb factor of 8 and a maximum cyclic shift of 8 in (a) and a maximum cyclic shift of 64 in (b)

With higher comb factor and smaller stepsize for the cyclic shift (stepsize ~1/max_CS) , the spacing between two correlation peaks is decreased, therefore ambiguity in UE detection increases due to propagation timing difference and also due to potential imperfect timing advance. Particularly for positioning use case, in the indoor scenario where relatively lower values of time difference between paths are observed, higher values of maximum cyclic shift than the ones currently allowed in Rel. 15 may be used.

Proposal 3: Allow UE to be configured with higher values of cyclic shift depending on the transmission comb factor.

### Multiplexing using different root sequences

For a given sequence length NZC, two Zadoff-Chu sequences with root index of q1 and q2 have a cross-correlation equal to 1NZC, if q1 -q2 is relative prime to the sequence length NZC used.  It is important to note that for positioning purposes, a wideband SRS signal will generally be used (i.e. a large NZC) and the cross-correlation between two sequences is 0 for all practical purposes. As an example, we consider we assume a carrier bandwidth of 20 MHz, a subcarrier spacing of 15kHz and a comb factor of 2. According to Table 6.4.1.4.3-1 in TS38.211, a possible value for MZC is 624 (52 RB). This results in a corresponding Zadoff-Chu sequence of length 619. For this sequence length, up to 618 Zadoff-Chu sequences exist. Nevertheless, in 5G-NR only 60 sequences can be selected by varying the parameters v and u. This restriction originates from the computation of the root index q of the Zadoff-Chu sequence as defined in chapter 5.2.2.1 of TS38.211 and the allowed range for the parameter u and v.

ForMZC≥3NscRB, the base sequence  is given by

where

The length  is given by the largest prime number such that.

The parameters u and v depend on higher-layer parameter groupOrSequenceHopping. If groupOrSequenceHopping equals “neither” v is set to 0, which further restricts the usable sequences to only 30 sequences.

In order to address deployment scenarios like factory halls, where the UE density is particularly high, it is necessary either to use large cyclic shifts or increase the number of sequences, to reduce the number of slots needed for a given update rate, compared to the possibilities for multiplexing users already possible today. Figure 7 shows the effect of increasing the number of UEs transmitting simultaneously on same time-frequency resources but multiplexed by using different sequence.  An assumption of equal received power is made here.

Figure 7: Code multiplexing of 8 UEs with the same received power.

### Usage scenario for comparison of code multiplex and cyclic shift

Assuming that several UEs (e.g. IoT devices) are tracked in parallel many symbols/timeslots may be required if a high update rate is desirable. Due to the low SINR requirements for positioning, SRSs from  UEs can be multiplexed using different time-frequency resources in order to reduce the total number of symbols or slots necessary for positioning usage. Rel. 15 of the standard already allows user to be configured with different cyclic shifts and sequence ID.

The key question to answer is whether the number of sequences and the number of cyclic shift need to be increased further in Rel. 16, at least for positioning applications. To better illustrate the problem, we consider the scenario in Figure 8, where four UEs each served by a different gNB share the resource in time frequency. This means transmission comb size (KTC), comb offset (kOffset) and symbol and slot are identical.

Figure 8: Example scenario used for the plots

We make further the following assumptions:

- gNB1 is the gNB were the signals from all UEs are received and processed.

- Power control is done with respect to its serving gNB.

- Each of the UE is roughly at the same distance from its serving gNB. This means the UEs have same timing advance and also transmit with roughly the same transmit power.

- Signal correlation at gNB1 is addressed. The UEs are at distance of 107m, 150m, 182m and 198m respectively. Although they have the same timing advance, the time difference of arrival of signal of the UEs depends on the relative distance differences between them.

- The following characteristics of the signal are considered

  - A transmission comb factor (KTC = 8), kOffset = 0.

  - A second symbol is inserted with kOffset = 4

  - 100 MHz signal bandwidth and subcarrier spacing of 30 kHz

  - The users are separated either using different cyclic shifts or using different sequences.

- The following assumptions on channel conditions are made

  - UMi_LOS channel

  - Noise and interference from other (neighboring) cells are not taken into account, as they are not critical to addressing the user separation issue.

The signal received at gNB1 is analyzed and discussed below. The input signal of gNB1 is the sum of all transmitted sequences. The following figures show the output of the correlator of gNB1.

Analysis for different cyclic shifts separation.

| Output of cyclic correlator (1 symbol). - For the selected configuration, we observe 32 correlation replica, where 4 replica arise when the received signal is correlated with reference signal having different cyclic shift. - 8 replica for each cyclic shift due to transmission comb factor of 8. | Figure 9: Output of cyclic correlation. |
| --- | --- |
| Result of de-staggering: We assume moderate Doppler shift and in such scenario, destaggering can be implemented by: CF1 = fft(rx1).* conj(ref_sym1); CF2 = fft(rx2).* conj(ref_sym2); corr = ifft(CF1+CF2); with CF1 = correlation sym1 in frequency domain CF2 = correlation sym2 in frequency domain corr = correlation in time domain Destaggering is similar to a coherent combining, which results in a signal equivalent to that having a transmission comb size of | Figure 10: Result of destaggering of SRS transmitted in two OFDM symbols. |
| The useful part of the correlation function is taken by selecting one replica, which arises due to the use of transmission comb factor. For the deployment scenario considered, the range difference applicable is roughly 200m. This is equivalent to 660ns, which corresponds to 81 samples with a sampling frequency 122.88MHz. According to the distances given above the signal of UE2 arrives 17samples (= (150m-107m)/c0/Ts ), UE3 30 samples (182m-107m) and UE4 37 samples (198m-107m) later than UE1 | Figure 11: Zoomed view of the group of correlation peaks in the first replica arising from the use of transmission comb factor. |
| For comparison, the same group of correlation peaks that would arise without sharing (i.e. each UE uses a different symbol or timeslot). The additional peaks in Figure 11 compared to Figure 12 result from “self-interference” in case of multiplexing several UEs to one set of REs. | Figure 12: Correlation with destaggering, same RE different shifts |

Analysis for different ZC sequences separation.

| The cross correlation properties of the ZC sequences in Figure 13 cause a ‘noise floor’ in the correlation function. | Figure 13: Correlation with destaggering, different REs |
| --- | --- |
| With code multiplex there are no/minor issues with the correlation replica arising from the use of transmission comb factor. The figure shows that using KTC = 8 combined with staggering/destaggering over 2 symbols is equivalent to using KTC=4. However, constraints on transmit power necessitates the use of higher transmission comb factor in combination with staggering, to achieve a larger coverage radius. | Figure 14: Correlation after destaggering. |

It is feasible to separate multiple UEs in cyclic shifts and sequences, within the same set of REs. Nevertheless, it can also be observed that the cyclic shift outperforms code multiplex. However, it requires a careful selection of cyclic shifts to resolve the ambiguity arising from the use of higher transmission comb factor and possibly non-ideal timing advance. Thus, use of higher cyclic shift values may be limited to indoor deployments with low ISD and lower transmission comb factor.

Observation 3: The cyclic shift outperforms code multiplex given that a careful selection of cyclic shifts is chosen to resolve the ambiguity.

Further consideration on interference issues:

It is important to note that for positioning applications, signals received with very low SINR are also detectable (especially if long sequences are selected). Hence, if intended UE is in a poor coverage (e.g. NLOS condition) to the expected RP/TRP and an interfering UE further away is in a LOS condition, as depicted in Figure 15.

Here it is assumed that both UE1 and UE2 use the same ZC-root index. The likelihood of this happening is quite high, if only 30 sequences groups are available, especially if a high density of UE for positioning is assumed.

Figure 15: Detection of wrong UE when same sequence is used.

Such scenario is unlikely to arise for use cases other than positioning because the SRS is intended to be measured at its serving gNB, and if the serving gNB is obstructed and another gNB is in good coverage, then either handover or cell-reselection (if connection lost) would take place.

In such situation, TRPs may detect signals from far away UEs (e.g. UE2 in the above example), which uses the same sequence as that of UE1. This causes ambiguity in resolving UE1 and UE2. At least for positioning purposes, it would be beneficial to increase the number of sequences available in order to resolve the ambiguity.

Observation 4: A low number of Zadoff-Chu sequences available for positioning increases the likelihood that two UEs served by different gNBs use same root sequence. This causes ambiguity in UE detection which is especially true at the TRPs with low received signal strength.

Proposal 4: Increase the number of SRS sequences available to address multiple UE separation.

# Conclusion

In this contribution, we have analyzed remaining details to be clarified for SRS and have made the following observations:

Observation 1: The zig-zag configuration allows for using few symbols to arrive at an effective lower transmission comb factor when destaggered, with a complexity in defining the symbol pattern.

Observation 2: Interference can be randomized if different UEs use different staggering patterns, the staggering pattern may be coordinated between gNBs and signaled to the UE by its s-gNB.

Observation 3: The cyclic shift outperforms code multiplex given that a careful selection of cyclic shifts is chosen to resolve the ambiguity.

Observation 4: A low number of Zadoff-Chu sequences available for positioning increases the likelihood that two UEs served by different gNBs use same root sequence. This causes ambiguity in UE detection which is especially true at the TRPs with low received signal strength.

Based on above observations, we make the following proposals:

Proposal 1: Allow for both regular and zig-zag pattern to be defined, signaling via DCI which pattern the UE is to choose.

Proposal 2: Allow the staggering pattern to be signaled to an UE using an index, per UE or per cell basis, for example using DCI.

Proposal 3: Allow UE to be configured with higher values of cyclic shift depending on the transmission comb factor.

Proposal 4: Increase the number of SRS sequences available to address multiple UE separation.

# References:

| [1] | 3GPP R1-1903053, "Uplink NR TDOA Design," Athens, Greece, 2019-03. |
| --- | --- |
| [2] | 3GPP R1-1909387, "UE Procedures for transmitting UL PRS," Prague, CZ, 2019-08. |
| [3] | 3GPP R1-1909386, "Details on Uplink PRS Signal Design," Prague, CZ, 2019-08. |
| [4] | 3GPP R1-1907088, "Details on Uplink PRS Signal Design," Reno, Nevada, USA, 2019-05. |