3GPP TSG RAN WG1 Meeting #99	R1-1911883

Reno, USA, November 18-22, 2019

Agenda Item:	7.2.4.2.1

Source:	Huawei, HiSilicon

Title:	Sidelink resource allocation mode 1

Document for:	Discussion and Decision

- Introduction

At RAN#84, a revised WI on NR V2X was agreed. The WI has the following objectives related to sidelink resource allocation [1]:

Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- […]

- Resource allocation [RAN1, RAN2]

  - Mode 1

    - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

In RAN1#  98bis, the following agreements for SL resource allocation mode 1 were achieved:

Agreements:

- In Mode-1, for a UE, for each of the configured MCS tables (for both DG & CG):

  - If no MCS is configured, UE autonomously selects MCS from the full range of values

    - Up to UE implementation

    - FFS details for the MCS table

  - If a single MCS is configured, the MCS is used by the UE

  - If a range of two or more MCSs are configured, UE autonomously selects the MCS from the configured values

    - Up to UE implementation

Agreements:

- To signal the gap between DCI reception and the first sidelink transmission scheduled by DCI:

  - A table of values is configured by RRC.

  - DCI determines which of the configured values is used.

  - FFS how to determine the slot for the first sidelink transmission (e.g., based on the indicated value, potential async between Uu & SL, different numerologies, etc.)

  - FFS if the gap is in physical or logical slots.

Agreements:

For reporting SL HARQ-ACK to the gNB:

- For dynamic grant and configured grant type-2 in SL, the Rel-15 procedure and signalling for DL HARQ-ACK are reused for the purpose of selecting PUCCH offset/resource and format in UL.

  - The configuration for SL is separate from Uu link for a UE

  - FFS how to indicate timing of transmission in PUCCH, including whether physical or logical slots are used

- For configured grant type-1 in SL, RRC is used to configure PUCCH offset/resource and format in UL (if supported)

Agreements:

- Two different UE-specific SL RNTIs are introduced for Mode-1 scheduling: one for CRC scrambling in DCI for a dynamic grant and the other one for CRC scrambling in DCI for a configured grant type-2.

  - The two above DCIs have the same size

Agreements:

- Multiple type-1 configured grants per UE are supported when LTE Uu controls NR SL

  - Up to the same max number of type-1 configured grants per UE when NR Uu controls NR SL

Working assumption:

- Each transmission in a resource provided by a configured grant contains PSCCH and PSSCH.

Agreements:

- For a configured grant in Mode 1 when using SL HARQ feedback:

  - There is only one HARQ-ACK bit for the configured grant

  - There is one PUCCH transmission occasion after the last resource in the set of resources provided by a configured grant.

In addition, the following agreement have been achieved through email discussions 98b-NR-12 and 98b-NR-13 after meeting #98b, respectively:

98b-NR-12:

- For unicast:

o     TX UE reports contents received in PSFCH (i.e., ACK/NACK) to gNB.

o     TX UE reports NACK if PSFCH is not detected

o     when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as working assumption) associated with the transmissions.

- For groupcast option 1:

o     TX UE reports ACK to the gNB if no PSFCH is detected.

o     TX UE reports NACK to the gNB if at least one PSFCH (i.e., NACK) is detected.

o     when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as a working assumption) associated with the transmissions.

o     FFS the cases when TX UE does not transmit/receive due to prioritization.

- For groupcast option 2:

o     TX UE reports ACK if all expected PSFCH resources are received and carry ACK.

o     TX UE reports NACK if at least one received PSFCH resource carries NACK or if no PSFCH is not detected.

o     FFS the case with PSFCHs corresponding to multiple PSCCH/PSSCH transmissions before generating the HARQ-ACK report.

o     FFS behavior when TX UE does not detect some expected PSFCH.

- FFS if no PSCCH/PSSCH is transmitted in a set of resources for configured grant.

- FFS whether/how to deal with the case of reaching the maximum number of HARQ re-transmissions for a TB.

98b-NR-13:

- NR supports reporting of multiple SL HARQ-ACKs in a single PUCCH resource.

  - The Rel-15 procedures for multiplexing DL HARQ-ACKs are reutilized.

  - Reports carry SL HARQ-ACKs for dynamic grants and/or configured grants.

    - A UE does not expected to be indicated to transmit SL HARQ-ACK information for more than one SL configured grant in a same PUCCH.

    - Note: A UE can be provided with multiple SL CGs with different (non-overlapping) slots for the corresponding PUCCH transmissions for SL HARQ-ACK reporting.

- NR supports multiplexing of SL HARQ-ACK(s) and DL HARQ-ACK(s) in a single PUCCH resource.

  - A UE does not expected to be indicated to transmit HARQ-ACK information for SPS PDSCH receptions and SL configured grants in a same PUCCH.

  - Note: A UE can be provided with multiple SL CGs/DL SPSs with different (non-overlapping) slots for the corresponding PUCCH transmissions for SL/DL HARQ-ACK reporting.

  - The PUCCH resource used for reporting the multiplexed HARQ-ACKs is determined by the last DCI among all DCIs associated with the reported HARQ-ACKs (e.g., carrying a SL grant, scheduling a PDSCH, etc.).

  - FFS whether the DL HARQ-ACK PUCCH resource set(s) or the SL HARQ-ACK PUCCH resource set(s) is used.

- For SL HARQ-ACK reporting, both Type-1 and Type-2 codebook are supported:

  - The same codebook type is used for SL HARQ-ACK and DL HARQ-ACK reporting.

  - SL HARQ-ACK bits are generated using the Rel-15 procedures and concatenated to the DL HARQ-ACK bits, which are independently generated using the corresponding procedures.

    - FFS changes or restrictions to the Rel-15 procedures for generating the SL HARQ-ACK bits.

  - FFS other details on how the codebook(s) are constructed

- SL HARQ-ACK is reported in PUSCH when reporting in PUCCH overlaps with a PUSCH transmission.

  - The Rel-15 procedures and signaling for multiplexing DL HARQ-ACKs in PUSCH are reutilized.

In RAN1 #98, further mode 1 related agreements were achieved:

Agreements:

- At least for dynamic grant, the timing and resource for PUCCH used for conveying SL HARQ feedback to the gNB are based on the indication(s) in the corresponding PDCCH

  - Details FFS

Agreements:

- DCI indicates the slot offset between DCI reception and the first sidelink transmission scheduled by DCI.

  - The minimum gap between DCI and the first scheduled sidelink transmission is not smaller than the corresponding UE processing time.

  - Details FFS

In RAN 1 #96b [2], the following agreements were achieved:

Agreements:

- A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB.

- A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions.

  - UE decides which TB to transmit in each of the occasions indicated by a given configured grant.

In RAN 1 #97 [3], the following agreement were achieved:

Agreements:

- For mode 1:

  - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH.

- NR sidelink does not support performing different transmissions of a TB using different configured grants.

Regarding LTE Uu configuration of NR sidelink, the agreements achieved so far are as follows:

Agreements @ RAN1#94bis

It is supported that LTE Uu provides at least necessary semi-static configuration for NR mode-2 SL communications

FFS details

Further study impact and benefits of LTE Uu managing NR mode-1 SL communications

Agreements @ RAN1-AH-1901

- LTE Uu to schedule NR sidelink mode 1 is supported:

  - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

  - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

RAN2#105bis

Agreements on inter-RAT resource allocation:

2:	LTE Uu supports configuration for NR mode-2 operation through RRC signaling and through SIB signaling. However, on demand or per valid area mode-2 configuration via SIB signaling is not supported.

1: 	New system information block type should be designed to support NR sidelink Tx and Rx resource pool configuration via LTE Uu. New system information will be defined as container (OCTET STRING) and actual information will follow what is defined in NR RRC.

2:	eNB should be able to configure the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

3:	NR Sidelink UE information and/or NR UE Assistance information will be transmitted as container (OCTET STRING) and actual information will be defined in NR RRC.

- In this paper, we discuss NR V2X mode 1 sidelink resource allocation/configuration. In particular, we discuss about NR SL mode-1 scheduled and configured grant (type-1 and type-2) resource allocations and HARQ feedback report to gNB. We also discuss LTE Uu being used to control NR sidelink. Specifically, we provide details on NR mode 1-Type 1 configured grant configuration by LTE Uu and how eNB configures the resources for NR mode 2.

Note that in companion paper [4], we discuss NR SL mode 2 resource allocation/configuration.

- NR SL mode 1 resource allocation

  - Dynamic Scheduling

In RAN1#96bis, it was agreed that a dynamic grant provides resources for one or multiple sidelink transmissions of a single TB.  LTE V2X mode 3 supports up to two transmissions scheduled by one DCI, where the gap between the two transmissions is signaled in DCI and SCI. Due to the higher reliability and lower latency requirements of NR V2X, more than two transmissions signaled by DCI should be supported.

Fig. 1 Reliability of data transmission with different numbers of repetitions for a TB size of 2000 bytes and 16QAM.

One of the performance requirements for advanced driving such as emergency trajectory alignment between UEs supporting V2X application is to achieve 10^-5 reliability and 3ms latency for packet sizes of 2000 bytes. Fig. 1 shows the BLER performance of different numbers of repetitions using a TBS of 2000 bytes and 16QAM using 48RB (close to 20MHZ BW). It can be seen from the figure that in order to achieve the 10^-5 reliability requirement at SNR of 5dB,  4 TB repetitions  are needed. Considering that in mode 2 it was agreed that SCI signaling can be designed to allow to indicate 1 or 2 or 3 resources at least of the same number of sub-channels with full flexibility in time and frequency position in a window W of a resource pool, one DCI/SCI should be able to signal at least 3 (re)transmissions of a single TB with full flexibility in time and frequency domains. With more or less the same overhead, it is possible to signal 4 (re)transmissions of a single TB in the same DCI/SCI with either full or reduced flexibility in both time and frequency domains.

Proposal 1: To meet latency and reliability requirements, NR V2X mode 1 dynamic grant should support at least 4 transmissions of a single TB.

Generally speaking, indicating the time frequency resources for each repetition separately is very costly in terms of signaling overhead. In order to efficiently schedule at least 4 transmissions of a single TB in one DCI/SCI, a time-frequency resource index scheduling all (re)-transmissions of a single TB should be included in the DCI/SCI. This can be achieved by a predefined or (pre)configured TFRP pool, which provides all the potential choices of time frequency resource combinations of all transmissions of a TB. It is also possible to indicate time and frequency resources separately. In this case, time domain repetition pattern can be indicated using a bitmap or an index of all potential combinations of time domain patterns within a given window size (e.g 16 slots). Frequency domain repetition pattern of all transmissions can be further simplified by indicating a frequency hopping offset, which can be in the granularity of sub-channels. The frequency hopping offset can reduce DCI/SCI overhead while keeping the number of bits for the frequency domain indication independent on the number of resources indicated in DCI/SCI.

Proposal 2: DCI/SCI should include a time-frequency index indicating resources for all repetitions of a single TB:

- Time domain repetition pattern can be indicated using a bitmap or an index of all potential time domain patterns within a given time window;

- Frequency domain repetition pattern can be indicated using a sub-channel based frequency pattern.

In mode 1, whether HARQ ACK/NACK feedback from Rx UE to Tx UE is supported is implicitly indicated by whether the resource for reporting sidelink HARQ ACK/NACK from Tx UE to gNB is allocated in the DCI.

Proposal 3: In mode 1, whether HARQ ACK/NACK feedback from Rx UE to Tx UE is supported is implicitly indicated by whether the resource for reporting sidelink HARQ ACK/NACK from Tx UE to gNB is allocated in the DCI.

For HARQ based retransmission, when both mode 1 and mode 2 resource allocation are configured simultaneously for a UE, a mapping relationship of HARQ process IDs is needed between gNB and Tx UE. Since the Tx UE’s HARQ buffer is shared by both mode-1 and mode-2, the HARQ process ID in DCI from gNB has to be mapped correcting into SCI by the Tx UE.

Proposal 4: For HARQ based (re-)transmission, when both mode 1 and mode 2 resource allocation are configured simultaneously for a UE, HARQ process usage needs to have a common understanding between gNB and Tx UE for each resource allocation mode.

Considering that UE-specific BWP is configured for sidelink communication, different resource pools within the SL BWP for a Tx UE can target different Rx UEs. If only one Tx resource pool is configured by RRC signaling, RRC reconfiguration is needed to support flexible communication between Tx UE and Rx UE. In order to avoid the transmission delay caused by RRC reconfiguration, flexible Tx resource pool selection within a SL BWP should be supported. A possible scheme is that multiple Tx resource pools are configured by RRC signaling, and one Tx resource pool is indicated in DCI. For example, if four Tx resource pools are configured in the SL BWP for a UE, Tx resource pool ID field with 2 bits can be designed in DCI to indicate the Tx resource pool for sidelink transmission.

Proposal 5: Tx resource pool ID is indicated in DCI to support flexible Tx resource pool selection within a SL BWP.

For the time interval between receiving a dynamic grant and transmitting PSCCH and PSSCH, the following procedure is used in LTE-V2X:

  - If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource  in the first subframe that is included in t0SL,t1SL,…,tTmaxSL and that starts not earlier than .

  - The set of subframes that may belong to a PSSCH resource pool for sidelink transmission mode 3 or 4 is denoted by  t0SL,t1SL,…,tTmaxSL where

    - 0≤tiSL<10240

    - the subframe index is relative to subframe#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0

    - the set includes all the subframes except the following subframes,

      - subframes in which SLSS resource is configured

      - downlink subframes and special subframes if the sidelink transmission occurs in a TDD cell

      - reserved subframes

    - The subframes are arranged in increasing order of subframe index

  - The value m is indicated by ‘Subframe offset’ field in the corresponding DCI format 5A according to Table -14.2.1-1 if this field is present and m=0 otherwise

- Table 14.2.1-1: Mapping of DCI format 5A offset field to indicated value m

| SL index field in DCI format 5A | Indicated value m |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | 3 |

For NR-V2X, since the flexible sub-carrier spacing is supported, the set of slots for PSCCH resource pool would be according to a definition such as:

  - The set of slots t0SL,t1SL,…,tTmaxSL that may belong to a PSSCH resource pool for sidelink transmission mode 1 or 2 is denoted by:

    - 0<tiSL<10240×Nslot ,   Nslot means the number of slots per millisecond under different SCS

    - the set includes all the slots except the following slot:

      - downlink slots if the sidelink transmission occurs in a TDD cell

      - Reserved slots

    - The remaining slots are arranged in increasing order of slot index

The duration in LTE-V2X contains TA adjustment, decoding DCI and preparing time at Tx UE before transmission. The parameter ‘4’ in the equation is determined due the UE processing capability in LTE-V2X, however, the configuration can be more flexible for NR. Considering the signaling overhead, a compact interval is needed in DCI to indicate configuration ‘4+m’. Therefore, considering the flexible numerology and UE capability in NR-V2X, the duration  should be supported, where  denotes the time unit in NR, is subcarrier spacing index, anddenotes the processing time.

Proposal 6:  The duration between receiving a dynamic grant and transmitting PSCCH and PSSCH should consider the flexible numerology and UE capability in NR-V2X.

In RAN1#96b, it was agreed that “a dynamic grant provides resources for one or multiple sidelink transmissions of a single TB”. In case of multiple sidelink transmissions, it is natural to allow the transmission to occur in non-consecutive slots to mitigate the half-duplex constraint. In this case, each transmission may be associated with its own PSFCH channel for feedback transmission if enabled. Similar to Mode 2 operation, if an ACK is received before the last transmission, UE can terminate the transmission early and not use the remaining resource for the retransmission, which can be saved for scheduling other UEs.

Proposal 7:  When dynamic grant provides multiple sidelink transmission resources for a single TB and HARQ feedback is enabled, early termination based on HARQ-ACK should be supported.

  - Configured grant scheduling

In RAN1 AH #1901, the following agreement was reached

Agreements:

- When NR Uu schedules NR SL mode 1, both type 1 and type 2 configured grants are supported for NR SL

Therefore, both configured grant type 1 and type 2 are supported for NR V2X mode 1 SL resource allocation. Importantly, the above agreement stipulates that both type 1 and type 2 configured grants, as specified in NR Uu, are supported for NR sidelink. This is so because configured grant type 1 and type 2 terminology has been recently introduced in NR Release 15 and did not exist in either LTE or LTE V2X. In our understanding, this implies that the main principles/features of NR Uu configured grant type 1 and type 2 are to be applied to NR SL. At least for type 1, we believe the main principle of NR configured grant is that the Tx UE is configured for data transmission through semi-static RRC signaling and that no resources for additional dynamic scheduling of data transmissions are included in the configured grant. This fundamental principle of CG type 1 resource allocation should be preserved when applying type 1 CG to NR SL, otherwise the previous agreement would not be respected. Moreover, in Uu, a CG by the gNB provides resources for transmission of PUSCH. Similarly, a configured grant for NR SL by the gNB shall provide resources for transmission of PSSCH. In addition, as per the RAN#84 guidance: “In order to expedite the V2X sidelink work, RAN1 is advised to reuse NR Uu design and/or LTE sidelink design to the greatest extent appropriate, unless there are strong reasons to deviate from these.” No such strong reasons to deviate from the NR Uu CG design have been identified. Furthermore, our simulation results show a significant benefit of SL transmission without SCI, and the necessity of such operation to meet the latency/reliability requirements.   Therefore we have the following proposal:

Proposal 8: A configured grant for NR SL by the gNB provides resources exclusively for data transmission by the UE on PSSCH:

- The working assumption agreed in RAN1#98b: “Each transmission in a resource provided by a configured grant contains PSCCH and PSSCH.” should not be confirmed.

For MCS selection, it is agreed in RAN1#98b that for both dynamic and configured grant, if a single MCS is configured, the MCS is used by the UE; while if no MCS or more than 1 MCSs are configured, UE autonomously selects the MCS from the full range of values or the configured values. Note that for configured grant Type 1, MCS is likely to be conservative since there is a need to maintain high reliability in order to meet the latency/reliability requirements of advanced V2X use cases. Besides, on the SL, interference rather than accurate adaptation to channel variations constitutes the main mitigating   factor, thus a semi-static selection of MCS by the Tx UE is enough, especially for the case of CG Type 1, where no SCI is needed. Therefore, for CG Type 1, Tx UE can select the MCS and send the selected MCS to Rx UE via PC5-RRC in case of unicast and group cast and periodic SCI in case of broadcast.

Proposal 9: In Type 1 CG, if no MCS or more than one MCS is configured, Tx UE selects a single MCS among the full range of MCS values or the configured MCS values and informs the Rx UE the selected MCS via PC5-RRC (for unicast and groupcast) or periodic SCI (for broadcast).

TB repetition not triggered by feedback, as well as frequency hopping are defined for NR Uu CG. Therefore, configured grant resource configurations should include at least time-frequency resource related information, as discussed in more detail in Section 2.2. For NR UL CG, the repetition is done in slots which are logically consecutive, i.e. a transmission is dropped if it is conflict with the UL/DL transmission direction configurations. Since in shared carriers the Uu transmissions will affect the available REs per slot, and the sidelink slot format indication may in general be in non-consecutive slots (whether flexible and/or uplink), this Uu constraint cannot apply to the sidelink. In sidelink, this has the added benefit of helping resolve persistent collisions and improving the reliability [4].

Proposal 10: For NR SL mode 1, repetition or blind retransmission in non-consecutive slots is supported.

In order for the gNB to optimize the mode-1 configured grant resource allocation while taking into consideration the sidelink half-duplex constraint, UE-specific TFRPs can be configured or indicated to the UEs for configured grant type 1 and type 2, respectively.

Proposal 11: TFRPs are configured via RRC for type-1 configured grant and indicated in DCI for configured grant type 2.

Latency, reliability and resource efficiency also need to be taken into account for the time-frequency resource pattern (TFRP) configuration. Each TB repetition can use the redundancy version according to (pre)configuration.

    - SL CG latency/reliability considerations

As all transmission parameters including scheduling information for configured grant type-1 resource allocation, and some information such as periodicity for configured grant type-2 resource allocation, are signaled to the Tx UE via semi-static RRC signaling, there is the opportunity to configure the Rx UE also via higher layer signaling instead of repeatedly indicating the same information in SCI for each TB, thereby saving non-negligible signaling overhead. This is all the more relevant because SCI can represent more than 50% overhead for certain vehicle platooning scenarios with relatively small packet sizes (50 bytes) including cooperative driving information exchange between a group of UEs supporting V2X application and reporting needed for platooning between UEs supporting V2X application and between a UE supporting V2X application and RSU [5]. In addition, for some advanced driving applications, such as emergency trajectory alignment between UEs supporting V2X application, it is required to deliver relatively large packet sizes (2000 bytes) within 3 ms latency and with very high reliability of 99.999%. For some other applications such as sensor information sharing between UEs supporting V2X applications, the same stringent latency and reliability requirements are expected to be met to deliver SL data rates as high as 50Mbps [5]. The primary target of configured grant schemes is to achieve high reliability within a small latency budget. Thus by saving the SCI overhead more repetitions or retransmissions can be allowed for configured grant transmissions compared with LTE V2X SPS solution (which uses SCI for each TB), when UE processing time latencies are taken into account. More repetitions or retransmissions within the small latency budget automatically translate into higher reliability. Also, for packet sizes of 2000 bytes and 50Mbps data rates too many radio resources would be wasted on SCI that would otherwise be used for data transmission in order to achieve the stringent latency and reliability requirements for the above referenced applications.

As a concrete example, consider two CG transmission schemes, namely scheme 1 (with standalone PSSCH) and scheme 2 (with associated SCI for each PSSCH), as illustrated in Fig. 2.a and Fig. 2b, respectively, and let us compare how these two schemes fare in terms of meeting the requirements for emergency trajectory alignment between UEs supporting V2X application, i.e. TB size of 2000 bytes, 3 ms latency and 99.999% reliability. Let us assume that 30kHz SCS and 48 RBs (~20 MHz) bandwidth are used by both schemes. Note that for slot based transmission, some symbols of a slot are not used for control or data transmission (i.e. 1 symbol for AGC, 2 DMRS symbols and 1 symbol for Tx/Rx switch). For Scheme 2, it is assumed that PSCCH and PSSCH are TDMed, and that SCI occupies two OFDM symbols of a slot. This is a rather optimistic assumption from a latency perspective for Scheme 2 in order to achieve the 99.999% reliability requirement for PSCCH.

Fig. 2a Scheme 1 with standalone PSSCH

Fig. 2b Scheme 2 with associated SCI for each PSSCH

Based on the above assumptions, our link level simulations show that fewer repetitions are needed to achieve 10^-5 reliability for PSSCH for Scheme 1 than for Scheme 2. Accordingly, Scheme 1 is able to meet the 3ms latency requirement with 4 blind retransmissions, whereas Scheme 2, which requires at least 5 repetitions to achieve the same level of reliability, is not.

Proposal 12: In order to meet the latency/reliability requirements of advanced NR V2X use cases, at least for SL configured grant type 1, no PSCCH is associated with PSSCH.

    - Rx UE configuration for SL CG

In order to configure Rx UEs for the reception of standalone PSSCH in the case of mode 1 CG, both RRC signaling conveyed from gNB through Uu interface or from Tx UE through PC5 interface can be considered, the latter option having the advantage of allowing to accommodate Rx UEs that are outside of gNB coverage. For Rx UEs which are under gNB coverage, either or both options can be supported, however, for the sake of unified design, configuring Rx UE through PC5 RRC is preferred.

PC5 RRC can be used to configure Rx UE(s) for unicast and groupcast scenarios. However, it is currently not an option for configuring Rx UEs in the case of broadcast scenario, as broadcast PC5 RRC is not supported in release 16. In this case, it is proposed that Tx UE sends a periodic SCI to configure Rx UEs for the broadcast scenario. Note that such periodic SCI need not be associated with any PSSCH. The configuration SCI needs to be periodic rather than being a one-shot SCI in order to allow for Rx UE(s) configuration update in situations where the Rx UEs within the Tx UE broadcast coverage change, as likely to be the case in a vehicular environment.

Proposal 13: For SL configured grant, Rx UE(s) are configured through PC5-RRC signaling for unicast and groupcast scenarios. A periodic SCI is used to configure Rx UEs in the case of broadcast scenario.

    - CG Type 1 resource configuration

In RAN1#96bis, it was agreed that configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. The set of resources correspond to a TFRP and which can be configured in a periodic manner. Therefore, the resource configuration should include the periodicity and starting time offset of the TFRP, and the size and locations of the time-frequency resources. Since repetition or blind retransmission has been agreed for NR V2X SL, a RV sequence to be used for repetitions should also be configured to the UE for SL CG transmissions. In addition, similar to NR UL CG, DMRS configuration and MCS should also configured to the UE for NR SL configured grant.

In RAN2#107, the following was agreed: “A mode-1 UE is allowed to continue using the configured SL grant type 1 when beam failure or physical layer problem in NR Uu occur. FFS how long the SL configured grant is considered valid.” While we assume that RAN2 will address the FFS in the agreement, the configuration for SL configured grant Type 1 needs to include a threshold as a form of indication to UEs on how long the configured grant can be used in case of NR Uu interruption.

Proposal 14: For SL configured grant Type 1 in Mode 1, the configuration includes at least the following content:

- An offset, periodicity, length, RV sequence, indication of time-frequency resources, DMRS configuration, retransmission configuration, MCS, and a threshold defining the validity of the grant in case of NR Uu interruption.

    - Multiple configured grant (CG) configurations for SL

In RAN2#96, it has been agreed to support multiple active sidelink configured grants. From RAN1 perspective, it should be clarified that multiple active configured grants are supported per SL BWP per cell, rather than e.g. having to be split among multiple cells. Furthermore, given that a resource pool (RP) may be used for dynamic scheduling, it is natural to assume that CG resources are also configured within RPs, where RPs are configured within the SL BWP. As multiple active configured grants can be used to support diverse packet sizes, QoS requirements, etc., multiple CGs per resource pool should be supported

Proposal 15: SL configured grant is configured within a resource pool. Multiple configured grant configurations per resource pool is supported.

    - CG HARQ Operation

In NR UL configured grant, multiple HARQ processes are supported and HARQ process ID is derived from the physical resources. A similar mechanism for determining HARQ process ID can be adopted for SL configured grant. The RV can be determined based on the RV sequence for repetitions that is configured to the UE.  In this case, an associated SCI to indicate the HARQ process ID and RV is not needed for SL CG transmissions.

Proposal 16: Similarly to NR UL configured grant, HARQ process ID of SL configured grant should be determined as a function of the physical resources.

If the Rx UE successfully receives a SL CG transmission (either the initial transmission or one of the retransmissions) of the TB before all the retransmissions have taken place, it would be efficient from a resource utilization perspective to inform the Tx UE so that retransmissions can be terminated.

Unlike NR UL configured grant transmission, the repetition resources for SL CG should not take place in consecutive slots, due to HD issue. Given the potential time gap between different repetitions of the same TB, there is a strong motivation for the Tx UE to terminate the blind retransmission early in the event of receiving an ACK. Therefore, Rx UE should send an ACK to the Tx UE if it successfully decodes the TB before the last blind retransmission.  In case all the blind retransmissions are needed, the Rx UE will send ACK/NACK after the last of them, as usual.

For groupcast transmission, the blind retransmission may be terminated if ACKs from all Rx UEs in the group are received by Tx UE.

Proposal 17: For a UE configured/scheduled with repetitions of a TB, early termination based on ACK received from Rx UE should be supported. If NACK is received after the last repetition, then Tx UE can retransmit the same TB using the CG resources.

In RAN1#96b, it was agreed that  “ Configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. UE decides which TB to transmit in each of the occasions indicated by a given configured grant.” In RAN1#98b, it was further agreed that “For a configured grant in Mode 1 when using SL HARQ feedback: There is only one HARQ-ACK bit for the configured grant; There is one PUCCH transmission occasion after the last resource in the set of resources provided by a configured grant.” Note that for dynamic grant, the multiple resources scheduled by a DCI is for multiple transmissions of a single TB. As only 1 PUCCH occasion is available for transmissions within a periodicity of configured grant, if multiple TBs are transmitted within one period of the configured grant, it is not clear to which TB the PUCCH will correspond. Since the PUCCH resource is configured to report sidelink HARQ feedback, which means if the Tx UE receives a NACK for the TB, it can report to the gNB such that gNB may schedule a retransmission for the TB. If UE receives multiple NACKs for multiple TBs in a periodicity of a configured grant, then it is not clear which TB the PUCCH is associated with. Even if we assign a specific TB (e.g. the last TB of the period) for PUCCH reporting, there is no mechanism to let gNB know there are NACKs received for other TBs transmitted in the period. Therefore, there should be some constraints on how UE can decides which TB to transmit within a periodicity of configured grant to make the current agreements work. If UE only transmits one TB within a periodicity, which is the case in Uu UL CG, then UE can report the HARQ feedback of that TB in the associated PUCCH resource. On the other hand, if UE transmits multiple TBs within a periodicity, but all the TBs transmitted other than the last one have been acknowledged, then UE can just use the PUCCH resource to report NACK for the last TB of the transmissions within the periodicity.

Proposal 18: Configured grant provides K resources within one periodicity, select one of the following options to clarify how “UE decides which TB to transmit in each of the occasions indicated by a given configured grant”:

- Option 1: UE can only transmit one TB on transmission occasions within one period, but UE can choose to retransmit the same TB on transmission occasions in a different period.

- Option 2: UE can choose to transmit a new TB on transmission occasions in one period only if it receives ACKs on all previous TBs transmitted within the period.

In RAN1#98b, it was also agreed that “Two different UE-specific SL RNTIs are introduced for Mode-1 scheduling: One for CRC scrambling in DCI for a dynamic grant and the other one for CRC scrambling in DCI for a configured grant type-2”. However, considering that PUCCH resource for reporting SL HARQ feedback is agreed to be supported in CG in meeting #98b, it is reasonable to assume UE reports NACK so that it can be scheduled by the gNB for retransmission. Similar to the NR Uu case, retransmission scheduling by gNB for SL CG transmission can be supported after reporting the HARQ feedback to gNB. In order to differentiate the retransmission for dynamic grant and configured grant for SL, the SL RNTI for CG (e.g. SL CS-RNTI) can also be used for retransmission scheduling. For Type 2 CG, UE differentiates whether the DCI addressed to SL CS-RNTI is an activation/deactivation DCI or a retransmission DCI through NDI value. In addition, similarly to NR Uu, Type 1 and Type 2 CG can share the same RNTI. Therefore, we have the following proposals:

Proposal 19: A UE specific SL CS-RNTI that is different than the RNTI used for SL dynamic grant is configured in RRC for CG in SL Mode 1:

- The same SL CS-RNTI is used for both Type 1 and Type 2 CG;

- For Type 1 CG, SL CS-RNTI is used for CRC scrambling of retransmission DCI;

- For Type 2 CG, SL CS-RNTI is used for CRC scrambling of activation, deactivation and retransmission DCI.

When UE receive a retransmission grant for CG transmission from the gNB, UE needs to be able to identify which TB the retransmission grant is targeting.  This can be done by using the HARQ process ID provided in the retransmission DCI, where UE can assume the retransmission is for the last TB of the period that is mapped to the HARQ process ID indicated in DCI. However, it is possible the UE may choose to use a different HARQ process ID in SCI as long as the same HARQ process ID is used for the same TB.

Proposal 20: When UE receives a retransmission grant addressed by SL CS-RNTI:

- UE assumes the retransmission grant is for the TB associated with the last CG transmission of the period that is mapped to the HARQ process ID indicated in the DCI.

  - DCI format

In LTE-V2X, DCI format 5A is used for scheduling PSSCH and PSCCH transmission, as shown in Table 2.

Table 2. LTE DCI format 5A contents.

| Field | Size |
| --- | --- |
| Carrier indicator | 3 bits |
| Lowest index of the subchannel allocation to the initial transmission | < 5 bits (for up to 20 sub-channels) |
| SCI format 1 field - Frequency resource location of initial transmission and retransmission | < 8 bits (for up to 20 sub-channels) |
| SCI format 1 field - Time gap between initial transmission and retransmission | 4 bits |
| SL index (for TDD configuration 0-6) | 2 bits |
| SL SPS configuration index (for SPS only) | 3 bits |
| Activation/release indication (for SPS only) | 1 bit |

In NR-Uu, two formats DCI 0-0 and DCI 0-1 as shown in Table 3 are defined for scheduling of PUSCH.

Table 3. NR DCI contents for formats 0-0 and 0-1.

| Field |  | Format 0-0 | Format 0-1 |
| --- | --- | --- | --- |
| Identifier |  | 1 bit | 1 bit |
| Resource information | CFI | – | 0 or 3 bits |
|  | UL/SUL | 0 or 1 bit | 0 or 1 bit |
|  | BWP indicator | – | 0-2 bits |
|  | Freq.-domain allocation | Variable (only type 1) | variable |
|  | Time-domain allocation | 0-4 bits | 0-4 bits |
|  | Frequency hopping | 0 or 1 bit | 0 or 1 bit |
| TB-related | MCS | 5 bits | 5 bits |
|  | NDI | 1 bit | 1 bit |
|  | RV | 2 bis | 2 bis |
| HARQ-related | Process number | 4 bits | 4 bits |
|  | DAI | – | 1-4 bits |
|  | CBGTI | – | 0, 2, 4, or 6 bits |
| Multi-antenna related | DMRS sequence initialization | – | 1 bit |
|  | Antenna ports | – | 2-5 bits |
|  | SRI | – | Variable |
|  | Precoding information | – | 0-6 bits |
|  | PTRS-DMRS assoc. | – | 0 or 2 bits |
|  | SRS request | – | 2 bits |
|  | CSI request | – | 0-6 bits |
| Power control | PUSCH power control | 2 bits | 2 bits |
|  | Beta offset | – | 0 or 2 bits |

In NR-V2X, the sidelink scheduling DCI will need to have additional fields compared to LTE-V2X due to the new features such multiple transmission of a single TB or sidelink HARQ reporting from Tx UE to gNB. Since neither the current NR-Uu DCI formats nor LTE DCI format 5A can provide complete information for scheduling NR-V2X, a new DCI format is needed to convey dynamic grants and configured grant type-2. The dynamic grant and configured grant type-2 DCIs are conveniently distinguished by different RNTIs. The detailed DCI contents are shown as following:

- Resource pool indicator: indicate a dedicated Tx resource pool

- Resource indicator: indicate the time and frequency resource

- PUCCH resource for reporting sidelink HARQ ACK/NACK feedback:

  - PSFCH-to-HARQ_feedback timing indicator: indicate the time interval between receiving the HARQ feedback from Rx UE and reporting the sidelink HARQ feedback to gNB

  - PUCCH-SL resource indicator: indicate sidelink HARQ feedback frequency resource on PUCCH

- HARQ related information:

  - HARQ process number

  - NDI

  - RV

  - DAI

- SL CG configuration index (for configured-grant type 2 only)

- Activation/release indication (for configured-grant type 2 only)

Proposal 21: A new DCI format is needed to convey dynamic grants and configured grant type-2.

  - Uu-based sidelink interference coordination

In order to ensure low interference in NR SL transmission mode 1, gNB can exploit the knowledge of UE geographic location (e.g., obtained via GPS and reported periodically by the UE to the network). Reuse of a time-frequency resource is then possible whenever UEs are sufficiently far apart. This is equivalent to imposing a minimum reuse distance. The disadvantage of this location-based reuse strategy is that it does not take into account the actual physical propagation of waves (i.e., the wireless channel). For example, whereas two vehicles on the highway may need to be a few miles away to transmit on the same resource with negligible interference, the situation may be very different in an urban environment where buildings shield most interference between nearby parallel streets. Finally, Rel-14 LTE sidelink focused on broadcast traffic. However, Rel-16 NR sidelink needs to support unicast, multicast/groupcast, and broadcast. While receiver feedback in case of broadcast is not a feasible solution due to potentially large number of receivers, for unicast and broadcast it might provide benefits when it comes to resource selection.

To support Mode 1 resource allocation, a UE (e.g., when requesting resources) may report measurements such as S-RSSI observed in a given resource or set of resources. Alternatively, the UE may report a set of best or preferred resources. Based on this information, the gNB sidelink scheduler can schedule sidelink transmissions in an interference-aware fashion.

Proposal 22: UEs may report measurements, or information derived from such measurements (e.g., preferred resources), to support sidelink scheduler.

  - Interference-aware sidelink scheduling

Physical sidelink resources can be used more efficiently if interference between nearby UEs can be mitigated. One way to avoid sidelink interference is e.g. by using multi-antenna transmission and/or reception techniques at the UEs, when corresponding sidelink measurements can be done by the UEs. In order that the UEs’ capability to mitigate sidelink interference can be considered by the gNB scheduler, this information needs to be reported by the UEs to the gNB. Then, the sidelink scheduler can schedule interference-aware sidelink transmissions potentially on the same resources, thus increasing resource reuse.

Figure 3. Interference suppression by receiving (left) and transmitting (right) UE. Reporting to gNB information on the ability to suppress sidelink interference can be used for link scheduling.

As also shown in Fig. 3, when UE i requests a sidelink resource for transmission to UE j, it may report to the gNB a list of one or more nearby UEs m toward which it can limit interference e.g. under a certain threshold when transmitting to UE j on the particular resource. Similarly, on the receive side, UE l may report a list including one or more nearby UEs n from which it can suppress interference when receiving from UE k on the particular resource.

Proposal 23: UEs report to the gNB a list of nearby UEs to/from which they can mitigate sidelink interference on particular resources, in order to assist gNB scheduling and resource allocation.

- SL HARQ feedback to gNB

During last meeting, it was agreed that the Rel-15 procedure and signaling for DL HARQ-ACK to select PUCCH offset/resource and format in UL is reused in sidelink for dynamic grant and configured grant type-2, while it is still open to discuss how to indicate the timing of PUCCH transmission.

In NR Uu, for the dynamic scheduling of PDSCH, a UE is required to provide HARQ-ACK information in the slot n+k, when the UE finishes PDSCH reception in slot n. The value k, namely the slot offset between PDSCH reception and PUCCH transmission, is provided in high layer parameter dl-DataToUL-ACK and indicated by the PDSCH-to-HARQ-timing-indicator field in DL grant.  Moreover, if the same procedure and signaling are reused for sidelink, how to interpret the k value, in another word, the slot offset should be start from which transmission to PUCCH.

Fig. 4. UE reports SL HARQ-ACK information to gNB

As shown in Fig. 4, option 1 and option 2 can be illustrated as T0 and T3 respectively. Option 1 is simple and straightforward, however, it is really a quite long time. It includes the slot offset between DCI and PSSCH/PSCCH, time gap between PSSCH and PSFCH, the Tx UE preparation time to transmit PUCCH and even the length of PSSCH and PSFCH. When the gNB indicate the k using option 1, a large time range is needed to cover which resulting in consuming large amount of bits in the DCI. On the contrary, following the previous agreements, the gap T1 between DCI scheduling sidelink transmission and the PSSCH is indicated by DCI, PSFCH feedback timing T2 is derived base on sidelinke k configuration which (pre-) configured per resource pool. Therefore, gNB only needs to indicate a timing from the PSFCH slot, both UE and gNB would figure out the slot to transmit and receive the PUCCH with SL HARQ-ACK. Furthermore, the time range is much smaller than the one in option 1, and much less bits are needed as well.

In case DCI schedules multiple repetitions of a single TB, then each repetition may have its own PSFCH resource. In order to save PUCCH resources for SL feedback reporting to gNB, only one PUCCH resource can be indicated for the multiple repetitions. Then the slot offset indicated in the DCI corresponds to the slot offset between PSFCH of the last repetition and PUCCH.

For configured grant, it is agreed that RRC is used to configure PUCCH offset/resource and format in UL in CG Type 1. It is also agreed one PUCCH resource is configured per periodicity. Therefore, the slot offset between the last PSFCH resource corresponds to the last transmission occasion within a periodicity and PUCCH should be indicated.

As a result, following proposals define the timing of PUCCH used for conveying SL HARQ-ACK to gNB.

Proposal 24: A slot offset between the transmissions of PSFCH and PUCCH is indicated in DCI to determine the timing for PUCCH used for carrying SL HARQ to gNB. In case of multiple repetitions of a single TB, the slot offset corresponds to the slot offset between the PSFCH resource of the last repetition and PUCCH.

Proposal 25: For configured grant, PUCCH resources share the same periodicity as the CG resources and a slot offset between the transmissions of last PSFCH and PUCCH is indicated in RRC for Type 1 CG and activation DCI for Type 2 CG.

Besides timing of sidelink HARQ reporting from Tx UE to gNB, the PUCCH resource indicator field in DCI to indicate frequency resource for HARQ feedback in NR Uu can also be reused for sidelink HARQ feedback.

Proposal 26: DCI contains a field of PUCCH-SL resource indicator to indicate the PUCCH frequency resource for sidelink HARQ feedback.

Considering the uniform design and spectrum efficiency, multiplexing the sidelink HARQ report with Uu link UCI transmission should be supported. Both dedicated resources for SL HARQ feedback, and joint transmission with Uu link UCI transmission need to be supported. Since all of the resources for HARQ feedback are allocated by gNB, no matter for Uu or sidelink, thus gNB can differentiate the PUCCH resource to be used for feedback from Uu and sidelink.

Proposal 27: For the sidelink HARQ ACK/NACK report from transmitter UE to gNB, both dedicated resources and joint transmission with Uu link UCI transmission need to be supported.

If dedicated resources for sidelink HARQ are supported, then UE should be allowed to transmit PUCCH containing Uu ACK/NACK as well as SL ACK/NACK in the same slot, otherwise Uu scheduling will be restricted. However, it is difficult to avoid PUCCH resources colliding between two such PUCCHs depending on gNB scheduling. The UE cannot transmit both if PUCCH resources are in conflict. In order to solve the problem, the UE can either drop one of them or multiplex them together. Dropping PUCCH for sidelink or PUCCH for Uu can depend on the priority of sidelink HARQ feedback, which can be the same as that of the corresponding sidelink data. If the UE multiplexes sidelink HARQ and Uu UCI, then both sidelink HARQ and Uu UCI can be transmitted.

Proposal 28: For the sidelink PUCCH resource and Uu PUCCH resource collision, dropping rules should be defined depending on the priority of sidelink HARQ.

- Enhancements for NR SL resource pool configuration

  - Reducing the impact of high mobility on sidelink resource allocation

When reserving periodic resources for a certain resource reselection period, the selected resources may quickly experience higher interference than expected at the time of resource selection. For example, on a two-way street, a quickly approaching vehicle may soon (i.e., within the resource reselection period) cause interference at a receiver, which the transmitter could not have predicted at the time of resource selection.

Observation 1: UE motion may adversely impact the performance of sidelink resource allocation.

To reduce the impact of high mobility on the performance of sidelink resource allocation, the gNB may configure a set of UE motion vector classes (e.g., velocity vectors) and associated resource pools. Based on this configuration, the UE may determine the class it belongs to by comparing its current state of motion with the configured classes. The UE may then select a resource pool based on the class it belongs to.

Figure 5 shows a motion-based resource pool configuration example in a highway scenario. In this example, the network configures four motion classes (slow and fast lanes in each direction) and allocates orthogonal resource pools to each motion class.

Figure 5. Motion-based resource pool configuration example (highway scenario).

In order to prevent the loss of spectral efficiency as a result of partitioning (i.e., due to pool underutilization), the size of the resource pool(s) assigned to a given class may be adapted based on the number of vehicles in the class and/or their current traffic demand.

In case of GF (type-1 configured grant) transmissions, in a highway scenario, one example of assigning TFRPs inside the resource pools is as follows: odd slots may only be used by eastbound UEs, whereas even slots may only be used by westbound UEs. The TFRPs are then defined so that an “eastbound TFRP” consists only of odd slots, whereas a “westbound TFRP” consists only of even slots. Such a “motion-aware TFRP” may be used for periodic or aperiodic traffic and it can be reconfigured depending on the traffic characteristics in each direction.

Proposal 29: Configuration of resource pools based on e.g. direction or speed of travel of UEs that use the pool is supported.

- LTE Uu Controlling NR SL

When NR sidelink mode-1 is controlled by gNB, dynamic scheduling, and configured grants of type 1 and type 2 are supported. When controlled by eNB, neither dynamic scheduling nor type 2 configured grant are supported, and the support of NR sidelink mode-1 is based on type 1 configured grants with some simplifications.

  - Configurations for NR mode-1 SL

    - NR Uu configured grant configuration

The design of NR sidelink structure and procedure is continuing to be discussed, and more agreements are still needed to make how NR sidelink mode-1 works clear. In the meantime, NR Uu configured grant configuration is a suitable a starting point to discuss the details of configuration for NR sidelink mode-1 type 1 by LTE Uu.

The configuration for the configured grant is as follows [6]:

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2).

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                		   OPTIONAL,   -- Need S,

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                    INTEGER (1..64)                                                     OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant               SEQUENCE {

timeDomainOffset                        INTEGER (0..5119),

timeDomainAllocation                    INTEGER  (0..15),

frequencyDomainAllocation               BIT STRING (SIZE(18)),

antennaPort                             INTEGER (0..31),

dmrs-SeqInitialization                  INTEGER (0..1)                                                  OPTIONAL,   -- Need R

precodingAndNumberOfLayers              INTEGER (0..63),

srs-ResourceIndicator                   INTEGER (0..15)                                                 OPTIONAL,   -- Need R

mcsAndTBS                               INTEGER (0..31),

frequencyHoppingOffset                  INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                   OPTIONAL,   -- Need R

pathlossReferenceIndex                  INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}                                                                                                           OPTIONAL,       -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

According to the agreement, the configuration from LTE Uu for NR sidelink mode-1 type 1 is restricted to time/frequency resources and periodicity. Hence, the green highlighted parameters of the NR Uu configured grant are needed.

Proposal 30: The function LTE Uu configuration of NR sidelink mode-1 type 1 is supported by the following parameters:

- resourceAllocation

- periodicity

- configuredGrantTimer

- timeDomainOffset

- timeDomainAllocation

- frequencyDomainAllocation

- frequencyHoppingOffset

    - More necessary configuration needed for NR mode-1 SL

The configuration restricted to time/frequency resources and periodicity aims to simplify/facilitate network management and/or UE implementation, based on an assumption that it is challenging for an eNB to obtain dynamic information about the NR sidelink without introducing new feedback information over LTE Uu and changes in eNB schedulers. Thus the agreement assumes the LTE network provides time and frequency domain resources, and leaves the selection and adaption of transmission schemes and parameters to sidelink transmission UEs, e.g., MCS/TBS, etc, in a way similar to how the LTE sidelink works, even in mode 3.

However, if some parameters as cyan highlighted from NR Uu configured grant configuration, e.g., , dmrs-Seqinitialization, etc, are up to sidelink transmission UE choice, such parameters need to be indicated dynamically on the NR sidelink, which would unnecessarily increase the size of control information, whether carried by SCI or DMRS. Therefore, such parameters would be better to be configured by LTE Uu configuration as well. The detailed parameters to be configured can be determined after the design of NR sidelink configured grant type 1 is complete.

Proposal 31: In addition to the parameters related to time/frequency resources & periodicity, some other parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1. The detailed parameters to be configured can be determined after the design of NR sidelink configured grant type 1 is complete.

In addition, RAN2 has agreed at RAN2#105bis [7] that multiple active configured sidelink grants are supported in NR sidelink mode 1. It is natural that NR sidelink mode 1 operation supports the same  number of type 1 configured grants when under LTE Uu control.

Proposal 32: NR SL mode 1, when controlled by LTE Uu, supports the same number of Type 1 configured sidelink grants as when controlled by NR Uu.

  - Configurations for NR mode-2 SL

Regarding NR mode-2 SL, it was agreed that mode 2 supports the sensing and resource (re)-selection procedures and FFS resource granularity for sensing and resource (re)-selection, e.g., PRB(s), slots, resources patterns, etc. Therefore, the resources for NR mode-2 SL sensing and resource (re)-selection procedures will be configured by gNB.

RAN2 has agreed at RAN2#105bis [3] that:

- LTE Uu supports configuration for NR mode-2 operation through RRC signaling and through SIB signaling. However, on demand or per valid area mode-2 configuration via SIB signaling is not supported.

- New system information block type should be designed to support NR sidelink Tx and Rx resource pool configuration via LTE Uu. New system information will be defined as container (OCTET STRING) and actual information will follow what is defined in NR RRC.

- eNB should be able to configure the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

More agreements in RAN1 are needed for NR sidelink mode-2 operation regarding sensing and resource (re)-selection working in the control of NR Uu. According to the RAN2 agreement, that new system information block will be designed to support NR sidelink Tx and Rx resource pool configuration and it will be defined as a container, and follow what is defined in NR RRC. Hence, for NR V2X mode 2 sidelink resource pool configuration via dedicated signaling, the signaling design should follow what is defined in NR dedicated signaling.

Proposal 33: For NR V2X mode 2 sidelink resource pool configuration via dedicated RRC signaling from LTE Uu, information contained in the dedicated signaling will follow what is defined in NR RRC dedicated signaling.

- Conclusions

In this contribution, we discussed NR V2X sidelink resource allocation by NR Uu and LTE Uu. The discussion and analysis lead to the following observation and proposals:

Proposal 1: To meet latency and reliability requirements, NR V2X mode 1 dynamic grant should support at least 4 transmissions of a single TB.

Proposal 2: DCI/SCI should include a time-frequency index indicating resources for all repetitions of a single TB:

- Time domain repetition pattern can be indicated using a bitmap or an index of all potential time domain patterns within a given time window;

- Frequency domain repetition pattern can be indicated using a sub-channel based frequency pattern.

Proposal 3: In mode 1, whether HARQ ACK/NACK feedback from Rx UE to Tx UE is supported is implicitly indicated by whether the resource for reporting sidelink HARQ ACK/NACK from Tx UE to gNB is allocated in the DCI.

Proposal 4: For HARQ based (re-)transmission, when both mode 1 and mode 2 resource allocation are configured simultaneously for a UE, HARQ process usage needs to have a common understanding between gNB and Tx UE for each resource allocation mode.

Proposal 5: Tx resource pool ID is indicated in DCI to support flexible Tx resource pool selection within a SL BWP.

Proposal 6:  The duration between receiving a dynamic grant and transmitting PSCCH and PSSCH should consider the flexible numerology and UE capability in NR-V2X.

Proposal 7:  When dynamic grant provides multiple sidelink transmission resources for a single TB and HARQ feedback is enabled, early termination based on HARQ-ACK should be supported.

Proposal 8: A configured grant for NR SL by the gNB provides resources exclusively for data transmission by the UE on PSSCH:

- The working assumption agreed in RAN1#98b: “Each transmission in a resource provided by a configured grant contains PSCCH and PSSCH.” should not be confirmed.

Proposal 9: In Type 1 CG, if no MCS or more than one MCS is configured, Tx UE selects a single MCS among the full range of MCS values or the configured MCS values and informs the Rx UE the selected MCS via PC5-RRC (for unicast and groupcast) or periodic SCI (for broadcast).

Proposal 10: For NR SL mode 1, repetition or blind retransmission in non-consecutive slots is supported.

Proposal 11: TFRPs are configured via RRC for type-1 configured grant and indicated in DCI for configured grant type 2.

Proposal 12: In order to meet the latency/reliability requirements of advanced NR V2X use cases, at least for SL configured grant type 1, no PSCCH is associated with PSSCH.

Proposal 13: For SL configured grant, Rx UE(s) are configured through PC5-RRC signaling for unicast and groupcast scenarios. A periodic SCI is used to configure Rx UEs in the case of broadcast scenario.

Proposal 14: For SL configured grant Type 1 in Mode 1, the configuration includes at least the following content:

- An offset, periodicity, length, RV sequence, indication of time-frequency resources, DMRS configuration, retransmission configuration, MCS, and a threshold defining the validity of the grant in case of NR Uu interruption.

Proposal 15: SL configured grant is configured within a resource pool. Multiple configured grant configurations per resource pool is supported.

Proposal 16: Similarly to NR UL configured grant, HARQ process ID of SL configured grant should be determined as a function of the physical resources.

Proposal 17: For a UE configured/scheduled with repetitions of a TB, early termination based on ACK received from Rx UE should be supported. If NACK is received after the last repetition, then Tx UE can retransmit the same TB using the CG resources.

Proposal 18: Configured grant provides K resources within one periodicity, select one of the following options to clarify how “UE decides which TB to transmit in each of the occasions indicated by a given configured grant”:

- Option 1: UE can only transmit one TB on transmission occasions within one period, but UE can choose to retransmit the same TB on transmission occasions in a different period.

- Option 2: UE can choose to transmit a new TB on transmission occasions in one period only if it receives ACKs on all previous TBs transmitted within the period.

Proposal 19: A UE specific SL CS-RNTI that is different than the RNTI used for SL dynamic grant is configured in RRC for CG in SL Mode 1:

- The same SL CS-RNTI is used for both Type 1 and Type 2 CG;

- For Type 1 CG, SL CS-RNTI is used for CRC scrambling of retransmission DCI;

- For Type 2 CG, SL CS-RNTI is used for CRC scrambling of activation, deactivation and retransmission DCI.

Proposal 20: When UE receives a retransmission grant addressed by SL CS-RNTI:

- UE assumes the retransmission grant is for the TB associated with the last CG transmission of the period that is mapped to the HARQ process ID indicated in the DCI.

Proposal 21: A new DCI format is needed to convey dynamic grants and configured grant type-2.

Proposal 22: UEs may report measurements, or information derived from such measurements (e.g., preferred resources), to support sidelink scheduler.

Proposal 23: UEs report to the gNB a list of nearby UEs to/from which they can mitigate sidelink interference on particular resources, in order to assist gNB scheduling and resource allocation.

Proposal 24: A slot offset between the transmissions of PSFCH and PUCCH is indicated in DCI to determine the timing for PUCCH used for carrying SL HARQ to gNB. In case of multiple repetitions of a single TB, the slot offset corresponds to the slot offset between the PSFCH resource of the last repetition and PUCCH.

Proposal 25: For configured grant, PUCCH resources share the same periodicity as the CG resources and a slot offset between the transmissions of last PSFCH and PUCCH is indicated in RRC for Type 1 CG and activation DCI for Type 2 CG.

Proposal 26: DCI contains a field of PUCCH-SL resource indicator to indicate the PUCCH frequency resource for sidelink HARQ feedback.

Proposal 27: For the sidelink HARQ ACK/NACK report from transmitter UE to gNB, both dedicated resources and joint transmission with Uu link UCI transmission need to be supported.

Proposal 28: For the sidelink PUCCH resource and Uu PUCCH resource collision, dropping rules should be defined depending on the priority of sidelink HARQ.

Proposal 29: Configuration of resource pools based on e.g. direction or speed of travel of UEs that use the pool is supported.

Proposal 30: The function LTE Uu configuration of NR sidelink mode-1 type 1 is supported by the following parameters:

- resourceAllocation

- periodicity

- configuredGrantTimer

- timeDomainOffset

- timeDomainAllocation

- frequencyDomainAllocation

- frequencyHoppingOffset

Proposal 31: In addition to the parameters related to time/frequency resources & periodicity, some other parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1. The detailed parameters to be configured can be determined after the design of NR sidelink configured grant type 1 is complete.

Proposal 32: NR SL mode 1, when controlled by LTE Uu, supports the same number of Type 1 configured sidelink grants as when controlled by NR Uu.

Proposal 33: For NR V2X mode 2 sidelink resource pool configuration via dedicated RRC signaling from LTE Uu, information contained in the dedicated signaling will follow what is defined in NR RRC dedicated signaling.

Observation 1: UE motion may adversely impact the performance of sidelink resource allocation.

References

- RP-190984, “Revised WID on 5G V2X with NR sidelink”, LG Electronics, RAN#84, Newport Beach, USA, June 2019.

- Chairman’s Notes, 3GPP RAN1 #96bis.

- Chairman’s Notes, 3GPP RAN1 #97.

- R1-1911884, “Sidelink resource allocation mode 2 for NR V2X”, Huawei, HiSilicon, RAN1#99, Reno, USA, Nov. 18-22, 2019.

- TS 22.186 v16.1.0, “Enhancement of 3GPP support for V2X scenarios; (Release 16)”.

- TS 38.331, “NR; Radio Resource Control (RRC) protocol specification (Release 15)”.

- RAN2 Chair notes, RAN2#105bis.

- R1-1906010, “Sidelink resource allocation mode 1”, Huawei, HiSilicon, RAN1#97, Reno, USA, May 2019.