3GPP TSG RAN WG1 Meeting #99	R1-1911897

Reno, USA, November 18-22, 2019

Agenda Item:	7.2.10.2

Source:	Huawei, HiSilicon

Title:	Remaining issues on SRS for NR positioning

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues on SRS for positioning purposes. Some of the related physical layer procedures are covered in our companion contribution [5].

Discussion

Frequency hopping for positioning SRS

Working assumption [1]: Frequency hopping of SRS for positioning is not supported.

As discussed in our RAN1 98b contribution [6], the usefulness of SRS frequency hopping for positioning purposes is questionable. As such, we propose to confirm the above working assumption.

Proposal 1: Frequency hopping of SRS for positioning is not supported (i.e., agree on the working assumption in RAN1 #98b).

Assuming that the above working assumption will be agreed, the fields related to the SRS frequency hopping were removed from the list of higher layer parameters in RAN1 98b [7]. The removed fields are b-SRS, b-hop, repetitionFactor, and freqDomainPosition (nRRC). Note that these are mandatory fields in SRS configuration. Therefore, to be able to reuse the Rel. 15 SRS mapping to physical resources description that is provided in Clause 6.4.1.4.3 of 38.211 also for SRS for positioning, UE should assume some values when the above fields are not configured. The value of zero should be assumed for freqDomainPosition when this field is not configured. For the sake of simplicity, the assumed values of b-SRS and b-hop can also be zero as,  when bhop≥BSRS,  frequency hopping is disabled. Finally, the assumed value for repetitionFactor should be equal to nrofSymbols NsymbSRS as, in this case, intra-slot frequency hopping is disabled.

Proposal 2: Following behavior is supported:

If b-SRS is not configured, UE assumes b-SRS is equal to zero.

If b-hop is not configured, UE assumes b-hop is equal to zero.

If freqDomainPosition (nRRC) is not configured, UE assumes freqDomainPosition is equal to zero.

If repetitionFactor is not configured, UE assumes repetitionFactor is equal to nrofSymbols.

SRS for positioning resource mapping

Agreement [1]: The supported relative RE offsets for combinations of the number of symbols (1,2,4,8,12) and comb sizes (2,4,8) for SRS are given in the table below. At least one pattern for each pair are to be selected from the following options: Number of symbols / Comb size; 1; 2; 4; 8; 12. 2; {0}; {0, 1}; FFS: {0, 1, 0, 1} and/or {0, 0, 1, 1}; N/A; N/A. 4; N/A; {0, 2}; FFS: {0,2,1,3} and/or {0,1,2,3} and or {0,2,3,1}; FFS: {0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3} and/or {0,0,1,1,2,2,3,3}; FFS: {0,2,1,3, 0,2,1,3, 0,2,1,3} and/or {0,1,2,3, 0,1,2,3,0,1,2,3} and/or {0,0,0,1,1,1,2,2,2,3,3,3}. 8; N/A; N/A; N/A; FFS: {0,4,2,6,1,5,3,7} and/or {0,4,1,5,2,6,3,7} and/or {0,1,2,3,4,5,6,7} and/or {0,4,6,2,5,7,3,1}; N/A.

In RAN1 98b, we agreed on the set of supported pairs of number of symbols NsymbSRS and comp size KTC for SRS for positioning. For each pair, up to 4 resource mapping patterns were listed for further study. It was agreed that at least one pattern for each pair should be selected.

Figure 1: Three candidate Rel. 16 positioning SRS resource mappings for NsymbSRS=8 and KTC=4 multiplexed with a non-staggered Rel. 15 SRS with NsymbSRS=4 and KTC=4.

As an example, Figure 1 shows all three agreed resource mapping candidates for NsymbSRS, KTC=8,4.  Each of the three resource mapping candidates has a justification:

- Non-staircase resource mapping (left sub-figure of Figure 1): In Figure 1, non-staircase resource mapping is given by the transmission comb offset of {0, 2, 1, 3, 0, 2, 1, 3}. The proponents of this resource mapping method refer to the fact that its first few symbols have the best effective comb. As such, if only the first few symbols are considered for the ToA measurement, the effect of the aliased peaks is better supressed.  For instance, the first two symbols of the positioning SRS in the left sub-figure of Figure 1 provide an effective comb of 2. In our opinion, such a property may be an advantage for a DL PRS but is hardly an advantage for the positioning SRS that is an UL transmission: UE has to transmit all configured NsymbSRS symbols anyway and it is only up to the network to decide whether or not to consider the first few symbols for positioning measurements. In the case that the network decides to only consider the first few symbols, the time that is saved in the PHY layer by ignoring the last few SRS symbols is minuscule compared to the overall latency of the positioning procedure that is almost entirely governed by the higher layers latency (transmission of the positioning measurements to LMF and the aggregation of these measurements to calculate the UE position and providing the UE position to the positioning client).

- Staircase resource mapping (middle sub-figure of Figure 1): In Figure 1, staircase resource mapping is given by the transmission comb offset of {0, 1, 2, 3, 0, 1, 2, 3}. This is a simple conventional staggering pattern that can be easily formulated in specification and possibly expanded in future releases to other values of NsymbSRS, KTC pairs.

- Group-bundled resource mapping (right sub-figure of Figure 1): In Figure 1, group-bundled resource mapping is given by the transmission comb offset of {0, 0, 1, 1, 2, 2, 3, 3}. Similar to the other two resource mapping methods, this method also provides the full staggering pattern. In addition, it has the advantage of a better collision-free multiplexing with Rel. 15 SRS. As can be observed from Figure 1, this mapping is the only mapping that can be FDMed with a Rel. 15 SRS with NsymbSRS, KTC=4,4 without an RE collision. In fact, irrespective to the starting symbol and transmission comb offset ∈0,1, 2, 3 of the Rel. 15 SRS with NsymbSRS, KTC=4,4, it has some colliding REs with both the non-staircase resource mapping and the staircase resource mapping. Additionally, there are other advantages of having two consecutive symbols with the same RE offset within a staggered SRS resource. For instance, it can be observed from the right sub-figure in Figure 1 that, starting from the third SRS symbol (which falls in the last 6 symbols of the slot used for Rel. 15 SRS usages), every two consecutive SRS symbols can concurrently be used as a two-symbol (NsymbSRS=2 ) SRS resource with KTC=4 for Rel. 15 usages. Note that sending a dual intent SRS for the Rel. 16 positioning and Rel. 15 SRS usages substantially reduces the uplink RS overhead and preserves UE power and makes the UL-based (UL+DL-based) positioning solutions more attractive candidates for implementation in a real world scenario.

Similar advantages as above also hold for group-bundled resource mapping with the transmission comb offset of 0, 0, 0, 1, 1, 1, 1, 2, 2, 2, 3, 3, 3 for the case NsymbSRS, KTC=12,4.

Observation 1: Group-bundled resource mapping with the transmission comb offset of {0, 0, 1, 1, 2, 2, 3, 3} for the case of NsymbSRS, KTC=8,4 and 0, 0, 0, 1, 1, 1, 1, 2, 2, 2, 3, 3, 3 for the case NsymbSRS, KTC=12,4 has the following two advantages compared to the staircase and the non-staircase resource mappings:

- It provides the possibility of FDM with Rel. 15 SRS resources without any collision.

- It can be used as a dual intent SRS for both positioning purposes and Rel. 15 usages.

Based on the above discussion, we believe that the group-bundled resource mapping should be supported in the applicable scenarios of NsymbSRS, KTC=8,4 and NsymbSRS, KTC=12,4. As a compromise, we propose to support all candidate resource mappings for each pair of NsymbSRS, KTC. Note that there are at most four different resource mapping candidates for each pair of NsymbSRS, KTC. These can be indicated to the UE using a 2-bit indicator in RRC. If such a compromise cannot be reached in RAN1 99 and most sources insist on supporting only one resource mapping for each pair of NsymbSRS, KTC, we can support the staircase resource mapping for every pair of NsymbSRS, KTC.

Proposal 3: Regarding resource mapping of SRS for positioning, support one of the following two alternatives (Alt 2 is the second preference):

Alt 1) For each pair of number of symbols and comb size, support all listed patterns in RAN1 #98b agreements

Alt 2) For each pair of number of symbols and comb size, support only the regular staircase pattern in the list.

Number of SRS resources/resource sets for positioning

Agreement [1]: - UE can be configured to transmit up to Y1 SRS resources for positioning per SRS resource Set - UE can be configured to transmit up to Y2 SRS resources for positioning across all SRS resource Sets - UE can be configured to transmit up to total Y3 SRS Resource Sets for positioning - FFS: Y1, Y2, Y3 - FFS: whether Y1 and/or Y2 and/or Y3 are per BWP or all BWPs

The number of SRS resources for positioning per SRS resource set Y1, SRS resources for positioning across all SRS resource sets Y2, and the total SRS resource sets for positioning Y3 can be finalized in the dedicated UE capability discussion sessions. Further, whether or not Y1, Y2, Y3 are per BWP, across all BWPs or per each frequency band can be clarified in the UE capability discussion. As a starting point of such discussions, we can refer to the relevant UE capability values for Rel. 15 SRS for beam management given in  38.331:

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

Note that the above values are per band. We believe that the maximum number of SRS resources per set for positioning can be similar to the maximum number of SRS resources per set for beam management (Y1=16). However since SRS for positioning may need to reach multiple cells, the total number of SRS resource sets for positioning per band should be up to 16. We propose:

Proposal 4: support the following:

- Maximum number of SRS resources per set for positioning is 16.

- Maximum number of SRS resource sets for positioning is 16 per band.

SRS bandwidth configuration

Agreement [1]: - The SRS for positioning purposes is transmitted within the active UL BWP of the UE.

It was agreed in RAN1 98b that SRS for positioning purposes is transmitted within the active UL BWP of the UE. However, it was not clarified if, similar to the Rel. 15 SRS, SRS for positioning is configured per UL BWP. Since it is already agreed that SRS for positioning is transmitted within the active UL BWP, it is intuitive to follow the Rel. 15 SRS configuration method and configure SRS for positioning per UL BWP.

Note that we have also reached the following agreement in RAN1 98b:

“RAN1 assumes that the SRS for positioning configuration will be included in the RRC configuration of a UE and that it is up to RAN2 and RAN3 WG’s scope to analyze further the system level aspects along with any further work on the design of higher layer signaling”.

Since RAN1 assumes that SRS for positioning is configured in RRC, and, hence, by the serving cell, UL BWPs of the UE are known to the configuring node (serving cell). Therefore, there is also no technical difficulty to configure SRS for positioning per UL BWP. In summary, configuring SRS for positioning per UL BWP is both intuitive and simple and is aligned with Rel. 15 SRS configuration per BWP. As such, we propose the following:

Proposal 5: Similar to Rel. 15 SRS configuration, SRS for positioning configuration is per UL BWP.

Anti-aliasing cyclic shift mapping to SRS symbols

Since the cyclic shift is used to multiplex multiple ports using artificial delay within the same physical resource, it is somehow contradictory to the intention of introducing staggering pattern in the first place. Moreover the sidelobes caused by multiplexing multiple ports with different cyclic shifts are much closer to the main lobe than those caused by the non-staggering RE mapping, as shown in Figure 2.

Figure 2 Sidelobes caused by multiplexing multiple ports with different cyclic shifts with non-staggering (left) and staggering (right) RE mapping

To overcome the aliasing issue caused by cyclic shift, phase hopping across symbols within an SRS resource in a slot can be considered. In this approach, similar to Rel. 15, a cyclic shift across all symbols of each port of the SRS resource is applied. Moreover, as shown in Eq. (1), a symbol-level phase hopping (phase rotation) ejφl' is applied on top of the existing SRS. By introducing the symbol-level phase hopping sequence, the cross-correlation peaks of two SRS ports with different cyclic shifts can be reduced to the cross-correlation of the two phase hopping sequences. The phase hopping sequence φl' can be configured by higher layer.

|  | r(pi)n,l'=ejφl'ru,vαi,δn | (1) |
| --- | --- | --- |

To support the phase hopping using ejφl', we need sequences that:

Are short to match the number of SRS symbols

Have low cross-correlation

- The base sequences defined by 5.2.2.2 [2] (CGS sequences) can meet the above requirements.

Figure 3 shows the auto-correlation and cross-correlation if the phase hopping sequence is based on NR length-12 CGS sequence. It can be observed that the sidelobes caused by different cyclic shifts are suppressed.

Figure 3 Sidelobes caused by symbol-level cyclic shifts with staggering RE mapping

To specify the CGS sequence used for phase hopping, UE needs to determine the sequence length MZC and the sequence group for φl'.

For sequence length MZC, one straightforward way is set MZC to be the smallest value within {6, 12} that is no less than the number of SRS symbols.

For the group, gNB should provide a higher layer parameter groupId associated with the SRS resource.

- Since only single-port SRS resource is supported, where the port is mapped to the cyclic shift provided by cyclicShift, the resource-specific groupId is only applicable to this cyclic shift associated with the port.

- Note that the phase hopping is independent from ru,vαi,δn, another low PAPR sequence that is mapped to RE in the pre-DFT domain for a symbol.

Proposal 6: Support phase hopping across symbols within an SRS resource, and include groupID in the SRS resource configuration for positioning.

Proposal 7: Modify the SRS sequence generation as

r(pi)n,l'=ejφl'ru,vαi,δn

φl' is selected from the CGS sequence defined in section 5.2.2.2 of TS 38.211 where

  - MZC is the smallest one within {6, 12} that is no less than the number of SRS symbols provided by nrofSymbols

  - Sequence group ID is provided by groupID.

- ru,vαi,δn is based on the existing mechanism of sequence configuration and generation.

SRS sequence generation

In Rel. 15, the SRS sequence is based on the Zadoff-Chu sequence given by ru,vα,δ(n), where u and v are respectively the sequence group and sequence number in the group that jointly determine the root of the Zadoff-Chu sequence [2]. u and v depend on the value of the higher layer parameter groupOrSequenceHopping and their calculation formulae are brought in Table 1 that is a snippet of Clause 6.4.1.4.2 in [2].  As can be observed from Table 1, both v (if v≠0) and u are functions of the higher layer parameter sequenceId nIDSRS . The length of nIDSRS is limited to 10 in Rel. 15, i.e., the total number of different SRS sequence group hopping patterns is limited to 1024. This may be enough for UEs within one cell. However, the requirement is different for NR positioning where each cell is expected to receive SRS from a UE in the coverage area of other neighboring cells and, therefore, SRS sequence distinction is needed within a wider range than a single cell coverage. As such, increasing the available number of SRS sequences is beneficial in terms of reducing SRS collision and further mitigating the UL interference. Since the number of SRS symbols has also increased, it provides more room to randomize the interference by supporting more sequence group hopping patterns introduced from extending the sequenceId.

Table 1: Sequence group and sequence number calculation from 38.211 Clause 6.4.1.4.2

The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Config IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Config IE and l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. - if groupOrSequenceHopping equals 'neither', neither group, nor sequence hopping shall be used and - if groupOrSequenceHopping equals 'groupHopping', group hopping but not sequence hopping shall be used and where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with cinit=nIDSRS at the beginning of each radio frame. - if groupOrSequenceHopping equals 'sequenceHopping', sequence hopping but not group hopping shall be used and where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame.

The gold sequence c. that is used to determine u and v in Table 1 is initialized by setting cinit  to nIDSRS. As described in Clause 5.2.1 of [2], cinit is a 31-bit sequence and setting cinit  to nIDSRS  underutilizes cinit by setting its last 21 MSBs to zero. Therefore, increasing the length of nIDSRS to any binary sequence with the length less than or equal to 31 bits does not result in any overflow of cinit or any other specification impact. Note that the same gold sequence c.  is also used to generate UL DMRS and is initialized by a 16-bit sequence ID.

Based on the above analysis, we conclude that increasing the length of nIDSRS to support more distinct SRS sequences does not require any other RAN1 specification change. In our view, a value of 16 bits should be more than enough. Therefore, we have the following proposal:

Proposal 8: Increase the bit width of the parameter sequenceId of the SRS resource configuration to 16 bits.

Single-port SRS for positioning

The field nrofSRS-Ports was removed from the list of higher layer parameters for SRS for positioning configuration in RAN1 98b; implying that only a single-port SRS for positioning is supported in Rel. 16. nrofSRS-Ports is a mandatory field in SRS configuration. Therefore, to be able to reuse the Rel. 15 SRS mapping to physical resources description that is provided in Clause 6.4.1.4.3 of 38.211 also for SRS for positioning, UE should assume nrofSRS-Ports is equal to one when nrofSRS-Ports is not configured.

Proposal 9: If nrofSRS-Ports is not configured, UE assumes nrofSRS-Ports is equal to one.

Reuse of Rel-15 SRS

Supporting Rel-15 SRS without the enhancement defined in RAN1 has the following benefits

- It allows Rel-15 UE to benefit from the NR positioning techniques, such as UTDOA and UAoA.

- It reduces the reference signal overhead since the SRS can be shared with other usages for the same UE, e.g., SRS for codebook based in FR1, for antenna switching in FR1, and for beam management in FR2.

- It enables to multiplex SRS used for positioning with SRS for other usages transmitted by a different UE.

  - Since the positioning SRS defined by RAN1 only supports staggering pattern for the time being, it cannot support efficient frequency domain multiplexing with other Rel-15 SRS.

- At least for the serving cell, any measurement can be carried out by Rel-15 SRS.

- No new capability needs to be introduced from UE side.

- The specification impact is small where only NRPPa needs to provide additional support for Rel-15 SRS configuration transfer between NG-RAN and LMF.

For Rel-15 UE, Rel-16 UL positioning only needs changes at NG-RAN and 5GC side without any additional UE feature. Furthermore, the Rel-15 SRS can be configured with maximum 4 symbols, which shows sufficient UL coverage for the commercial use case, as evaluated in the SI [4]. It is up to gNB to resolve the correlation ambiguity due to the non-staggering pattern.

Note that the ambiguity range for comb-4 SRS with 30kHz SCS is approximately given by

130×4×10-3×3×108=2500m

Even when 120kHz SCS in FR2 is used, the ambiguity range is approximately 625m. It is not so difficult for a gNB to resolve such an ambiguity range.

Proposal 10: Support reuse of Rel-15 SRS for NR positioning in addition to any enhanced SRS in Rel. 16.

There is no RAN1 impact.

Send an LS to RAN2 and RAN3.

Remaining issues regarding pathlossReferenceRS and spatialRelationInfo RS

Our views on the remaining details of pathlossReferenceRS and spatialRelationInfo RS configuration for SRS for positioning are provided in details in our companion contribution [5]. In this section, we briefly outline these views.

In [5], we discuss that UL-based positioning target serving or neighboring cells may send SSBs on different frequency layers. SRS configuration in RRC (SRS-Config IE) includes the configuration of multiple SRS resource sets each of which potentially targeting a different serving or neighboring cell. Therefore, configuring SSBs as pathlossReferenceRS or spatialRelationInfo RS on multiple SSB frequency layers in SRS for positioning configuration should be supported. We have proposed in [5] the following.

Proposal 11: Support configuring SSBs as pathlossReferenceRS or spatialRelationInfo RS on multiple SSB frequency layers in SRS for positioning configuration.

- Maximum number of configured SSB frequency layers for each SRS for positioning configuration is N=4.

When SSB is configured as a pathlossReferenceRS, the UE needs to know the SSB Tx power. This is addressed in RAN1 98b by inclusion of ss-PBCH-BlockPower in the higher layer parameter list where it is left to RAN2 WG to decide whether this field is indicated in DL PRS configuration or SRS for positioning configuration. We believe that indicating ss-PBCH-BlockPower in DL PRS configuration is infeasible due to the following reasons:

- It is possible that UE is not configured with any DL PRS while being configured with SRS for positioning. In such a case, if  ss-PBCH-BlockPower is only a part of DL PRS configuration, UE cannot calculate the DL pathloss when an SSB is configured as a pathlossReferenceRS.

- ss-PBCH-BlockPower is not required for any DL-based positioning measurement.

As such, we propose the following in [5].

Proposal 12: Support indicating ss-PBCH-BlockPower in SRS for positioning RRC configuration and update the higher layer parameter list accordingly.

It was agreed in RAN1 98b to support configuration of a spatial relation between two SRS resources for positioning. In [5], we have argued that it should be possible to use a positioning SRS resource or a Rel. 15 SRS resource in a BWP as the reference spatialRelationInfo SRS for the positioning SRS resources configured in another BWP or another serving cell. The agreement in RAN1 98b is not clear about this issue and we have suggested in [5] to clarify this in RAN1 99.

Proposal 13: Support indicating a configured SRS resource in any BWP of any serving cell as the spatialRelationInfo RS for a SRS resource for positioning.

The use of CSI-RS as a pathlossReferenceRS for posititoning SRS is still FFS. It is natural to at least use NZP-CSI-RS as a pathlossReferenceRS for a positioning SRS that targets a serving cell. Note that, NZP-CSI-RS can be used as a pathlossReferenceRS for SRS in Rel. 15. We have proposed the following in [5].

Proposal 14: Support indicating NZP-CSI-RS resource as pathlossReferenceRS for a serving cell. Similar to Rel. 15, provide NZP-CSI-RS-ResourceId to the UE to indicate the NZP-CSI-RS resource as pathlossReferenceRS.

Whether or not to support CSI-RS for RRM as a pathlossReferenceRS or spatialRelationInfo RS is still FFS. If a CSI-RS resource for RRM is transmitted from a target serving or neighboring cell of the positioning SRS, indicating it as a pathlossReferenceRS or spatialRelationInfo RS to the UE does not come at any additional cost as this resource has already been configured in MeasObjectNR and detected by the UE. Moreover, CSI-RS for RRM can be the only choice for pathlossReferenceRS or spatialRelationInfo RS if the target cell of the positioning SRS in a SCell without SSB. As such, we believe RAN1 should agree to support indicating CSI-RS resource for RRM as a pathlossReferenceRS or spatialRelationInfo RS. Note also that, it was agreed in RAN1 98b that RAN1 assumes SRS for positioning is configured in RRC. If this assumption is confirmed by RAN2, it is very straightforward to use CSI-RS resources for RRM in MeasObjectNR as pathlossReferenceRS or spatialRelationInfo RS for positioning SRS since both MeasObjectNR and positioning SRS are configured in RRC. We have proposed the following in [5].

Proposal 15: Support configuring CSI-RS resources for RRM from the serving and neighboring cells as pathlossReferenceRS or spatialRelationInfo RS.

Finally, the following Table is also provided in [5] that includes all necessary parameters for configuration and indication of a pathlossReferenceRS or a spatialRelationInfo RS for SRS for positioning

Table 2: Necessary parameters for configuration and indication of a pathlossReferenceRS or a spatialRelationInfo RS for SRS for positioning from [5].

| Reference signal | Configuration parameters | Configuration level | Indicating parameters | Indication level |
| --- | --- | --- | --- | --- |
| SSB for spatial relations | - ssbFrequency - ssbSubcarrierSpacing - - smtc1 - - deriveSSB-IndexFromCell. | SRS-Config (Per UL BWP) | - - An index that uniquely identifies the SSB frequency layer (e.g. ssb-Reference-ConfigId in [5]) - - SSB index - - Physical Cell ID | Per SRS for positioning resource |
| SSB for pathloss |  |  | - -An index that uniquely identifies the SSB frequency layer (e.g. ssb-Reference-ConfigId in [5]) - -SSB index - -Physical Cell ID - -ss-PBCH-BlockPower | Per SRS for positioning resource set |
| DL PRS resource for spatial relations | Already configured in LPP AssistanceData |  | -DL-PRSConfigId (To uniquely identify TRP. See [8]), -PRS resource set index, -PRS resource index. | Per SRS for positioning resource |
| DL PRS resource for pathloss |  |  | DL-PRSConfigId (To uniquely identify TRP. See [8]), -PRS resource set index, -PRS resource index. | Per SRS for positioning resource set |
| Positioning SRS resource or Rel. 15 SRS resource for spatial relations | Already configured in SRS-Config in RRC |  | -Rel. 15 SRS resource ID or Rel. 16 Positioning SRS resource ID -UL BWP ID -Serving cell ID | Per SRS for positioning resource |
| NZP-CSI-RS resource for spatial relations (applicable when target cell is a serving cell) | Already configured in NZP-CSI-RS-Resource in RRC |  | -Serving cell ID -NZP-CSI-RS-ResourceId | Per SRS for positioning resource |
| NZP-CSI-RS resource for pathloss (applicable when target cell is a serving cell) |  |  | -NZP-CSI-RS-ResourceId | Per SRS for positioning resource set |
| CSI-RS for RRM resource for spatial relation | Already configured in MeasObjectNR in RRC |  | -Measurement object ID (measObjectId) -Physical Cell ID -CSI-RS-Index | Per SRS for positioning resource |
| CSI-RS for RRM resource for pathloss |  |  | - -Measurement object ID (measObjectId) - -Physical Cell ID - -CSI-RS-Index - -Transmit power | Per SRS for positioning resource set |

Updates on higher layer parameters related to SRS for positioning configuration

We have provided an update to the higher layer parameters related to SRS for positioning configuration in the Appendix. The parameters related to the configuration and the indication of a pathlossReferenceRS or a spatialRelationInfo RS for SRS for positioning are detailed in our companion contribution [5] while the background on the parameters to indicate resource mapping index for a pair of NsymbSRS, KTC and to indicate per-symbol phase shift for SRS for positioning resource is provided in Section 2.2 and Section 2.5 of the current contribution, respectively.

Conclusions

In this contribution, we have provided our considerations on the remaining issues of SRS for NR positioning. Based on the discussion, we have the following proposals and observations.

Proposal 1: Frequency hopping of SRS for positioning is not supported (i.e., agree on the working assumption in RAN1 #98b).

Proposal 2: Following behavior is supported:

If b-SRS is not configured, UE assumes b-SRS is equal to zero.

If b-hop is not configured, UE assumes b-hop is equal to zero.

If freqDomainPosition (nRRC) is not configured, UE assumes freqDomainPosition is equal to zero.

If repetitionFactor is not configured, UE assumes repetitionFactor is equal to nrofSymbols.

Proposal 3: Regarding resource mapping of SRS for positioning, support one of the following two alternatives (Alt 2 is the second preference):

Alt 1) For each pair of number of symbols and comb size, support all listed patterns in RAN1 #98b agreements

Alt 2) For each pair of number of symbols and comb size, support only the regular staircase pattern in the list.

Proposal 4: support the following:

- Maximum number of SRS resources per set for positioning is 16.

- Maximum number of SRS resource sets for positioning is 16 per band.

Proposal 5: Similar to Rel. 15 SRS configuration, SRS for positioning configuration is per UL BWP.

Proposal 6: Support phase hopping across symbols within an SRS resource, and include groupID in the SRS resource configuration for positioning.

Proposal 7: Modify the SRS sequence generation as

r(pi)n,l'=ejφl'ru,vαi,δn

φl' is selected from the CGS sequence defined in section 5.2.2.2 of TS 38.211 where

  - MZC is the smallest one within {6, 12} that is no less than the number of SRS symbols provided by nrofSymbols

  - Sequence group ID is provided by groupID.

- ru,vαi,δn is based on the existing mechanism of sequence configuration and generation.

Proposal 8: Increase the bit width of the parameter sequenceId of the SRS resource configuration to 16 bits.

Proposal 9: If nrofSRS-Ports is not configured, UE assumes nrofSRS-Ports is equal to one.

Proposal 10: Support reuse of Rel-15 SRS for NR positioning in addition to any enhanced SRS in Rel. 16.

There is no RAN1 impact.

Send an LS to RAN2 and RAN3.

Proposal 11: Support configuring SSBs as pathlossReferenceRS or spatialRelationInfo RS on multiple SSB frequency layers in SRS for positioning configuration.

- Maximum number of configured SSB frequency layers for each SRS for positioning configuration is N=4.

Proposal 12: Support indicating ss-PBCH-BlockPower in SRS for positioning RRC configuration and update the higher layer parameter list accordingly.

Proposal 13: Support indicating a configured SRS resource in any BWP of any serving cell as the spatialRelationInfo RS for a SRS resource for positioning.

Proposal 14: Support indicating NZP-CSI-RS resource as pathlossReferenceRS for a serving cell. Similar to Rel. 15, provide NZP-CSI-RS-ResourceId to the UE to indicate the NZP-CSI-RS resource as pathlossReferenceRS.

Proposal 15: Support configuring CSI-RS resources for RRM from the serving and neighboring cells as pathlossReferenceRS or spatialRelationInfo RS.

- References

- 3GPP RAN1, Chairman's Notes RAN1 98b, Chongqing, China, Oct 14-20, 2019.

- 3GPP TS 38.211, “Physical channels and modulation”.

- 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”.

- 3GPP TR 38.855, Study on NR positioning support.

- R1-1911899, Remaining issues on physical layer procedures to support positioning measurements, Huawei, HiSilicon, RAN1#99, Reno, US, Nov. 18 - 22, 2019.

- R1-1911343, SRS design for NR positioning, Huawei, HiSilicon, RAN1#98b, Chongqing, China, Oct 14-20, 2019.

- R1-1911564, Update to the list of higher layer parameters for NR Positioning (RAN1-98bis), Intel Corporation, 3GPP RAN1#98bis, Chongqing, China, Oct. 14th – 20th, 2019.

- R1-1911896, Remaining issues on DL PRS for NR positioning, Huawei, HiSilicon, RAN1#99, Reno, US, Nov. 18 - 22, 2019.

Appendix: Updates on higher layer parameters related to SRS for positioning configuration

| Sub-feature group | Parent IE in ASN.1 | RAN2 ASN.1 name (suggestions/examples) | New or existing parameter | Parameter name in specification | Description | Value range | Default value | UE specific/Cell specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters to search for the configured SSBs as pathlossReferenceRS or spatialRelationInfo RS on each SSB frequency layer. See [5]. |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | SRS-Config | SSB-Reference-Config | New | SSB-Reference-Config | An IE that contains the necessary fields for configuring SSBs for pathloss or spatial relations per frequency layer. These fields are ssbFrequency, ssbSubcarrierSpacing, smtc1, deriveSSB-IndexFromCell. See [5] for further details. |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SSB-Reference-Config | ssb-Reference-ConfigId | New | ssb-Reference-ConfigId | ID of SSB-Reference-Config | INTEGER(1..maxNrofssb-Reference-Config) maxNrofssb-Reference-Config= [4] |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SSB-Reference-Config | ssbFrequency | Existing (New in SRS config) | ssbFrequency | SSB frequency in NR ARFCN value of the SSB frequency layer associated with SSB-Reference-Config |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SSB-Reference-Config | ssbSubcarrierSpacing | Existing (New in SRS config) | ssbSubcarrierSpacing | Subcarrier spacing of the SSB frequency layer associated with SSB-Reference-Config |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SSB-Reference-Config | smtc1 | Existing (New in SRS config) | smtc1 | SSB-MTC window for SSB search on SSB frequency layer indicated in SSB-Reference-Config |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SSB-Reference-Config | deriveSSB-IndexFromCell | Existing (New in SRS config) | deriveSSB-IndexFromCell | If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier. See [3][5] for further details. |  |  | UE-specific |  |
| Parameters to indicate a SSB resource as a pathlossReferenceRS or spatialRelationInfo RS on each configured SSB frequency layer. See [5]. |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | ssb-Reference-ConfigId | New | ssb-Reference-ConfigId | The ID of SSB-Reference-Config to uniquely identify the SSB frequency layer |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | ssb-Index | Existing | ssb-Index | SSB ID of the indicated SSB resource |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | cellId | Existing (New in SRS config) | cellId | Physical Cell ID of the indicated SSB resource |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS | ss-PBCH-BlockPower | Existing (New in SRS config) | ss-PBCH-BlockPower | SSB resource transmit power to calculate pathloss |  |  | UE-specific |  |
| Parameters to indicate a DL PRS resource as a pathlossReferenceRS or spatialRelationInfo RS. See [5]. |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | DL-PRSConfigId | Existing (Exisiting in DL PRS config. New in SRS config.) | DL-PRSConfigId | DL-PRSConfig IE represents a TRP. See [8] for further details. |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | DL-PRS-ResourceSetId | Existing (Exisiting in DL PRS config. New in SRS config.) | DL-PRS-ResourceSetId | The ID of the DL PRS Resource set of the indicated DL PRS resource |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | DL-PRS-ResourceId | Existing (Exisiting in DL PRS config. New in SRS config.) | DL-PRS-ResourceId | The ID of the indicated DL PRS resource |  |  | UE-specific |  |
| Parameters to indicate SRS for positioning or Rel. 15 SRS as a spatialRelationInfo RS. See [5]. (Note: Only the use of positioning SRS is agreed in RAN1 98b) |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | SRS-SpatialRelationInfo | SRS-ResourceId | Existing | SRS-ResourceId | The ID of the SRS for positioning or Rel. 15 SRS indicated as spatial relation RS |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SRS-SpatialRelationInfo | BWP-Id | Existing | BWP-Id | The ID of the UL BWP in which the spatial relation SRS is configured. |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SRS-SpatialRelationInfo | ServCellIndex | Existing | ServCellIndex | The ID of the serving cell in which the spatial relation SRS is configured |  |  | UE-specific |  |
| Parameters to indicate NZP-CSI-RS as a pathlossReferenceRS towards a serving cell or spatialRelationInfo RS towards a serving cell. See [5]. (Note: The use of NZP-CSI-RS as a spatialRelationInfo RS towards a serving cell is agreed in RAN1 98b. The use of NZP-CSI-RS as a pathlossReferenceRS is FFS) |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | NZP-CSI-RS-ResourceId | Existing | NZP-CSI-RS-ResourceId | The ID of NZP-CSI-RS that is indicated as a pathlossReferenceRS towards a serving cell or spatialRelationInfo RS towards a serving cell |  |  | UE-specific |  |
| NR UL PRS/SRS Configuration | SRS-SpatialRelationInfo | ServCellIndex | Existing | ServCellIndex | The ID of the serving cell in which the spatial relation NZP-CSI-RS is configured |  |  | UE-specific |  |
| Parameters to indicate CSI-RS for RRM as pathlossReferenceRS or spatialRelationInfo RS. See [5]. (Note: The use of CSI-RS for RRM as pathlossReferenceRS or spatialRelationInfo RS is FFS) |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | measObjectId | Existing (New in SRS config) | measObjectId | The ID of measurement object in which the indicated CSI-RS resource for RRM is configured. |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | cellId | Existing (New in SRS config) | cellId | Physical Cell ID of the indicated CSI-RS resource for RRM |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS and SRS-SpatialRelationInfo | CSI-RS-Index | Existing (New in SRS config) | CSI-RS-Index | The ID of the indicated CSI-RS resource for RRM |  |  |  |  |
| NR UL PRS/SRS Configuration | pathlossReferenceRS | CSI-RS-Tx-Power | New | CSI-RS-Tx-Power | CSI-RS for RRM resource transmit power to calculate pathloss |  |  |  |  |
| Parameter to indicate resource mapping index for a pair of NsymbSRS, KTC. See Section 2.2. (Note: It is FFS whether one or more candidate resource mappings is applicable for each pair of NsymbSRS, KTC.) |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | SRS-Resource | resourceMappingId | New | resourceMappingId | A 2-bit field to indicate the SRS for positioning resource mapping index for a pair of NsymbSRS, KTC. See Section 2.2 |  |  |  |  |
| Parameter to indicate per-symbol phase shift for SRS for positioning resource. See Section 2.5. (Note: Proposed to be used for anti-aliasing cyclic shift mapping.) |  |  |  |  |  |  |  |  |  |
| NR UL PRS/SRS Configuration | SRS-Resource | groupId | New | groupId | Parameter to indicate per-symbol phase shift for SRS resource. See Section 2.5 |  |  |  |  |