3GPP TSG RAN WG1 Meeting #99                                                                        R1-1911941

Reno, USA, November 18th - 22nd, 2019

Source:              ZTE

Title:                   Discussion on carrier index in SRS triggering

Agenda Item:     7.1.2

Document for:   Discussion and Decision

Discussion

In Rel-15, aperiodic SRS can be triggered by DCI format 0_1, 1_1 or DCI format  2_3. However, if SRS is triggered by a DCI format 0_1 and 1_1, the carrier index of triggered SRS resource set(s) is not clear since there is no corresponding description in the current specifications.

Observation: Based on the current Rel-15 specifications, the carrier index of triggered SRS resource set(s) is not clear if  SRS is triggered by DCI format 0_1 or 1_1.

There are two interpretations for this issue as follows

- Alt.1: The carrier index of the triggered SRS is the same as the triggering DCI.

- Alt.2: Carrier indicator in DCI format 0_1 and 1_1 is to indicate the CC index of the triggered SRS resource set(s).

From our view, Alt.2 is more naturally supported and provides more NW flexibility in Rel-15. So we propose to clarify it in the current specifications.

Proposal: It should be clarified in the current specifications that the carrier index of triggered SRS resource set(s) is indicated by Carrier indicator in DCI if SRS is triggered by DCI format 0_1 or 1_1.

Based on the proposal, we suggest the following TP for both section 7.3.1.1.2 (DCI format 0_1) and 7.3.1.2.2 (DCI format 1_1) in TS 38.212.

---------------------------------------------------------------TP for 38.212 ------------------------------------------------------------

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214]. The serving cell of triggered SRS is indicated by Carrier indicator field.

--------------------------------------------------------------------------------------------------------------------------------------------

Conclusion

In this contribution, we provide our views on some issues of the current spec.

Observation: Based on the current Rel-15 specs, the carrier index of triggered SRS resource set(s) is not clear if  SRS is triggered by DCI format 0_1 or 1_1.

Proposal: It should be clarified in the current specifications that the carrier index of triggered SRS resource set(s) is indicated by Carrier indicator in DCI if SRS is triggered by DCI format 0_1 or 1_1.

References

- TS 38.212, NR; Multiplexing and channel coding, v15.7.0