3GPP TSG RAN WG1 #99                               		R1-1912006

Reno, Nevada, US, Nov 18th – 22nd, 2019

Source:	vivo

Title:	Other aspects for 2-step RACH and UE features

Agenda Item:	7.2.1.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views on the payload size of MsgA PUSCH, and the UE features for 2-step RACH. Performance evaluation results on the payload size and MCL (Maximum Coupling Loss) for PUSCH of msgA are provided.

- UE features for 2-step RACH

A list of UE features for 2-step RACH are summarized in [2]. Based on current summary of UE features for 2-step RACH, we have following comments.

| 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection based on RSRP threshold - Fall back from 2-step RACH to 4-step RACH - Separate RO with 4-step RO configuration - Shared RO with 4-step RO configuration - msgA PUSCH occasions configuration - Intra-slot frequency hopping for msgA PUSCH - Up to two msgA PUSCH configurations in an UL BWP - Validation of msgA PRACH and PUSCH - Preamble and PUSCH resource unit mapping - msgA PUSCH scrambling - Multiple DMRS sequences for msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - DCI format with CRC scrambled by msgB-RNTI - Monitoring of msgB - HARQ-ACK feedback to a MsgB that contains successRAR - For MsgA PRACH support all preamble formats - For MsgA PRACH, support 15 kHz and 30 kHz in FR1 and 60 and 120 kHz in FR2. - For MsgA PUSCH, support 15 kHz, 30 kHz and 60 kHz in FR1 (maybe 60 kHz can be optional). Support 60 kHz and 120 kHz for FR2 - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1 and configuration type 2. |  |
| --- | --- | --- | --- |
| 9-2 | 2-step RACH for NR-unlicensed | - LBT for msgA PRACH and PUSCH - Interlaced msgA PUSCH | 9-1 |

Regarding UE feature 9-1, currently the whole functionalities of 2-step RACH including MsgA and MsgB are included as a capability. In fact, such a large UE capability may complicate the UE implementation. We suggest to further discuss which functionalities in 9-1 are necessary as the basic feature of 2-step RACH. For example, DMRS configuration type 2 is an optional feature in Rel-15. The necessity to include it as the basic 2-step RACH feature needs to be clarified.

Proposal 1: For UE feature 9-1, further discuss and identify the necessary basic functionalities of 2-step RACH.

Regarding UE feature 9-2, 2-step RACH for NR-unlicensed feature includes LBT and interlaced msgA PUSCH. As discussed in the NR-U feature, LBT and interlaced msgA PUSCH are separate features. Therefore, we suggest to align with the NR-U feature, i.e. LBT and interlaced msgA PUSCH are separated as different UE capabilities.

Proposal 2: For UE feature 9-2, LBT and interlaced msgA PUSCH are separated as different UE capabilities.

- Evaluations for MCL

The number of PRBs and OFDM symbols for a PUSCH occasion affect the performance of PUSCH transmission for msgA due that the code rate is dependent on the resource allocation. For 2-step RACH, payload size of 72 bits for PUSCH of msgA is agreed as start point and other values are not precluded. The link-level evaluation aims to acquire a threshold demodulation SNR at the target BLER (e.g 1%). Furthermore, maximum coupling loss (MCL) is calculated based on the threshold SNR, where MCL is the limiting value of the coupling loss at which a service can be delivered. The following table is based on the inputs and calculations for MCL from TR 36.888 [1]. In this section, we focus on 72bits for evaluation. Evaluations of other payload size can be found in our companion paper [3]. In the simulation, the number of PRBs is assumed as 2, 3, 4, 6, or 12 and the number of OFDM symbols for PUSCH is 6 or 14.

| (1) Power in channel bandwidth (dbm) | 23dbm is used in typical UE and do not consider CA scenarios. |
| --- | --- |
| (2) Thermal noise density (dbm/Hz) | -174 |
| (3) Receiver Noise Figure (dB) | NF is related to the front end Low Noise Amplifier. A conservative NF=5 for gNB. |
| (4) Interference margin | Consider an average interference margin in network =10dB |
| (5) Occupied channel bandwidth (Hz) | SCS*12*N_RB |
| (6) Required SNR (dB) | Derive from LLS |
| (7) MCL | Calculate: (1)-{(2)+(3)+(4)+10*log10((5))}-(6) |

Figure 1: BLER performance with different #PRBs and #OFDM symbols

| TBS=72 |  | 2 PRBs |  | 3 PRBs |  | 4 PRBs |  | 6 PRBs |  | 12 PRBs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH target BLER=1% |  | Threshold: SNR | MCL | Threshold: SNR | MCL | Threshold: SNR | MCL | Threshold: SNR | MCL | Threshold: SNR | MCL |
| Rx: 2 | 6-OS | 7.96 | 115.5 | 5.18 | 116.5 | 3.07 | 117.3 | 0.46 | 118.2 | -3.54 | 119.2 |
|  | 14-OS | 2.57 | 120.9 | 0.37 | 121.3 | -1.26 | 121.7 | -3.31 | 122.0 | -6.97 | 122.6 |
| Rx: 4 | 6-OS | 1.8 | 121.6 | -0.52 | 122.2 | -2.10 | 122.5 | -4.06 | 122.7 | -7.42 | 123.1 |
|  | 14-OS | -2.92 | 126.3 | -4.59 | 126.3 | -5.84 | 126.3 | -7.59 | 126.2 | -10.56 | 126.2 |

Figure 2: MCL vs. different PRB sizes

Observation 1: From MCL perspective, larger PRB size can achieve better performance than smaller PRB size, when assuming the same number of OFDM symbols.

Observation 2: From MCL perspective, 14-OS can achieve better performance than 6-OS, with the same PRB size.

- Conclusion

In this contribution, we discuss the payload size of MsgA PUSCH, and the UE features for 2-step RACH. The observations are summarized as follows:

Observation 1: From MCL perspective, larger PRB size can achieve better performance than smaller PRB size, when assuming the same number of OFDM symbols.

Observation 2: From MCL perspective, 14-OS can achieve better performance than 6-OS, with the same PRB size.

Proposal 1: For UE feature 9-1, further discuss and identify the necessary basic functionalities of 2-step RACH.

Proposal 2: For UE feature 9-2, LBT and interlaced msgA PUSCH are separated as different UE capabilities.

- References

- 3GPP TR 36.888 V12.0.0

- R1-1911751, “RAN1 UE features list for Rel-16 NR”, AT&T, NTT DOCOMO, INC., RAN1 #98bis.

- R1-1906126, “Evaluation for 2-step RACH”, vivo, RAN1#97

- Appendix A

Table 1: Link level evaluation assumption

| Assumption | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Waveform | CP-OFDM |
| Subcarrier spacing | 30 KHz, |
| Modulation order | QPSK |
| PUSCH resource | 2,3,4,6,12PRB, 14OS or 6OS |
| DMRS | Type1, double-symbol |
| Antenna configuration | BS: 2Rx or 4Rx , UE: 1Tx |
| Chanel model | TDL-C, DS=300ns, 3km/h |
| Detection algorithm | Channel estimation: MMSE filter Equalizer: MMSE-IRC |
| Frequency offset | Ideal , 0ppm |
| Timing offset | Ideal , 0Ts |