3GPP TSG RAN WG1 #99		R1-1912010

Reno, USA, November 18th – 22nd, 2019

Source:	vivo

Title:	Discussion on physical DL channel design in unlicensed spectrum

Agenda Item:	7.2.2.1.2

Document for:	Discussion and Decision

- Introduction

According to WID on NR-based access to unlicensed spectrum [1], the following parts should be specified:

-	Physical layer aspects including [RAN1]:

-	For DL data channel, support of multiple PDSCH starting positions;

-	Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption;

-	DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

In this paper, physical DL channel design including COT structure indication, search space group switching and  etc. are discussed in the following sections.

- Discussion

SFI-based COT indicator and related UE behavior

Time domain COT indicator

Agreement:

Add a COT duration bit-field per serving cell in GC-PDCCH, i.e., DCI format 2_0

- The following are configurable by RRC:

  - Presence of this bit-field

  - Location of this bit-field in the DCI

  - Length of this bit-field in the DCI

    - FFS: Whether a single value will suffice in which case, the length is not configurable

  - Encoding of the bit field value, i.e., what COT duration corresponds to which bit-field value

- If a UE receives this bit-field, it applies the knowledge about end-of-COT at least for the purpose of UL transmission LBT category switching in a gNB-acquired COT

- If this field is not present, UE should use SFI indication to determine end-of-COT (if SFI is available)

  - FFS: details for this SFI-based mechanism

- FFS: Whether the duration is encoded as e.g., total length or remaining length.

- FFS: Granularity the signalled duration

In RAN1#98bis, the following agreement is made:

In LAA/eLAA/FeLAA, UEs monitor DCI format 1C in common PDCCH with CRC scrambled by CC-RNTI to obtain some COT information, i.e. ‘Subframe configuration for LAA’, ‘UL duration and offset’ and ‘COT sharing indication for AUL’. First, ‘Subframe configuration for LAA’ indicates the partial DL subframe for current subframe or next subframe, which conveys part of DL COT information. Second, ‘UL duration and offset’ indicates the subframes for UL transmission in eNB’s COT. Third, ‘COT sharing indication for AUL’ is used to enable/disable AUL transmission to share eNB’s COT.

In NR Rel-15, SFI is used to convey time domain information (i.e. the dynamic UL/DL/Flexible) to control the UE behaviour for certain high layer transmission/reception such as configured grant. COT information in time domain in NR-U could also serve similar purpose, e.g. enable UL transmission such as configured grant to share gNB’s COT by using Cat 2 LBT. More specifically, at least the COT ending symbol should be indicated to convey the time domain COT information. The following two alternatives could be considered to indicate the COT ending symbol according to current agreement:

  - Additional field indicates COT ending symbol position or

  - SFI indication to determine COT ending symbol position if the above field is not present

For the latter alternative, one straightforward way is to use the time length of indicated SFI (e.g. 10 slots) as the length of remaining COT. However in some case this may not be aligned with one of the NR Rel15 SFI indication restrictions as the following text in TS 38.213 [2]:

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_0.

One example is provided below in Figure 1: COT starts between two configured GC-PDCCH occasions and thus the COT will span two GC-PDCCH periods. If using SFI indication length as COT information in this case, SFI should indicates the number of slots from current GC-PDCCH occasion to COT ending, which is smaller than a PDCCH monitoring periodicity for DCI format 2_0. This doesn’t meet NR Rel15 SFI indication restrictions.

Figure 1  GC-PDCCH configuration together with COT information

A possible way to solve this is to specify some rules to determine COT information based on current SFI indication framework. For example, the last continual ‘F’ symbols of the indicated SFI combination is considered as out of COT for NRU. In another word, the last non-‘F’ symbol of one indicated SFI combination for a number of slots is the COT ending symbol. In the example as shown in Figure 1, SFI could also indicates the SFI combination until next GC-PDCCH where the symbols from COT ending to next GC-PDCCH are indicated as ‘Flexible’.

Proposal 1: NRU supports SFI to determine end of COT duration by the following rule, i.e. the last non-Flexible symbol of one indicated SFI combination for a number of slots is COT ending symbol.

Frequency domain COT indicator

It is agreed in last meeting that the indication of the gNB’s available LBT bandwidths can be carried in a bitmap in the GC-PDCCH. The bitmap size is determined based on the bandwidth of the wideband BWP. E.g., the bandwidth of the BWP is 80MHz, the bitmap can be 4 bits, and each bit corresponds to the transmission status of one LBT subband. If gNB indicates 1100 to UEs, then UEs will monitor in LBT bandwidth 1 and 2. The indication enhancement at the beginning of DL transmission burst is not necessary since only the first one or two slots will be affected.  One option is to indicate all the LBT bandwidths or carriers to the UEs at the beginning of the DL transmission burst, UEs can blindly detect in all the LBT bandwidths or carriers as they do outside the COT. When the GC-PDCCH for specific LBT bandwidth is ready, UEs can perform blind decoding only in the indicated LBT bandwidths. If the CG-PDCCH is not configured, UEs can perform blind decoding in all the LBT bandwidths.

Besides, the explicit indication via UE-specific PDCCH can also be considered as a further enhancement when GC-PDCCH is not configured. UE can obtain the LBT bandwidth by detecting its scheduling PDCCH. Furthermore, explicit indication via UE specific PDCCH can bring additional flexibility, different UEs can be indicated with different information. E.g., if gNB indicates 1100 to UE1 and 1101 to UE2, then UE1 will monitor in LBT subband 1 and 2, and UE2 will monitor in LBT subband 1, 2 and 4. When the gNB initiated COT expires, UE1 and UE2 both start to monitor the full bandwidth.

Proposal 2: The gNB’s transmitted LBT bandwidths can be also explicitly indicated to UE via a bitmap in UE-specific PDCCH.

UE behaviour based on SFI-based COT indication

In NR Rel-15, the UE behaviour related to SFI is summarized below in Table 1 when SFI is configured and semi-static UL/DL configuration indicates the set of symbols as flexible or is not provided:

Table 1	NR Rel-15 UE behaviour based on SFI reception [2]

|  | Indicated as ‘D’ Does not expect DCI(s) indicating UL transmission (apply to all rows) | Indicated as ‘U’ Does not expect DCI(s) indicating UL transmission (apply to all rows) | Indicated as ‘F’ | Not detected |
| --- | --- | --- | --- | --- |
| PDCCH monitoring symbols | Perform regular PDCCH monitoring on this subset of symbols | Cancel PDCCH monitoring on this subset of symbols | Cancel PDCCH monitoring on this subset of symbols | Perform PDCCH monitoring |
| PDSCH or CSI-RS transmission symbols configured by high layer | Perform regular PDSCH reception/CSI-RS measurement when all the symbols indicated as ‘D’ | Cancel PDSCH reception/CSI-RS measurement when one of the symbols indicated as ‘U’ | Cancel PDSCH reception/CSI-RS measurement when one of the symbols indicated as ‘F’ | Cancel PDSCH reception/CSI-RS measurement |
| SRS transmission symbols configured by high layer | Cancel SRS transmission on this subset of symbols | Perform regular SRS transmission on this subset of symbols | Cancel SRS transmission on this subset of symbols | Cancel SRS transmission satisfying certain timeline requirement |
| PUCCH, or PUSCH, or PRACH transmission on symbols configured by high layer | Cancel PUCCH/PUSCH/PRACH transmission when one of the symbols indicated as ‘D’ | Perform regular PUCCH/PUSCH/PRACH transmission when all the symbols indicated as ‘U’ | Cancel PUCCH/PUSCH/PRACH transmission when one of the symbols indicated as ‘F’ | Cancel PUCCH/PUSCH/PRACH transmission satisfying certain timeline requirement |
| Symbols not configured by high layer | Expect DCI indicating DL transmission only | Expect DCI indicating UL transmission only | Expect DCI indicating DL transmission and UL transmission | Expect DCI indicating DL transmission and UL transmission |

For NR-U, it is not appropriate to have semi-static UL/DL configuration since the transmission states depend on LBT results, i.e. semi-static UL/DL configuration could indicate all symbols as flexible. If no change, the above UE behaviour is the baseline for NR-U. However, there will be several differences compared to licensed operation.

First, it is clearly seen from Table 1 that when SFI is configured but not detected, the symbols which are configured as SRS/PUCCH/PUSCH/PRACH transmission by high layer are cancelled if satisfying certain timeline requirement, i.e. N2 symbols after last symbol of configured coreset for SFI GC-PDCCH. This is not appropriate for NR-U since SFI GC-PDCCH in unlicensed cell may not be transmitted due to LBT failure. In this case, it will cause the high layer configured transmissions cancelled in these not indicated slots by GC-PDCCH. Especially for uplink transmissions such as configured grant PUSCH, UE could perform Cat 4 LBT to initiate configured grant PUSCH transmission even GC-PDCCH is not transmitted due to LBT failure. Therefore when SFI GC-PDCCH is configured in unlicensed serving cell, the UE behaviour should be changed, i.e. the high layer configured transmissions should not be cancelled.

Proposal 3: The UE behavior when SFI GC-PDCCH is configured but not detected depends on the type of configured monitoring serving cell for SFI GC-PDCCH:

- When SFI GC-PDCCH is configured in licensed serving cell, UE follows NR Rel-15 behaviour for the indicated serving cell(s), i.e. cancels the high layer configured transmissions;

- When SFI GC-PDCCH is configured in unlicensed serving cell, UE behaviour is changed, i.e. not to cancel the high layer configured transmissions.

Second, COT ending symbol will be incorporated into SFI transmission as proposed above and additional UE behaviour based on this additional information should be defined. One straightforward way is to enable UE’s transmission to share gNB’s COT, i.e. LBT type is changed from Cat 4 to Cat 2. More specifically, high layer configured UL transmission, e.g. one SRS/PUCCH/PUSCH/PRACH occasion should perform Cat 4 LBT to proceed with the transmission and change to Cat 2 LBT when all the symbols for it are indicated as ‘U’ before COT ending symbol.

Proposal 4: UE could use Cat 2 LBT for one SRS/PUCCH/PUSCH/PRACH occasion configured by high layer when all its symbols are indicated as ‘U’ before COT ending symbol.

Third, the frequency domain COT indication is one new agreed parameter to be transmitted together with SFI and potentially COT ending symbol. But the UE behaviour is not clearly defined yet. For the indicated LBT subband as not available, UE will cancel any reception and transmission on this LBT subband before COT ending symbol, i.e. PDCCH monitoring, PDSCH reception, CSI-RS measurement, SRS/PUCCH/PUSCH/PRACH transmission; UE recover the reception and transmission when outside COT ending symbol.

Proposal 5: For the indicated LBT subband as not available, UE will cancel any reception and transmission on this LBT subband before COT ending symbol; UE recover the regular reception and transmission when outside COT ending symbol.

gNB control from Cat 4 LBT to Cat 2 LBT

It is one typical case that PDSCH-to-HARQ-timing-indicator in the DCI indicates to UE that the HARQ-ACK feedback is to be transmitted outside the current COT. This is similar to the mechanism of HARQ-ACK feedback indicator in the same COT except for UE LBT category. Since HARQ-ACK feedback is outside the shared COT, category 1 LBT or category 2 LBT may not be applicable. UE may perform category 4 LBT to access channel and then transmit HARQ-ACK in the indicated PUCCH resource.

There can be a situation described as follows. The gNB indicates UE in a COT (named COT1) to report HARQ-ACK feedbacks with a PUCCH resource that falls outside COT1 by applying category 4 LBT to access the channel. Then gNB acquires another COT (named COT2), and the indicated PUCCH resource falls inside COT2, now UE can apply category 1 LBT or category 2 LBT to report HARQ-ACK feedbacks in the indicated PUCCH resource. Meanwhile, if gNB schedules in COT2 new PDSCH transmissions for UE, of which corresponding HARQ-ACK feedbacks will also be reported in the indicated slot, then gNB can indicate UE to apply category 1 LBT or category 2 LBT to access the channel, in order to simplify UE behavior and improve  probability of successful channel access. At this time, two LBT categories are indicated by gNB for the HARQ-ACK feedbacks in a slot, which LBT category UE should follow in this scenario needs to be defined, i.e. UE always follow the latest indication of LBT category.

Figure 2  LBT category confusion for PUCCH

Proposal 6: The LBT category for one PUCCH is determined by the latest DCI indication, i.e. gNB could control PUCCH transmission from Cat 4 LBT to Cat 2 LBT by LBT category overriding.

Search space group switching

Agreement:

- A UE can be provided with at least two groups (FFS: more than two groups) of search space sets for PDCCH. The UE can be configured to switch between the groups, indicated based on at least the following alternatives.

  - Alt 1: implicitly e.g. after detection of [FFS: DL burst, (WB-)DM-RS, GC-PDCCH and/or PDCCH] and/or e.g., based on information on COT structure.

  - Alt 2: explicitly in GC-PDCCH and/or PDCCH

- Search space sets that are not part of the configured groups (e.g., a common search space set) will always be monitored by the UE regardless of the search space set indication

- A single search space set can be part of more than one group.

- It is up to RAN2 to optimize the signaling to minimize overhead.

In RAN1#98bis, the following agreement is made:

From the above agreement, UE could be configured with two separate search space (SS) group but only one group is active in a certain time duration. For robustness, it is better to configure one default SS group which is active after RRC configuration on SS, switching to another BWP or cell, DRX on and etc. In these cases, UE performs PDCCH monitoring in default SS group.

Proposal 7: One default SS group is configured for UE to perform PDCCH monitoring after RRC configuration on SS, switching to another BWP or cell, DRX on and etc.

When operating in default SS group, UE switches to another non-default SS group once the following rule is met, i.e. any PDCCH in the default SS group is detected. For NRU, this default SS group is used for PDCCH monitoring outside of the COT, which has more frequent monitoring occasion. Since PDCCH detection needs certain processing time, the valid time point to start SS group switching should be a certain time after time of PDCCH detection. For gNB’s implementation flexibility, this valid time delay could be RRC configured but should be larger or equal to the PDCCH processing time.

Proposal 8: When operating in default SS group, UE switches to another non-default SS group at a certain configured time after any PDCCH (including GC-PDCCH) in default SS group is detected.

Once operating in non-default SS group, it means UE is operating inside gNB’s COT according to the above proposal. So UE needs to switch back to default SS group when the COT ends. However, the COT information is not mandatory to be available at UE side. Additional timer-based mechanism needs to be introduced for switching back to default SS group. In summary, the following proposal is made.

Proposal 9: When operating in non-default SS group, UE switches back to another default SS group once any of the following condition is met:

- After COT ending symbol indicated by gNB, or

- After a timer duration when no any PDCCH is detected from the detected PDCCH

One example is shown below: SS1 is the default SS group UE is monitoring where 2 symbols monitoring period is configured. Once LBT successes, gNB transmits PDCCH in SS1. This PDCCH will trigger UE to switch to SS2 after a RRC configured duration, within which SS1 is still valid. Then UE switches to SS2 for PDCCH monitoring until COT ending. After it, UE switches back to SS1.

Figure 3  Example for SS group switching

CSI-RS related UE behavior

In NR Rel15, UE could be configured with periodic/semi-persistent CSI-RS associated with CSI reporting. For each CSI reporting, one CSI reference slot will be defined, i.e. nCSI_ref  slots before the CSI reporting slot. nCSI_ref depends on SCS setting and number of configured CSI-RS resource. For example, assuming SCS is 15KHz for both DL and UL, and only one CSI-RS resource is configured, nCSI_ref is smallest value greater than or equal to 4 such that it is a valid DL slot.

As shown in Figure 4(a), if a UE is not configured with time Restriction for Channel Measurements, the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource associated with the CSI resource setting.

As shown in Figure 4(b), if a UE is configured with time Restriction for Channel Measurements, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS associated with the CSI resource setting.

Figure 4   Periodic CSI-RS transmission and CSI reporting

However for NRU, configured periodic/semi-persistent CSI-RS may not be transmitted due to LBT failure, e.g. CSI-RS #2 is not transmitted due to LBT failure. In this case, UE should skip measurement on this non-transmitted CSI-RS. For case a, UE needs to omit the measurement occasion without CSI-RS transmission; for case b, UE should select the most recent transmitted CSI-RS no later than the CSI reference resource. The basic condition to achieve this is that UE needs to determine whether configured P/SP CSI-RS is transmitted or not.

Proposal 10: UE needs to determine whether configured P/SP CSI-RS is transmitted or not before performing measurement.

DL burst detection signal

In LTE LAA, together with transmitting PDSCH, the always-on CRS could be used for LAA UEs to detect whether there is DL transmission or not. In implementation, LAA UEs will monitor CRS first and then start blind PDCCH monitoring after detecting CRS, which will avoid unnecessary PDCCH blind decoding and thus save UE power. However, CRS has been removed in NR Rel15 and NR-U will inherit this feature. One simple way is to start PDCCH blind decoding in configured search space directly, which results in a waste of UE power due to the uncertainty of channel availability in NRU. Therefore, it is necessary to introduce low complexity signal to facilitate identification of DL transmission for NRU. The following options are identified

- Alt. 1: PDCCH DM-RS

  - Alt. 1-1: UE specific PDCCH DM-RS

    - Alt. 1-1-1: Narrow band DM-RS (e.g. 1, 2 or 4 CCEs)

    - Alt. 1-1-2: Wideband DM-RS (e.g. 8 CCEs)

  - Alt. 1-2: GC-PDCCH DM-RS

    - Alt. 1-2-1: Narrow band DM-RS (e.g. 4 CCEs)

    - Alt. 1-2-2: Wideband DM-RS (e.g. 8 CCEs)

- Alt. 2: CSI-RS

- Alt. 3: PSS/SSS like signal

The comparison of the above options is summarized in terms of performance, complexity, spec impact, flexibility and information capability in the following table:

Table 2		Pros and Cons comparison of 4 options

| Schemes | Performance | Complexity | Spec impact | Flexibility | Information |
| --- | --- | --- | --- | --- | --- |
| Alt. 1-1-1 | Bad | Bad | Good | Bad | Normal |
| Alt. 1-1-2 | Good | Good | Normal | Bad | Normal |
| Alt. 1-2-1 | Normal | Good | Good | Good | Good |
| Alt. 1-2-2 | Good | Good | Normal | Normal | Good |
| Alt. 2 | Good | Good | Good | Good | Normal |
| Alt. 3 | Normal | Good | Bad | Bad | Normal |

From performance aspect, the comparison is determined from evaluation of miss detection performance based on the simulation assumptions provided in Annex. As shown in Figure 5, wideband DM-RS (Alt. 1-1-2 and Alt. 1-2-2) and CSI-RS (Alt. 2) could achieve better performance, e.g. the performance of PDCCH DMRS in 8 CCEs has about 3dB gain compared with PDCCH DMRS in 4 CCEs. For Alt. 1-1-1, its performance is bad since there are less PRBs used for narrow band DM-RS with low aggregation level. Except Alt. 1-1-1, other Alts can achieve acceptable performance in terms of miss-detection performance assuming 1% target false alarm rate, i.e. <1% misdetection rate when SNR=-2dB, where almost 100% NRU UEs are operating above according to system level evaluation observation.

Figure 5	  Performance of DL burst detection signal

From complexity aspect, it is determined by the number of correlation times for different alternatives. UE specific PDCCH is most probably configured with multiple aggregation levels and candidate positions to enhance scheduling flexibility, e.g. AL=1,2,4. For Alt. 1-1-1, UE needs to detect the narrow band DM-RS blindly in all candidate PDCCH positions which leads to higher complexity. Wide band DM-RS could alleviate this since DM-RS is spanning in the whole CORESET by Alt. 1-1-2. For GC-PDCCH, it is broadcast to a group of UEs and thus it is always with large ALs, e.g. AL=4,8. With large AL, the number of PDCCH candidates is limited, i.e. 1 or 2. Then the complexity for Alt. 1-2-1 could be kept low. Obviously, Alt. 1-2-2 will have even less complexity. For other Alts, the complexity will be low if small number of sequences (e.g. 1-4) is assumed for one UE.

From spec impact aspect, Alt. 1-1-1, Alt. 1-2-1 and Alt. 2 are mandatory existing NR signals in Release 15 so NRU could reuse them with little spec impact. Since wideband DM-RS is optional in NR Release 15, it needs to become mandatory if it is used for NRU as DL burst detection signal. For Alt. 3, although PSS/SSS is existing signal, it is only used in SSB as one integrated block. If PSS/SSS without PBCH is used for DL burst detection, this is one new kinds of signal which new configuration design. Besides, they should be carefully designed to meet some basic requirement such as OCB requirement, multiplexing with other channel (e.g. PDSCH) and etc. This should introduce more spec impact than others.

From flexibility aspect, different Alts are analyzed in time domain and frequency domain:

-	In time domain, for Alt. 1-1-1 and Alt. 1-1-2, the DM-RS monitoring position is coupled with search space of UE specific PDCCH scheduling data. Referring to discussions in Section 2.1, frequent UE specific PDCCH monitoring is the basis of multiple PDSCH starting positions, e.g. PDCCH monitoring every 2 symbols as shown in Figure 3. Thus, Alt. 1-1-1 and Alt. 1-1-2 may occur in multiple symbol positions so that UE needs to monitor multiple positions within one slot. Similarly, for Alt. 1-2-1 and Alt. 1-2-2, the DM-RS monitoring position is coupled with search space of GC-PDCCH which could be flexibly configured since it is not scheduling data. For Alt. 2, CSI-RS could also be configured flexibly by RRC signalling. For Alt. 3, in order to avoid misunderstanding with PSS/SSS in SSB, there will be some limitation, e.g. better not to be used in DRS window. In general, there will be some restrictions for Alt. 1-1-1, Alt. 1-1-2 and Alt. 3 in time domain;

-	In frequency domain, wideband DM-RS has obvious limitation since it needs to span the whole configured CORESET even when PDCCH is in part of it. As a result, PDCCH multiplexing between multiple UEs is limited in the CORESET, i.e. only PDCCH for the UEs with the same beamforming and configured scrambling ID could be multiplexed. This restriction will apply to both Alt. 1-1-2 and Alt. 1-2-2. For Alt. 3, PSS/SSS’s transmission bandwidth is 7.2MHz, which is not meeting OCB requirement in unlicensed band. Besides, PSS/SSS may mislead cell search UEs when they coincide with sync raster. Thus, there will some restrictions for Alt. 1-1-2, Alt. 1-2-2 and Alt. 3 in frequency domain.

From information aspect, although the DM-RS sequence is fixed according to PDCCH configuration and transmission time and can’t convey additional information, the coupled PDCCH could deliver some information to the UE(s). For Alt. 1-1-1 and Alt. 1-1-2, the coupled UE specific PDCCH could convey additional information to one scheduled UE only; For Alt. 1-2-1 and Alt. 1-2-2, the coupled GC-PDCCH could convey information to a group of UEs as also described in Section 2.3. CSI-RS and PSS/SSS could carry information bits by different sequences to all UEs, which will degrade the detection performance. From the evaluation result, carrying 2bits information along with CSI-RS and SSS is still acceptable in terms of performance. Thus, Alt. 2 and 3 is feasible to carry small information bits to the UEs.

Based on the above analysis and summary in Table 2, Alt. 1-2-1, Alt. 1-2-2 and Alt. 2 are better candidates for DL burst detection signal. Whether or which RS is used for DL burst detection is up to UE’s implementation depending on the configuration. For example, if one slot is configured with GC-PDCCH, the DM-RS in GC-PDCCH could be used by UE for DL burst detection. Then the following proposal is made:

Proposal 11: DL burst detection is up to UE’s implementation based on network configurations and there is no need to discuss this in RAN1 anymore.

DM-RS design Type B PDSCH in NRU case

In NR Rel-15, for PUSCH Type B mapping, when single-symbol is configured, all length of symbols within a slot are supported; when double-symbol is configured, length of symbols larger than 4 are supported. This brings UL scheduling flexibility and UL transmission efficiency. However, for PDSCH Type B mapping, when single-symbol is configured, only lengths of 2, 4, and 6(ECP)/7(NCP) symbols are supported; when double-symbol is configured, only lengths of 6(ECP)/7(NCP) symbols are supported. This will restrict gNB’s DL scheduling and thus lead to DL transmission inefficiency.

It was agreed to support NR PDSCH Type B mapping with the length of symbols from 2 to 13 for NRU case in RAN1 #97 meeting. For DMRS enhancements for NR PDSCH Type B mapping in NRU case, at least the following aspects should be taken into account for DMRS position design,

- Reuse NR Rel-15 PDSCH/PUSCH DMRS positions as much as possible.

- The last additional DMRS symbol(s) position should locate as near to the end as possible to make better DMRS demodulation performance.

- Reserve 1 or 2 OFDM symbols for UE processing to achieve early demodulation.

- Support up to 3 additional DMRS symbols for single-symbol DMRS case, and up to 1 additional DMRS for double-symbol DMRS case.

- Support both NCP and ECP cases.

Based on above principles, for PDSCH mapping Type B, DMRS positions for single-symbol DMRS case and double-symbol DMRS case are shown in Table 3 and Table 4 respectively. Note that the proposed DM-RS position for length 9 and 10 for NRU is different with the following agreed DMRS position in DSS case in last meeting as shown in Table 5. Since there is no such case for NRU to rate match around CRS, the DMRS design for NRU doesn’t need to follow DSS case. If following DSS case for length 9 and 10 PDSCH, the channel estimation will become worse since the number of symbols between any two successive DM-RS is not equal. Furthermore, it will introduce two different DMRS position hypothesis (i.e. (, 7), (,4, 7)) and thus increase UE implementation complexity with more DMRS position hypothesis.

Table 3   DMRS positions for single-symbol DMRS

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  | , 4 | , 4 | , 4 |
| 7 |  | , 4 | , 4 | , 4 |
| 8 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 8 | , 4, 8 | , 4, 8 |
| 11 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | ,10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | ,10 | , 5, 10 | , 3, 6, 9 |
| 14 | - | - | - | - |

Table 4    DMRS positions for double-symbol DMRS

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |
| --- | --- | --- |
|  | 0 | 1 |
| <4 | - | - |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  | ,5 |
| 9 |  | ,5 |
| 10 |  | ,7 |
| 11 |  | ,7 |
| 12 |  | ,9 |
| 13 |  | ,9 |
| 14 | - | - |

Table 5    DMRS positions for double-symbol DMRS

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | ,4,7 | ,4,7 |
| 10 |  | , 7 | ,4,7 | ,4,7 |

Proposal 12: For PDSCH Type B mapping, support DMRS positions Table 3 and Table 4 for single-symbol DMRS case and double-symbol DMRS case respectively.

- Conclusion

In this contribution, we focus on the design of DL signals and channels for NRU, and have the following proposals:

Proposal 1: NRU supports SFI to determine end of COT duration by the following rule, i.e. the last non-Flexible symbol of one indicated SFI combination for a number of slots is COT ending symbol.

Proposal 2: The gNB’s transmitted LBT bandwidths can be also explicitly indicated to UE via a bitmap in UE-specific PDCCH.

Proposal 3: The UE behavior when SFI GC-PDCCH is configured but not detected depends on the type of configured monitoring serving cell for SFI GC-PDCCH:

- When SFI GC-PDCCH is configured in licensed serving cell, UE follows NR Rel15 behaviour for the indicated serving cell(s), i.e. cancel the high layer configured transmissions;

- When SFI GC-PDCCH is configured in unlicensed serving cell, UE behaviour is updated, i.e. not cancel the high layer configured transmissions.

Proposal 4: UE could use Cat 2 LBT for one SRS/PUCCH/PUSCH/PRACH occasion configured by high layer when all its symbols are indicated as ‘U’ before COT ending symbol.

Proposal 5: For the indicated LBT subband as not available, UE will cancel any reception and transmission on this LBT subband before COT ending symbol; UE recover the regular reception and transmission when outside COT ending symbol.

Proposal 6: The LBT category for one PUCCH is determined by the latest DCI indication, i.e. gNB could control PUCCH transmission from Cat 4 LBT to Cat 2 LBT by LBT category overriding.

Proposal 7: One default SS group is configured for UE to perform PDCCH monitoring after RRC configuration on SS, switching to another BWP or cell, DRX on and etc.

Proposal 8: When operating in default SS group, UE switches to another non-default SS group at a certain configured time after any PDCCH (including GC-PDCCH) in default SS group is detected.

Proposal 9: When operating in non-default SS group, UE switches back to another default SS group once any of the following condition is met:

Proposal 10: UE needs to determine whether configured P/SP CSI-RS is transmitted or not before performing measurement.

Proposal 11: DL burst detection is up to UE’s implementation based on network configurations and there is no need to discuss this in RAN1 anymore.

Proposal 12: For PDSCH Type B mapping, support DMRS positions Table 3 and Table 4 for single-symbol DMRS case and double-symbol DMRS case respectively.

- References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- TS 38.213 v15.6.0, Physical layer procedures for control, June. 2019.

- TR 38.889, Study on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- Annex-Simulation parameters

Table 2 	Simulation parameters for miss-detection performance of DL burst detection signal

| Simulation parameters | values |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel model | TDL-C Low 100ns 10Hz 1X2 |
| Subcarrier spacing | 30 kHz |
| Target false alarm rate for CSI-RS/SSS | 1% |
| Number of RS sequence candidates for CSI-RS/SSS | 1 and 4 candidate sequences for 1bits and 2bits carried by RS sequence, respectively |
| CORESET BW | 48 RB |
| PDCCH Aggregation Level | 2/4/8 |