3GPP TSG RAN WG1 #99		R1-1912013

Reno, USA, November 18th – 22nd, 2019

Source:	vivo

Title:	Discussion on initial access procedure for NR-U

Agenda Item:	7.2.2.2.2

Document for:	Discussion and Decision

- Introduction

According to WID on NR-based access to unlicensed spectrum [1], the following parts should be specified:

-	Physical layer procedure(s) including [RAN1, RAN2]:

- 	Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

- 	Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1)

In this paper, the enhancements to initial access related procedures are discussed based on WID guidance and the previous agreements in the following sections.

- Discussion

DRS transmission and detection

Type 0 PDCCH configuration

Coreset#0 Configuration:

Regarding Coreset#0 limitations for NRU, the following agreements are made until now:

Agreement:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

Agreement:

- Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U

In licensed spectrum, the configuration of SSB resource are indicated in 4 MSB bits in pdcch-ConfigSIB1 in MIB. Taking into account the above agreements only, the following highlighted entries in Table 13-1 and 13-4 of TS 38.213 can be used for NRU UEs to determine Coreset#0 configuration according to the above agreements.

Table 13-1: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

Table 13-4: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

In addition, RAN4 has agreed that there is only one sync raster placed at the proximate edge of each 20MHz LBT subband. In this case, the offset value between SSB and Coreset#0 here in Table 13-1 and 13-4 doesn’t make sense since Coreset#0 will be locating outside the 20MHz LBT subband if using such large value (e.g. 12, 14, 16 and etc.) For NRU, there is no need to have flexible Coreset#0 location in frequency domain, the offset value could be fixed to 0. Only the following configuration for Coreset#0 is needed for {30, 30} kHz SCS case:

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 0 |
| 1 | 1 | 48 | 2 | 0 |

Proposal 1: For NRU, frequency domain position for Coreset#0 in one LBT subband could be fixed and 4 bits for configuring Coreset#0 in frequency domain could be saved in MIB.

Type0 PDCCH Configuration:

Regarding Type 0 PDCCH monitoring slots, the following conclusion/agreements are made:

TR38.889 Conclusion:
Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (referred to as the NR-U DRS) can be beneficial.
Agreement:
If Q is known, candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present.

In other words, the RMSI including coreset#0 and PDSCH is preferred be confined in the same slot. If SSB and its associated RMSI is separately in different slots, it may bring about the following drawbacks:

1) gaps between SSBs need to be filled to composite continuous transmission burst and the filled gap transmission (e.g. UE specific signal) may not be allowed in the COT obtained through Cat 2LBT for DRX transmission.;

2) the time interval between SSB and RMSI are extended and may exceed Cat2 DRS transmission limit.

On the other hand, confining SSB and its associated monitoring occasion in the same slot could avoid these problems and increase the transmission opportunity of both SSB and RMSI significantly. This could be realized by the following configuration by jointly considering the above agreement on Type 0 PDCCH monitoring for QCLed SSBs:

| Index | PDCCH monitoring occasions (SFN and slot number) | Number of search space per slot | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- | --- |
| 0 | nc=nSSB,iQCL (mod (iQCL, Q)= mod (i, Q)) | 1 | 0 |
| 1 | nc=nSSB,iQCL (mod (iQCL, Q)= mod (i, Q)) | 2 | {0, if is even}, {7, if is odd} Note: limited to only number of configured Coreset#0 symbols is 1 |

Proposal 2: For NRU, SSB and monitoring occasions of its associated Type 0 PDCCH are confined in the same slot and only 1 bit is used to configure Type 0 PDCCH in time domain.

Configuration of QCL parameter Q

For the QCL assumptions inside and across DRS transmission windows, the following agreements have been made so far:

Agreement:

For a cell (either serving or a neighbour cell), UE may assume a QCL relation between SS/PBCH blocks within or across DRS transmission or measurement windows that have the same value of modulo(A, Q), once Q is known to the UE

- A is the PBCH DMRS sequence index.

Agreement:

For purposes of SSB QCL derivation, the following values of Q are supported: {1, 2, 4, 8}.

- FFS: Further down-selection of allowed values.

Conclusion:

- The maximum number of PBCH DMRS sequences used in a cell is unchanged from Rel-15 (=8).

- The number of PBCH DMRS sequences used in a cell is independent of Q.

Agreement:

If 2 bits can be found to not be necessary in the current MIB, then Parameter Q is indicated in the MIB by a serving cell. Otherwise, Q is indicated in the RMSI.

Agreement:

For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states,

- Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell.

- Support signaling from the serving cell of a Q value for a listed neighbour cell.

- If Q is signaled for a listed cell, it overrides any common Q value per frequency

Agreement:

No further down selection is made for set of Q values, i.e., Q = {1,2,4,8} (resolves FFS of prior agreement on Q values)

- UE does not expect to be configured with two Type-0 PDCCH monitoring occasions in a slot with Q=1.

- RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8. This assumption does not have any other specification impact.

- Include this agreement in an LS to RAN4 asking them to take this into account. The LS should also provide an overview on the definition of Q.

For the Type 0 PDCCH monitoring, it is already agreed that QCLed SSBs will share the same monitoring slots which involve QCL information. In NR Rel-15, the Type 0 PDCCH configuration is included in MIB for initial access UEs to receive RMSI. Naturally, the QCL parameters determining Type 0 PDCCH configuration should also be included in MIB. According to the discussions in 2.1.1, at least 4 bits indicating Coreset#0 offsets could be spared to signal QCL parameter Q.

Proposal 3: QCL parameter Q is signalled in MIB for NRU serving cell.

DRS transmission period and window

For DRS transmission period and window, the following agreements have been made so far:

Agreement:

Support configuration of the DRS transmission window duration of a UE’s serving cell(s)

- DRS transmission window periodicity is the same as the configuration of SSB burst periodicity

- Note: RAN4 performance requirements depending on the DRS transmission window size, e.g., RRM, will assume a DRS transmission window size of 5ms

Longer DRS windows could increase the probability of successful DRS transmission. On the other hand, it will result in more power consumption since UE needs to search SSB within  longer DRS window. So DRS window duration should be flexible and could be configured in different cases. For example, in low interference case, shorter DRS window duration is enough and would be preferred to be configured. Thus, the following proposal is made:

Proposal 4: NR-U supports flexible configuration of DRS transmission window durations (e.g. {1ms, 2ms, 3ms, 4ms, 5ms}).

Candidate positions of actually transmitted SSBs

For the candidate positions of actually transmitted SSB, the following agreement has been made in last meeting:

Agreement:

The ssb-PositionsInBurst IE is configured as in Rel-15 NR

- FFS: Interpretation of the field for NR-U

As agreed in last meeting, the size of ssb-PositionsInBurst IE will remain the same as NR Rel15, i.e. 8. However for NRU, the maximum number of SSBs in one period becomes 20 to increase SSB transmission opportunity. Thus the re-interpretation of these 8 bits for 20 SSBs is needed. Taking into account that the maximum QCL value is 8, each bit in ssb-PositionsInBurst could indicate validity of one SSB group with the same QCL, where the QCL index is the bit position in ssb-PositionsInBurst. In this way, the first Q bits are valid for the candidate SSB indication. Besides, only SSBs inside configured DRS transmission window could be candidate positions of actually transmitted SSBs. One example is given below as shown in Figure 1: SSB# (0,2,4,6,8,10) are the candidate positions of actually transmitted SSBs when DRS transmission window is 3ms and the high layer parameters are configured as Q=4 and ssb-PositionInBurst=10100000.

Figure 1  Example for candidate positions of actually transmitted SSBs

Proposal 5: Each bit in ssb-PositionsInBurst could indicate validity of one SSB group within DRS transmission window with the same QCL, where the QCL index is the bit position in ssb-PositionsInBurst.

RO association with SSB

In Rel-15, there is no QCL assumption between SSBs with different SSB indices in a SSB burst set, and each RACH occasion can be associated to the actually transmitted SSBs, as indicated in ssb-PositionInBurst, in FR1. When multiple SSBs are associated to the same RO, each SSB is associated with a subset of the preambles.

As discussed in section 2.1.2, QCL assumption for SSBs with different SSB indices/position indices in a DRS transmission window can be assumed when indicated. It is a simple way to reuse the SSB and RO mapping rule in Rel-15 for NRU. However, with the QCL assumption between SSBs, the SSB RO association rule can be enhanced to further reduce the access delay for PRACH transmission. Assuming Q=4, the number of actually transmitted SSBs indicated by ssb-PositionInBurst is 8, the number of FDMed RACH occasion is 4, the number of time domain PRACH occasions in a RACH slot is 2, and each SSB is mapped to 1 RACH occasion, in this case, only one PRACH occasion can be used for each SSB selected for PRACH transmission in a RACH slot, as illustrated in Figure 2(a). However, if the QCLed SSBs, e.g. {0,4},{1,5},{2,6},{3,7}, share the same RO, the number of ROs for each SSB in a PRACH slot can be doubled, as shown in Figure 2 (b). In other words, SSB and RO mapping rule can be enhanced by taking the QCL assumptions into consideration, which can be named as QCLed SSBs and RO mapping rule.

Figure 2 QCLed SSB group and RO mapping rule

Alternatively, the SSB and RO association indicated by RRC signaling can remain the same as that in Rel-15, with additional rule that if UE selects one SSB for PRACH procedure, PRACH can also be transmitted in ROs associated with the SSBs QCLed with the SSB that UE selected. If more than one RACH occasions can be used in one PRACH instance, UE can randomly select one to transmit PRACH.

Proposal 6: SSB and RO mapping rule can be enhanced by taking the QCL assumptions into consideration, following options can be considered.

- The SSB&RO mapping rule is revised to QCLed SSB group and RO mapping rule.

- Reuse the SSB&RO mapping in Rel-15, with additional rule that if UE selects one SSB for PRACH procedure, the ROs associated with the SSBs QCLed with the SSB that UE selected can also be used.

Radio link monitoring

In RAN1#96, the following agreements were made on enhancements for RLM in NRU.

Agreement:

An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

- FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

- FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

- FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

FFS: Mechanism to handle missing RLM-RS due to LBT failure

In the licensed spectrum, both SSB and CSI-RS can be configured as RLM-RS and should be transmitted periodically. When the radio link quality assessed on any configured RLM-RS resource is better than the threshold Qin, in-sync is indicated. And when the radio link qualities assessed on all of the configured RLM-RS resources are worse than the threshold Qout, out-of-sync is indicated. However, in the unlicensed band, RLM-RS transmission may be blocked due to channel unavailability, which will lead to out-of-sync (OOS) indication more frequently, and then radio link failures will be declared more frequently.

To avoid this issue, the indication criterion should be enhanced to ensure the accuracy of the radio link quality. For each configured RLM-RS, UE should determine whether the RLM-RS is detected, if the RLM-RS is not detected by UE, UE does not need to evaluate the hypothetic PDCCH BLER against Qin and Qout, and the number of times when the RLM-RS is not detected could be counted in each indication period. To verify the reliability of RLM-RS appearance detection, some evaluation is performed based on the simulation assumptions given in Table 1 and the results are shown in Figure 3.

Table 1: Simulation results for misdetection performance for RLM-RS

| Simulation parameters | values |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel model | TDL-C Low 100ns 10Hz 1X2 |
| Subcarrier spacing | 30 kHz |
| Target false alarm rate for CSI-RS/SSS/DRS(SSS+CSI-RS+PBCH DMRS) | 1% |
| RMSI CORESET BW | 48 RB |
| Initial BWP size | 48 RB |
| DCI size | 39bits |

Figure 3  Miss-detection performance for RLM-RS

Note that, as specified in [4], SNR region for in-sync is the SNR which can ensure BLER of RMSI PDCCH with AL=4 lower than 2%, and SNR region for out-of-sync is the SNR which cannot ensure BLER of RMSI PDCCH with AL=8 higher than 10%. Based on the simulation assumptions in Table 1 and simulation results in Figure 3, the in-sync and out-of-sync SNR region are [-inf, -4dB] and [2dB, inf], respectively. In licensed band, SSS and CSI-RS are used as RLM-RS, while in unlicensed band, it has been identified to be beneficial to include CSI-RS in DRS, thus both SSB and CSI-RS may be contained in DRS. In this case, all the RSs in DRS, including SSS, CSI-RS and PBCH DMRS, can be jointly utilized to detect whether RLM-RS is actually transmitted. As illustrated in Figure 3, SSS, CSI-RS and DRS are evaluated. The misdetection rates of SSS\CSI-RS\DRS are quite low even in low SNR region in real deployments, which means the RLM-RS misdetection performance is still reliable.

Observation 1: UE can detect whether the RLM-RS is actually transmitted with high reliability.

Based on this observation, one straightforward solution is skipping non-transmitted samples when performing IS and OOS evaluation, i.e. RAN4 evaluation period extension by considering RLM-RS transmission failure due to LBT. However, the reporting state when there is no transmitted RLM-RS sample in the indication period will remain the same as the previous one with RLM-RS transmission. So this will  duplicate previous reporting state by repeating it for N times if no RLM-RS samples in N contiguous indication periods as shown in the following figure.

Figure 4  Example for evaluation period extension in RAN4

To solve the above problem, the radio link quality indication mechanism can be enhanced by taking into account the number of RLM-RSs not detected as transmitted. If all RLM-RSs are determined as not transmitted, the ‘RLM-RS missing’ state can be indicated to the higher layer.

Alternatively, the OOS indication criterion can be enhanced by taking the times of RLM-RS not detected into account, without introducing a new state indicated to the higher layer. For example, if all configured RLM-RSs are determined as not detected in N consecutive indication periods, OOS is indicated to the higher layer.

Proposal 7: Radio link quality indication in physical layer should be enhanced considering that the RLM-RS transmission may be blocked in the unlicensed spectrum due to LBT.

- ‘RLM-RS missing’ state can be indicated to the higher layer when none of the RLM-RSs is detected in an indication period, or

- OOS can be indicated to the higher layer when none of the RLM-RSs is detected in a certain number of consecutive indication periods.

- Conclusion

In this contribution, we focus on the design of initial access procedure for NR-U, and have the following proposals:

Proposal 1: For NRU, frequency domain position for Coreset#0 in one LBT subband could be fixed and 4 bits for configuring Coreset#0 in frequency domain could be saved in MIB.

Proposal 2: For NRU, SSB and monitoring occasions of its associated Type 0 PDCCH are confined in the same slot and only 1 bit is used to configure Type 0 PDCCH in time domain.

Proposal 3: QCL parameter Q is signaled in MIB for NRU serving cell.

Proposal 4: NR-U supports flexible configuration of DRS transmission window durations (e.g. {1ms, 2ms, 3ms, 4ms, 5ms}).

Proposal 5: Each bit in ssb-PositionsInBurst could indicate validity of one SSB group within DRS transmission window with the same QCL, where the QCL index is the bit position in ssb-PositionsInBurst.

Proposal 6: SSB and RO mapping rule can be enhanced by taking the QCL assumptions into consideration, following options can be considered.

Proposal 7: Radio link quality indication in physical layer should be enhanced considering that the RLM-RS transmission may be blocked in the unlicensed spectrum due to LBT.

- References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- R1-1906127, Discussion on initial access signals and channels, May. 2019.

- TR 38.889, Study on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- TS 38.133-f30, Requirements for support of radio resource management, Sep. 2018.