3GPP TSG RAN WG1 #99                                                 				        R1-1912131

Reno, Nevada (US), November 11th – 18th, 2019

Source:	ICS

Title:	Preliminary evaluation of OFDM based waveforms for NR to support NTN

Agenda Item:  7.2.5.1

Document for:  Discussion

# Introduction

The study item “Study on NR to support non-terrestrial networks” focused on the definition of NTN deployment scenarios, adaptation of channel models for NTN and identification of key impact areas on the new-radio (NR) interface that need further evaluations to support NTN [1]. The typical satellite channel impairments that impose serious challenges in the physical layer design to support NR operations via satellites are large propagation delay, high Doppler shift due to mobility, high non-linear degradation, power-limited link budget and phase noise [1].

The RAN1 physical layer objectives that were approved and updated in the study item “Study on Solutions for NR to Support Non-Terrestrial Networks” in RAN#82 meeting [2] are reported as

- Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

  - Physical layer control procedures (e.g. CSI feedback, power control)

  - Uplink Timing advance/RACH procedure including PRACH sequence/format/message

  - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

- Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

Moreover, in 3GPP TR 38.811 (V15.1.0) [1], it is specifically reported that,

7.2	NR features/protocols potentially affected

Table 7.2-1: NR Features/protocols that may require some adaptations to support operations via Satellite or HAPS

| Non-Terrestrial Network design constraints | Impacted NR areas | NR Features that may require adaptations to support NR operation via Satellite or HAPS |
| --- | --- | --- |
| Propagation channel | Physical Layer | The sub-carrier spacing (SCS) of the OFDM signal may be extended with greater SCS values to accommodate larger Doppler (To Be Confirmed). |
| Power limited link budget | Physical Layer | To maximise the throughput / power ratio, the operation point in the power amplifier at satellite or at the UE shall be set as close as possible to the saturation point, when needed. To support this, several techniques can be considered and possibly combined together: - Extended multicarrier modulation and coding schemes especially for the UL that features low Peak to Average Power Ratio (PAPR) that are more robust against distortions - PAPR reduction and nonlinear distortion mitigation through signal processing techniques (e.g. pre-distortion mechanisms). |
| Terminal mobility | Physical layer | - The extension of SCS values could be considered to support very high speed mobility UEs. |

In the framework of the above RAN1 physical layer objectives, this contribution provides the results of numerical evaluation of NR waveforms with different numerologies under satellite channel impairments according to the guidelines set by 3GPP regarding the key LEO satellite parameters and simulation assumptions for the integrated scenario. The comparison of candidate waveforms with different numerologies, multicarrier waveforms with low PAPR, pre-distortion mechanisms and the effect of extending subcarrier SCS on Doppler are also presented in the contribution

# OFDM based candidate waveforms

In 3GPP RAN1 84#bis, it has been agreed that OFDM based waveform with scalable numerology will be adopted for NR [3]. Filtered-OFDM (f-OFDM) and windowed-OFDM (W-OFDM) enables mixed numerologies, preserve the transceiver structure of OFDM and have reduced out of band emission. Hence these were proposed in order to preserve the advantages of OFDM as well as maintain backward compatibility [4] [5]. PAPR reduction methods that have smaller changes to NR radio protocols need to be considered when NR is adapted for NTN and hence DFT-spread-f-OFDM (DFT-s-f-OFDM) and DFT-spread-W-OFDM (DFT-s-W-OFDM) which have lower PAPR than their counterparts, are also considered. The generic block diagram of the OFDM based transceiver [6] for the candidate waveforms is shown in Figure 1.

Figure 1: Transceiver block diagram of OFDM based orthogonal waveform

The OFDM based NR air interface waveforms can generally be represented as

sn=k=-∞∞m=0Mdk,mp(n-kM)ej2πmn-kMF

where dk,m represents the actual data symbols or DFT spread version of the data symbols, modulated at k-th symbol index and k-th sub-carrier index, p(n) denotes the prototype filter and F is the sub-carrier spacing. In W-OFDM, the per-sub-carrier filtering with p(n) is effectively carried out by time-domain windowing using a pulse with smooth edges in time domain and subsequently sharp decay in frequency domain. For f-OFDM, the conventional CP-OFDM signal is first synthesized using IFFT operation with p(n) as rectangular pulse and then inserting cyclic prefix. The f-OFDM signal y(n) is then generated by filtering the CP-OFDM signal through a band-pass filter with impulse response, f(n), to obtain yn=sn*f(n) where * denotes linear convolution. The choice of transmit/receive window and filter functions, receiver processing and detailed implementation aspects are provided in [7].

# Preliminary simulation results

In this section, we present the performance evaluation of the candidate waveforms with different numerology under satellite channel impairments using relevant metrics such as total degradation at target BER, OOB emission based on spectral analysis, PAPR, BER etc. The numerical analysis of the 5G NR candidate waveforms and the impact of different numerologies in integrated satellite mobile communication scenario is presented. Simulation assumptions, power amplifier models and pre-distortion mechanisms can be found in the appendix.

  - Impact of non-linear distortion by High Power Amplifier

The robustness of the candidate waveform against nonlinear PA distortion mainly depends on the PAPR of the waveform. Complementary cumulative distribution function (CCDF) versus PAPR comparison of the candidate waveforms for the number of used sub-carriers, Nused=NFFT and Nused=NFFT2 are presented in Figure. 2.

Figure 2: PAPR comparison for candidate waveforms. The solid and dotted curves correspond to Nused=NFFT and Nused=NFFT/2 respectively.

The greater the PAPR, the greater will be the non-linear distortion. Minimizing the distortion by increasing the back-off of the amplifier operating point reduces the amplifier efficiency accordingly. The DFT-s-W-OFDM and DFT-s-f-OFDM have lower PAPR values of 2.7 dB and 5.6 dB respectively at CCDF = 10-3. The f-OFDM and W-OFDM have PAPR values that are greater than 11 dB.

Observation 1: The fully loaded DFT spread waveforms corresponding to Nused=NFFT are equivalent to single carrier systems and hence have lower PAPR than that corresponding to Nused=NFFT/2.

Observation 2: The windowed OFDM based waveforms have lower PAPR than the filtered waveforms.

The out of band emission (OOBE) of the waveforms are analyzed using the power spectral density (PSD) plots under HPA non-linearity and are compared for different numerologies in Figure 3. The subcarrier spacing corresponding to each numerology μ is 2μ×15 kHz. The PSD plots corresponding to OBO = 3dB for μ=0, μ=1 and μ=2 are presented in Figure 3a, Figure 3b and Figure 3c respectively. The HPA introduces spectral spreading which is evident from the large side-lobe levels without PD operation, but is suppressed with PD. The OOBE levels of all the waveforms can be brought down to less than -50 dB with PD at higher OBO.

Observation 3: The DFT spread waveforms with lowest PAPR shows the best OOBE suppression and the OOBE levels are less than -50 dB at OBO = 3 dB.

Observation 4: A spectral widening, leading to an increased transition band, is noticeable for windowed OFDM based waveforms at larger sub-carrier spacing compared to the filtered waveforms.

Figure 3(a) OBO=3dB, μ=0, Subcarrier spacing = 15 kHz

Figure 3 (b) OBO=3dB, μ=1, Subcarrier spacing = 30 kHz

Figure 3 (c) OBO=3dB, μ=2, Subcarrier spacing = 60 kHz

A distinctive measure for quantifying the trade-off between the amplifier OBO and the nonlinear signal distortion is total degradation which is a function of OBO. TD is defined as TD=OBOdB+EbN0NL-EbN0Lwhere EbN0NLand EbN0Lare the per-bit signal to noise ratio (SNR) required to maintain a target bit error rate (BER) under non-linear distortion due to the actual TWTA and a perfectly linear TWTA respectively. The TD versus OBO performance comparison for an un-coded system with a target BER of 10-4 is presented in Figure. 4. The minimum TD occurs at 1.95 dB, 2.5 dB and 2.52 dB for W-OFDM, DFT-s-f-OFDM and f-OFDM respectively.

Figure 4: Total Degradation versus OBO

Observation 5: DFT-s-W-OFDM shows the best total degradation performance which has the least distortion at smallest OBO value.

  - Impact of Doppler, Phase Noise and Multipath Channel

    - Effect of subcarrier bandwidth variation on residual Doppler shift

In-order to investigate the effect of sub-carrier bandwidth variation on residual Doppler shift, BER versus SNR performance comparison of the candidate waveforms for different LEO satellite scenario and numerologies under the effect of non-line-of-sight (N-LOS) multipath fading, Doppler shift, phase noise and HPA non-linearity at OBO=3dB, is presented in this section. Two different LEO satellite scenario are considered for each waveform, corresponding to maximum user equipment velocity of 1000 km/hr. The tapped delay line (TDL) channel model in 3GPP TR 38.811 (V15.0.0) for non-line-of-sight satellite to user link in a terrestrial-NTN integrated scenario in sub-urban environment is used in the simulations.  The TDL model is used to generate the impulse response corresponding to the multipath channel with Rayleigh fading coefficients at satellite elevation angle (EA) of 100 and delay spread of 10ns. Half-rate convolutional coding is used for channel coding and one-tap zero-forcing equalization employed. The parameters of the S-band satellite phase-noise mask in [8] is used for the simulation of phase noise.

The residual carrier frequency offset (CFO) values corresponding to S band are calculated and normalized using the sub-carrier bandwidths of the NR numerologies for different LEO satellite scenarios in [9] which are presented in Table 1. Here, ϵ is the normalized CFO, fd represents maximum residual Doppler shift, v is the user equipment velocity, h and d denote the LEO satellite altitude and beam foot print diameter respectively.

Table 1 Residual CFO, ϵ for different numerologies for S band

| h (km) | d (km) | v (km/h) | fd (ppm) | Num-0, 15 kHz | Num-1, 30 kHz | Num-2, 60 kHz |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ϵ | ϵ | ϵ |
| 600 | 200 | 1000 | 4.3 | 0.573 | 0.286 | 0.143 |
|  |  | 0 | 4.14 | 0.552 | 0.276 | 0.138 |
|  | 20 | 1000 | 0.44 | 0.058 | 0.029 | 0.014 |
|  |  | 0 | 0.42 | 0.056 | 0.028 | 0.014 |
| 1200 | 200 | 1000 | 2.1 | 0.280 | 0.140 | 0.070 |
|  |  | 0 | 2.01 | 0.268 | 0.134 | 0.067 |
|  | 40 | 1000 | 0.42 | 0.056 | 0.028 | 0.014 |
|  |  | 0 | 0.4 | 0.053 | 0.026 | 0.013 |

Figure 5(a) : W-OFDM						Figure 5(b) : DFT-s-W-OFDM

Figure 5(c) : f-OFDM						Figure 5(d) : DFT-s-f-OFDM

Figure 5: BER versus SNR performance at S band OBO=3 dB, maximum user velocity, v=1000 km/hr, Elevation Angle=10o, Delay spread=10 ns.

The maximum residual Doppler shift when LEO satellite altitude, h=1200 km and beam foot print diameter, d=200 km is 4.2 kHz and that when h=600 km and d=20 km is 880 Hz. The reduction in BER with the increase in sub-carrier bandwidth from 15 kHz (μ=0) to 60 kHz (μ=2) is illustrated in Figure 5. This happens because, as the sub-carrier bandwidth increases, the inter carrier interference (ICI) produced due the Doppler shift decreases.  For the LEO satellite scenario corresponding to fd=4.2 kHz, h=1200 km and d=200 km, it is evident that the BER is drastically reduced when numerology μ=2 is chosen, for all the waveforms.

Observation 6: The change in NR numerology has significant effect on the BER performance in the integrated satellite terrestrial scenario as the large Doppler for LEO satellites can be accommodated by increasing the sub-carrier bandwidth.

Observation 7: Even though increasing sub-carrier bandwidth results in a lower CP length, simulation results indicate that, it is sufficient to protect the signals against multipath interference.

    - Candidate waveform comparison under Doppler, Phase Noise and Multipath Channel

The BER versus SNR performance comparison of the candidate waveforms for different number of used sub-carriers  at OBO=3 dB, LEO satellite altitude, h=600 km, elevation angle of 10o, beam footprint diameter, d=20 km and S-band phase noise are shown in Figure 6. Figure 6(a) and Figure 6(b) shows the BER comparison plots corresponding to delay spread=0, μ=0 and delay spread=10 ns, μ=2 respectively.

Observation 8: When the number of used sub-carriers, Nused=NFFT/2, the filtered OFDM based waveforms show better BER performance than the windowed OFDM based waveforms when delay spread is zero. For fully loaded OFDM systems, i.e. Nused=NFFT, the windowed OFDM based waveforms show better BER performance at high SNR points.

Observation 9: In the presence of multipath delay spread, residual Doppler shift and phase noise, if pre-distortion is employed, f-OFDM provides the best BER performance among the other considered waveform candidates.

Observation 10: DFT-s-f-OFDM incur significant performance losses compared to other waveforms in the high SNR region.

Observation 11: With appropriate numerology, pre-distortion and channel equalization f-OFDM and W-OFDM have better robustness against the satellite channel impairments than the DFT spread counterparts despite the large PAPR.

Figure 6(a) Delay spread =0, μ=0

Figure 6(b) Delay spread =10ns,  μ=2

Figure 6: BER versus SNR comparison of waveforms at S band, OBO=3 dB, LEO Satellite altitude=600 km, maximum user velocity, v=1000 km/hr, Elevation Angle=10o, beam footprint diameter, d=20 km.

Proposal 1: Capture the link level simulation results into TR38.821.

Proposal 2: Investigate performance evaluation of the NR candidate waveforms for LEO satellite access in mixed numerology scenario through link level simulations.

# Conclusion

In this contribution, the performance of 5G NR numerologies and the candidate waveforms are analysed using key satellite parameters and simulation assumptions in the context of RAN1 objectives associated with solutions for NR to support NTN. The following observations and proposals are proposed:

Observation 1: The fully loaded DFT spread waveforms corresponding to Nused=NFFT are equivalent to single carrier systems and hence have lower PAPR than that corresponding to Nused=NFFT/2.

Observation 2: The windowed OFDM based waveforms have lower PAPR than the filtered waveforms.

Observation 3: The DFT spread waveforms with lowest PAPR shows the best OOBE suppression and the OOBE levels are less than -50 dB at OBO = 3 dB.

Observation 4: A spectral widening, leading to an increased transition band, is noticeable for windowed OFDM based waveforms at larger sub-carrier spacing compared to the filtered waveforms.

Observation 5: DFT-s-W-OFDM shows the best total degradation performance which has the least distortion at smallest OBO value.

Observation 6: The change in NR numerology has significant effect on the BER performance in the integrated satellite terrestrial scenario as the large Doppler for LEO satellites can be accommodated by increasing the sub-carrier bandwidth.

Observation 7: Even though increasing sub-carrier bandwidth results in a lower CP length, simulation results indicate that, it is sufficient to protect the signals against multipath interference.

Observation 8: When the number of used sub-carriers, Nused=NFFT/2, the filtered OFDM based waveforms show better BER performance than the windowed OFDM based waveforms when delay spread is zero. For fully loaded OFDM systems, i.e. Nused=NFFT, the windowed OFDM based waveforms show better BER performance at high SNR points.

Observation 9: In the presence of multipath delay spread, residual Doppler shift and phase noise, if pre-distortion is employed, f-OFDM provides the best BER performance among the other considered waveform candidates.

Observation 10: DFT-s-f-OFDM incur significant performance losses compared to other waveforms in the high SNR region.

Observation 11: With appropriate numerology, pre-distortion and channel equalization f-OFDM and W-OFDM have better robustness against the satellite channel impairments than the DFT spread counterparts despite the large PAPR.

Proposal 1: Capture the link level simulation results into TR38.821.

Proposal 2: Investigate performance evaluation of the NR candidate waveforms for LEO satellite access in mixed numerology scenario through link level simulations.

# References

[1] TR 38.811 V15.1.0, Technical Specification Group Radio Access Network; Study on New Radio (NR) to support non terrestrial networks (Release 15), June 2019

[2] RP-182444, Revised SID: Study on solutions for NR to support non-terrestrial networks, 3rd Generation Partnership Project (3GPP), SID revised-97, RAN#82, December 2018.

[3] R1-163961, Final Report of 3GPP TSG RAN WG1 #84bis v1.0.0, April 2016.

[4] R1-165425, f-OFDM scheme and filter design, Huawei, HiSilicon, RAN1 #85

[5] R1-164685, OFDM based waveform single user evaluation, Qualcomm Incorporated, RAN1 #85

[6] R1-162152, OFDM based flexible waveform for 5G, RAN1 84 BIS, April 2016.

[7] R1-162199, Waveform Candidates, RAN1 84 BIS, April 2016.

[8] R1-1905216, Key satellite parameters and simulation assumptions for NTN, RAN1-96, April 2019.

[9] R1-1907390, On Doppler shift compensation and Timing Advance in NTN, RAN1-97, May 2019

# Appendix

## A Simulations parameters and assumptions

Table II Parameter settings for link level evaluation

| Parameters | Settings |
| --- | --- |
| Frequency band | S |
| Basic Numerology Parameters | FFT size, NFFT=1024, Cyclic prefix length, NCP=72 |
| MCS | 16QAM: 1/2 |
| W-OFDM parameters | Transmit window roll of ratio: 4% Transmit window roll of ratio: 1% |
| f-OFDM | Filter order : NFFT/2 |
| Channel model | TDL model N-LOS satellite to user link in sub-urban environment [1] |
| Phase noise mask | S-Band; 10 Hz; 100 Hz; 1 kHz; 10 kHz; 100 kHz; 1 MHz; 10 MHz. dBc/Hz (SSB); -29; -59; -69; -74; -83; -95; -101. |
| Channel estimation | Ideal |

## B Power amplifier (PA) model and pre-distortion method

A highly nonlinear AM/AM characteristics of a typical HPA provided by ESA is used for the simulations. The characteristics can be modelled according to the generic memoryless modified Saleh model. According to the model, the transformation function Ta is defined as

Ta=αaη1+βaγν+ϵ

The different parameters for the AM/AM and AM/PM characteristics of the HPA characteristics are obtained using curve fitting toolbox of MATLAB. The parameters are given in Table III. The modelled characteristics are compared with the actual AM/AM and AM/PM characteristics in Figure 7.

Figure 7: AM/AM and AM/PM curves

The phase distortion is pre-calculated using the model and subtracted from the original phase of the signal before input to the HPA so that phase distortion is cancelled out. The amplitude distortion is corrected using inverse function based pre-distortion method. For the monotonically increasing region of the characteristics, a simplified model derived from modified 6 parameter Saleh model, with less parameters can be used to approximate the AM/AM characteristic which is given as Ta=αaη(1+βaη+ϵ. An inverse function of T-1b can be derived from Ta as T-1b=exp⁡1ηln(b-ϵα-β(b-ϵ), which is used for pre-distortion. The curves corresponding to the actual AM/AM characteristics, Ta, T-1a and TT-1a

are shown in Figure 8.

Figure 8: HPA function linearized by inverse function based pre-distortion