3GPP TSG RAN WG1 Meeting #99			                                                                 	R1- 1912181

Reno, NV, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.2.2.2

Source:	Charter Communications

Title:	On NR-U Initial Access Procedures

Document for:	Discussion/Approval

# Introduction

The NR-U WI is in progress in RAN1 [1]. In the RAN1 #98BIS meeting, the following agreements were reached on initial access procedures [2]:

Agreement:

If 2 bits can be found to not be necessary in the current MIB, then Parameter Q is indicated in the MIB by a serving cell. Otherwise, Q is indicated in the RMSI.

Agreement:

For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states,

- Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell.

- Support signaling from the serving cell of a Q value for a listed neighbour cell.

  - If Q is signaled for a listed cell, it overrides any common Q value per frequency

Agreement:

- No further down selection is made for set of Q values, i.e., Q = {1,2,4,8} (resolves FFS of prior agreement on Q values)

- UE does not expect to be configured with two Type-0 PDCCH monitoring occasions in a slot with Q=1.

- Note: This does not imply a change in Rel-15 behavior for Type-0 PDCCH monitoring in two consecutive slots.

- Note: This agreement does not imply any changes to Rel-15 PDCCH monitoring requirements on BD/CCE limits per slot

- RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8. This assumption does not have any other specification impact.

  - Include this agreement in an LS to RAN4 asking them to take this into account. The LS should also provide an overview on the definition of Q.

Agreement:

- The RSSI measurement timing configuration (RMTC) includes the following:

- Time-domain parameters:

- Periodicity (e.g., 40/80/160/320/640 ms)

- Measurement duration in terms of OFDM symbols with a configured reference subcarrier spacing

- Offset of RMTC measurement duration

- Frequency-domain parameters:

- Measurement bandwidth at least in units of LBT bandwidths

  - FFS: Units other than LBT bandwidths

  - Note: RAN4 can determine if the bandwidth used for the measurement within an LBT bandwidth can be less than the signaled measurement bandwidth.

- Measurement ARFCN for inter-frequency measurements

- Configurable L3 filtering as in RSSI for LTE-LAA

Conclusion:

No new medium contention/load metrics other than channel occupancy are introduced

Agreement:

Support configuration of the DRS transmission window duration of a UE’s serving cell(s)

- DRS transmission window periodicity is the same as the configuration of SSB burst periodicity

- Note: RAN4 performance requirements depending on the DRS transmission window size, e.g., RRM, will assume a DRS transmission window size of 5 ms

Agreement:

The ssb-PositionsInBurst IE is configured as in Rel-15 NR

- FFS: Interpretation of the field for NR-U

This contribution discusses open issues as captured above, in [3], and others.

# NR-U Initial Access Procedures

## Indication of Q by Serving Cell

The parameter Q has the following definition. For a cell (either serving or a neighbor cell), UE may assume a QCL relation between SS/PBCH blocks within or across DRS transmission or measurement windows that have the same value of modulo(A, Q) where A is the PBCH DMRS sequence index, once Q is known to the UE. In other words, Q can be considered as the number of non-QCL SSBs that the UE can expect to receive. Two bits are needed to indicate which element of the set of Q values {1, 2, 4, 8} has been employed by the serving cell.

For the indication of Q, the alternatives under discussion for the serving cell are:

- MIB if two unnecessary bits can be identified

- SIB1 as fallback solution

The Rel-15 24-bit MIB is structured as

MIB ::=                             SEQUENCE {

systemFrameNumber                   BIT STRING (SIZE (6)),

subCarrierSpacingCommon             ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset                INTEGER (0..15),

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1,

cellBarred                          ENUMERATED {barred, notBarred},

intraFreqReselection                ENUMERATED {allowed, notAllowed},

spare                               BIT STRING (SIZE (1))

}

where PDCCH-ConfigSIB1 is further defined as

PDCCH-ConfigSIB1 ::=                SEQUENCE {

controlResourceSetZero              ControlResourceSetZero,

searchSpaceZero                     SearchSpaceZero

}

Here, ControlResourceSetZero (CORESET#0 RBs, number of symbols and RB offset from SSB) and SearchSpaceZero (PDCCH monitoring occasions) each comprise 4 bits.

It is assumed that the existing spare bit is not utilized for Q.

For NR-U, not all possible CORESET#0 configurations are necessary. It has been agreed that:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

- Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U.

The existing 4-bit sets of resource blocks and slot symbols of CORESET#0 in TS 38.213 Tables 13.1 (15 kHz SCS for SSB/15 kHz SCS for PDCCH), Table 13.2 (15 kHz SCS for SSB/30 kHz SCS for PDCCH) have up to 6 rows of configurations, Table 13.3 (30 kHz SCS for SSB/15 kHz SCS for PDCCH) has 3 rows of configurations (and an additional 7 reserved rows), and Table 13.4 (30 kHz SCS for SSB/30 kHz SCS for PDCCH) has 5 rows of configurations corresponding to 3-symbol CORESET#0, which are not applicable to NR-U. Tables 13.5 and 13.6 for bands with minimum channel bandwidths of 40 MHz also each contain at least 6 reserved rows in addition to 3-symbol CORESET configurations.

Therefore, for all SCS combinations applicable to NR-U in FR1, 2 bits (4 rows) can be repurposed from existing ControlResourceSetZero field for indication of Q in MIB. There is also an additional bit available from subCarrierSpacingCommon since the SCS for all SSBs and Coreset #0 on a NR-U carrier is always the same and it is reasonable to assume that Msg.2/4 for initial access, paging and broadcast SI-messages also use the same SCS.

Observation 1: For all SCS combinations applicable to NR-U in FR1, 2 bits (4 rows) can be repurposed from existing ControlResourceSetZero field for indication of Q in MIB.

## RLM and RRM

Some progress was made in previous meetings for SSB-based RLM. Namely, for SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window. This is a straightforward conclusion since the UE does not expect to receive SSBs outside the DRS transmission window.

The case of CSI-RS-based RLM does not need to be as restrictive. Both multiplexing of CSI-RS with DRS and transmission of CSI-RS outside DRS transmission window are supported in NR-U. In Rel-15 NR, a UE may be configured with both CSI-RS and SSB-based RLM-RS. In fact, aperiodic CSI-RS transmissions outside the DRS transmission window can be beneficial in terms of additional opportunities for RLM-RS transmissions. Therefore, using CSI-RS outside the DRS transmission window can help UEs to quickly determine in-sync conditions for the serving cell, especially if the DRS transmission window has a low periodicity.

Proposal 1: CSI-RS-based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for at least in-sync evaluations.

Finally, consider the case of wideband NZP CSI-RS configured for RRM or RLM purposes in a wideband BWP that spans multiple LBT bandwidths. In Rel-15, CSI-RS can be configured to span between 24 PRBs to the maximum number of PRBs in multiples of 4.

If a CSI-RS configured for RRM measurements is partially transmitted in frequency due to LBT failure on one or more LBT bandwidths, then the corresponding measurement quantity (e.g., CSI-RSRP or CSI-RSRQ) may need to be adjusted. Taking RSRP for example, CSI-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. If some REs that have been configured to carry CSI-RS are unused due to LBT failure, including these REs in the RSRP computation will skew the results. Therefore, it is proposed that the UE excludes CSI-RS REs that are unused due to LBT failure on LBT subbands when computing RRM measurements. The knowledge of unused LBT bandwidths can be acquired from GC-PDCCH, as was agreed in the last meeting.

Proposal 2: For CSI-RS-based RRM measurements, UE does not include CSI-RS resource elements in non-occupied LBT sub-bands when the CSI-RS resource allocation spans multiple LBT bandwidths.

# Summary

In this contribution we commented on initial access procedures for NR-U. The corresponding proposals are summarized as follows.

Observation 1: For all SCS combinations applicable to NR-U in FR1, 2 bits (4 rows) can be repurposed from existing ControlResourceSetZero field for indication of Q in MIB.

Proposal 1: CSI-RS-based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for at least in-sync evaluations.

Proposal 2: For CSI-RS-based RRM measurements, UE does not include CSI-RS resource elements in non-occupied LBT sub-bands when the CSI-RS resource allocation spans multiple LBT bandwidths.

# References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, RAN#82

- Chairman’s Notes, RAN1#98BIS, October 2019.

- R1-1911685, Feature lead summary #3 of Enhancements to initial access procedure, Charter Communications