3GPP TSG RAN WG1 #99																					R1-1912207

Reno, USA, November 18th – 22nd, 2019

Source:	Intel Corporation

Title:	Remaining opens of sidelink in-device coexistence

Agenda item:	7.2.4.4

Document for:	Discussion and Decision

Introduction

At the RAN#83 meeting, the work item on NR V2X was approved [1] with one of the objectives for in-device coexistence:

Solutions for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks - TDM-based solutions as per the study outcome [RAN1, RAN2, RAN4] - FDM-based solutions with static power allocation as per the study outcome [RAN4] - This will not consider the case where LTE and NR sidelinks are in the same frequency band. - No impact to LTE specifications at least from RAN1 and RAN2 perspective.

The agreements made by RAN1 WG for this topic are provided in Annex A for reader convenience. These RAN1 agreements are used for further discussion in this contribution which is dedicated to solutions of in-device coexistence issues. Our views on other NR-V2X design aspects are summarized in our companion contributions [2]-[8].

In-Device Coexistence

In-Device Coexistence Conflicts

The following types of in-device coexistence conflicts were identified by RAN1:

Conflict 1. NR PC5 TX vs LTE PC5 TX;

Conflict 2. NR PC5 TX vs LTE PC5 RX;

Conflict 3. LTE PC5 TX vs NR PC5 RX;

Conflict 4. LTE PC5 RX vs NR PC5 RX.

- Up to date RAN1 has addressed at some extent the TX/TX and RX/TX conflicts by developing solution where RAT having packet with relatively higher priority can be prioritized for transmission or reception. In addition it was concluded that potential RX/RX conflicts are left up to UE implementation.

- In the next section, we discuss additional options to solve in-device co-existence problems with a focus on TDM solutions.

Impact on Physical Layer

In-device coexistence problems may have impact on physical layer. In case of TDM solutions the following impact on RAN1 is foreseen:

Resource selection procedures on each RAT, so that if one RAT is aware about ongoing or reserved resources at other RATs, the resource selection step can be enhanced to take into account transmission on the other RAT (e.g. select non-overlapping in time resources)

Control logic to drop packets (transmissions) on one of the sidelink RATs and declaration of DTX or DRX state on one of the sidelink RATs

Sidelink measurements

  - Inform RAT-A that its transmissions can be prioritized due to level of congestion on RAT-B

  - Can be used to determine leakage among RATs

In our view, considering that both LTE and NR will support semi-persistent transmissions, the information about reserved resources can be gathered from one RAT and passed to another one RAT. In this case, resource selection procedure running on RAT-B can exclude sidelink resources overlapping with sidelink resources reserved at RAT-A. This mechanism should be supported on top of TX/TX and TX/RX conflict resolution that was addressed at the last meeting based on sidelink priority. In this specific scenario, NR-V2X resource selection logic can avoid selecting resources overlapped with resources reserved by LTE-V2X and vice versa.

  - NR sidelink resource selection procedure supports exclusion of resources conflicting with LTE sidelink transmission

    - Exclusion of resources conflicting with LTE sidelink transmission is subject to priority and radio-layer conditions considerations

  - NR sidelink resource selection procedure does not exclude resources conflicting with LTE sidelink transmission if:

    - NR sidelink transmission priority has higher priority than LTE sidelink transmission (or NR sidelink transmission priority is higher than pre-configured priority level)

    - LTE sidelink channel is congested (i.e. CBR is above pre-configured threshold so that sidelink transmission and reception is not guaranteed anyway)

It was agreed at RAN1#98 meeting to introduce S-RSSI measurements and to use it in NR-V2X congestion control for CBR estimation. The transmission on RAT-A can contribute to incorrect CBR measurements on RAT-B due to cross-RAT leakage. In order to correct this, LTE transmissions can be taken into account in CBR measurements by NR V2X. The cross-RAT leakage may also be a reason for lack of sensing or invalid sensing results on conflicting resources. In order to take this into account coordination function should inform NR PC5 RAT about all transmissions on LTE PC5 RAT.

  - Coordination function should inform NR PC5 RAT about all transmissions on LTE PC5 RAT, so that cross-RAT leakage can be properly handled in

    - Congestion control measurements by NR

    - NR sensing and resource selection procedures

Coordination w/ Network and In-Device Coexistence

In case of eNB and gNB controlled modes simultaneously acting across PC5 RATs, the in-device coexistence conflicts should be avoided by proper network scheduling. In case of in-device conflicts between network controlled modes and UE autonomous modes, UE may apply the same logic to resolve coexistence conflicts as in UE-autonomous resource allocation modes, except it cannot handle them in terms of resource allocation (UE is not expected to reselect scheduled by gNB/eNB resource).

In mixed scenario, when one RAT-A operates in network controlled mode and RAT-B operates in UE-autonomous mode, the network does not have information on resources selected in UE-autonomous mode. In this case, solutions used for co-existence conflicts for UE-autonomous modes can be reused.

  - In-device coexistence conflicts for network-controlled modes are addressed in the same way as for UE-autonomous modes

  - In device coexistence conflicts in mixed mode scenario, are addressed based on prioritization or using NR resource selection mechanisms

In-device Coexistence Conflicts b/w Uu and PC5 Air-interfaces

When UE has multiple Uu and PC5 air-interfaces there may be additional in-device coexistence conflicts with Uu transmissions on shared licensed carriers. Therefore at least the following conflicts may need to be addressed:

Prioritization of NR Uu UL transmissions over NR PC5 SL or LTE PC5 SL

Prioritization of LTE Uu UL transmissions over NR PC5 SL or LTE PC5 SL

In order to address these problems network may configure which sidelink transmission priority levels should have higher or lower priority than NR UL transmissions and/or LTE UL transmissions.

  - Prioritization of NR Uu UL transmissions over NR PC5 SL or LTE PC5 SL as well as LTE Uu UL transmissions over NR PC5 SL or LTE PC5 SL is supported

Conclusions

In this contribution, we have analyzed in-device coexistence problems and potential solutions to address those. Based on discussion, we have following proposals:

  - NR sidelink resource selection procedure supports exclusion of resources conflicting with LTE sidelink transmission

    - Exclusion of resources conflicting with LTE sidelink transmission is subject to priority and radio-layer conditions considerations

  - NR sidelink resource selection procedure does not exclude resources conflicting with LTE sidelink transmission if:

    - NR sidelink transmission priority has higher priority than LTE sidelink transmission (or NR sidelink transmission priority is higher than pre-configured priority level)

    - LTE sidelink channel is congested (i.e. CBR is above pre-configured threshold so that sidelink transmission and reception is not guaranteed anyway)

  - Coordination function should inform NR PC5 RAT about all transmissions on LTE PC5 RAT, so that cross-RAT leakage can be properly handled in

    - Congestion control measurements by NR

    - NR sensing and resource selection procedures

  - In-device coexistence conflicts for network-controlled modes are addressed in the same way as for UE-autonomous modes

  - In device coexistence conflicts in mixed mode scenario, are addressed based on prioritization or using NR resource selection mechanisms

  - Prioritization of NR Uu UL transmissions over NR PC5 SL or LTE PC5 SL as well as LTE Uu UL transmissions over NR PC5 SL or LTE PC5 SL is supported

References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LGE, Huawei, Shenzhen, China, March, 2019

- R1-1912203, “Physical structure details for NR V2X sidelink communication”, Intel Corporation, Reno, USA, November, 2019

- R1-1912204, “NR V2X sidelink communication in resource allocation mode-1”, Intel Corporation, Reno, USA, November, 2019

- R1-1912205, “Design of resource allocation mode-2 for NR V2X sidelink communication”, Intel Corporation, Reno, USA, November, 2019

- R1-1912206, “Synchronization for NR V2X sidelink”, Intel Corporation, Reno, USA, November, 2019

- R1-1912208, “Physical layer procedures for NR V2X sidelink”, Intel Corporation, Reno, USA, November, 2019

- R1-1912209, “Congestion control for NR V2X sidelink”, Intel Corporation, Reno, USA, November, 2019

- R1-1912210, “LTE sidelink communication under NR Uu control”, Intel Corporation, Reno, USA, November, 2019

Annex A – List of RAN1 WG Agreements on In-device Coexistence for NR-V2X Communication

In this section, we provide list of RAN1 WG agreements made for in-device coexistence topic during the previous meetings.

RAN1#94 Agreements - For the study of LTE-V2X and NR-V2X sidelink co-existence, at least the following scenarios are considered from the UEs perspective: - LTE sidelink and NR sidelink do not have any coordinated procedures - LTE sidelink and NR sidelink have coordinated procedures and half-duplex constraints are assumed - RAN1 will focus on this scenario in the SI - RAN1 focus on at least the following potential solutions for coexistence at least until the next meeting: - TDM of LTE V2X and NR V2X sidelink transmissions - FDM of LTE V2X and NR V2X sidelink transmissions RAN1#94bis Agreements - In the context of in-device coexistence between NR and LTE V2X sidelinks (not co-channel), - TDM solutions are those that prevent overlapping or simultaneous NR and LTE V2X sidelink transmissions. - FDM solutions are those that involve simultaneous transmissions of NR and LTE V2X sidelink transmissions and defining mechanisms for sharing the total device power between the two. RAN1#95 Agreements - Consider solutions for sidelink coexistence for the following: - Potential LTE V2X Tx and NR V2X Tx - Potential LTE V2X Tx and NR V2X Rx - Potential LTE V2X Rx and NR V2X Tx - FFS the case of potential LTE V2X Rx and NR V2X Rx, e.g., whether or not it can be handled implementation - RAN1 will identify both TDM and FDM solutions for coexistence. The specific support for each solution is FFS. - For FDM solutions: - For both dynamic and semi-static power allocation solutions, RAN1 assumes synchronization between NR and LTE V2X sidelinks, for a NR V2X UE when NR and LTE V2X sidelinks are intra-band - The case of inter-band is FFS - Note: If the identified solutions can be applied to systems that are not synchronized, then RAN1 may revisit this assumption. RAN1 – AdHoc-1901 - For TDM solutions for in-device coexistence between LTE and NR V2X: - Time Alignment - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers - For long term time scale TDM solutions for in-device coexistence between LTE and NR V2X: - For a UE with coexistence impact, non-overlapping (in time domain) resource pools are (pre-)configured for NR V2X and LTE V2X sidelinks - No information is exchanged between LTE and NR sidelinks within the UE - Long term time scale TDM solution is feasible from RAN1 point of view - Note: although feasible, it is expected that such a solution may have impact on latency, reliability and data rate requirements for some applications - No additional modifications to LTE specifications are needed - Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X, - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement - Inter-band FDM Solutions for coexistence - For static power assignment of Pc,max for each carrier - Synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink. - This FDM solution is feasible for resolution of Tx/Tx coexistence conflicts - If the band separation is large enough (based on RAN4 indication), then this FDM solution for coexistence is feasible for Tx/Rx coexistence - If the band separation is NOT large enough, then this FDM solution is not feasible for resolution of Tx/Rx coexistence conflicts - For dynamic power sharing between carriers, - FFS details of FDM solutions and whether they are feasible RAN1#96 Agreements - From RAN1 point of view, short term TDM solutions for NR and LTE V2X in-device coexistence is considered to be feasible for a UE when the load for the UE from LTE side and from NR side is at or below an acceptable level - For each occurrence of Tx/Tx overlap and of Tx/Rx overlap, one RAT is prioritized over another - High-level principles of prioritization (e.g., BSM is deemed to have a higher priority, etc.) of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation - From RAN1 point of view, for both intra-band and inter-band Tx/Tx FDM solutions for in-device coexistence are considered to be feasible, at least if the following conditions are met: - For the intra-band case for dynamic power sharing, NR and LTE transmissions are fully overlapped in the time domain, i.e., NR transmissions have to span the entire LTE TTI such that the total power across the transmissions is constant. - For intra-band and inter-band FDM dynamic power sharing solutions, the following additional conditions apply: - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers - For purposes of dynamic power sharing between LTE and NR Tx, - High-level principles of prioritization (e.g., BSM is deemed to have a higher priority, etc.) of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation - Rx/Rx coexistence are feasible for intra- & inter-band from RAN1 point of view - High-level principles of Rx/Rx coexistence of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation - Based on the study from physical layer specification perspective, in-device coexistence of LTE and NR sidelink is feasible for intra- & inter-band under the respective conditions & solutions for TX/TX, TX/RX, & RX/RX - In the TR, also provides a reference to the respective sections RAN1#96bis Agreements Conclusion: - RAN1 does not see any specification impact for support of Long Term Time-Scale TDM for coexistence of NR and LTE sidelinks Working assumption: - For Tx/Tx overlap, - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.) - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.) - RAN1 does not assume any impact to LTE physical layer specifications RAN1#97 Agreements - For Tx/Tx overlap, - Confirm the working assumption made in RAN1#96bis - UE capability is defined for short-term time-scale TDM for in-device coexistence - For Rx/Rx overlap, - Up to UE implementation to manage receptions of LTE and NR sidelinks. - RAN1#98 Agreements - Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then - It is up to UE implementation to handle LTE Tx/NR Rx overlap - It is up to UE implementation to handle NR Tx and LTE Rx overlap - RAN1 understand that NR V2X priority field and PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs - Ask SA2 to confirm the understanding. If understanding is incorrect, please provide solution - RAN1#98Bis Agreements - For Tx/Rx overlap, - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received - For sidelink synchronization signal/channel (including S-SSB and LTE SLSS/PSBCH) priority for a UE is (pre)-configured per UE - The (pre)-configured priority is used in the same way as the priority for other channel/signals w.r.t. prioritization for handling in-device co-existence - Note: it is understood that the same priority (pre)-configuration is intended for all the related UEs - The priority of PSFCH is set as the priority of the corresponding PSSCH. - Post- RAN1#98Bis e-mail Discussion Outcome - UE reports its capability to the network of whether it supports short-term time scale TDM solutions. - Resource allocation related information is not reported to other RAT.