3GPP TSG RAN WG1 #99	R1- 1912294

Reno, NV, 18th – 22nd November, 2019

Agenda item:		7.2.10.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Measurements for NR Positioning

Document for: 	Discussion and Decision

# Introduction

As part of the recently approved WID [1], RAN1 has the following objectives related to measurements for NR positioning:

Define UE measurements based on DL reference signals applicable for NR positioning. The following UE measurements are specified for serving, reference, and neighboring cells [RAN1]

  - DL RSTD (reference signal time difference) measurements for NR positioning

  - DL RSRP (reference signal received power) measurements for NR positioning

  - UE RX-TX time difference measurements for NR positioning

Define gNB measurements based on UL reference signals applicable for NR positioning. The following gNB measurements are specified [RAN1]:

  - UL RTOA (relative time of arrival) measurements for NR positioning

  - UL Angle of Arrival (AoA) measurements (including Azimuth and Zenith Angles) for NR positioning

  - UL RSRP (reference signal received power) measurements for NR positioning

  - gNB RX-TX time difference measurements for NR positioning

This contribution presents Nokia’s view on measurements for NR positioning. For Nokia’s view on the downlink reference signal design for NR positioning see [2]. In addition, see our companion contributions [3] and [4] for our views on UL reference signal design and physical layer procedures for NR positioning respectively.

# Discussion

## Quality metrics

Agreement:​ - Support reporting separate metric(s) corresponding to quality of each of the following measurements: - RSTD​ - UE Rx-Tx time difference​ - UL-RTOA​ - gNB Rx-Tx time difference​ - UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA) - FFS: The details of these quality metrics

At RAN1#97 the following agreement was reached:

At RAN1#98-bis good progress on the timing measurement quality metrics was reached. New quality metrics are required for UL angle of arrival measurements. A measurement comprises two angles, the azimuth of arrival (AoA) and the zenith of arrival (ZoA). In general, the errors of the two angle measurements are correlated. They are only uncorrelated in special cases where the axes of the error distribution depending on the antenna geometry are aligned with the coordinate curves.

Observation 1: In the general case the measurement errors of azimuth of arrival (AoA) and zenith of arrival (ZoA) are correlated.

The measurement errors can be characterized by a two-dimensional probability density function (PDF). In the following we describe two different options.

The most frequently used PDF is that of the Gaussian distribution. For a vector x the elements of which are the two measured angles it can be written as

pGaussx= 1(2π)2Σexp-12x-μTΣ-1x-μ.

μ is the mean value and Σ is the 2x2 covariance matrix. Assuming that the mean of the Gaussian error is zero, it is fully characterized by the covariance matrix alone. Due to its symmetry, the covariance matrix can be described by 3 real valued parameters.

Observation 2: Using a Gaussian PDF for characterizing zero-mean angle measurement errors for azimuth and zenith angles requires reporting of three (quantized) real values.

The natural way of defining a PDF for measurements of two angles, which correspond to points on the surface of the 3-dimensional unit sphere, is the Kent distribution. It is defined by its PDF

pKentx= 1c(κ,β)expκγ1Tx+ βγ2Tx2-γ3Tx2.

κ describes the concentration or spread and β the ellipticity. Both of them are real-valued. The vector γ1 is the mean direction, i.e., the three dimensional unit vector pointing to the point on the surface of the sphere that corresponds to the two angles, and vectors γ2 and γ3 are the major and minor axis respectively. The vectors γ1, γ2, γ3 are three-dimensional real-valued vectors and form together an orthonormal real-valued 3x3 matrix (γ1,γ2,γ3). Because the matrix is orthonormal, it can be fully described by 3 real-valued parameters. Note that the vector x is here also a three-dimensional unit vector, which defines a point on the surface of the sphere corresponding to the two angles in the spherical coordinate system. Assuming that the measurement values of the two angles have already been reported and the mean of the measurement error is zero, there is no need to report γ1 again. γ1 contains just two independent coordinates because it is normalized to unit length. The remaining parameters to be reported for characterizing the measurement errors are κ, β, γ2 and γ3. Since the normalized vectors γ2 and γ3 are mutually orthogonal and orthogonal to γ1, they are fully characterized by a single real valued parameter. Together with κ and β we have to report 3 real-valued parameters, which can of course be quantized.

Observation 3: Using a Kent PDF for characterizing zero-mean angle measurement errors for azimuth and zenith angles requires reporting of three (quantized) real values. This is the same amount of reporting information as for the covariance matrix of a Gaussian PDF.

An advantage of the Kent distribution is that it is less impacted by the nonlinearities of the spherical coordinate system than the Gaussian covariance matrix.

Proposal 1: Consider different options like a Gaussian PDF or a Kent PDF for defining a quality metric for two-dimensional angle measurements and define the quality metric to be used for reporting.

When we use a local spherical coordinate system (r',θ', φ') embedded in a Cartesian coordinate system (x’, y’,z’) for measuring and reporting angles that describe a direction in space, we identify the direction with a point on the unit sphere. The distortions introduced by the spherical coordinate system can be described by the metric tensor

1000r2000r2sin2θ'

where r=1 on the unit sphere.

The element r2sin2θ' indicates that a change dφ' of the angle φ' corresponds to a change ds=sinθ' dφ' of the point position describing the direction which depends on the angle θ' where ds is the line element on the unit sphere. I.e., if we reported errors of the angle φ' as a quality metric, the same number would correspond to different errors in the measured direction depending on the angle θ' and thus the quantization for reporting would have different impact for different angles θ'.

Observation 4: The actual change in direction corresponding to a measurement error in φ' depends on the second angle θ’.

There are several options for avoiding this. The first option is to use Kent distribution for reporting the error distribution. The Kent distribution doesn’t have this problem. The second option is to report errors of the transformed variable  φ''=sinθ' φ'. The third option is to rotate the coordinate system after having measured the angles θ'0 and φ'0 in such a way that in the rotated coordinate system r'',θ'',φ'' the direction described by θ''=π2 and φ''=0 points towards the measured direction described by θ'0 and φ'0. This can be done in a similar way as the transform from the local coordinate system (LCS) to the glogbal coordinate system in TR 38.901. In a first step we would rotate the coordinate system around the z’-axis such that in the rotated coordinate system (r,θ,φ) the angle φ=0 corresponds to φ'=φ'0. In a second step we would tilt the coordinate system around the new y-axis in such a way that the direction (θ=π2,  φ=0) corresponds to (θ=θ0',  φ=0). The coordinate system (r,θ,φ) is then denoted by r'',θ'',φ''.

In the rotated coordinate system r'',θ'',φ'' the metric tensor is

1000r2000r2sin2θ''

and the error distribution is located around θ''=π2 where sinθ''=1 and therefore the impact of the term sinθ'' is minimized. As a quality metric we would report then the covariance matrix

covθ''φ''

with respect to the angles θ'' and φ'' in the rotated coordinate system.

Proposal 2: Use a rotated coordinate system for reporting the error covariance matrix as a quality metric for the angle measurements. The coordinate system is rotated in such a way that the x’’-axis of the rotated system points towards the measured direction of arrival.

Note that the direction of arrival and the quality metric are reported in different coordinate system with a known relation to each other given by the measured direction. Therefore no additional reporting of the rotation angles is required.

Observation 5: No additional reporting of rotation angels is required to use the aboved rotated coordinate system for the quality metric.

Optionally the coordinate system could be further rotated around the x’’-axis in order to make the covariance matrix close to diagonal. In order to avoid the requirement to report this rotation angle, the antenna array can be oriented by convention in the local coordinate system in such a way that the off-diagonal elements of the covariance matrix ere minimized.

Proposal 3: The quality metric for the angle-of-arrival measurement includes the following fields:

- Three real values corresponding to the three degrees of freedom of the real-valued, symmetric covariance matrix

- FFS: Information on the resolution

- FFS: Number of samples used for determining the quality metric

## UE/gNB Reporting

At RAN1#98 the following agreements were made:

Agreement: - UL RTOA is defined with the respect to the subframe timing associated with the UE - Multiple SRS resources for positioning purposes can be used to determine the received UL subframe timing of the first arrival path of the UE. - FFS: The resource ID(s) or resource set ID(s) used for determining the timing of the UE and possibly the Rx beam used at the gNB in the UL RTOA measurements can be requested for reporting in the measurement report. Agreement: - gNB Rx-Tx time difference is defined with respect to the subframe timing associated with the UE - Multiple SRS resources for positioning purposes can be used to determine the received UL subframe timing of the first arrival path of the UE. - FFS: The resource ID(s) or resource set ID(s) used for determining the timing of the UE and possibly the Rx beam used at the gNB in the gNB Rx-Tx time difference measurements can be requested for reporting in the measurement report.

Next we discuss the FFS from the above agreement. The Rx beams used by the gNB can carry additional information about the direction of the UE. Additionally it may be useful for the LMF to know which SRS resources were used by the gNB to make either RTOA or gNB Rx-Tx time difference. As one example the LMF may need to know which SRS resource was used to measure the gNB Rx-Tx time difference so it can properly calculate the RTT between a UE and gNB.

At RAN1#98-bis the following agreement on the DL PRS was reached

Agreement: A DL PRS resource can be associated with one or more of the following in either GCS or LCS - azimuth angle - elevation angle - Note: The details of the message (e.g., resolution, range, etc.) can be defined in RAN3.

Therefore if the gNB uses the same beams for RX as it uses for transmission of the DL PRS there will be angular information of those beams. Reporting to the LMF which RX beam the gNB uses for an UL timing measurement provides additional information to that can assist in the positioning calculation.

Proposal 4: For both UL RTOA and gNB Rx-Tx time difference measurements support LMF requesting reporting of the resource ID(s) or resource set ID(s) used for determining the timing of the UE and the Rx beam used by the gNB for the measurement.

## Multiple Path Reporting

It is well known that when making timing measurements that multipath is a major issue. Finding the correct path for RSTD estimation is critical to improved positioning accuracy. During the SI phase many companies assumed that UEs make use of thresholding in order to find the first peak above a given value to help detect the correct peak. Thresholding is useful but the details of how the UE determines the peaks for RSTD measurements are not expected to be specified. In addition, thresholding does not remove all errors associated with finding the correct peak. Reporting of multiple peaks can allow further processing by the location server to attempt to correct errors.

Observation 6: It is beneficial to optionally have multiple RSTD peaks reported to the location server when possible.

In LTE the RSTD measurement procedure supported the UE reporting AdditionalPaths if requested and detected [5]. Specifically the AdditionalPath reporting allowed the UE to report a relative path timing for additional paths that it detects as well as the quality of that detected timing. From TS 36.355 those two fields were defined as follows:

AdditionalPath field descriptions

relativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing used for the rstd value in units of 0.5 Ts, with Ts=1/(15000*2048) seconds. A positive value indicates that the particular path is later in time than the detected path used for RSTD; a negative value indicates that the particular path is earlier in time than the detected path used for RSTD.

path-Quality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

Proposal 5: Additional path reporting should be supported in NR for at least the RSTD and RTOA measurements. The AdditionalPath fields of relativeTimeDifference and path-Quality from LTE can be used as a starting point in NR.

Proposal 6: RAN1 to discuss additional path reporting for UE Rx-Tx time difference and gNB Rx-Tx time difference.

At RAN1#98 it was agreed that

- At least the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in RSTD measurements can be configured for reporting in the measurement report.

In addition to reporting the additional path information proposed above it is also useful for the UE or gNB to report the resource ID(s) or resource set ID(s) used for determing the additional path timing.

Proposal 7: Support reporting of the resource ID(s) or resource set ID(s) in the AdditionalPath field of at least the RSTD and RTOA measurement reports.

# Conclusion

In this contribution we make the following observations:

Observation 1: In the general case the measurement errors of azimuth of arrival (AoA) and zenith of arrival (ZoA) are correlated.

Observation 2: Using a Gaussian PDF for characterizing zero-mean angle measurement errors for azimuth and zenith angles requires reporting of three (quantized) real values.

Observation 3: Using a Kent PDF for characterizing zero-mean angle measurement errors for azimuth and zenith angles requires reporting of three (quantized) real values. This is the same amount of reporting information as for the covariance matrix of a Gaussian PDF.

Observation 4: The actual change in direction corresponding to a measurement error in φ' depends on the second angle θ’.

Observation 5: No additional reporting of rotation angels is required to use the aboved rotated coordinate system for the quality metric.

Observation 6: It is beneficial to optionally have multiple RSTD peaks reported to the location server when possible.

In this contribution we make the following proposals:

Proposal 1: Consider different options like a Gaussian PDF or a Kent PDF for defining a quality metric for two-dimensional angle measurements and define the quality metric to be used for reporting.

Proposal 2: Use a rotated coordinate system for reporting the error covariance matrix as a quality metric for the angle measurements. The coordinate system is rotated in such a way that the x’’-axis of the rotated system points towards the measured direction of arrival.

Proposal 3: The quality metric for the angle-of-arrival measurement includes the following fields:

- Three real values corresponding to the three degrees of freedom of the real-valued, symmetric covariance matrix

- FFS: Information on the resolution

- FFS: Number of samples used for determining the quality metric

Proposal 4: For both UL RTOA and gNB Rx-Tx time difference measurements support LMF requesting reporting of the resource ID(s) or resource set ID(s) used for determining the timing of the UE and the Rx beam used by the gNB for the measurement.

Proposal 5: Additional path reporting should be supported in NR for at least the RSTD and RTOA measurements. The AdditionalPath fields of relativeTimeDifference and path-Quality from LTE can be used as a starting point in NR.

Proposal 6: RAN1 to discuss additional path reporting for UE Rx-Tx time difference and gNB Rx-Tx time difference.

Proposal 7: Support reporting of the resource ID(s) or resource set ID(s) in the AdditionalPath field of at least the RSTD and RTOA measurement reports.

# References

- RP-190723, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, RAN#83, Shenzhen, China, March 18-21, 2019.

- R1-1912292, “Views on DL reference signals for NR Positioning”, Nokia, Nokia Shanghai Bell, RAN1#99, Reno, Nevada, USA.

- R1-1912293, “Views on UL reference signals for NR Positioning”, Nokia, Nokia Shanghai Bell, RAN1#99, Reno, Nevada, USA.

- R1-1912295, “Views on physical layer procedures for NR Positioning”, Nokia, Nokia Shanghai Bell, RAN1#99, Reno, Nevada, USA.