3GPP TSG RAN WG1 Meeting #99	R1-1912337

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.1.1

Source: 	SONY

Title:	Remaining issues on Two-Step RACH Channel Structure

Document for:	Discussion and Decision

# Introduction

The work item on 2-step RACH for NR aims to specify 2-step RACH as general MAC procedure covering both physical layer and higher layer aspects [1]. In RAN1#98bis meeting the following agreements were made [2]:

Agreements:

- The initialization ID for msgA PUSCH scrambling is:

  - cinit = RA-RNTI216+RAPID210+nID

  - nID is a cell-specific higher-layer parameter if configured; otherwise nID =NIDcell

  - RA-RNTI is as same as Rel.15

  - FFS whether or not to replace the RAPID by DMRS index, if 1-to-multiple mapping between preambles and PRUs is supported.

- For the configuration of the msgA PUSCH waveform

  - Use a separate cell-specific parameter “msgA-transformPrecoder” to indicate the waveform of msgA PUSCH

    - If the parameter is not configured, msgA PUSCH follows the waveform of msg3.

- Different configuration periodicity for msgA PRACH and PUSCH is not supported in Rel.16

- Regarding the potential overlapping of msgA PUSCH occasions for a PUSCH configuration:

  - Limit the msgA PUSCH configuration so that overlapping between PUSCH occasions is not expected

- An msgA PUSCH occasion is considered as valid only if the following criteria are satisfied

  - it does not overlap (in time and frequency) with any 4-step or 2-step RACH occasions, and

  - FFS it does not span across the slot boundary, and

  - in addition, if a UE is provided TDD-UL-DL-ConfigurationCommon, a 2-step PUSCH occasion is considered as valid if the following criteria are satisfied

    - it is within UL symbols, or

    - it does not precede a SS/PBCH block in the PUSCH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block transmission symbol

      - FFS whether Ngap needs to be revisited

  - FFS other criteria (the gap between preamble and data for MsgA, etc.)

- At least support separate LBTs for msgA PRACH and PUSCH respectively, for 2-step RACH for NR-U

  - Strive to specify mechanisms to reduce LBTs

- All msgA PUSCH occasions and the associated msgA RACH occasions are confined within a single 20 MHz carrier/LBT bandwidth

- The starting of msgB window should follow that defined for 2-step RACH regardless of failure of LBT for msgA PUSCH.

- The ordering of the msgA PRACH preambles within an msgA association period is

  - First, in increasing order of preamble indexes within a single PRACH occasion

  - Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

  - Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

  - Fourth, in increasing order of indexes for PRACH slots

- The PRACH preambles are mapped to valid PUSCH resource units (PRUs) within an msgA association period in the following order:

  - First, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

  - Second, in increasing order of DMRS indexes within a single PUSCH occasion

    - FFS DMRS indexes for DMRS ports and/or sequences

  - Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot

  - Fourth, in increasing order of indexes for PUSCH slots

  - For multiple configurations, the mapping is between the PRUs under each msgA PUSCH configuration and the preambles in the corresponding preamble group

    - Each msgA PUSCH configurations can identify sub-sets of DMRS port/sequence combination

- For the RRC configuration of MCS and TBS for msgA PUSCH

  - Signalling MCS only

    - FFS the table and value range

- For a UE in RRC_IDLE/INACTIVE state, do not support more than 2 msgA PUSCH configurations for Rel.16

- For a UE in RRC_CONNECTED state,

- Support up to two msgA PUSCH configurations in an UL BWP

  - If msgA PUSCH configuration is not configured for the UL BWP, it can follow that of initial BWP.

  - (Working Assumption) Reuse the preamble group based method as defined for RRC_IDLE/INACTIVE state.

  - FFS: Whether the number of msgA PUSCH configuration(s) should be aligned with that of UEs in RRC RRC_IDLE/INACTIVE state.

  - To confirm whether PRACH configuration and msgA PUSCH configuration are both BWP specific or cell specific.

- For the definition of PRU, support both DMRS ports and DMRS sequences at least for CP-OFDM

  - More than 1 DMRS sequence can be configured, FFS the value

  - FFS whether/how to support multiple sequences for DFT-s-OFDM

  - The conditions under which only DM-RS ports are to be specified. FFS details

In this contribution, we provide our views on the 2-step RACH procedure particularly on the channel structure for the PUSCH in msgA of two-step RACH.

# Discussion

  - MsgA PUSCH Configuration

The WID objectives describe the preamble and MsgA PUSCH as TDM’ed [1]. Furthermore, RAN1 has agreed on the same periodicity for msgA PRACH and PUSCH. Hence, there should be a timing relationship or a time-gap between the preamble and MsgA. The time-gap can be used for example as the guard time period after preamble transmission, time-alignment to support various SCS, etc. The essential part is the gNB can configure the time-gap between preamble and MsgA PUSCH.

Proposal 1: gNB configures the relative time-offset between preamble and msgA PUSCH (msgAPUSCH-timeDomainOffset) by selecting one value from a possible set of values.

The transport block size (TBS) of MsgA PUSCH has also been investigated. The TBS should at least be enough to carry the contents of legacy Msg3. Hence, it only requires a small TBS size (i.e. 56 or 72 bits). In addition, it would also be beneficial if  small data can also be carried out in MsgA PUSCH. This may require larger TBS. In the last few RAN1 meetings, 500 and 1000 bits have been suggested as candidates. When in RRC-INACTIVE [or RRC-CONNECTED?] mode, a UE can use larger TBS in good channel conditions. In RAN1 #98bis, it was agreed that the gNB provides the MCS in the RRC configuration (in addition to the previously agreed number of PRBs). The UE can use large TBS by selecting the combination of higher MCS index and larger PRB. Considering robustness is important in MsgA PUSCH, the MCS options should only use QPSK modulation with some selected coding rates.

Proposal 2: Support Transport block size (TBS) with {56, 72, 500, 1000} bits.

As previously agreed, a UE can be configured with multiple 2-step RACH configurations. For RRC_INACTIVE/IDLE state, support up to two msgA PUSCH configurations for Rel.16. For RRC_CONNECTED state, also support two MsgA PUSCH configurations for each UL BWP. Considering 2-step RACH can be targeted to support various use-cases and scenarios, all four alternatives to indicate the selected 2-step RACH configurations can be supported. At least, alt.1 has been agreed as a working assumption for all RRC states in RAN1 #98bis.

Proposal 3: The following mechanisms /alternatives for the indication of different 2-step RACH configurations are supported considering 2-step RACH can be designed to support various RRC states:

- Alt.1: Using different preamble groups

- Alt 2: Using different preamble groups and/or RO partitioning

- Alt.3: Using UCI based indication

- Alt. 4: Using different DMRS ports/sequences

  - msgA PUSCH Scrambling

It was agreed in the previous meeting that RA-RNTI will be used for msgA PUSCH scrambling. Given that multiple PUSCH could occupy one PUSCH occasion, it would be desirable that each of the PUSCHs in any PUSCH occasion have different scramble IDs. If one-to-multiple mapping between preamble and PRU is supported, DMRS index has to be included into the c_init to distinguish multiple PUSCHs in the same PUSCH occasion, since the various PUSCHs cannot always have different scramble IDs if the RNTI uses only preamble index, and n_ID. Here, we propose to replace the RAPID by DMRS index in the generation of c_init as discussed in RAN1#98bis. On the other hand, we consider one-to-multiple mapping to have some potential issues, such as increase in the PRU collision rate. Hence, we prefer one-to-multiple mapping is not supported in Rel.16.

Proposal 4: One-to-multiple mapping between preamble and PRU is not supported in Rel-16. If supported, DMRS index should be included in the c_init for msgA PUSCH scrambling.

  - MsgA DMRS sequence

In RAN1#98 meeting, it was agreed to support both DMRS ports and DMRS sequence at least for CP-OFDM, and these FFS points were left;

- the number of DMRS sequence;

- whether to support multiple sequence for DFT-s-OFDM.

For the number of DMRS sequence for CP-OFDM, if the number of DMRS sequence supports more than 2, much specification change is necessary, while the discussion time is very limited to complete Rel-16. Therefore, the number of DMRS sequence should be up to 2. In Rel-15 NR, 2 scrambling IDs are configured for DMRS sequence generation for CP-OFDM. MsgA PUSCH of 2-step RACH should reuse the same mechanism as much as possible to minimize specification impact.

For DFT-s-OFDM, no multiple DMRS sequence was defined in Rel-15 NR specification. As discussed in the above, since the discussion time is very limited to complete Rel-16, multiple DMRs sequence should not be supported in Rel-16 and only DMRS antenna port should be applied to MsgA PUSCH resource.

Proposal 5: For CP-OFDM, the number of DMRS sequence should be up to 2.

Proposal 6: For DFT-s-OFDM, multiple DMRS sequence should not be supported in Rel-16.

  - MsgA PUSCH Periodicity

We consider 2-step RACH can be beneficial for legacy NR operation in some scenarios and also many specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC. Option 2 (relative location) provides less signaling overhead and also possible low-latency of PUSCH occasion (PO) after the RACH occasion (RO). In addition, as Option 1 is configured separately, the gNB has more freedom and flexibility in allocating PO. We prefer the gNB to have MsgA configuration with Option 2. After the UE transmits the preamble, the UE can immediately transmit PUSCH with a relative timing from the preamble transmission. We expect in such circumstances that the UE uses a fixed PUSCH transport block size (TBS). In addition, some UEs may require to transmit larger transport block sizes or PUSCH transmission with repetition. In such cases, we consider a large PUSCH allocation is required. Furthermore, MsgA PUSCH configuration with Option 1 is applicable in this case and the gNB can provide the allocation regardless of the RACH configuration.

Observation 1: 2-step RACH can be used to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and small data transmissions.

Observation 2: MsgA PUSCH with different periodicity can be considered for targeting some other use-cases (e.g. small data transmissions) in future release.

# Conclusion

In this contribution, we discussed the msgA channel structure of 2-step RACH procedure and conclude on the following observation and proposals:

Proposal 1: gNB configures the relative time-offset between preamble and msgA PUSCH (msgAPUSCH-timeDomainOffset) by selecting one value from possible set of values.

Proposal 2: Support Transport block size (TBS) with {56, 72, 500, 1000} bits.

Proposal 3: The following mechanisms /alternatives for the indications of different 2-step RACH configurations are supported considering 2-step RACH can be designed to support various RRC states:

- Alt.1: Using different preamble groups

- Alt 2: Using different preamble groups and/or RO partitioning

- Alt.3: Using UCI based indication

- Alt. 4: Using different DMRS ports/sequences

Proposal 4: One-to-multiple mapping between preamble and PRU is not supported in Rel-16.If supported, DMRS index should be included in the c_init for msgA PUSCH scrambling.

Proposal 5: For CP-OFDM, the number of DMRS sequence should be up to 2.

Proposal 6: For DFT-s-OFDM, multiple DMRS sequence should not be supported in Rel-16.

Observation 1: 2-step RACH can be used to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and small data transmissions.

Observation 2: MsgA PUSCH with different periodicity can be considered for targeting some other use-cases (e.g. small data transmissions) in future release.

# References

- RP-190711, “Revised work item: 2-step RACH for NR”, ZTE, Sanechips, Shenzhen, China, March, 2019.

- Chairman notes on RAN1#98bis, ETSI MCC