3GPP TSG RAN WG1 #99		R1-1912352

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.6.3

Source: 	Sony

Title:	Remaining issues in PUSCH Enhancements

Document for:	Discussion / decision

# Introduction

In RAN1#98bis, we made the following agreements:

Agreements:

- Do not support PUSCH mapping type A for Option 4.

Agreements:

- Rel-16 enhanced PUSCH scheme (including dynamic indication of the number of repetitions) is supported for DCI format 0_1 and new UL DCI format (for DG and type 2 CG).

- Rel-16 enhanced PUSCH scheme is not supported for DCI format 0_0 for DG and type 2 CG

Agreements:

For the dynamic indication of the number of repetitions for dynamic grant:

- Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table

  - The maximum TDRA table size is increased to 64

  - No other spec impact is expected

Agreements:

- Support dynamic indication of the number of repetitions for Rel-15 PUSCH with slot aggregation using DCI formats 0_1 & the new UL DCI format

  - The dynamic indication is done by using the same Rel-16 mechanism (Jointly coding the number of repetitions with SLIV in TDRA table)

Agreements:

For frequency hopping for Rel-16 PUSCH, the number of actual hopping locations in frequency is 2.

Agreements:

In case frequency hopping is enabled for Rel-16 PUSCH, to determine the frequency locations of the two hops, reuse Rel-15 RRC parameters and equations for format 0_1, and introduce new RRC parameters (same as those of Rel-15) for new DCI UL format.

- FFS time domain hopping pattern

Agreements:

In terms of how to interpret L and K for Rel-16 PUSCH transmissions (for both DG & CG), Alt. 1 is adopted.

- That is, for the Rel-16 PUSCH with enhanced repetition transmission, the time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.

Conclusion:

Definitions:

- “Rel-16 PUSCH transmission scheme”: Option 4

- “Rel-15 PUSCH transmission scheme”: the transmission is done according to Rel-15 behavior, either with or without slot aggregation. With slot aggregation, the number of repetitions can be either semi-statically configured (as in Rel-15) or dynamically indicated (as agreed for Rel-16).

Agreements:

For DG and retransmission of CG, introduce one RRC parameter for each of the DCI format 0_1 and the new UL DCI format, to indicate whether UE follows the behavior for “Rel-16 PUSCH transmission scheme” or the behavior for “Rel-15 PUSCH transmission scheme”.

- FFS: whether to restrict that “Rel-16 PUSCH transmission scheme” cannot be enabled for both DCI formats simultaneously

For Type 1 CG, introduce an RRC parameter per CG configuration to indicate whether UE follows the behavior for “Rel-16 PUSCH transmission scheme” or the behavior for “Rel-15 PUSCH transmission scheme”.

Agreements:

For Type 2 CG, UE uses the PUSCH transmission scheme (“Rel-16 PUSCH transmission scheme” or “Rel-15 PUSCH transmission scheme”) associated with the activating DCI format.

Agreements:

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 1-4 is not further considered for both DG and CG

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 1-2 is not further considered for DG.

Agreements:

For the interaction with DL/UL directions, if dynamic SFI is configured, Option 2-2 and 2-3 is not further considered for DG.

Agreements:

- For both DG and CG with “Rel-16 PUSCH transmission scheme”, if dynamic SFI is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs at least around semi-static DL symbols.

  - FFS segmentation also around dynamically indicated invalid symbols for UL transmissions in the UL grant (if supported for DG and/or Type 2 CG) and/or semi-statically configured invalid symbols for UL transmissions (if supported)

  - FFS how to handle the conflict with dynamic DL transmission for CG

This contribution considers some remaining issues on PUSCH enhancements for eURLLC in Rel-16.

# Discussions

## Interactions with dynamic SFI

Dynamic SFI is transmitted using DCI Format 2_0 which can convert a semi-static Flexible symbol into an UL or a DL symbol, thereby providing flexibility for the gNB to manage the slot resources.  However, it is noted that dynamic SFI is not transmitted with the same reliability as an UL grant for URLLC [1].  For example, DCI Format 2_0 may have a target BLER of 10-2 whilst the UL Grant for URLLC PUSCH requires BLER of 10-5 to 10-6.  That is dynamic SFI is 1000 to 10,000 times less reliable than an UL Grant for URLLC PUSCH.

Observation 1: Dynamic SFI carried by DCI Format 2_0 can be 1000 to 10,000 times less reliable than an UL Grant for URLLC PUSCH.

Even if dynamic SFI matches the reliability of an URLLC UL grant, the combination of dynamic SFI and the URLLC UL grant would not provide the required URLLC reliability [2].  Hence, dynamic SFI does not provide the required reliability for URLLC.  Consequently , dynamic SFI cannot be used for URLLC transmission, albeit it can be used for eMBB transmission for the same UE.

Observation 2: The use of dynamic SFI would not provide the required URLLC reliability for PUSCH transmission.

Proposal 1: URLLC PUSCH transmission does NOT depend on dynamic SFI.

For Dynamic Grant the remaining options where the UE behaviour is NOT dependent on dynamic SFI are:

- Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

- Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

Option 1-1 removes the flexibility of dynamic SFI since all semi-static flexible symbols are used for PUSCH transmissions.  This may starve the DL of DL resources for URLLC PDSCH.

Observation 3: Option 1-1 where all flexible symbols are used for URLLC PUSCH transmission removes the benefit of dynamic SFI and may deprive DL resources for URLLC PDSCH.

Option 1-3 retains the benefit of dynamic SFI and also ensures the reliability of the SFI by transmitting the SFI in the UL Grant for the URLLC PUSCH.  However, the SFI-Index requires max{log2(maxSFIIndex+1), 1} bits, which can be significant.  An alternative is to use a single bit to indicate whether the semi-static Flexible symbols can all be used for PUSCH or not.  This is the preferred option as it allows some of the benefits of dynamic SFI, meets the reliability of URLLC and uses only a single bit.

Proposal 2: For interactions with dynamic SFI in DG, a single bit is used in the UL grant to indicate whether all semi-static flexible symbols or none of the them are used for PUSCH transmission.  Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

For Configured Grant, the only option left if UE behaviour does not depend on dynamic SFI is:

- Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

Hence, we propose this option.

Proposal 3: For interactions with dynamic SFI in CG, semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

## Handling of Orphan Symbol

When a PUSCH is split into multiple repetitions, some of the repetitions may only be 1 OFDM symbol long, which by itself may not contain any information especially when a front loaded DMRS is used, i.e. an “orphan” symbol is created.  This orphan symbol can be dropped or it can simply be an extension to another adjacent PUSCH repetition (i.e. without any interrupted symbols in between).  We have a preference that this orphan symbol is utilized since the resource is already scheduled for the UE.

Proposal 4: When a split PUSCH repetition contains only 1 symbol, this symbol is utilized by an adjacent PUSCH repetition, i.e. the adjacent PUSCH repetition is extended by 1 symbol.

## SLIV Calculation

In Rel-15, the SLIV for a PUSCH with starting symbol S and duration L, is calculated as [3]:

if  then

else

where

Where this calculation requires that the PUSCH does not cross a slot boundary, i.e. S+L ≤ 14.  However, the Rel-16 PUSCH for eURLLC is allowed to cross a slot boundary and if the Rel-15 SLIV calculation is also directly used for S+L>14, it will lead to ambiguity as the SLIV values represent two different sets of {S, L} as shown in Table 1 (where entries in red are for S+L>14).  For example, SLIV=27 can represent {S=13, L=2} or {S=0, L=14}.

Table 1: SLIV allowing S+L>14

| Start, S | Length, L |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 14 | 28 | 42 | 56 | 70 | 84 | 98 | 97 | 83 | 69 | 55 | 41 | 27 |
| 1 | 1 | 15 | 29 | 43 | 57 | 71 | 85 | 99 | 96 | 82 | 68 | 54 | 40 | 26 |
| 2 | 2 | 16 | 30 | 44 | 58 | 72 | 86 | 100 | 95 | 81 | 67 | 53 | 39 | 25 |
| 3 | 3 | 17 | 31 | 45 | 59 | 73 | 87 | 101 | 94 | 80 | 66 | 52 | 38 | 24 |
| 4 | 4 | 18 | 32 | 46 | 60 | 74 | 88 | 102 | 93 | 79 | 65 | 51 | 37 | 23 |
| 5 | 5 | 19 | 33 | 47 | 61 | 75 | 89 | 103 | 92 | 78 | 64 | 50 | 36 | 22 |
| 6 | 6 | 20 | 34 | 48 | 62 | 76 | 90 | 104 | 91 | 77 | 63 | 49 | 35 | 21 |
| 7 | 7 | 21 | 35 | 49 | 63 | 77 | 91 | 105 | 90 | 76 | 62 | 48 | 34 | 20 |
| 8 | 8 | 22 | 36 | 50 | 64 | 78 | 92 | 106 | 89 | 75 | 61 | 47 | 33 | 19 |
| 9 | 9 | 23 | 37 | 51 | 65 | 79 | 93 | 107 | 88 | 74 | 60 | 46 | 32 | 18 |
| 10 | 10 | 24 | 38 | 52 | 66 | 80 | 94 | 108 | 87 | 73 | 59 | 45 | 31 | 17 |
| 11 | 11 | 25 | 39 | 53 | 67 | 81 | 95 | 109 | 86 | 72 | 58 | 44 | 30 | 16 |
| 12 | 12 | 26 | 40 | 54 | 68 | 82 | 96 | 110 | 85 | 71 | 57 | 43 | 29 | 15 |
| 13 | 13 | 27 | 41 | 55 | 69 | 83 | 97 | 111 | 84 | 70 | 56 | 42 | 28 | 14 |

An easy way to solve this ambiguity without changing the structure of the TDRA table, i.e. without introducing any new columns/fields to the TDRA table or any new bits in the DCI, is to increase the range of SLIV.  This can be done by adding an offset “OSLIV ” to the SLIV calculation as follows:

if  then

SLIV=14⋅L-1+S+OSLIV

else

SLIV=14⋅14-L+1+14-1-S+OSLIV

where

OSLIV = 0; 	for 0 < S+L ≤ 14

OSLIV =105; 	for S+L > 14

The UE will recognize that any SLIV ≥ 105 means S+L>14, i.e. the PUSCH crosses a slot boundary.  This will give a table with unique SLIV values for each combination of {S, L}as shown in Table 2.

Table 2: SLIV with offset OSLIV

| Start, S | Length, L |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 14 | 28 | 42 | 56 | 70 | 84 | 98 | 97 | 83 | 69 | 55 | 41 | 27 |
| 1 | 1 | 15 | 29 | 43 | 57 | 71 | 85 | 99 | 96 | 82 | 68 | 54 | 40 | 131 |
| 2 | 2 | 16 | 30 | 44 | 58 | 72 | 86 | 100 | 95 | 81 | 67 | 53 | 144 | 130 |
| 3 | 3 | 17 | 31 | 45 | 59 | 73 | 87 | 101 | 94 | 80 | 66 | 157 | 143 | 129 |
| 4 | 4 | 18 | 32 | 46 | 60 | 74 | 88 | 102 | 93 | 79 | 170 | 156 | 142 | 128 |
| 5 | 5 | 19 | 33 | 47 | 61 | 75 | 89 | 103 | 92 | 183 | 169 | 155 | 141 | 127 |
| 6 | 6 | 20 | 34 | 48 | 62 | 76 | 90 | 104 | 196 | 182 | 168 | 154 | 140 | 126 |
| 7 | 7 | 21 | 35 | 49 | 63 | 77 | 91 | 210 | 195 | 181 | 167 | 153 | 139 | 125 |
| 8 | 8 | 22 | 36 | 50 | 64 | 78 | 197 | 211 | 194 | 180 | 166 | 152 | 138 | 124 |
| 9 | 9 | 23 | 37 | 51 | 65 | 184 | 198 | 212 | 193 | 179 | 165 | 151 | 137 | 123 |
| 10 | 10 | 24 | 38 | 52 | 171 | 185 | 199 | 213 | 192 | 178 | 164 | 150 | 136 | 122 |
| 11 | 11 | 25 | 39 | 158 | 172 | 186 | 200 | 214 | 191 | 177 | 163 | 149 | 135 | 121 |
| 12 | 12 | 26 | 145 | 159 | 173 | 187 | 201 | 215 | 190 | 176 | 162 | 148 | 134 | 120 |
| 13 | 13 | 132 | 146 | 160 | 174 | 188 | 202 | 216 | 189 | 175 | 161 | 147 | 133 | 119 |

Proposal 5: For Rel-16 eURLLC PUSCH, to support PUSCH crossing slot boundary (S + L > 14), an offset OSLIV is added to the SLIV calculation as follows:

if (L-1)≤7 then

SLIV=14⋅L-1+S+OSLIV

else

SLIV=14⋅14-L+1+14-1-S+OSLIV

where

OSLIV = 0; 	for 0 < S+L ≤ 14

OSLIV =105; 	for S+L > 14

## DMRS

Since PUSCH mapping Type A is not supported in Rel-16 PUSCH then each nominal PUSCH repetition would have a front loaded DMRS.  When a nominal PUSCH repetition is segmented into multiple repetitions, only the first PUSCH segment contains DMRS and the other segments would not have any DMRS.  It is therefore proposed that a front loaded DMRS is transmitted at the start of each segmented PUSCH.

Proposal 6: When a nominal PUSCH repetition is segmented into multiple repetitions at the slot/UL period boundary, a front loaded DMRS is transmitted at the beginning of each repetition of the segmented PUSCH.

## TBS Determination

The TBS can be determined based on one of the PUSCH repetitions or based on all the repetitions.  The TBS based on a single repetition, such as the 1st repetition, may not provide sufficient RE resources for a low code rate.  If a low code rate is required and the PUSCH repetition duration is short (e.g. 2 or 3 symbols), then a large number of PRBs may be required.  There may not be sufficient frequency resource to provide a high number of PRBs and furthermore, a PUSCH transmission using a large amount of frequency resource would result in low Power Spectral Density which is not desirable, especially for a UE at the cell edge.  Hence, we prefer that the TBS is based on the resources from all repetitions, where the encoded information bits can be placed in a circular buffer such that each PUSCH repetition is a different RV (i.e. contains different parts of the circular buffer).

Proposal 7: The TBS is determined based on total allocated resources.

## Maximum PUSCH Length

It has been proposed that the length L of a single PUSCH transmission can be larger than 14 symbols (up to 28 symbols) [4].  The rationale for having a PUSCH with L > 14 is that for an intended PUSCH duration, e.g. 28 symbols, the overall transmission would require fewer repetitions. For example, in Figure 4, the intended PUSCH duration is 28 symbols, i.e. L×K=28.  If L ≤ 14 (restricted to 14 symbols), then to minimize the number of repetitions, this would require L=14, repetition K=2 and if the PUSCH starts at the 8th symbol of the slot, it would lead to 4 overall repetitions since each of the nominal PUSCH repetitions is split into two at the slot boundaries. On the other hand, if L > 14, then for an intended PUSCH duration of 28 symbols, the allocation can be L=28 and K=1 which would result in 3 overall repetitions as the single nominal PUSCH repetition is split into 3 segments.  For the case where L=28 & K=1, there is one less DMRS symbol compared to the other case (L=14 & K=2) since it has one less PUSCH repetition.  We do not see this as a major benefit for having L > 14.  It should also be noted that the extra DMRS can actually improve the decoding performance e.g. allowing more spatial diversity or improving the channel estimation.

Figure 4: PUSCH with duration of 28 symbols

Proposal 8: The duration of the PUSCH is not larger than 14 symbols, i.e. L ≤ 14.

# Conclusion

In this contribution, we discuss some remaining issues in PUSCH enhancements for eURLLC.  We observe the following:

Observation 1: Dynamic SFI carried by DCI Format 2_0 can be 1000 to 10,000 times less reliable than an UL Grant for URLLC PUSCH.

Observation 2: The use of dynamic SFI would not provide the required URLLC reliability for PUSCH transmission.

Observation 3: Option 1-1 where all flexible symbols are used for URLLC PUSCH transmission removes the benefit of dynamic SFI and may deprive DL resources for URLLC PDSCH.

We therefore propose the following:

Proposal 1: URLLC PUSCH transmission does NOT depend on dynamic SFI.

Proposal 2: For interactions with dynamic SFI in DG, a single bit is used in the UL grant to indicate whether all semi-static flexible symbols or none of the them are used for PUSCH transmission.  Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

Proposal 3: For interactions with dynamic SFI in CG, semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

Proposal 4: When a split PUSCH repetition contains only 1 symbol, this symbol is utilized by an adjacent PUSCH repetition, i.e. the adjacent PUSCH repetition is extended by 1 symbol.

Proposal 5: For Rel-16 eURLLC PUSCH, to support PUSCH crossing slot boundary (S + L > 14), an offset OSLIV is added to the SLIV calculation as follows:

if (L-1)≤7 then

SLIV=14⋅L-1+S+OSLIV

else

SLIV=14⋅14-L+1+14-1-S+OSLIV

where

OSLIV = 0; 	for 0 < S+L ≤ 14

OSLIV =105; 	for S+L > 14

Proposal 6: When a nominal PUSCH repetition is segmented into multiple repetitions at the slot/UL period boundary, a front loaded DMRS is transmitted at the beginning of each repetition of the segmented PUSCH.

Proposal 7: The TBS is determined based on total allocated resources.

Proposal 8: The duration of the PUSCH is not larger than 14 symbols, i.e. L ≤ 14.

# References

[1] R1-1906808, “On PUSCH enhancements for eURLLC,” Intel Corporation, RAN1#97

[2] R1-1908779, “PUSCH Enhancements for URLLC,” Sony, RAN1#98

[3] TS38.214, “Physical layer procedures for data (Release 15)” v15.6.0

[4] R1-1910547, “PUSCH Enhancements for NR URLLC,” Ericsson, RAN1#98bis