3GPP TSG RAN WG1 #99							R1- 1912384

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	6.2.2.3

Source: 	LG Electronics

Title: 	Discussion on multiple transport blocks scheduling in NB-IoT

Document for:	Discussion and decision

- Introduction

In the RAN#85 meeting, WID of NB-IoT enhancement was agreed [1]. Also, several agreements on scheduling of multiple DL/UL transport blocks were made [2]. In this contribution, we discuss and provide our view on scheduling of multiple DL/UL transport blocks for NB-IoT.

- Discussions

  - Unicast

HARQ-ACK feedback

One of the major differences from the legacy HARQ process could be a method of HARQ-ACK reporting. According to the agreement in a previous meeting, individual HARQ-ACK feedback for each HARQ process is supported for multi-TB scheduling. In a simplest way, reusing legacy HARQ-ACK feedback channel design with some modification on scheduling can be considered. However, it should be noted that power consumption efficiency and resource overhead problem of multiple HARQ process are not considered in legacy HARQ-ACK feedback channel design. Thus, enhanced HARQ-ACK feedback design should be discussed further.

Unlike legacy HARQ process with individual DCI, UE could notice exact number of HARQ process at once using single DCI. So, transmitting multiple HARQ-ACK feedback using single HARQ-ACK channel can be work in this method. In this point of view, HARQ-ACK bundling could be considered. UE in deep coverage may require HARQ-ACK bundling, which represents one-bit bundled HARQ-ACK information, to reduce the latency due to the HARQ-ACK reporting.

In a previous meeting, it was agreed that the repetitions for one transport block can be either contiguously transmitted or interleaved. In interleaved transmission is applied, it can be expected that the similarity of channel condition between TBs would be increased. Thus, applying HARQ-ACK bundling may have benefits at least when repetition of TBs are interleaved. Also, likewise MTC, it can be expected that scheduled TBs may have similar SINR condition if repetition is not applied to NPDSCH transmission. Thus there is no reason not to apply bundling for non-repetition case if interleaved transmission is enabled by a higher layer signal.

Proposal 1: Bundled HARQ-ACK feedback is implicitly enabled when interleaved transmission is enabled by a higher layer signal

If individual HARQ-ACK is supported, HARQ-ACK resources are scheduled for the number of scheduled HARQ processes. In this case, it would be beneficial to transmit HARQ-ACK feedback for ACK reporting only, while DTX is used for NACK representation. In case of DTX, UE can save the power for a HARQ-ACK transmission. However, if DTX is used for representing NACK for individual HARQ processes, eNB may not distinguish all NACK case from DCI missing case. To solve the problem, representing NACK on a scheduled HARQ-ACK resource only for all NACK case can be considered.

Proposal 2: If individual HARQ-ACK is used in multi-TB scheduling, UE transmits HARQ-ACK for ACK reporting while DTX is used for NACK representation.

- Explicit NACK transmission can be considered to represent NACK for all scheduled HARQ processes

  - SC-PTM

In NB-IoT, SC-PTM can be used for multicast. It can be used for various types of service. In some SC-PTM applications, data traffic to be transmitted by multicast channel may exceed the maximum TBS which can be used for SC-MTCH transmission (e.g. firmware update). Also, SC-PTM can be used for periodic data which may have stable traffic pattern (e.g. billboard). In these cases, eNB can expect the required traffic pattern to be transmitted. Thus, instead of transmitting DCI for every SC-MTCH transmission, configure pre-defined transmission pattern for multicast purpose would be beneficial in some scenario.

To indicating scheduling information of multiple TBs for SC-MTCH two candidate approaches could be considered. First, indicating scheduling information using DCI could be considered. For example, number of TBs and their resource allocation can be conveyed by DCI. In this approach, it seems necessary to introduce new DCI design. Thus, search space configuration and G-RNTI value should be configured separately only for multi-TB capable Rel-16 UEs. New DCI based dynamic multiple TBs scheduling can have benefits in scheduling flexibility point of view. In this case, design principle of multi-TB scheduling DCI for unicast can be shared to reduce the effort of standardization. However, legacy multi-TB non-capable UEs that requires same SC-PTM service with multi-TB capable UEs cannot recognize the new DCI. Thus eNB may need to transmit same SC-MTCH multiple times, resulting in increased the DL resource overhead. It should be noted that one of the main purpose of introducing the multi-TB transmission is to reduce the network overhead.

Observation 1: It is beneficial to the network in terms of scheduling flexibility but the network overhead would be increased if new DCI with separate G-RNTI which can dynamically schedule multiple NPDSCHs for SC-MTCH is introduced.

Alternatively, using same DCI and G-RNTI with legacy UE for multi-TB scheduling in SC-MTCH can be considered. If the same DCI and G-RNTI is used for Rel-16 multi-TB capable UEs, SC-MTCH transmission for legacy SC-MTCH would be easily shared with Rel-16 multiple TBs capable UEs by configuring same G-RNTI. Thus, in resource efficiency point of view, reusing same DCI and G-RNTI may be more beneficial than new DCI design approach. Note that there is no remaining bit or state in legacy DCI format for SC-MTCH, so dynamic scheduling of multiple SC-MTCH cannot be performed. Instead, pre-defined DCI skipping rule in UE perspective can be introduced that can be configured by SC-PTM configuration message in SC-MCCH. For example, if Rel-16 multi-TB capable UE obtain grant for SC-MTCH, it can skip following search spaces but can expect multiple SC-MTCH transmission. Number of SC-MTCH transmission that can be expected by UE with a single DCI can be configured by SC-MCCH. In this approach, SC-MTCH transmission in eNB perspective is same as for legacy UEs. In UE perspective, power consumption efficiency could be increased by reducing blind decoding attempts. Figure 1 shows an example of Rel-16 UE behavior with DCI skipping method.

Observation 2: It is beneficial to both UE and network in terms of power and downlink resource efficiency if UE is allowed to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Proposal 3: For multiple SC-MTCH transmission, introduce DCI skipping mechanism which allows UE to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Figure 1 Example of skipping DCI method for SC-MTCH

- Conclusion

In this contribution, we discuss and provide our view on the multiple transport blocks scheduling for NB-IoT. The proposals of this contribution are summarized as follows.

Observation 1: It is beneficial to the network in terms of scheduling flexibility but the network overhead would be increased if new DCI with separate G-RNTI which can dynamically schedule multiple NPDSCHs for SC-MTCH is introduced.

Observation 2: It is beneficial to both UE and network in terms of power and downlink resource efficiency if UE is allowed to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Proposal 1: Bundled HARQ-ACK feedback is implicitly enabled when interleaved transmission is enabled by a higher layer signal

Proposal 2: If individual HARQ-ACK is used in multi-TB scheduling, UE transmits HARQ-ACK for ACK reporting while DTX is used for NACK representation.

- Explicit NACK transmission can be considered to represent NACK for all scheduled HARQ processes

Proposal 3: For multiple SC-MTCH transmission, introduce DCI skipping mechanism which allows UE to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

- Reference

- RP-192313, “WID revision: Additional enhancements for NB-IoT,” Futurewei

- R1-1911575, RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT, Futurewei