3GPP TSG RAN WG1 Meeting #99	            R1-1912450

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.2.2.2

Source:	Samsung

Title:	Enhancements to Initial Access Procedure for NR-U

Document for:	Discussion and Decision

# Introduction

Agreement:

If 2 bits can be found to not be necessary in the current MIB, then Parameter Q is indicated in the MIB by a serving cell. Otherwise, Q is indicated in the RMSI.

Agreement:

For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states,

- Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell.

- Support signaling from the serving cell of a Q value for a listed neighbour cell.

  - If Q is signaled for a listed cell, it overrides any common Q value per frequency

Agreement:

- No further down selection is made for set of Q values, i.e., Q = {1,2,4,8} (resolves FFS of prior agreement on Q values)

- UE does not expect to be configured with two Type-0 PDCCH monitoring occasions in a slot with Q=1.

- Note: This does not imply a change in Rel-15 behavior for Type-0 PDCCH monitoring in two consecutive slots.

- Note: This agreement does not imply any changes to Rel-15 PDCCH monitoring requirements on BD/CCE limits per slot

- RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8. This assumption does not have any other specification impact.

  - Include this agreement in an LS to RAN4 asking them to take this into account. The LS should also provide an overview on the definition of Q.

Agreement:

- The RSSI measurement timing configuration (RMTC) includes the following:

- Time-domain parameters:

- Periodicity (e.g., 40/80/160/320/640 ms)

- Measurement duration in terms of OFDM symbols with a configured reference subcarrier spacing

- Offset of RMTC measurement duration

- Frequency-domain parameters:

- Measurement bandwidth at least in units of LBT bandwidths

  - FFS: Units other than LBT bandwidths

  - Note: RAN4 can determine if the bandwidth used for the measurement within an LBT bandwidth can be less than the signaled measurement bandwidth.

- Measurement ARFCN for inter-frequency measurements

- Configurable L3 filtering as in RSSI for LTE-LAA

Conclusion:

No new medium contention/load metrics other than channel occupancy are introduced

Agreement:

Support configuration of the DRS transmission window duration of a UE’s serving cell(s)

- DRS transmission window periodicity is the same as the configuration of SSB burst periodicity

- Note: RAN4 performance requirements depending on the DRS transmission window size, e.g., RRM, will assume a DRS transmission window size of 5 ms

Agreement:

The ssb-PositionsInBurst IE is configured as in Rel-15 NR

- FFS: Interpretation of the field for NR-U

In RAN1 #98b meeting [1], the following agreements regarding the initial access and mobility procedure were made:

This contribution discusses the details of the agreed study points and other aspects related to the initial access and mobility procedure for NR-U:

- Enhancement to SS/PBCH Block Transmission

- Enhancement to Random Access Procedure

- Enhancement to RRM Measurement

- Enhancement to RLM Procedure

# Enhancement to SS/PBCH Block Transmission

## DRS transmission window duration

It was agreed that the duration of DRS transmission window can be configurable and the maximum configurable value is 5 ms. One FFS point is the granularity for the configuration, and we believe the granularity same as SMTC should be sufficient. Hence, we have the following proposal.

Proposal 1: Duration of DRS transmission window can be configurable from {1, 2, 3, 4, 5} ms.

## QCL parameter indication

In the last meeting, it was agreed that the QCL parameter for serving cell can be indicated by MIB or SIB1, depending on the available number of bits in the MIB. From our analysis in [3], at least 2 reserved bits can be available from the field controlResourceSetZero in MIB. Also taking into consideration that the bit subCarrierSpacingCommon in MIB could also be available, we have the following proposal. The intention for Approach 2 is to leave more reserved configurations for CORESET #0.

Proposal 2: The QCL parameter for serving cell can be indicated by

- Approach 1: 2 MSB of controlResourceSetZero in MIB; or

- Approach 2: combination MSB of controlResourceSetZero and subCarrierSpacingCommon in MIB.

## Actually transmitted SS/PBCH blocks

In NR Rel-15, actually transmitted SS/PBCH blocks are indicated to UE by RMSI content and RRC parameter. This cell-specific configuration is mainly for the purpose of rate matching for serving cell. After a UE detects any of the SS/PBCH block within a burst of SS/PBCH blocks, and decoding the corresponding RMSI, the UE is not only aware of the timing information indicated by the received SS/PBCH block, but also aware of whether the other SS/PBCH blocks in the burst are transmitted or not based on the indication of actually transmitted SS/PBCH blocks in RMSI. Meanwhile, by reading the indication of actually transmitted SS/PBCH blocks, any UE after initial access is aware of the resources reserved for SS/PBCH block transmission in the serving cell, even without receiving SS/PBCH block, and hence performs rate matching based on the reserved resources. However, for NR-U, due to the wrapped-around transmission, a UE cannot be fully aware of the transmission of the other SS/PBCH blocks in the burst based on the the indication of actually transmitted SS/PBCH blocks, by only receiving one of the SS/PBCH blocks in the burst, and the UE also has confusion on the rate matching behaviour in term of the reserved resources. For example, an illustration of this issue is shown in Figure 1, wherein the actually transmitted SS/PBCH blocks are both full bitmap (i.e., all the 8 SS/PBCH blocks) in Scenario 1 and Scenario 2. In Scenario 1, the burst of SS/PBCH blocks starts from the first slot, while in Scenario 2, the burst of SS/PBCH blocks starts in a later slot based on a wrapped-around operation. If a UE only receives SS/PBCH block #5 (filled with red in the figure), the UE cannot distinguish actual transmission is according to Scenario 1 or Scenario 2.

Figure 1 Illustration of the issue with actually transmitted SS/PBCH blocks.

The solution proposed in this contribution is extending the indication of the actually transmitted SS/PBCH blocks such that it is not impacted by the outcome of LBT. For this, we propose the “potentially actually transmitted SS/PBCH block locations”, which is defined as the indication of actually transmitted SS/PBCH blocks with a modulo operation based on the value Q within the DRS transmission window. Figure 2 illustrates an example, wherein the indication of actually transmitted SS/PBCH blocks is “11000000”, and modulo value Q = 8. Then, the “potentially actually transmitted SS/PBCH block locations” within a DRS transmission window with size 20 are given by {0, 1, 8, 9, 16, 17}. The potentially actually transmitted SS/PBCH block locations should be included as part of the reserved resource for PDSCH rate matching, and the PDSCH of concern can be any PDSCH in general other than the PDSCH of RMSI.

Figure 2 Illustration of “potentially actually transmitted SS/PBCH block locations”.

Based on above discussion, we have the following proposal for actually transmitted SS/PBCH blocks and rate matching.

Proposal 3: The indication of actually transmitted SS/PBCH blocks for NR-U is based on the length-8 bitmap supported in Rel-15.

- A UE assumes the number of actually transmitted SS/PBCH blocks is equal to or smaller than Q;

- A UE only expects to be configured as “1” in the first Q bits of the length-8 bitmap.

Proposal 4: A UE assumes the resources mapped for potentially actually transmitted SS/PBCH block locations within a DRS transmission window are not available for PDSCH, wherein the PDSCH is not for RMSI.

- The potentially actually transmitted SS/PBCH block locations are defined by the indication of actually transmitted SS/PBCH blocks with a modulo operation based on the value Q.

## Type0-PDCCH Monitoring

In the last RAN1 meeting, it was agreed that candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present. The remaining issue is how to associate the monitoring slots with SS/PBCH blocks. In Rel-15, for multiplexing pattern 1, the number of monitoring slots associated to a SS/PBCH block is 2, and the slots are determined based on parameter O and M, which were originally designed for licensed band. The following aspects are observed such that the determination of the monitoring slots may not be proper for unlicensed band.

First of all, the transmission on unlicensed band should strive for being contiguous such that there is no gap between RMSI slots, which makes the value of M as 2 not properly supported for NR-U, and hence, there is no need to monitor 2 slots for NR-U.

Then, even for the configuration with M = 1, the total duration of transmission burst variates with successful LBT location, which challenges the definition of LBT associated to this type of transmission. An illustration of this issue is shown in Figure 3, wherein Q is assumed as 4, and transmission of SSB (in blue) and RMSI (in green) are in the same half frame. If M = 1, the total duration of SSB and associated RMSI variates subject to the location where LBT succeeds, and this may not be properly defined for unlicensed operation in term of deciding its LBT type and priority class.

Figure 3 Illustration of variable transmission duration using Rel-15 configurations.

Moreover, the value of O for determining the slot index of monitoring occasion is defined in a sense of absolute timing, such that there is no guarantee that no gap exists between the burst of SSB and the burst of RMSI. For some value of O, there is for sure to leave a gap between SSB and RMSI, the fully up to gNB’s implementation to guarantee contiguous transmission in time domain may not be efficient.

Based on above discussion, we propose to simply the Type0-PDCCH monitoring for NR-U, and the details are as follow:

Proposal 5: A UE assumes

- the periodicity of monitoring PDCCH in the Type0-PDCCH CSS set is the same as the periodicity of SS/PBCH block;

- the number of slot for monitoring PDCCH in the Type0-PDCCH CSS set is one;

- the slot index for monitoring PDCCH in the Type0-PDCCH CSS set is the same as the one containing the associated SS/PBCH block.

# Enhancement to Random Access Procedure

This section discusses enhancements to the random access procedure, including valid RO determination due to the uncertainty of SSB transmission position, and other NR-U random access enhancements from the baseline Rel-15 4-step random access procedure to decrease the expected random access delay caused by potential LBT failures.

## Valid RO determination

In a NR TDD band, the system information includes the semi-static UL/DL configuration to provide the duration of the DL part, UL part as well as flexible part. In the meantime, due to complex situation of possible semi-static UL/DL configuration (variable period, variable durations of DL/UL), the validation rule is finally promoted to apply to the PRACH configuration indicated via the PRACH configuration index of a table. In short, the RO indicated by the PRACH configuration index will be invalid if it is collided with DL part or a part before the end of the last SSB in the flexible part. Such rule in addition to the gap definition after the last symbol of DL part/SSB helps the UE to determine the actual valid RO within the PRACH configuration period. Note that each PRACH configuration period may have different valid ROs, thus it produces the different consistence of the SSB-RO association period, i.e., different number of PRACH configuration period in one SSB-RO association period. For example, one SSB-RO association period could be 4 PRACH configuration periods at one time, but becomes 2 PRACH configuration periods in a later time. To ease the burden at the UE side, instead of the always (re)-calculating the association period, NR Rel-15 defines the association pattern period which consists of one or multiple SSB-RO association period(s), in which the mapping pattern between the association pattern periods will be purely repeated.

For NR-U, the uncertainty of the LBT outcome will add the variance to the (actually transmitted) SSB positions, as well as other DL signal transmission, making the situation of channel utilization more complicated. For example, different from the deterministic SSB positions in Rel-15, the NR-U SSB transmissions can start at a finite set of starting positions within the DRS transmission window. Method of effective determination of valid RO needs to be studied, using PRACH configuration design from NR Rel-15 as a baseline.

As shown in below figure, the validation of RO is related to how the SSB will be actually transmitted. Based on the description in section 2.5, the proposed “potentially actually transmitted SS/PBCH block locations” could be obtained by UE. Thus, such information could be used for the RO validation determination.

Figure 4 Illustration of RO validation for NR-U

Using the same example as in Section 2.5 on the SSB configurations, i.e., wherein the indication of actually transmitted SS/PBCH blocks is “11000000”, and modulo value Q = 8. Then, the “potentially actually transmitted SS/PBCH block locations” within a DRS transmission window with size 20 are given by {0, 1, 8, 9, 16, 17}. In this case, the last potential position where a SSB could be transmitted is candidate position #17, which we can use this position as the reference point to determine the start of the valid RO. Due to the expansion of the candidate position size (i.e., 0~19) is larger than the actually transmitted SSB size (i.e., 0~7), more RO will be invalid compared to the situation in licensed NR. However, this invalidation may not happen in every PRACH configuration period, as one example in the last figure, the next PRACH configuration period will face no collision with SSB then all the RO could be valid (note that here we ignore the UL-DL configuration impact on the validation).

In addition that the RO in the UL part is valid, a PRACH occasion in a PRACH slot is valid if it starts at least Ngap symbols after the last symbol of the last candidate position of the transmitted SS/PBCH block, wherein Ngap can reuse the definitions from Rel-15.

Proposal 6: A PRACH occasion in a PRACH slot is valid if it starts at least Ngap symbols after the last symbol of the last candidate position of the transmitted SS/PBCH block.

For PRACH transmissions on FBE NR-U, the RACH occasion validation rule can also be enhanced from that of Rel-15 NR. Specifically, since the fixed frame period includes both the COT and the idle period, wherein the idle period is at least 5% of the COT. As a result, since no transmission is allowed during the idle period of a fixed frame period of FBE, the RACH occasion(s) that overlap with the IDLE period of a fixed frame period associated with the gNB shall be considered as invalid.

Proposal 7: A RO that overlaps with the IDLE period of a fixed frame period for FBE NR-U is invalid.

## Multiple Msg3 transmission opportunities

Similar to PRACH, a standalone transmission of Msg3 also requires a CAT-4 LBT operation. Although Msg3 can be subject to a CAT-2 LBT when the Msg3 is configured to be within the COT of RAR, RAR can include more than one UL grants (scheduled msg.3 transmission) such that it’s not always possible to put all the scheduled Msg.3(s) into the same RAR COT, e.g., when the user number is relatively large or when COT for RAR is relatively short. As a result, Msg3 transmission/re-transmission can also be subject to a significant delay due to the LBT requirement.

Observation 1: COT based solution could not solve the impact of LBT to msg.3 transmission.

Therefore, enhancement to support multiple Msg3 transmission opportunities for NR-U can also be beneficial (which has been concluded in RAN2) to reduce the Msg3 transmission delay as well as the overall random access procedure delay. In particular, the multiple Msg3 transmission opportunities can be configured through a method without the MAC format changing, which is still using a single RAR with a single UL grant including a single time-domain resource allocation. However, additional RAN1 support is needed to indicate number of times N_tx that UE could use the indicated time domain resources in UL grant and the step gap delta_T between two candidate transmission opportunities. More details on multiple Msg3 transmission opportunities can be found in our companion contribution [3].

Proposal 8: From scheduling multiple-msg.3 point of view, UE should be provided the number of times N_tx that UE could use indicated time domain resources in UL grant and the step gap delta_T between two candidate transmission opportunities.

## Time/frequency-domain enhancements for NR-U RACH resources

Agreement:

Following options have been identified for potential RACH resource enhancements in NR-U beyond the flexibility already available in Rel-15:

- Frequency-domain enhancement

  - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

- Time-domain enhancements

  - For connected mode UE, scheduling of PRACH resources via DCI.

    - Triggered PRACH within TXOP can use a new resource

  - For idle mode UE, scheduling of PRACH resources via paging

    - Note: potential inefficiency in network resource due to paging across multiple cells

  - Additional, new RACH resources are used immediately following detection of DRS transmission

  - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

    - Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

    - FFS: How to handle potential multiple RARs to same UE

  - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

In RAN1 #94bis [2], several frequency-domain and time-domain potential NR-U RACH resource enhancements have been identified.

Among the identified enhancement options, time-domain enhancement option (d) and option (e) wherein multiple PRACH transmission and time domain grouping method can be utilized as the most general approaches in compensating the potential PRACH LBT failures for both contention-based and contention-free random access. Option (d) and option (e) are most beneficial because each standalone PRACH transmission is subject to a CAT-4 LBT operation, which corresponds to a non-deterministic LBT duration and therefore a non-deterministic ending position of the LBT operation. The non-deterministic ending position for PRACH LBT can be compensated through option (d) and option (e), by configuring multiple consecutive time-domain RACH occasions, such that the UE can utilize one or multiple of its configured ROs after LBT is completed. Without option (d) and option (e), the PRACH transmission delay will be significantly increased since UE will be unable to utilize its configured RO if the PRACH LBT fails or completes after the configured RO; and the UE needs to initiate a new PRACH LBT procedure in order to utilize its next available RO.

By contrast, each of the remaining options would have certain constraints. Specifically, for the frequency-domain enhancement in [2], since it has already been agreed that the initial active UL BWP is (approximately) 20 MHz and that LBT will be performed in units of 20 MHz for NR-U, performing LBT on a per sub-band basis for NR-U PRACH will have minimal channel access benefit for initial access UEs.

The PRACH performance of option (a) is not guaranteed, since the DCI-scheduled RO is a one-time and non-repeatable PRACH resource, and there lacks sufficient evidence in demonstrating the reliability of one-time PRACH preamble transmission even for CFRA in licensed band. In addition, the DCI-scheduled RO is also subject to at least a CAT-2 LBT, thus enhancements such as option (d) or option (e) would still be necessary to increase the PRACH LBT success chance. Using paging to configure the PRACH resource with time-domain option (b) could be a huge resource waste as already noted in the agreement, this often causes the PRACH resource reserved in multiple cells and expected such UE could utilize only one of them. For the time-domain enhancement option (c), the RACH resources that can be utilized immediately following detection of a DRS are very limited or even unavailable, when the DRS is subject to CAT-4 LBT with lowest priority class value, and/or when the number of SS/PBCH blocks within DRS is high (e.g., 8). Therefore, UEs utilizing such RACH resources (if any) could be subject to high collision probability.

As a result, the time-domain enhancement option (d) and (e) can be the prioritized RACH enhancement options for NR-U; while the other enhancement options require further study to see if there is any benefits.

Proposal 9: NR-U shall prioritize time-domain enhancement option (d) and (e) for PRACH.

## Enhancement to increase transmit opportunities for Msg1 in 4-step RACH

Due to the LBT requirement for transmitting PRACH, the failure of PRACH LBT can cause significant delay to the overall RA procedure. As a result, enhancements to increase the transmission opportunities of Msg1 in 4-step RACH can be used to decrease the PRACH transmission delay.

As a first enhancement, NR-U can try to inherit the SSB-RO association design in the NR Rel-15, and add an associated LBT before each RO to enhance the transmit opportunities for Msg1 as discussed in Section 3.1. For example, NR Rel-15 supports configuring one or more than one SSB for each RO, and it may end up with multiple ROs, associated with the same SSB, not consecutive to each other in time domain. Then, if the LBT for an earlier RO fails, Msg1 still has opportunities to be transmitted in later configured ROs within the association period, subject to the LBT results of the later ROs. An example of this enhancement is illustrated in Figure 5.

Figure 5 Illustration of the first enhancement method.

As another enhancement, NR-U can configure multiple consecutive ROs associated with the same SSB, and an associated LBT can be performed before each RO (there can be gap symbol(s) between neighbouring ROs among the consecutive ROs to account for LBT overhead). As a result, a UE can be configured with multiple consecutive time-domain ROs for RACH attempt, which can also increase the transmit opportunities for Msg1. Equivalently, this example can be considered as configuring an associated RO window to each SSB, wherein multiple consecutive time-domain ROs can be configured within the window. Configuring multiple consecutive time-domain ROs can greatly reduce the NR-U PRACH transmission delay, since CAT-4 LBT for NR-U PRACH has a non-deterministic duration, and the UE can utilize the first available RO among its configured ROs after successful a PRACH LBT.

In order to support configuring multiple time-domain ROs to be associated with SSBs in a consecutive manner, the SSB-RO mapping can be enhanced from NR Rel-15, e.g, time-domain first mapping; especially when the enhancement can be combined with FDMed multiple ROs. Figure 6 gives such an example where each SSB is configured with 4 consecutive ROs in the time domain (with LBTs associated to each RACH occasion), and two sets of FDMed ROs in the frequency domain. Note the PRACH LBT shall be performed over the initial active UL BWP, so the LBTs associated with FDM’ed ROs in Figure 6 may actually overlap in frequency domain.

Figure 6 Illustration of the second enhancement method.

Proposal 10: NR-U shall configure a UE with multiple Msg1 transmission opportunities within an SSB-RO association period to decrease the PRACH transmission delay.

# Enhancement to RRM Measurement

In the last meeting, it was agreed that the LTE LAA framework is reused for NR-U, with the enhancement of introducing frequency domain information. There is one FFS point is, whether the configuration unit for the BW of RSSI could be smaller than a LBT bandwidth.

Note that the main functionality for supporting RSSI measurement is for interference detection on the channel, especially for the coexistence scenario among NR-U, LTE LAA, and Wi-Fi. Since the existing system may operate on parts of NR-U BWP and NR-U downlink transmission may occupy parts or whole of the BWP where CCA is successful at gNB, it would be beneficial to support RSSI measurement over the sub-band to reflect interference of each part of BWP separately, and the motivation to support a further finer unit seems not strong. Hence, we have the following proposal.

Proposal 11: For RSSI measurement, using a unit of LBT bandwidth for measurement bandwidth configuration is sufficient, and no other unit with smaller granularity is needed.

In the last meeting, it was agreed that for neighboring cell RRM measurement, a QCL parameter can be configured per frequency layer and per cell, and the serving cell RRM measurement is still FFS.

Since the configuration of a QCL parameter is supported for serving cell as well, that QCL parameter can be reused for serving cell RRM measurement.

Proposal 12: The QCL parameter configured for serving cell in MIB/SIB1 can be reused for serving cell RRM measurement.

Meanwhile, some remaining details about RRM measurement should be clarified, e.g. how to utilize the QCL assumption for RRM measurement. In Rel-15, the SS/PBCH blocks to be measured is configured by a bitmap in RRC parameter ssb-ToMeasure, however, for NR-U, more measurement locations for SS/PBCH blocks should be supported due to LBT. Candidate SS/PBCH blocks in the SMTC should be measured, if the index of the candidate SS/PBCH block is indicated by ssb-ToMeasure after a modulo operation with respect to Q, where Q is the associated QCL parameter for the RRM measurement.

Proposal 13: Candidate SS/PBCH blocks in the SMTC should be measured, if the index of the candidate SS/PBCH block is indicated by ssb-ToMeasure after a modulo operation with respect to Q, where Q is the associated QCL parameter for the RRM measurement.

# Enhancement to RLM Procedure

For radio link monitoring, the downlink radio link quality of the primary NR or NR-U cell shall be monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. In NR, the UE can be configured for a cell with a set of resources (e.g. SS/PBCH block and/or CSI-RS) for radio link monitoring by higher layers. The physical layer in the UE shall indicate out-of-sync to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE shall indicate in-sync to higher layers. In order to operate NR-U without any assistance from licensed carrier, the radio link monitoring is also performed by a NR-U cell. In NR-U, it is possible that gNB fails to access the channel so that RLM-RS cannot be transmitted in a set of slots configured for RLM at a UE side. In this case, the UE may declare out-of-sync unnecessarily. Therefore, modifications on Rel-15 RLM procedure were discussed in RAN1#96, and it was agreed that an RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations. In this contribution, we also propose the DRS transmission periodicity is the same as SSB, such that there is no SSB transmitted outside DRS transmission window. However, even within DRS window, SSBs may be dropped due to LBT failure. Whether to take such occasional absence of SSB into account for out-of-sync evaluation needs further consideration of the feasibility to differentiate SSB DTX and really poor link quality. If UE can distinguish these two cases, either L1 handling or defining a new indicator type such as DTX indication to higher layers can be considered. If an indication of gNB’s COT is transmitted to UE, SSB in the COT can be used for in-sync evaluation.

Proposal 14:

- RLM-RS within the RLM measurement window (including both SSB and/or CSI-RS) is used for both in-sync and out-of-sync evaluation.

- RLM-RS outside the RLM measurement window, if any, is used for in-sync evaluation, and is not used for out-of-sync evaluation.

Proposal 15: If it is feasible for UE to distinguish SS/PBCH block DTX and poor link quality, mechanism to handle missing SS/PBCH block due to LBT failure can be considered.

# Conclusion

The observations and proposals made in this contribution are summarized below:

Observation 1: COT based solution could not solve the impact of LBT to msg.3 transmission.

Proposal 1: Duration of DRS transmission window can be configurable from {1, 2, 3, 4, 5} ms.

Proposal 2: The QCL parameter for serving cell can be indicated by

- Approach 1: 2 MSB of controlResourceSetZero in MIB; or

- Approach 2: combination MSB of controlResourceSetZero and subCarrierSpacingCommon in MIB.

Proposal 3: The indication of actually transmitted SS/PBCH blocks for NR-U is based on the length-8 bitmap supported in Rel-15.

- A UE assumes the number of actually transmitted SS/PBCH blocks is equal to or smaller than Q;

- A UE only expects to be configured as “1” in the first Q bits of the length-8 bitmap.

Proposal 4: A UE assumes the resources mapped for potentially actually transmitted SS/PBCH block locations within a DRS transmission window are not available for PDSCH, wherein the PDSCH is not for RMSI.

- The potentially actually transmitted SS/PBCH block locations are defined by the indication of actually transmitted SS/PBCH blocks with a modulo operation based on the value Q.

Proposal 5: A UE assumes

- the periodicity of monitoring PDCCH in the Type0-PDCCH CSS set is the same as the periodicity of SS/PBCH block;

- the number of slot for monitoring PDCCH in the Type0-PDCCH CSS set is one;

- the slot index for monitoring PDCCH in the Type0-PDCCH CSS set is the same as the one containing the associated SS/PBCH block.

Proposal 6: A PRACH occasion in a PRACH slot is valid if it starts at least Ngap symbols after the last symbol of the last candidate position of the transmitted SS/PBCH block.

Proposal 7: A RO that overlaps with the IDLE period of a fixed frame period for FBE NR-U is invalid.

Proposal 8: From scheduling multiple-msg.3 point of view, UE should be provided the number of times N_tx that UE could use indicated time domain resources in UL grant and the step gap delta_T between two candidate transmission opportunities.

Proposal 9: NR-U shall prioritize time-domain enhancement option (d) and (e) for PRACH.

Proposal 10: NR-U shall configure a UE with multiple Msg1 transmission opportunities within an SSB-RO association period to decrease the PRACH transmission delay.

Proposal 11: For RSSI measurement, using a unit of LBT bandwidth for measurement bandwidth configuration is sufficient, and no other unit with smaller granularity is needed.

Proposal 12: The QCL parameter configured for serving cell in MIB/SIB1 can be reused for serving cell RRM measurement.

Proposal 13: Candidate SS/PBCH blocks in the SMTC should be measured, if the index of the candidate SS/PBCH block is indicated by ssb-ToMeasure after a modulo operation with respect to Q, where Q is the associated QCL parameter for the RRM measurement.

Proposal 14:

- RLM-RS within the RLM measurement window (including both SSB and/or CSI-RS) is used for both in-sync and out-of-sync evaluation.

- RLM-RS outside the RLM measurement window, if any, is used for in-sync evaluation, and is not used for out-of-sync evaluation.

Proposal 15: If it is feasible for UE to distinguish SS/PBCH block DTX and poor link quality, mechanism to handle missing SS/PBCH block due to LBT failure can be considered.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #98bis, Chongqing, China, Oct 14th – 20th, 2019.

[2] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #94bis, Chengdu, China, October 8th –12th, 2018.

[3] R1-1912446, Initial access signals and channels for NR-U, Samsung