3GPP TSG RAN WG1 #99	R1-1912466

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.4.7

Source: 	Samsung

Title: 	On NR Uu controlling LTE sidelink

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #84, the WI [1] on 3GPP NR V2X was endorsed including the following objective to specify NR Uu controlling LTE sidelink:

2. Specify support for NR Uu to provide control for LTE sidelink - Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and - Sidelink mode 3-like RRC-configured SPS scheduling with DCI-based activation/deactivation as per the agreement in RAN1#97 [RAN1, RAN2].

Also, in RAN1#97 meeting, RAN1#98 meeting and RAN1#98bis meeting, following agreements were made [2-4]:

Agreements:

- RRC-based activation/deactivation is not supported

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

Agreements:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

Agreements:

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

    - UE reports a single value subject to UE capability

In this contribution, we present our views on the remaining issues of NR Uu controlling LTE Mode 3-like sidelink.

Agreements:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has 8 values

  - The size of the DCI field is fixed at 3 bits

# Discussions

Configuration of necessary parameters

In order to support NR Uu controlling LTE sidelink, some necessary parameters should be configured by NR gNB. Besides reusing all of the existing LTE sidelink related configurations transmitted from LTE eNB, some additional parameters are required. For example, the bandwidth of LTE sidelink should be configured by NR gNB, which is currently configured as UL system bandwidth in the system information in LTE system. In addition, LTE UL-DL configuration should also be configured by NR gNB to determine the available subframe for LTE sidelink transmission.

Proposal 1: NR gNB should configure following parameters by higher layer signalling:

- Bandwidth of LTE sidelink

- LTE UL-DL configuration

Content of the DCI for activation/deactivation of LTE SL SPS

Regarding the DCI content for activation/deactivation of LTE SL SPS, all of the fields in the LTE DCI format 5A should be reused. In addition, some DCI fields related to PUCCH resource in NR DCI format 1_0 should be reused to support ACK feedback of LTE SL SPS release, e.g., PUCCH resource indicator and PDSCH-to-HARQ feedback timing indicator. The two DCI fields can be reserved when LTE SL SPS is activated. In addition, if LTE sidelink and NR SUL can share the same carrier, the DCI filed for UL/SUL indication is required.

Proposal 2: The DCI for activation/deactivation of LTE SL SPS at least includes all of the fields in the LTE DCI format 5A and PUCCH resource related DCI fields in the NR DCI format 1_0.

Padding the DCI for activation/deactivation of LTE SL SPS

Considering a new DCI format is introduced for activation/deactivation of LTE SL SPS, DCI padding should be performed to not increase the total number of DCI blind decoding, i.e., not increase the total number of DCIs with different size. Following DCI padding can be considered:

- Padding bits are generated until the payload size equals to that of the new introduced DCI format scheduling NR sidelink. In Mode 1 resource allocation, new NR DCI format should be introduced for scheduling NR sidelink, including scheduling dynamic SL grant and configured SL grant. Thus, DCI padding among these new introduced DCI formats scheduling sidelink is straightforward.

- Padding bits are generated until the payload size equals to that of the DCI format 0_0 scheduling UL. If the DCI scheduling LTE sidelink and the DCI format 0_0 scheduling UL are configured within the same search space, the DCI padding should be performed.

Similar to current NR DCI padding mechanism, whether to perform above DCI padding can be dependent on the total number of different DCI sizes configured to monitor for the cell. If the total number of different DCI sizes is no more than 4, above DCI padding may be unnecessary.

Proposal 3: For the DCI for activation/deactivation of LTE SL SPS, padding bits are generated until the payload size equals to that of the DCI format 0_0 in NR Uu.

# Conclusions

In this contribution, we presented our views on remaining issues of NR Uu controlling sidelink. Based on the discussion, the following proposals are provided:

Proposal 1: NR gNB should configure following parameters by higher layer signalling:

- Bandwidth of LTE sidelink

- LTE UL-DL configuration

Proposal 2: The DCI for activation/deactivation of LTE SL SPS at least includes all of the fields in the LTE DCI format 5A and PUCCH resource related DCI fields in the NR DCI format 1_0.

Proposal 3: For the DCI for activation/deactivation of LTE SL SPS, padding bits are generated until the payload size equals to that of the DCI format 0_0 in NR Uu.

# References

- RP-190984, “Revised WID on 5G V2X with NR sidelink”, LG Electronics

- Final Chairman’s notes for RAN1#97 Meeting

- Final Chairman’s notes for RAN1#98 Meeting

- Final Chairman’s notes for RAN1#98bis Meeting