3GPP TSG RAN WG1 #99			R1-1912657

Reno, USA, November 18th – 22nd, 2019

Title:	[DRAFT] LS response on multi PDCCH-based and single PDCCH-based multi-TRP operation

Response to: 	R2-1914020, LS on multi PDCCH-based and single PDCCH-based multi-TRP operation

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	[RAN1], Ericsson

To:	RAN2

Contact Person:

#### Name:	Mattias Frenne

E-mail Address:	mattias.frenne@ericsson.com

Attachments:	None

1. Overall Description:

In a received LS to RAN1, RAN2 asks three questions related to multi-TRP operation. In this response, RAN1 would like to provide answers to these three questions.

Total number of CORESETs per serving cell

The LS in [2] indicates that the number of CORESETs is increased per PDCCH-config from 3 to 5:

On multiple PDCCH based multi-TRP/panel transmission,

- For multi-PDCCH based multi-TRP operation, increase the maximum number of CORESETs per “PDCCH-config” to 5, according to UE capability.

Question 1. Does the total number of CORESETs per cell need to be increased from current 12 corresponding to 3 CORESETs per BWP?

RAN1 Response:

No, 12 CORESET per cell is enough to support the intended use cases for multi-TRP/multi-panel operation.

PDSCH TCI state options reflected in DCI per TRP

Currently there is one MAC CE activating 8 TCI states for PDSCH reception. Without any change for mPDCCH mTRP transmission, a PDCCH transmission [for one TRP] will point to one of these 8 TCI states, thus the TRPs are sharing the 8 activated TCI states.

Question 2. Does RAN1 think the current operation is sufficient for mPDCCH mTRP operation?

RAN1 Response: Yes.

In RAN2#107bis, RAN2 reached the following agreements for single PDCCH-based multi TRP/panel transmission.

Agreements 1. We will adopt a dynamic MAC CE based approach. 2. RAN2 understands that the UE does not need to know via MAC CE that a TCI state corresponds to a specific TRP

Question 3. RAN2 would like to ask RAN1 for confirmation that the understanding in RAN2 agreements is correct.

RAN1 Response: Yes, RAN1 confirms this understanding.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above answers into account in their work for multi-TRP.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN1 Meeting #100	24th – 28th February 2020	Athens, Greece.

TSG-RAN1 Meeting #100bis	20th – 24th April 2020		Busan, Korea.