3GPP TSG RAN WG1 #99									        R1-1912699

Reno, USA, November 18th– 22nd, 2019

Agenda Item:	7.2.2.2.4

Source:	InterDigital, Inc.

Title:	Configured grant UCI for NR-U

Document for:	Discussion and Decision

# Introduction

During the last RAN1 meetings, several agreements on configured grant UCI content and multiplexing were reached, listed in appendix. This contribution discusses remaining details.

# CG-UCI content and mapping

CG-UCI content:

Configured grant UCI (CG-UCI) was agreed to be supported to carry control information related to CG-PUSCH transmission. The content was discussed in the previous RAN1 meetings and the feature lead summary [3] captures different proposals. During the RAN1#AH1901 meeting, it was agreed that at least HARQ process ID, new data indicator, redundancy version and COT sharing information should be included in the CG-UCI. In our view, another parameter that should also be carried by CG-UCI is the channel access priority class (CAPC). The gNB needs to be aware of the LBT priority class used to access the channel so that it can use the channel to transmit traffic with similar or higher priority. There are two options for the gNB to determine the CAPC used by a UE. First option could be to determine the CAPC based on the received UL data. Meaning that the gNB needs to successfully decode the uplink transport block and extract the priority information from MAC CEs (e.g. from the logical channel ID). This option requires more processing time as the data needs to be decoded and processed. The processing time can be longer than the indicated possible starting transmission time by a UE within the shared COT. Furthermore, the gNB needs to correctly decode the TB to extract the priority class information. Second option is to include the CAPC in the CG-UCI. With this approach, determining the priority class is not subject to successfully decoding the TB and thus leads to faster determination. The CG-UCI can for example carry 2 bits to indicate the priority class if four priority classes are supported. The priority class indication can be part of the COT sharing bit information. We thus propose the following:

Proposal 1: The UE indicates the LBT priority class in CG-UCI, when it initiates a channel occupancy.

For the remaining CG-UCI content that was proposed during last RAN1 meeting [3], Table 1 highlights our views regarding the necessity of each content.

Table 1. Analysis of candidate CG-UCI contents

| CG-UCI content | Needed or not | Note |
| --- | --- | --- |
| LBT type | No | In our view, the default channel access type should be cat4 LBT for CG transmissions. In case the UE shares its COT, COT sharing information can be provided to the gNB. By providing COT sharing information, it is implied that LBT cat4 was used. In other cases, the UE should rely on gNB instructions to determine the type of LBT (e.g. cat2 LBT using gNB initiated COT) and there is no need to indicate it to the gNB. |
| LBT priority class | Yes, in case of COT sharing | Described in our first paragraph. |
| CRC | Depending on the CG-UCI payload | Similar encoding of UCI in NR Rel-15 can be applied to CG-UCI. If the CG-UCI payload is above 12 bits, CRC can be added to CG-UCI. |
| UE ID | Yes, if CRC is attached to CG-UCI | UE ID can be conveyed using CRC scrambling with UE ID. |
| Resource configuration index | No | It is not clear why it is needed. |
| Transmission parameters (such as MCS, PMI, RI, SRI) | No | The transmission parameters should be part of RRC configuration similar to CG transmission in NR Rel-15. Different active CG configuration is supported (in eURLLC) and can have different transmission parameters. |
| CBGTI and CBGFI | Yes, in case CBG retransmissions is supported using CG transmissions. | Supporting CBG level retransmission using configured grant transmission was left for further study. |
| PUSCH starting start/end point. | No | It was agreed that UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position. There is some discussion on allowing sub-symbol starting position to multiplex multiple UEs within the same resource. In our view, the offset will be configured by the gNB and thus there is no need to indicate the starting point. |

Based on this analysis, we propose the following:

Proposal 2: Table 1 is considered for the support of CG-UCI contents for CG transmission in NR-U.

Some of the proposed CG-UCI content are only relevant for some specific use cases. For example, COT sharing information is relevant only for the case of a UE initiating the COT. It is possible that the gNB acquires the channel and transmits the COT structure indication so that the UE can use cat2 LBT. In that case, COT sharing information is not needed to be included in the CG-UCI. Another example is the CBGTI/CBGFI which are only relevant for CBG-based transmissions. Furthermore, including all the agreed contents for every CG-UCI transmission will increase the overhead on the CG transmission. For example, in the case of two symbols PUSCH, with one symbol carrying DMRS, there might not be enough resources left for data transmission. We think that the agreed contents should not always all be present in the CG-UCI and can be dynamically selected based on the operating scenario. A UE can be configured along with CG-PUSCH configuration with a set of possible contents that can be used. UE can select the CG-UCI content according to the scenario. For example, if a UE initiates and shares a COT with the gNB it can indicate the COT sharing information to the gNB.

CG-UCI and UCI multiplexing:

During the last meeting, it was agreed that CG-UCI is included in every CG-PUSCH transmission. Furthermore, CG-UCI is mapped as per Rel-15 rules with CG-UCI having highest priority (CG-UCI is mapped on the symbols starting after first DMRS symbol). The maximum of 3 separately encoded UCIs in configured grant transmission, and CG-UCI, CSI part 1 and part 2 can be sent on CG-PUSCH at least when CG-UCI and HARQ is not multiplexed. For the case of PUCCH carrying HARQ-ACK overlapping with CG-PUSCH, two alternatives were discussed during last meeting: first alternative is to drop CSI part 2 when the number of UCI types multiplexed in a CG-PUSCH is greater than 3 with priority order of CG-UCI, HARQ-ACK and CSI part 1. A drawback of this option, as it was highlighted during the meeting, is the HARQ-ACK size mismatch between the gNB and the UE in case of misdetection of the last scheduling DCI. The second alternative is to skip the configured grant transmission and prioritize the PUCCH carrying HARQ-ACK feedback. The issue of misdetection exists in NR Rel-15 and it was considered as a corner case, and the gNB can blindly detect the mismatch case.

Proposal 3: In case of PUCCH overlapping with CG-PUSCH, UE drops CSI part 2 when the number of UCI types multiplexed in a CG-PUSCH is greater than 3 UCIs.

# Conclusion

In this contribution, we presented our views and proposals regarding NR-U configured grant enhancements. The following are proposed:

Proposal 1: The UE indicates the LBT priority class in CG-UCI, when it initiates a channel occupancy.

Proposal 2: Table 1 is considered for the support of CG-UCI contents for CG transmission in NR-U.

Proposal 3: In case of PUCCH overlapping with CG-PUSCH, UE drops CSI part 2 when the number of UCI types multiplexed in a CG-PUSCH is greater than 3 UCIs.

# References

- RP-191581 Guidance on essential functionality for NR-U; Qualcomm.

- R1-1910939 Channel access in NR-U; InterDigital.

- R1-1909476 Feature lead summary on enhancement to configured grants; vivo.

- Chairman’s Notes RAN1#96, Athens, Greece, February 25th – 1st, 2019

- Chairman’s Notes RAN1 AdHoc 1901, Taipei, Taiwan, January 21st – 25th, 2019

# Appendix

At the RAN1#98bis meeting: Agreement: CG-UCI is mapped as per Rel-15 rules with CG-UCI having the highest priority (CG-UCI is mapped on the symbols starting after first DMRS symbol) Agreement: To determine the number of REs used for CG-UCI, the mechanism of beta-offset in Rel-15 NR for HARQ-ACK on CG-PUSCH is reused. - A new RRC parameter to configure the beta-offset for CG-UCI is defined. FFS: Value range Agreement: CG-UCI is included in every CG-PUSCH transmission (confirms working assumption from RAN1#98) Agreement: - CG-UCI, CSI-part1, CSI-part 2 can be sent on CG-PUSCH at least when CG-UCI and HARQ-ACK feedback is not multiplexed on a CG-PUSCH At the RAN1#98 meeting: Agreement: The number of separately encoded UCIs multiplexed in a PUSCH transmitted using a configured grant is not changed from Rel-15 (maximum of 3). At the RAN1# 96 meeting: Agreement: When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following - The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE - FFS: - How the duration is signalled - Whether the UE should signal continued use of the COT for its own transmissions - LBT priority class At the RAN1#AH1901 meeting: Agreement: CG-UCI should at least include the following information: - HARQ ID - NDI - RV - COT sharing information, FFS details - FFS: other information including UE ID