3GPP TSG RAN WG1 #99			R1-1912813

Reno, USA, November 18th – 22th, 2019

Agenda Item:	7.2.4.4

Source:	Apple

Title:	Remaining Issues on In-device Coexistence

Document for:	Discussion/Decision

# Introduction

In RAN1 #98bis meeting [1], the following agreements on in-device coexistence between NR and LTE sidelinks have been made.

Agreements:

- For Tx/Rx overlap,

  - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received

    - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received

Agreements:

  - For sidelink synchronization signal/channel (including S-SSB and LTE SLSS/PSBCH) priority for a UE is (pre)-configured per UE

    - The (pre)-configured priority is used in the same way as the priority for other channel/signals w.r.t. prioritization for handling in-device co-existence

    - Note: it is understood that the same priority (pre)-configuration is intended for all the related UEs

  - The priority of PSFCH is set as the priority of the corresponding PSSCH.

In this contribution, we discuss the remaining issues on in-device coexistence between LTE sidelink and NR sidelink.

# Discussion

In the short-term time scale TDM solutions for in-device coexistence between LTE sidelink and NR sidelink, it was agreed that the data with higher priority in either sidelink is processed in case of the Tx/Tx or Tx/Rx overlap.

The QoS parameters of NR sidelink data are provided by upper layers (e.g., V2X applications). In NR V2X, there are also AS layer messages. RAN2 agreed that PC5-RRC signaling and PC5-S signaling are carried on sidelink control channel (SCCH) and the logical channel priority of SCCH carrying a PC5-RRC message or a PC5-S message is specified as a fixed value.

One exception of AS layer message is sidelink CQI/RI reporting. It was agreed [1] that in sidelink unicast, CQI/RI reporting is transmitted in PSSCH via higher layer signaling (e.g., MAC CE).

It is likely that sidelink CQI/RI reporting is a MAC CE and PSSCH carries only CSI/RI reporting. Then it is necessary to define the corresponding priority level of the PSSCH. This priority is used not only for facilitating the in-device coexistence between LTE sidelink and NR sidelink, but also for mode 2 UE resource selection procedure.

Since CSI/RI reporting is used for sidelink unicast, the priority level of the corresponding PSSCH transmission could be associated with the corresponding sidelink unicast data priority. Hence, the priority level of this AS layer message should be (pre-)configured.

Proposal 1: The priority of PSSCH conveying only sidelink CQI-RI reporting should be (pre-)configured.

# Conclusion

In this contribution, we discussed the remaining issues on in-device coexistence between NR and LTE sidelinks. We have the following proposal:

Proposal 1: The priority of PSSCH conveying only sidelink CQI-RI reporting should be (pre-)configured.

# References

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #98bis Meeting, Chengdu, China, Oct. 2019.