3GPP TSG RAN WG1 #99		R1-1912820

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.6.5

Source:	Apple Inc.

Title:	Remaining Issues on Enhanced Inter UE Tx prioritization/multiplexing

Document for:	Discussion/Decision

# Introduction

In R16 enhanced Inter UE Tx prioritization/multiplexing, the main topics of discussion are (1) UE UL Cancellation for eMBB UEs and, (2) Enhanced uplink Power Control for URLLC UEs.  The agreements that have been made on this agenda item for both the SI and WI up to and including RAN1 #98bis are summarized in [1].

In RAN1 #98bis, the final detailed design for enhanced UL power control for DG-PUSCH was agreed on and it was concluded that there should be no enhancement for CG-power control in Rel-16 for inter-UE multiplexing. However, for the UL cancellation scheme, there were discussions on the detailed design issues with some of the design details as FFS.

As such, in this contribution, we discuss the open issues related to unresolved items related to the UL cancellation mechanism and provide possible solutions on different designs aspects such as UL CI monitoring, UL CI signaling design and the utilization of the non-overlapped cancelled resources.

# UE UL Cancellation Mechanism

In the UE UL cancellation mechanism, the gNB uses a GC-PDCCH to send an Uplink Cancellation Indication (UL CI) to the eMBB UE. On the eMBB UE detecting an UL cancelation indication, it stops transmitting its current transmission without the option of resuming its transmission when the interrupting signal in done, except for specific signals such as the SRS. As such the URLLC UE is able to transmit to the gNB with no interference. This is illustrated for an FDD system in Figure 1.

# UL CI Monitoring

For the eMBB UE to stop its transmission, it is necessary for it to monitor for the UL CI in the GC-PDCCH. In RAN1 #98bis, the following related agreements were made:

RAN1 #98bis Agreement: Regarding UL CI monitoring, support the following: - A new RNTI (e.g. CI-RNTI) is used for UL CI - FFS: Monitoring periodicity larger than [5] slot is not supported for UL CI - The aggregation level(s) and the number of PDCCH candidates configured by RRC - FFS possible restrictions, e.g., the ones associated with SFI - The DCI payload size for UL CI is configured by RRC - FFS possible values

Figure 1: URLLC Multiplexing using and UL cancellation signal to the eMBB UE

Given that the UL CI is meant to cancel the eMBB UE transmission and allow for low latency URLLC transmission, a large monitoring periodicity should not be supported. As such, having a periodicity larger than 5 slots may be counterproductive. Note that the MO should be related to the size of the reference region. A monitoring occasion periodicity that is much larger than the maximum size of the reference region in which the location of the resources to be cancelled is identified, means that there will be a large set of resources that cannot be cancelled. As the largest value under discussion is 29 OFDM symbols or 2 slots, a monitoring periodicity above 2 slots should not be supported.

Proposal 1: the largest periodicity of the MO should be related to the largest configurable size of the reference region.

Signaling the UE CI requires that the eMBB UE perform additional PDCCH monitoring results in additional PDCCH monitoring complexity for the eMBB UE. As such, there is a need to limit the overall increase in monitoring occasions. There is a need for the UL CI to be reliably decoded as it impacts the reliability of the URLLC transmission. This is because an error prone UL CI signal may result in some eMBB UEs interfering with URLLC transmissions. As such, we should limit the PDCCH aggregation levels (ALs) to the highest  ALs to reduce the number of PDCCH candidates in the blind decoding procedure e.g. 4, 8 and 16. Secondly, we can limit the number of PDCCH candidates per monitoring occasion to e.g. 2. This could help limit the overall increase in PDCCH monitoring complexity required for the eMBB UE to support UL cancellation, especially since it may have to monitor more often to take into account the low latency requirements of the URLLC traffic.

Proposal 2: Limit the AL and PDCCH MO to reduce the overall complexity increase.

# UL CI Signaling Design

The design of the UL CI signal has been discussed in the WI. The following relevant agreements were made:

RAN1 #97 Agreements: - Support at least group common DCI for cancelation indication RAN1 #98 bis Agreement: - The reference time region where a detected UL CI is applicable is determined by the following: - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation - FFS X can be configured to be larger than the minimum processing time for UL cancelation - The duration of the reference time region is configured by RRC - FFS Possible values (e.g. 2OS, 4OS, 7OS, 14OS, 28OS?) - FFS DL symbols are excluded from the reference time region Support the following for UL CI - Each UL cancelation indicator per serving cell has an RRC configurable field size of X bits - One value of X is 14 - FFS other values (e.g. X can be N (N>0) times of 7) - The time domain granularity for the reference time region is configured by RRC - FFS the possible values (e.g. the time region can be divided into [1],[2],[4],[7],[14],…portions) - FFS valid configurations according to the duration of the time reference region - The frequency domain granularity is determined based on the configured time domain granularity and the configured bit field size of each indicator - The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar as DL PI) over the time and frequency partitions within the reference region - FFS dynamic 2D-bitmap

The agreements define a reference region within which a detected UL CI is applicable. The reference region is defined in relation to the PDCCH CORESET containing the UL CI. The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar to the  DL PI in Rel 15) over the time and frequency partitions within the reference region. This is illustrated below in Figure 2.

Figure 2: Reference Region Definition

As illustrated in Figure 2, the reference region starts X OFDM symbols after receipt of the PDCCH CORESET that contains the UL CI. However. whether X can be configured to be larger than the minimum processing time for UL cancelation is still an unresolved issue.

If the eMBB UE is configured with Capability 2 processing limits, it may be able to react to the UL CI at approximately the same time as that the URLLC UE is able to decode its scheduling PDCCH. As such, the eMBB UE can expect to cancel its transmission at the beginning of the reference region and, as such, limit the latency loss of the URLLC UE. This means that the minimum processing time for cancellation X, can be set to Capability 2 processing limits. However, if the eMBB UE is configured with Capability 1 processing limits, it may not be able to react to the UL CI as quickly.  The eMBB UE can expect to cancel its transmission later in the reference region, at a time when it has decoded the UL CI and can identify where in the reference region to stop its transmission.

Considering the UL CI may be intended for a group of UEs which may be configured with different processing Capabilities in a CC, if X is less than the minimum processing time for UL cancellation of a UE (e.g. with Capability 1), the UE may ignore the indication from the UL CI, e.g. in the case where the sum of the reference region time duration and the X is less than the minimum processing time for UL cancellation of the UE. Alternatively, X should be configurable to value larger than the minimum processing time for UL cancellation for all the UEs signaled in the UL CI signal.

Proposal 3: A Minimum processing time for cancellation X = Capability 2 processing limits should be supported.

Proposal 4: X should be configurable to value larger than the minimum processing time for UL cancellation for all the UEs signaled in the UL CI signal.

Proposal 5: A UE with UL CI processing time less than the sum of X and the time duration of the reference region may ignore the UL CI.

On the actual signal design, the design should be flexible enough to allow for very granular identification of the cancelled resources in the reference region. This is because an UL CI cancels the transmission of every eMBB UE with the resource it identifies. A coarse granularity could result in a lot of eMBB UEs being cancelled, thereby impacting the efficiency of the network.

# Conclusion

In this contribution, we discussed the remaining issues Enhanced Inter UE Prioritization/Multiplexing. Based on the discussion, the following proposals have been made:

Proposal 1: the largest periodicity of the MO should be related to the largest configurable size of the reference region.

Proposal 2: Limit the AL and PDCCH MO to reduce the overall complexity increase

Proposal 3: A Minimum processing time for cancellation X  = Capability 2 processing limits should be supported.

Proposal 4: X should be configurable to value larger than the minimum processing time for UL cancellation for all the UEs signaled in the UL CI signal.

Proposal 5: A UE with UL CI processing time less than the sum of X and the time duration of the reference region may ignore the UL CI.

# Reference

[1] R1-1911728, RAN1 agreements for Rel-16 eURLLC