3GPP TSG RAN WG1 #99		R1-1912833
Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.13.2

Source:	Apple Inc.

Title:	Remaining issues on enhancements to single Tx switched uplink solution for EN-DC

Document for:	Decision

# Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) operation, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity (EN-DC), requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is capable to transmit LTE UL and NR UL simultaneously, UE is allowed to signal one of two different UL operation modes, per agreement from RAN #77[1]:

- Dual UL transmission: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single UL transmission: in this mode, at any given time, UE is only required to transmit uplink in at most one RAT, i.e. either LTE or NR. We also refer to it as Single Tx Switched UL.

Specifically, which band combinations are allowed for single Tx switched UL is decided by RAN4 [1]. Compared to dual UL transmission, Single Tx Switched UL has several performance benefits especially for the UE in power limited scenario (i.e. cell edge) as well as implementation advantages [2].

The Single Tx Switched UL design should cover both the case of LTE TDD PCell (e.g. DC_41_n41) and LTE FDD PCell (e.g. DC_3_n77). So far, R15 specification has been completed for the LTE FDD PCell scenario.

In RAN4 #86bis meeting [3], it was agreed to allow Single Tx Switched UL operation for intra-band EN-DC with LTE TDD PCell in band 41, mainly due to large power reduction associated with dual Tx in intra-band.

In RAN #81 [16] and #84 [17], it was further agreed to include the following as one of the objectives for the R16 work item on “MR-DC and CA enhancements”[16]:

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1]. Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG - Enable the Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC, possibly including any conclusions on the previous objective (6) for LTE FDD Pcells [RAN1].

In RAN1 #96, a similar “SUO case 1” with DL-reference HARQ timing as in the EN-DC with LTE FDD PCell was agreed for EN-DC with LTE TDD PCell [4]:

Agreements: For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config - For DL HARQ timing, the DL-reference UL/DL configuration is applied - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6, - FFS: whether/how to support TDD pattern 0/6 for LTE PCell - UE is not expected to transmit on the MCG and SCG simultaneously - For type 2 UE (i.e., UE without dynamic power sharing capability): - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration - For type 1 UE (i.e., UE with dynamic power sharing capability): - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration - If there is a collision, - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH - In other UL subframes, UE behaviour to be expected to specified with details FFS - E.g., drop LTE PUSCH, drop NR PUSCH, etc. - FFS for the case of NR SRS & NR PRACH - Note: the impact of switching time (if non-zero) will be further studied. - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination) - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

In RAN1#96bis [5], some further clarifications and progress were made on the HARQ timing for the LTE DL CA scenario, as well as LTE PUCCH formats, as referenced below:

Agreements: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed: - For the LTE TDD PCell: use the PDSCH ACK timeline as in Table 10.1.3.1-1 in 36.213. - For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD- TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - FFS - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) Agreements: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, - Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported. - FFS PUCCH format and resource determination when the UE only receives DAI=1 - FFS whether or not there are any additional issues related to SPS

In RAN1#98 [6], progress were made on DL HARQ timing for LTE TDD S-Cells, HARQ-offset for SUO case 1, and supportable LTE PCell’s TDD patterns for SUO case 1, as referenced below:

Agreements: - Regarding the FFS part (in change marks) in the agreement from RAN1 #96bis: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following is agreed: - For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell (already agreed in RAN1 #96bis) - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) - Support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell - Note: from UE perspective, it is expected that HARQ-offset value doesn’t violate the DL/UL configuration (in SIB1). - For type 1 UE, the feature is optional. FFS for type 2 UE. Agreements: R15 specification on “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” (i.e., case1 HARQ timing in single UL), is applied to EN-DC UE capable of dual UL Tx in EN-DC with LTE FDD PCell to mitigate DL de-sensing due to Harmonics, at least including: - UE behavior specified in 36.213 and 36.212 - FFS: all uplink subframes can be scheduled for LTE Agreements: When UE is configured with DL-reference config on the LTE PCell to support single UL in EN-DC with LTE TDD PCell, regarding the case of LTE PCell’s TDD pattern 0 & 6 (as configured in SIB1) - Not supported in R16.

In RAN1 #98bis, further conclusions were made on LTE PUCCH formats during fallback, RACH resources configuration, RRC signaling for utilizing single Tx mechanisum on dual Tx UE, as referenced below:

Agreements: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, Using only PUCCH formats 3/4/5. - A default PUCCH format 3 resource is used when the UE only receives PDCCH with DAI=1 and/or SPS PDSCH. - The default one is corresponding to ARI=00 - Note: the existing multiplexing rule for HARQ-ACK, SR, and CSI onto PUCCH format 3 remains unchanged - No PUCCH resources are reserved for SPS PDSCH for the UE Agreements: If a UE is configured with Case1 HARQ timing in EN-DC, with LTE TDD Pcell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config. - For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized - For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions Agreements: - Introduce a new UE capability of supporting DL reference configuration for dual-Tx UL (e.g., to handle DL de-sensing from harmonics) Agreements: - For the single-Tx case, for FDD LTE Pcell, - All uplink subframes can be scheduled for LTE for type 1 UEs - In which case, NR transmission is dropped for when the LTE and NR transmissions collide - Note: there is no change of UL scheduling timing for LTE Agreements: - For the dual-Tx case, for FDD LTE Pcell, - All uplink subframes can be scheduled for LTE at least for type 1 UEs - Note: there is no change of UL scheduling timing for LTE Agreements: - Support new RRC signaling “tdm-PatternConfig-r16” to configure a UE with a DL-reference config to the MCG PCell (LTE FDD) according to Rel-16 behavior - FFS: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes

Based on above agreements so far, we believe most of the design issues identified under the “potential enhancements for single Tx switched uplink” scope for R16 have been completed.

In the next section, we provide a brief summary of the progress/conclusions made during the R16 enhancements, and provide our views on finalizing a last couple of remaining details/clarifications .

# Brief summary of design conclusions on R16 single Tx enhancements and open issues

Based on the agreements referenced in section 1, we’ve worked on the following aspects of enhancements for the R16 enhancements to single Tx switched UL.

| Sub-topics | Status | Notes |
| --- | --- | --- |
| Case 1 HARQ timing for EN-DC with LTE TDD PCell | complete | Specified the PDSCH to UL ACK mapping for both LTE TDD PCell and LTE FDD/TDD SCell. Concluded that UL HARQ timing is the same as without reference config |
| Supportable TDD patterns (as in SIB1) for the LTE TDD PCell | complete | LTE D/U pattern 0 & 6 are not supported |
| HARQ-Offset support | complete | Support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell Same was agreed for the case of LTE FDD PCell in R15 |
| Collision handling for single-Tx | complete | For type 1 UE: NR transmission is dropped For type 2 UE: no collision expected |
| LTE PUCCH formats on the LTE PCell | complete | Only LTE PUCCH format 3/4/5 are used. PUCCH format 3 resource for ARI=00 is used if UE only receives DAI=1 and/or SPS PDSCH |
| RACH resource configuration | Partial complete | For UE configured with case 1 HARQ timing in EN-DC, the PRACH resource configuration not limited to UL subframes given by the DL-ref config: - Agreed for the case of LTE TDD PCell - WA for the case of LTE FDD PCell |
| Opportunistic scheduling LTE in all UL slots | complete | For single-Tx case for both LTE FDD PCell and LTE TDD PCell: - Type 1 UE: all uplink subframes can be scheduled for LTE - Type 2 UE: LTE PUSCH only allowed in UL subframes designated as UL in the DL-ref config |
| Utilize single-Tx for dual-Tx capable UE to mitigate DL de-sensing | complete | Introduce 2 new RRC parameters |

One of the major remaining issue was the working assumption part in the RAN1 #98bis agreement on RACH resource configuration, as shown in section 1. Recall that, in R15, for single Tx in EN-DC for LTE FDD PCell, it was agreed that the RACH configuration for SUO case 1 UE is not different from legacy UEs, as follows:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

The working assumption part of the agreement from RAN1 #98bis was generally aligned with the R15 mechanism, except that type 1 UE should transmit LTE PRACH in case of collision in UL subframes not designated as UL in the DL-reference config. Considering the fact that type 1 UE is able to detect the collision and drop NR, it is desirable that network configuration and UE behaviour are common between the case of EN-DC with LTE TDD PCell and LTE FDD PCell. Therefore, we propose to change the WA part to agreement.

Proposal 1: confirm to working assumption in the agreement on PRACH resource configuration made in RAN1 #98bis, i.e., update the agreement as follows:

If a UE is configured with Case1 HARQ timing in EN-DC, with LTE TDD Pcell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

# Summary

In this contribution, we discuss some of the remaining issues from the latest agreements on single Tx switched UL. We have the following the proposals:

Proposal 1: confirm to working assumption in the agreement on PRACH resource configuration made in RAN1 #98bis, i.e., update the agreement as follows:

If a UE is configured with Case1 HARQ timing in EN-DC, with LTE TDD Pcell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

# References

- RP-172085, “Signaling for single/dual UL Tx”, RAN2 Chairman, 3GPP RAN #77, Sapporo, Japan, Sept 11-15, 2017.

- R1-1902774, “Enhancements to single Tx switched uplink for EN-DC”, Apple, 3GPP RAN1#96, Athens, Greece, February 25th – March 1st, 2019

- “Chairman Note, RAN4#86bis”, 3GPP RAN4 #86bis, Melbourne, Australia, USA 16 - 20 April, 2018.

- “Chairman Notes, RAN1 #96”, 3GPP RAN1 #96, Athens, Greece, February 25th – March 1st, 2019.

- “Chairman Notes, RAN1 #96bis”, 3GPP RAN1 #96bis, Xi’an, China, April 8th-12th, 2019.

- “Chairman Notes, RAN1 #98”, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- “Chairman Notes, RAN1 #98bis”, 3GPP RAN1#98bis, Chongqing, China, October 14thth – 20th, 2019.

- R1-1910041, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1910108, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1910837, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1910681, “Discussions on single uplink solution for EN-DC”, Intel, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1908515, “Enhancements to single Tx switched UL for EN-DC”, Samsung, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1910608, “PUCCH formats for 1Tx switched uplink operation for TDD EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1911014, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1910978, “Remaining issues on enhancements to Single Tx Switched Uplink solution for EN-DC”, Apple Inc., 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1911061, “Enhancements to single UL operations for EN-DC”, MediaTek Inc., 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- R1-1911138, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm Inc., 3GPP RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- RP-180680, “Solutions for single Tx TDM”, RAN #80, La Jolla, CA, U.S.A., June 11st-14th, 2018.

- “Chairman Note, RAN#81”, 3GPP RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- “Chairman Note, RAN#84”, 3GPP RAN #84, Newport Beach, U.S.A., 3rd – 6th June, 2019.

- PR-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- PR-191600, “Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #84, Newport Beach, U.S.A., 3rd – 6th June, 2019.

- “Chairman Notes, RAN1 Ad-Hoc #1901”, 3GPP RAN1 Ad-Hoc #1901, Taipei, Taiwan, 21st – 25th January 2019.