3GPP TSG RAN WG1 #99			R1-1912845

Reno, NV, November 18th – 22nd, 2019

## Agenda item:	7.2.8.3

Source: 	AT&T

Title: 	Remaining Issues on Multi-Beam Enhancements

Document for:	Discussion/Decision

# Introduction

The following agreements have been made in RAN1#98bis on multi-beam enhancements [1]:

On Enhancements on Multi-Beam Operations

Agreement

Confirm the working assumption made in RAN1#97, with the following updates.

For the supported feature of simultaneous update/indication of a single spatial relation per group of PUCCH by using one MAC CE, the following configuration options for the group are supported:

- At least up to two groups per BWP.

  - FFS: Details on configuring the groups including whether to use implicit method or explicit method

    - For example, each corresponding to different TRP/panel, at least for multi-TRP/panel case

    - Another example, each corresponding to different active spatial relation at least for single TRP case

  - If there is no consensus to support more than two groups within RAN1#98bis, only up to two groups will be supported in Rel-16

  - Note: the terminology of “group” may or may not be in specifications.

Agreement

At least for the agreed feature of simultaneous update/indication of a single spatial relation per group of PUCCH resources by using MAC CE, explicit higher layer signalling on PUCCH resource grouping is supported.

- Signalling details are up to RAN2

Agreement

For the agreed feature of simultaneous update/indication of a single spatial relation per group of PUCCH resources by using MAC CE,

- Support up to 4 groups per BWP.

Conclusion

For the agreed feature of single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs,

- In the RRC perspective, the candidate (up to 128) TCI-states are still independently configurable by RRC for each CC/BWP.

Agreement

For SRS resource(s) configured in an SRS resource set with usage = ‘nonCodebook’ with configuration of associatedCSI-RS, the agreed feature of a default spatial relation is not applied.

- Note: Accordingly, the previous agreement made in RAN1#98 is updated as follows (RED):

  - At least for UEs supporting beam correspondence, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement' and except for SRS with usage = ‘nonCodebook’ with configuration of associatedCSI-RS, is not configured in FR2, a default spatial relation for the dedicated-PUCCH/SRS is applied.

Working Assumption

The default spatial relation for dedicated-PUCCH/SRS for a CC in FR2, at least when no pathloss RSs are configured by RRC is determined by

- Default TCI state or QCL assumption of PDSCH, i.e.,

  - in case when CORESET(s) are configured on the CC, the CORESET with the lowest ID in the most recent monitored downlink slot, or

  - in case when any CORESETs are not configured on the CC, the activated TCI state with the lowest ID applicable to PDSCH in the active DL-BWP of the CC

- Above applies at least for UEs supporting beam correspondence

- Above applies at least for the single TRP case

- FFS: Details on UE behavior in the absence of the activated TCI state

- FFS: Details on default spatial relation in multicarrier scenario

- FFS: Details on which RS to use for pathloss measurement

- FFS: Details on how to handle this issue in case pathloss RSs are configured

Agreement

When a set of TCI-state IDs for PDSCH are activated by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the same set of TCI-state IDs are applied for the all BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same set of PDSCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS: How many combinations of CCs can be configured by RRC and relevant UE capability

Agreement

Select one among the following alternatives on whether/how to revise the existing mechanism on higher layer filtered RSRP for pathloss measurement, when a pathloss RS is updated by MAC CE (if agreed) in RAN1#99.

- Alt.1: L1-RSRP based pathloss measurement is applied when the pathloss RS is updated by MAC CE.

- Alt.2: Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Note, before the higher layer filtered RSRP is applied, UE uses L1-RSRP for pathloss estimation.

- Alt.3: Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Note: Filtered RSRP value for previous pathloss RS will be used before the application time.

- Alt.4: Reuse higher layer filtered RSRP for pathloss measurement, with the same behavior with higher layer filtered RSRP as in Rel-15.

  - Note: UE is expected to track all the RRC-configured candidate pathloss RSs.

  - Note: The maximum configurable pathloss RSs by RRC is up to UE capability.

- Note: If there is no consensus to the down-selection above, the specification in Rel-16 only allows the maximum configurable pathloss RSs by RRC is 4 (same as Rel-15). For less than or equal to X candidate pathloss RSs configured by RRC, the existing mechanism on higher layer filtered RSRP for pathloss measurement can be reused.

  - Baseline is X=4.

  - The pathloss RS selected by the MAC CE, if agreed, is among the X RRC configured candidate pathloss RSs.

- Note: For Alt.1, Alt.2, Alt.3, and Alt.4, the maximum configurable pathloss RSs by RRC is increased from Rel-15, e.g., 8, 16, or 64.

  - For Alt.1, Alt.2, Alt.3, such pathloss reference signals are for configuration purpose only.

Agreement

When a TCI-state ID is activated for a CORESET by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the TCI-state ID is applied for the CORESET(s) with the same CORESET ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same PDCCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

Working assumption

Pathloss reference RS for PUSCH can be activated/updated via a MAC CE

- The MAC CE message can activate/update the value of PUSCH-PathlossReferenceRS-Id corresponding to sri-PUSCH-PowerControlId.

  - Note(Informative): In TS38.331, the mapping is given by SRI-PUSCH-PowerControl, in which the linkage is between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS(s) if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

Working assumption

Pathloss reference RS for AP-SRS/SP-SRS can be activated/updated via a MAC CE.

- A UE can be configured with multiple pathloss RSs by RRC and one of them can be activated/updated via the MAC CE for a SRS resource set.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

- Send an LS to RAN4 asking opinion on this working assumption.

Agreement

For the purpose of simultaneous TCI state activation across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - Send LS to RAN2 to inform above

Working assumption

For the purpose of simultaneous Spatial Relation update across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - The lists are independent from those for simultaneous TCI state activation

  - Send LS to RAN2 to inform above

Working assumption

When a Spatial Relation Info is activated for a SP/AP SRS resource by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same SRS resource IDs for multiple CCs/BWPs is up to capability discussion.

- FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS on the power control details (without RAN2 impact)

On Scell BFR and L1-SINR

Agreement

Regarding the LS from RAN2 on BFR for question 3:

Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell).

- RAN1’s response is YES. Additionally, it is agreed that there will be no further RAN1specification impact to address this issue.

Send an LS to RAN2.

Agreement

For SCell BFR, reuse beam failure detection procedure specified in Rel-15, where the beam failure detection is performed per SCell.

- The “beam failure detection procedure specified in Rel-15” includes the procedure on beam failure detection for each beam failure instance, interval for beam failure instance and corresponding parameter, i.e. beamFailureInstanceMaxCount.

  - The parameters related to BFD are configured per BWP per cell

- Note: BFRQ transmission timing and condition will be decided by RAN2 for simultaneous multiple SCells failure cases

  - Include as part of LS to RAN2  to be draft by Yushu (Apple)

Agreement

- The new beam RS is mandatorily configured if SCell BFR is configured

  - Include as part of LS to RAN2  to be draft by Yushu (Apple)

Working Assumption

In addition to previous agreement that PUCCH-BFR is configured in PCell/PSCell, it is also agreed that PUCCH-BFR can be configured in PUCCH-SCell if PUCCH group is configured

- For non-DC case, down-select one of the following alternatives in RAN1#99

  - Alt1a: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per PUCCH group

    - If more than 1 PUCCH-BFR resources are configured for a UE, UE can pick one of them to transmit BFRQ

  - Alt1b: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per PUCCH group

    - PUCCH-BFR resource is shared among the CCs belonging to the respective PUCCH group

  - Alt2: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per UE

  - The down-selection is based on the assumption of SR configuration behavior supported in current spec

The above PUCCH group refers to the existing PUCCH group description in TS38.213.

Agreement

At least for PDCCH, after K symbols after receiving response to step 2 MAC-CE, UE applies the new beam indicated in step 2 MAC-CE at least for the DL reception on the failed SCell if a new beam is identified.

- Applies for all CORESETs in the failed SCell

- FFS: Any other channel

- FFS: value of K

Agreement

- When PUCCH-BFR collides with SRS, the SRS should be dropped

Agreement

For maximum number of SCell BFD RS, support up to 2 BFD RS for per BWP without introducing additional UE capability

- FFS: whether to specify UE behaviour if number of configured CORESETs is more than 2

Agreement

For eMBB, when PUCCH-BFR collides with other PUCCH that does not carry SR, reuse the dropping/multiplexing rule specified in Rel-15 for collision handling between SR and other PUCCH except the case when PUCCH-BFR based on PUCCH format 0 collides with HARQ-ACK based on PUCCH format 1

- FFS: When PUCCH-BFR based on PUCCH format 0 collides with HARQ-ACK based on PUCCH format 1

Agreement

- The bit length is fixed to be 4 bits for differential L1-SINR

- The step size is fixed to be 1 dB for differential L1-SINR

Agreement

The value of k for L1-SINR to determine the priority value of CSI report equals to 0.

Agreement

For NZP-IMR based interference measurement, option 1a is supported

- In a CSI-reportConfig, gNB configures a list of N CMR(s) and another list of N IMR(s), and they are 1:1 mapped

- For each SINR, interference is measured based on each associated NZP-IMR only

- UE may assume that the NZP CSI-RS resource for channel measurement and NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'QCL-TypeD’

  - FFS: Whether QCL-TypeD can be configured to each NZP IMR

- FFS: Each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer

- FFS: Additional support of option 2a (without RRC signalling impact)

Note: There is no consensus in RAN1 on the support of option 2b/2c (which introduces IMR index reporting for L1-SINR)

Conclusion

How to measure interference for L1-SINR from configured ZP/NZP IMR resources is up to UE implementation.

In this contribution we provide our views on the remaining issues for multi-PDCCH based multi-TRP and single-PDCCH multi-TRP, as pertaining to operation in FR2 bands.

# SCell Beam Failure Recovery

In RAN1#98bis, the following working assumption was made on BFR for SCell with both UL and DL.

Working Assumption

In addition to previous agreement that PUCCH-BFR is configured in PCell/PSCell, it is also agreed that PUCCH-BFR can be configured in PUCCH-SCell if PUCCH group is configured

- For non-DC case, down-select one of the following alternatives in RAN1#99

  - Alt1a: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per PUCCH group

    - If more than 1 PUCCH-BFR resources are configured for a UE, UE can pick one of them to transmit BFRQ

  - Alt1b: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per PUCCH group

    - PUCCH-BFR resource is shared among the CCs belonging to the respective PUCCH group

  - Alt2: For a UE, up to 1 PUCCH-BFR resource for a BWP can be configured per UE

  - The down-selection is based on the assumption of SR configuration behavior supported in current spec

The above PUCCH group refers to the existing PUCCH group description in TS38.213.

If more than one PUCCH group is available, in addition to the BFRQ procedure in the DL only case, when an SCell is configured with both UL and DL, the UE should be allowed to send BFRQ in SCell. This provides more robustness in the beam failure recovery procedure, with the ability of the UE to send the BFRQ of one SCell via a different non-failed SCell. That is why we propose to confirm the working assumption.

Proposal 1: Confirm working assumption that PUCCH-BFR can be configured in PUCCH-SCell if PUCCH group is configured.

# Enhancements on UL/DL Beam Selection to reduce latency and overhead

Other methods should be considered to reduce latency and overhead for UL/DL beam selection, in addition to those considered thus far, as per the objectives of the WI.  In particular, we propose two such methods.

## UE-triggered Beam Management

The basic concept of NZP-CSI-RS resource set repetition ‘on’ is to repeat the transmission for multiple times to allow the receiver to sweep its receive beams to measure the receive beam quality and identify the receive beam at the UE. The network does not know how many beams or resources the UE needs to sweep to identify the best receiving beams. In fact, the details of the receive beam measurement at the UE in the beam management procedure are transparent to the network, and the network triggers the receive beam measurement without knowing the status of the receive beam at the UE.

Having the beam management procedure solely triggered by the network presents a potential problem since the network does not have readily information on when the receiver beam training procedure needs to be triggered, leading to a large overhead, and potential data transmission interruption when the UE is performing antenna panel switching for example.

Observation 1: Network triggered Rx beam management procedure may lead to an increased overhead

To reduce the overhead of the Rx beam sweeping procedure, while keeping the identification of the best beam at the receiver up to the UE implementation, we propose an on-demand receive beam procedure. The UE may identify the need for a Rx beam switch before a beam management procedure is triggered by the network. This identification can be the result of a failure in PDSCH decoding, for example. It can also result from a comparison of different Rx beam RSRPs when two different RSs are sent with the same TCI state, and received on different receive beams.

Once the UE identifies that there is a need to do receive beam training, it sends the network a request to trigger the beam management procedure. This request can be signaled at the first available transmission opportunity on PUCCH, on a reserved RACH state or in UCI on PUSCH.

When the network receives the request from the UE, the network will trigger a beam management procedure. The network may choose for example to trigger a P1/P2/P3 beam management procedure, or just a P3 procedure.

Proposal 2: a UE triggered beam management procedure should be studied in NR for overhead reduction

## Lower overhead receive beam sweeping

Another venue for reducing the overhead in the beam management procedure is to vary the number of OFDM symbols where the NZP-CSI-RS resources are configured with repetition ‘on’, such that configuring the NZP-CSI-RS resources with repetition ‘on’ with a smaller repetition number results in a smaller overhead. This can be based on the quality of the Rx beam report sent to the transmitter, and based on feedback from the UE.

Observation 2: Varying the number of NZP-CSI-RS resources configured with repetition ‘on’ can result in a lower beam management overhead

For UL transmit beam and for DL receive beam selection, the ability to indicate the selected beams, which can correspond to a given panel, or beams from the same panel, is important for UL beam transmission.

Furthermore, considering the issue from a DL perspective, having an indication to the gNB in the DL beam management report about the beam group at the UE is beneficial for efficiency of the UL and DL beam management operation. This is for example, important for applications such as IAB with large number of beams expected at the mobile terminal (MT) of the IAB node, as compared to a regular mobile UE. It is also applicable to V2X with multiple antenna panels at the vehicular nodes.

Proposal 3: DL beam reporting enhancements to include receive beam information should be considered for efficiency and overhead reduction.

# Conclusions

In this contribution we described our views on enhancements for Release 16 MIMO WI related to multi-beam. We made the following observations and proposals:

Observation 1: Network triggered Rx beam management procedure may lead to an increased overhead

Observation 2: Varying the number of NZP-CSI-RS resources configured with repetition ‘on’ can result in a lower beam management overhead

Proposal 1: Confirm working assumption that PUCCH-BFR can be configured in PUCCH-SCell if PUCCH group is configured.

Proposal 2: a UE triggered beam management procedure should be studied in NR for overhead reduction

Proposal 3: DL beam reporting enhancements to include receive beam information should be considered for efficiency and overhead reduction.

# References

[1] RAN 1 Chairman’s notes, RAN1#98bis, Chongqing, China, 14th-20th October, 2019.