3GPP TSG RAN WG1 Meeting #99		R1-1912924

Reno, USA, November 18-22, 2019

Agenda Item:	7.2.14

Source:	Huawei, HiSilicon

Title:	Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols

Document for:	Discussion and Decision

- Introduction

Rate matching patterns, also called reserved resources, are introduced in NR Rel-15. It is commonly understood that rate matching pattern is an effective and efficient mechanism to support co-existence between NR and LTE. Particularly, a single PRB can be reserved within a NR bandwidth to deploy a NB-IoT carrier. Then, NR and LTE can be co-existed in a high spectral efficient manner which is also a requirement in TR 38.913 [1].

In RAN1#90bis, the mechanism of PDSCH rate matching pattern with RB symbol level granularity was agreed to cover future/backward compatible resource, which results in specification of subclause 5.1.4.1 of TS 38.214 [2].

Agreements: - On the RB-symbol level, UE can be configured with one or multiple DL resource set(s), each resource set configuration includes a first bitmap of RB granularity and a second bitmap of OFDM symbols within a slot for which the first bitmap applies (i.e. the intersection of two bitmaps). - These resource set(s) can be identified as resource sets(s) for which the PDSCH is or is not mapped based on the L1 signalling. - FFS whether or not the presence of the information field to indicate the resource set is RRC configurable – conclude by Friday - FFS whether a resource set is applicable in every slot or not (e.g., via a periodic configuration, etc.) - Conclude by Friday whether or not there is RRC impact - FFS the case of one DCI scheduling multi-slot transmission - Note: covers future/backward compatible resource, parts of CORESETs, multiple CORESETs - FFS L1 signaling is GC PDCCH or scheduling DCI - CORESET(s) configured to a UE for monitoring can be included in resource set(s) - If included, the entire COREST is assumed for rate matching when applicable - These resource set(s) can be identified as resource sets(s) for which the PDSCH is or is not mapped based on the L1 signalling. FFS the configuration details of rate matching for semi-static case (w/o L1 signalling)

In the latest version of TS 38.214 [2], the following statement is written in Subclause 5.1.4.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

In [3], the issue on valid PDSCH scheduling conditions where rate matching (RM) patterns overlap with PDSCH DMRS REs was discussed, and the following two alternatives were provided for clarification and down-selections.

- Alternative 1: PDSCH DMRS REs indicated by the frequency domain resource assignment field in a DCI are always transmitted and not overlapping with any RM pattern.

- Alternative 2: Only for available PDSCH PRB allocated by both DCI and RM patterns, its DMRS REs are transmitted. If transmitted, no overlapping with any RM pattern.

In RAN1#97 meeting, there was a conclusion that alternative 1 is RAN1’s common understanding in Rel-15 [4]. Companies thought it was too late for Rel-15 UEs even though Alt. 2 clearly outperforms Alt. 1.

Conclusion: - Alt 1 in R1-1907489 is RAN1’s common understanding - No spec update is necessary

In RAN1#98 meeting, there was discussion on PDSCH rate matching regarding the conditions of “collision between PDSCH DMRS REs and REs not available for PDSCH” in the NR Rel-16 TEI agenda item [6]. In [6], it was proposed to improve the UE behavior to tackle the collision between PDSCH DMRS REs and REs not available for PDSCH including SSB and rate matching patterns.

In RAN1#98bis meeting, the proposal was refined and proposed in [7] and got supports of 14 companies. As a result, a conclusion was made as follows,

Conclusion: Following items are to be discussed further for Rel-16 NR TEI. - Downgrading configuration of SRS for antenna switching as proposed in R1-1911456 - RACH configuration considering TDD configuration for FR1 as proposed in R1-1911191 - Rate matching pattern overlapping with PDSCH DMRS symbols as proposed in R1-1910417 - TRS for FR1 as proposed in R1-1911434 FFS: Following items are to be discussed for Rel-16 NR TEI. - QCL Type D conflict between PDSCH and CSI-RS in FR2 as proposed in R1-1910686 - Aperiodic RS triggering offset as proposed in R1-1911520

To follow up, this contribution proposes to improve the UE behavior to tackle the collision between PDSCH DMRS REs and REs not available for PDSCH in TEI16.

In Appendix, our observations on the proposal from R1-1911191 about RACH configuration are provided.

- Discussion

Regarding the conditions of “collision between PDSCH DMRS REs and REs not available for PDSCH”, the following questions should be answered.

Q1: Whether or not a SSB should be always covered by additional rate-matching pattern whose symbol length is longer than the length of all possible scheduled symbols

- Option 1.1: Yes. An additional RM pattern (in orange as shown in Figure 1 below) must be always configured to fully cover the SSB resources (in red). Since the semi-static RM pattern has to cover all possible symbols dynamically scheduled by a DCI, it has a fixed symbol length as 12 symbols or more.

- Option 1.2: No. There is no additional RM pattern configured to cover SSB resources but UE behavior is specified. Once any DM-RS RE of a PRB indicated by DCI collides with SSB, the UE assumes that the entire PRB is not available in all scheduled symbols for both PDSCH and DM-RS, i.e. the effective rate-matched resource is the entire PRB. (in black as shown in Figure 1 below)

Opt. 1.1                                                            Opt. 1.2

Figure 1 Illustrations of Opt. 1.1 (Left) and Opt. 1.2 (Right) for PDSCH slot scheduling

Comparing Option 1.1 and Option 1.2, the latter option is absolutely better than the former one. The reasons are in the following two aspects.

- Higher spectrum efficiency (SE) and scheduling flexibility: For the case of sub-slot scheduling, the scheme in the figure of the right below has been supported by Rel-15 operation and should be retained by Rel-16 operation, which has higher SE than the figure of the left.

Opt. 1.1								 Opt. 1.2

Figure 2 Illustrations of Opt. 1.1 (Left) and Opt. 1.2 (Right) for PDSCH sub-slot scheduling

- Less cost of RM patterns: The maximum number of RM patterns configured to a UE is 4, according to RRC IE maxNrofRateMatchPatterns in TS 38.331. It is very limited number and some of RM-patterns have been used to rate-match CORESETs so that PDSCH can be scheduled at the same OF symbols as CORESETs. Additionally, in case of LTE-NR DL sharing, one RM pattern has been used for LTE FDD PSS/SSS while two RM patterns have been used for LTE TDD PSS/SSS. Therefore, it is important not to waste RM patterns for NR SSBs, no mention the case that multiple NR SSBs can be configured in a FDM manner.

There is no difference of UE implementation in terms of DL channel estimation and PDSCH demodulation because additional RM pattern configured for SSB simply does not relieve any UE burden. And there is also no impact on UE timeline for processing PDSCH since the SSB presence is static and the first thing the UE is aware of. Thus, it can be observed that there is absolutely no disadvantage of Option 1.2 compared to Option 1.1. Besides, it was agreed for Rel-15 that PDSCH is rate-matched around SS blocks, which does not rely on any additional signaling of rate-matching patterns.

RAN1 NR#3 (201709) Agreements: - Confirm working assumption of: - UE-specific RRC signaling with full bitmap can be used for indicating the actually transmitted SS blocks for both sub6GHz and over6GHz cases - The actually transmitted SS blocks is indicated in RMSI for both sub6GHz and over6GHz cases - Indication is in compressed form in above 6 GHz case - Indicated resources are reserved for actually transmitted SS blocks - Data channels are rate matched around actually transmitted SS blocks RAN1#91 Agreements: - In Rel-15, no support of dedicated signalling of the location of additional multiple SS/PBCH blocks in frequency domain of a cell for rate matching purposes

Therefore, the following proposal is given.

Proposal 1: A UE shall assume that a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH is not available for both PDSCH and DM-RS in all the symbols scheduled by the DCI if any PDSCH DM-RS RE in the PRB in any scheduled symbol overlaps with any REs corresponding to SSB.

Q2: Whether or not the configured rate-matching pattern should always cover all possible scheduled symbols

- Option 2.1: Yes. Any RM pattern (in orange as shown in Figure 3 below) must be semi-statically configured in slot granularity, i.e. fully cover all OF symbols in a slot (in orange as shown in ) unless it does not overlap with any potential DMRS symbols.

- Option 2.2: No. It is symbol level that the time-domain granularity for a RM pattern overlapping with potential DMRS symbols. A UE behavior is specified similar to the SSB case Option 1.2. Once any DM-RS RE of a PRB indicated by DCI collides with a RM pattern, the UE assumes that the entire PRB is not available in all scheduled symbols for both PDSCH and DM-RS, i.e. the effective rate-matched resource is the entire PRB. (in black as shown in Figure 3 below)

Opt. 2.1										Opt. 2.2

Figure 3 Illustrations of Opt. 2.1 (Left) and Opt. 2.2 (Right) for PDSCH slot scheduling

Obviously, Option 2.2 is better than Option 2.1 in terms of spectrum efficiency (SE) and scheduling flexibility. The scheme in the right figure of Figure 4 below has been supported by Rel-15 operation and should be retained by Rel-16 operation. (The blue symbol with texture is not transmitted with PDSCH DMRS but a potential symbol for DMRS.)

Opt. 2.1										Opt. 2.2

Figure 4 Illustrations of Opt. 2.1 (Left) and Opt. 2.2 (Right) for PDSCH slot/sub-slot scheduling

Note that, the key difference between Opt. 2.1 and Opt. 2.2 is that the RM pattern configured by network always occupies all symbols of a single slot in case of Opt. 2.1, and hence a UE can assume that the RM pattern always collides with both front-loaded DMRS and additional DMRS (if configured), but it does not in case of Opt. 2.2.

  - For both Opt. 2.1 and Opt. 2.2, once the DCI scheduling a PDSCH is decoded, the PRBs containing DMRS can already be determined according to the frequency domain resource assignment and the configured RM pattern(s) before starting channel estimation as shown in the following Figure 5. From UE implementation perspective, the only difference between Opt 2.2 and Opt. 2.1 is to additionally verify whether the scheduled symbol overlapping with RM pattern is DMRS symbol or not during determining the available PRBs, as highlighted in red in Table 1 of the Pseudo codes for UE processing procedures including Rel-15 behavior, Opt. 2.1 and Opt. 2.2. It can be clearly observed that resulting complexity increment of Opt. 2.2 is negligible. Therefore, it can be concluded that Opt. 2.2 has the same processing timeline budget as that for Opt. 2.1.

  - Besides, even for the case that UE perform channel estimation and PDSCH demodulation symbol by symbol, the UE can surely determine the PRBs containing DMRS before starting channel estimation. As a result, there is no difference between the two cases where RM pattern collides only with front-loaded DMRS and RM pattern collides only with additional DMRS.

Observation: For both Opt. 2.1 and Opt. 2.2, UE already determines PRBs containing DMRS before starting channel estimation. The only UE complexity increment is highlighted in red in the Table 1 which is negligible.

Figure 5 Illustration of UE processing timeline

Based on above analysis, it can be concluded that there is no disadvantage of Opt. 2.2 compared to Opt. 2.1 since Opt. 2.2 does not cause impact on UE timeline for processing PDSCH.

Table 1. Pseudo codes for UE processing procedures for Rel-15, Opt. 2.1 and Opt. 2.2

Pseudo codes for Rel-15 behavior If DCI is decoded determine DMRS symbols according to time domain resource assignment determine PRBs containing DMRS according to frequency domain resource assignment for k=1 to #PRG(K) estimate channels for PDSCH in PRG(k) end for j=1 to #RM (e.g. 4) if scheduled PRB within Freq_bitmap of RM j if scheduled symbol within Time_bitmap of RM j // RB-symbol level rate matching rate-match out the entire PRB in the symbol for PDSCH end end end end

Pseudo codes of Opt. 2.1 If DCI is decoded determine DMRS symbols according to time domain resource assignment determine PRBs containing DMRS according to frequency domain resource assignment for j=1 to #RM (e.g. 4) if scheduled PRB within Freq_bitmap of RM j if scheduled symbol within Time_bitmap of RM j // determine the available PRBs containing DMRS and PDSCH rate-match out the entire PRB in the symbol for both DMRS and PDSCH end end end for k=1 to #PRG(K) estimate channels for PDSCH in PRG(k) end end

Pseudo codes of Opt. 2.2 If DCI is decoded determine DMRS symbols according to time domain resource assignment determine PRBs containing DMRS according to frequency domain resource assignment for j=1 to #RM (e.g. 4) if scheduled PRB within Freq_bitmap of RM j if scheduled symbol within Time_bitmap of RM j // determine the available PRBs containing DMRS and PDSCH if the symbol contains DMRS rate-match out the entire PRB in all the scheduled symbols for both DMRS and PDSCH else rate-match out the entire PRB in the symbol for PDSCH end end end end for k=1 to #PRG(K) estimate channels for PDSCH in PRG(k) end end

Then, the following proposal is given.

Proposal 2: A UE shall assume that a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH is not available for both PDSCH and DM-RS in all the symbols scheduled by the DCI if any PDSCH DM-RS RE in the PRB in any scheduled symbol overlaps with any REs corresponding to rate-matching patterns, i.e. Opt. 2.2 is supported with specified UE behavior.

Q3: Whether PDSCH DMRS can be “rate-matched” out in case of wideband PRG or not

- Option 3.1: Yes.

- Option 3.2: No.

Opt 3.2 is preferred, because of the following text in TS 38.214. It implies that UE needs contiguous DMRS PRBs for channel estimation in case of “wideband” PRG.

If is determined as "wideband", the UE is not expected to be scheduled with non-contiguous PRBs and the UE may assume that the same precoding is applied to the allocated resource.

Proposal 3: Only PRG size 2 and 4 are addressed for allowing PDSCH DMRS (within the resources indicated by a DCI) not available due to SSB and rate-matching patterns.

Q4: Whether fractional PRG is allowed or not

- Option 4.1: Fractional PRG is not allowed.

- Option 4.2: Fractional PRGs are allowed but with limited number, e.g., 4.

Firstly, it is worth noticing that the RM pattern or SSB may not always contain an integer number of intact PRGs. In other word, RM pattern or SSB may not be aligned with PRG boundary. In this case, there would exist some PRGs containing PRBs not available for PDSCH DMRS. Such fractional PRGs may cause extra complexity or performance degradation for UE channel estimation.

Comparing the two options, Option 4.2 is preferred since Option 4.1 has the following disadvantages.

- It cannot support Option 1.2 to solve the popular issue of SSB. And it prevents gNBs from supporting forward compatibility and LTE-NR coexistence.

- On the contrary, Option 4.2 solves all important issues above. With limited number of fractional PRG, the UE complexity improvement and performance degradation will be negligible.

- Rel-15 UEs are mandatory to support fractional PRGs because they have supported the following, which should be inherited by Rel-16 UEs,

  - Downlink resource allocation type 1, in which both the indicated starting PRB and ending PRB are allowed to be not aligned with PRG boundary.

  - Active BWP boundary is allowed to be not aligned with PRG boundary.

The minimal number of fractional PRGs is 4, shown in Figure 5 below.

Opt. 4.2

Figure 6 Illustrations of the minimal # of fractional PRGs for a basic application

On top of Figure 6, more than 4 fractional PRGs may be required for the following cases.

- Multiple SSBs are configured in FDM manner for the coverage of skycrapers.

- The eMTC/NB-IoT is configured within the bandwidth.

- LTE PSS/SSS/PBCH is configured within the NR bandwidth for LTE-NR DL sharing.

- PDSCH for eMBB UEs may be rate-matched around search space/CORESET configured for URLLC UEs which are spanning at different OF symbols in a slot as the UE feature 3-5b.

Thus, it is preferred for UE to support 6 fractional PRGs.

Proposal 4: For PDSCH rate-matching purpose, fractional PRGs are allowed but with limited number. The minimum number of fractional PRGs that UEs support for rate-matching with SSB and rate-matching patterns is [4 or 6].

- A fractional PRG contains at least one PRB where PDSCH DMRS REs overlap with any REs corresponding to SSB or rate matching patterns.

Q5: Whether to increase the maximum number of rate matching patterns configurable to a UE or not

For the collision issue between DMRS and RM pattern, the only spec impact in TEI-16 is to specify UE behavior to handle the case, which is not related to the number of configured rate matching patterns. To minimize UE complexity, the maximum number of rate matching patterns configurable to a UE should NOT be increased, and it should be kept the same as that in Rel-15.

Proposal 5: The maximum number of rate matching patterns configurable to a UE, including both the original RB-symbol level rate-matching patterns and the rate-matching patterns that may overlap with DMRS, is the same as Rel-15.

In addition, there is some discussion on whether there is a need to clarify that non-available DM-RS RE includes REs which are indicated as CDM group without data or just includes REs which DMRS sequence is mapped on. It should be noted that this issue is only existed for RE-level rate matching. While in this TEI scope, only RB-symbol level rate-matching pattern and SSB are considered, and thus such clarification is not needed in this case.

Scope of PDSCH rate-matching around SSB and rate-matching patterns

If it is preferred to conclude the scope of Rel-16 TEI on PDSCH rate-matching around SSB and rate-matching patterns before jumping into any discussion of technical details, then the following proposal is given based on above analysis,

Proposal 6: The objectives for Rel-16 TEI on PDSCH rate-matching around SSB and rate-matching patterns are the following,

- Specify UE behavior for rate-matching around SSB as Opt. 1.2

- Specify rate-matching according to rate-matching pattern which can overlap with scheduled PDSCH DMRS, as Opt 2.2

  - For the rate-matching pattern which may overlap with DMRS,

    - It is semi-statically acknowledged by a UE that the rate-matching pattern is probably overlapping with PDSCH DMRS

      - For a rate-matching pattern with no such acknowledgement, the UE behavior for the rate-matching pattern is not changed

    - UE behavior is specified for the case where a UE identifies overlaps between its PDSCH DMRS and the rate-matching pattern

      - FFS: It is only front-loaded DMRS that the UE uses to determine the presence of overlap

  - The number M of the rate matching patterns that can overlap with DMRS is limited

  - The maximum number of rate matching patterns configurable to a UE, including both the original RB-symbol level rate-matching patterns and the rate-matching patterns that may overlap with DMRS, is the same as Rel-15

  - Applicable for both PDSCH mapping Type A and Type-B

Note: The case of wideband PRG is not in scope of this Rel-16 TEI proposal.

Note: The RE-level rate-matching is not in scope of this Rel-16 TEI proposal.

- Conclusions

In this contribution, the issue on PDSCH scheduling conditions with rate matching pattern overlapping with PDSCH DMRS symbols is discussed. An example CR is attached for discussion. The following observation and proposals are given:

Observation: For both Opt. 2.1 and Opt. 2.2, UE already determines PRBs containing DMRS before starting channel estimation. The only UE complexity increment is highlighted in red in the Table 1 which is negligible.

Proposal 1: A UE shall assume that a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH is not available for both PDSCH and DM-RS in all the symbols scheduled by the DCI if any PDSCH DM-RS RE in the PRB in any scheduled symbol overlaps with any REs corresponding to SSB.

Proposal 2: A UE shall assume that a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH is not available for both PDSCH and DM-RS in all the symbols scheduled by the DCI if any PDSCH DM-RS RE in the PRB in any scheduled symbol overlaps with any REs corresponding to rate-matching patterns, i.e. Opt. 2.2 is supported with specified UE behavior.

Proposal 3: Only PRG size 2 and 4 are addressed for allowing PDSCH DMRS (within the resources indicated by a DCI) not available due to SSB and rate-matching patterns.

Proposal 4: For PDSCH rate-matching purpose, fractional PRGs are allowed but with limited number. The minimum number of Fractional PRGs that UEs support for rate-matching with SSB and rate-matching patterns is [4 or 6].

- A fractional PRG contains at least one PRB where PDSCH DMRS REs overlap with any REs corresponding to SSB or rate matching patterns.

Proposal 5: The maximum number of rate matching patterns configurable to a UE, including both the original RB-symbol level rate-matching patterns and the rate-matching patterns that may overlap with DMRS, is the same as Rel-15.

Proposal 6: The objectives for Rel-16 TEI on PDSCH rate-matching around SSB and rate-matching patterns are the following,

- Specify UE behavior for rate-matching around SSB as Opt. 1.2

- Specify rate-matching according to rate-matching pattern which can overlap with scheduled PDSCH DMRS, as Opt 2.2

  - For the rate-matching pattern which may overlap with DMRS,

    - It is semi-statically acknowledged by a UE that the rate-matching pattern is probably overlapping with PDSCH DMRS

      - For a rate-matching pattern with no such acknowledgement, the UE behavior for the rate-matching pattern is not changed

    - UE behavior is specified for the case where a UE identifies overlaps between its PDSCH DMRS and the rate-matching pattern

      - FFS: It is only front-loaded DMRS that the UE uses to determine the presence of overlap

  - The number M of the rate matching patterns that can overlap with DMRS is limited

  - The maximum number of rate matching patterns configurable to a UE, including both the original RB-symbol level rate-matching patterns and the rate-matching patterns that may overlap with DMRS, is the same as Rel-15

  - Applicable for both PDSCH mapping Type A and Type-B

Note: The case of wideband PRG is not in scope of this Rel-16 TEI proposal.

Note: The RE-level rate-matching is not in scope of this Rel-16 TEI proposal.

References

3GPP TR 38.913 v15.0.0: "Study on Scenarios and Requirements for Next Generation Access Technologies ".

3GPP TS 38.214 v15.6.0: "NR; Physical layer procedures for data".

R1-1907489, “Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols”, Huawei, HiSilicon.

“Chairman’s notes RAN1 #97”, Reno, May 13th –17th, 2019.

3GPP TS 38.331 v15.6.0: "NR; Radio Resource Control (RRC) protocol specification".

R1-1909433, “TEI for PDSCH rate matching and views on DSS”, Huawei, HiSilicon, Prague, Czech, 2019.

R1-1910417, “Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols”, Huawei, HiSilicon, Chongqing, China, 2019

Appendix A

- Discussion on TEI-16 RACH configuration

- In RAN1#98bis, one TEI about RACH configuration is proposed to handle the flexibility issue of RACH configuration and four possible alternatives are provided [1]. More specifically, for Alt1 and Alt2, new RRC parameters are introduced. For Alt3 and Alt4, new configurations or additional modifications are needed for the current RACH configuration table. One common concern for the proposed solutions is that the benefit for Rel-16 UEs is unclear when Rel-15 UEs and Rel-16 UEs coexist in the network. More specifically, whenever there are Rel-15 UEs in the network, the RACH configuration period has to be configured as 10ms to serve Rel-15 UEs. Although longer period can be configured for the Rel-16 UEs, the overheads of the RACH resources remains the same, i.e., 10ms period. On the contrary, it may even increase the overheads if the existing RACH resources cannot be reused by Rel-16 UEs and additional RACH resources dedicated to Rel-16 UEs only are configured.

- Observation A.1: The benefit of the four proposed solutions on RACH configuration is unclear when Rel-15 UEs and Rel-16 UEs coexist in the network.

- Furthermore, another potential implementation-based solution is stated in [1] which configures shorter RACH configuration period but only maps SS/PBCH blocks to a subset of the RACH occasions to achieve an equivalent longer RACH configuration period. For example, the RACH configuration period 10 ms is configured but every 20ms RACH resources are mapped with SS/PBCH blocks. However, such solution is supported by current specifications. More specifically, for the mapping between SS/PBCH block and RACH occasions, the association period is specified to be the smallest value in the set determined by the PRACH configuration period according to Table 8.1-1 in Section 8.1 of [2] such that all the actual transmitted SS/PBCH blocks in one SS/PBCH block period are mapped at least once to the PRACH occasions within the association period. It can be seen that the smallest value for all possible PRACH configuration periods is 1. Then, for certain PRACH configuration period, the PRACH resources in each period should be mapped to the SS/PBCH blocks. Only when there remains a set of PRACH occasions that are not mapped to SS/PBCH blocks after an integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period and this remaining set of PRACH occasions cannot cover one mapping cycle, these PRACH occasions are not used for PRACH transmissions.

- Observation A.2: The implementation-based solution, which configures shorter RACH configuration period but only maps SS/PBCH blocks to a subset of the RACH occasions to achieve an equivalent longer RACH configuration period, is not supported by the latest specification.

Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period [2]

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

- Conclusions

- Based on above discussions, the following observations are provided,

- Observation A.1: The benefit of the four proposed solutions on RACH configuration is unclear when Rel-15 UEs and Rel-16 UEs coexist in the network.

- Observation A.2: The implementation-based solution, which configures shorter RACH configuration period but only maps SS/PBCH blocks to a subset of the RACH occasions to achieve an equivalent longer RACH configuration period, is not supported by the latest specification.

- References

- R1-1911191, “Discussion on NR Rel-16 TEI for RACH configuration”, NTT DOCOMO, INC., Ericsson, SoftBank, Sharp, Nokia

3GPP TS 38.213 v15.7.0: "NR; Physical layer procedures for control".