3GPP TSG RAN WG1 Meeting #99	                                                                          R1-1912935

Reno, Nevada, US

November 18th – 22nd, 2019

Agenda item:    7.2.2.1.1

Source:              Qualcomm Incorporated

Title:                  Initial access signals and channels for NR-U

Document for:  Discussion and Decision

# Introduction

The work item description on NR based access to unlicensed spectrum [1] contains the objectives of the WI which include the following:

Physical layer aspects:

- NR-U Discovery Reference Signal (DRS) containing at least SS/PBCH block burst set transmission and possibly CSI-RS, RMSI-CORESET(s)+PDSCH(s), OSI and paging with properties and extensions from NR Rel-15 in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2).

- 60kHz based SSB/PBCH block is outside the scope of the WI.

- DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

- PRACH including possible extension of PRACH format(s) in line with agreements during the SI phase (TR 38.889, Section 7.2.1.2) to support minimum bandwidth requirement given by regulation.

- Determine the applicability of Rel-15 NR formats to NR-U operation. RAN1 should decide whether 60 kHz subcarrier spacing for PRACH is supported, based on a unified design with 15 kHz and 30 kHz PRACH for meeting occupied channel bandwidth (OCB) requirements.

Physical layer procedure(s):

- Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

- Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1

# NR-U DRS

In RAN1 the following agreements have been captured so far:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

  - Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

- The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

  - TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

  - Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

  - FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

- “Support of Pattern 1 is recommended for multiplexing of SS/PBCH block(s) and CORESET(s)#0 in NR-U, where Pattern 1 is understood as CORESET#0 and SS/PBCH block occur in different time instances, and CORESET#0 bandwidth overlaps with the transmission bandwidth of the SS/PBCH block.

- As one element to facilitate a NR-U DRS design without gaps in the time domain, the CORESET#0 configuration(s) and/or Type0-PDCCH common search space configuration(s) may need enhancements compared to NR Rel-15, such as additional time domain configurations of the common search space.”

- Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for:

  - Meeting OCB requirement

  - Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

  - Support of stand-alone NR-U deployments

  - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment

  - Resolution of PCI confusion in an NR-U deployment

  - Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals

- The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial

  - Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

- Down-select from the following options for SSB pattern (symbol index starts at 0)

  - Option 1: SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot

  - Option 2: SSBs are at symbols (2,3,4,5) and (9,10,11,12) in the slot

- The down-selected pattern applies no matter if SSB SCS is indicated by higher layer or not, and no matter if RMSI is transmitted or not.

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

- Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U

- Select one of the following options in RAN1 #97:

  - Alt 1: Legacy SSB positions in a slot

    - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot

    - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot

      - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot

  - Alt2: New SSB positions in a slot

    - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

    - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot

  - Alt 3: Legacy SSB positions in a slot

    - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

    - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot

- For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further.

  - Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97

- Conclusion: There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

- To support RMSI transmission for ANR purpose on a carrier with an SSB not on a sync raster consider the following alternatives:

  - Alt 1: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point. After the UE detects the SSB it will proceed with RMSI decoding whereby the frequency location of the CORESET #0 scheduling the PDSCH carrying the RMSI is implicitly known.

  - Alt 2: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point and the MIB in SSB will point to the frequency location of the coreset #0.

    - FFS: number of CORESET 0 offsets configurable in MIB when SSB is not on sync raster entry

    - Note: Signalling of offset only necessary if more than one off-sync-raster point offset is agreed

## 2.1	SSB and RMSI transmission

Following the conclusion from RAN1 #97, no new Type0-PDCCH monitoring pattern will be adopted in Rel.16 NR-U. Instead, legacy Rel.15 NR Type0-PDCCH monitoring design will be used directly. To summarize, Rel.15 NR Type0-PDCCH monitoring is defined in Section 13 of TS38.213. For FR1, for an SS/PBCH block index i, a UE will monitor Type0-PDCCH CSS over two consecutive slots. The monitoring starting slot and the starting symbol of the PDCCH are functions of SS/PBCH block index i, and the parameters defined in Table 13-11 of TS38.213 (copied below). The parameters are controlled by a 4 bit field in PBCH. Only SSB/RMSI pattern 1 multiplexing is supported for 15 & 30 kHz SCS.

Table 13-11 of TS 38.213: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR1

| Index | O | Number of search space sets per slot | M | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 2 | 2 | 1 | 1 | 0 |
| 3 | 2 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 6 | 7 | 1 | 1 | 0 |
| 7 | 7 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 8 | 0 | 1 | 2 | 0 |
| 9 | 5 | 1 | 2 | 0 |
| 10 | 0 | 1 | 1 | 1 |
| 11 | 0 | 1 | 1 | 2 |
| 12 | 2 | 1 | 1 | 1 |
| 13 | 2 | 1 | 1 | 2 |
| 14 | 5 | 1 | 1 | 1 |
| 15 | 5 | 1 | 1 | 2 |

Out of the existing ways to configure the Type0-PDCCH monitoring, some of them will allow RMSI to be transmitted in the same slot as the corresponding SS/PBCH block (eg. O=0 and M=1/2 as demonstrated in Figure 1), while others may allow RMSI to be transmitted a few ms later (eg. O=2, M=1 as demonstrated in Figure 2). For the later case, for NR-U operation, it is possible that the RMSI transmission has to be in its own Channel Occupancy, instead of sharing the COT of DRS transmission. In that case, in the same DMTC period, the SS/PBCH blocks transmission and RMSI transmission may be independent (subject to separate LBT). Even though under some conditions, the DRS transmission itself can use Cat 2 LBT, the RMSI transmission in a different COT has to use Cat 4 LBT.

Figure 1. Same-slot Type0-PDCCH mapping (when O=0, M=1/2).

Figure 2: Different-slot Type-0 PDCCH mapping (e.g. O=2, M=1)

Observation 1. Legacy Type0-PDCCH time domain monitoring configuration has multiple options to support both DRS configurations where RMSI PDSCH shares the same slot as SSB, as well as DRS configurations where RMSI PDSCH uses a different slot than SSB

Previously, multiple companies proposed to consider RMSI PDSCH rate matching around SSB. Now that legacy NR Type0-PDCCH monitoring is used, the SS/PBCH block and the corresponding RMSI PDSCH may not be transmitted together. Defining a proper rate matching mechanism become more difficult. Furthermore, RAN4 also agreed to sync raster locations close to the edge of each 20MHz channel. In that case, continuous frequency domain resource allocation supported for DCI format 1_0 for RMSI PDSCH scheduling has enough flexibility to assign most of the remaining RBs in the SSB slots to RMSI PDSCH. Therefore we propose to not support additional RMSI PDSCH rate matching behaviour.

Proposal 2. RMSI PDSCH rate matching around SSB is not supported in NR-U.

## 2.2	CORESET #0 position within a 20MHz subband

RAN1 already concluded that NR-U “CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.” As such:

- 30 kHz SCS: at most four (PRB-granularity) offsets would be possible in a 20 MHz subband (illustrated in Figure 3).

- 15 kHz SCS: at most eleven (PRB granularity) offsets would be possible in a 20 MHz subband (not illustrated)

RAN4 also agreed a set of GSCN points (on a 1.44MHz grid), one per 20MHz, as listed in Table 1. The GSCN choices place the SSB close to the edge of each 20MHz but leaves at least 3 RB of guard towards the edge.

Though we may have a little flexibility in placing the coreset #0 of 48 RBs in 20MHz, it is highly questionable if there is any use case to take advantage of the flexibility. On the other hand, for better protection of the coreset #0 and better coexistence, it is preferable to put the coreset #0 in the middle of the 20MHz subband.

Figure 3: Illustration of Type0-CORESET frequency offset for 30 kHz SCS in NR-U

Proposal 3. 30KHz SCS CORESET #0 location within each 20MHz LBT bandwidth is fixed as in Table 1 (3rd column), and can be captured by a per 20MHz LBT bandwidth RB level offset from SSB on sync raster to coreset #0.

Proposal 4. For 15KHz SCS, CORESET #0 location can overlap with 30KHz CORESET #0 defined (align at the center of the lowest RE).

Table 1. Coreset #0 placement for each 20MHz channel

| 20MHz channel center (MHz) | GSCN | RB offset from CORESET #0 to SSB | Left Guard for CORESET #0 (MHz) | Right Guard for CORESET #0 (MHz) | Left Guard for 50RB initial UL BWP (MHz) | Right Guard for 50RB initial UL BWP (MHz) | Coreset center to channel center (KHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5160 | [8996] | 2 | 1.345 | 1.375 | 0.985 | 1.015 | -15 |
| 5180 | 9010 | 2 | 1.505 | 1.215 | 1.145 | 0.855 | 145 |
| 5200 | 9024 | 3 | 1.305 | 1.415 | 0.945 | 1.055 | -55 |
| 5220 | [9037/9038] | 0/3 | 1.105/1.465 | 1.615/1.255 | 0.745/1.105 | 1.255/0.895 | -255/105 |
| 5240 | 9051 | 0 | 1.265 | 1.455 | 0.905 | 1.095 | -95 |
| 5260 | 9065 | 0 | 1.425 | 1.295 | 1.065 | 0.935 | 65 |
| 5280 | 9079 | 1 | 1.225 | 1.495 | 0.865 | 1.135 | -135 |
| 5300 | 9093 | 1 | 1.385 | 1.335 | 1.025 | 0.975 | 25 |
| 5320 | 9107 | 2 | 1.185 | 1.535 | 0.825 | 1.175 | -175 |
| 5340 | [9121] | 2 | 1.345 | 1.375 | 0.985 | 1.015 | -15 |
| 5480 | [9218] | 1 | 1.385 | 1.335 | 1.025 | 0.975 | 25 |
| 5500 | 9232 | 2 | 1.185 | 1.535 | 0.825 | 1.175 | -175 |
| 5520 | 9246 | 2 | 1.345 | 1.375 | 0.985 | 1.015 | -15 |
| 5540 | 9260 | 2 | 1.505 | 1.215 | 1.145 | 0.855 | 145 |
| 5560 | 9274 | 3 | 1.305 | 1.415 | 0.945 | 1.055 | -55 |
| 5580 | 9287 | 0 | 1.105 | 1.615 | 0.745 | 1.255 | -255 |
| 5600 | 9301 | 0 | 1.265 | 1.455 | 0.905 | 1.095 | -95 |
| 5620 | 9315 | 0 | 1.425 | 1.295 | 1.065 | 0.935 | 65 |
| 5640 | 9329 | 1 | 1.225 | 1.495 | 0.865 | 1.135 | -135 |
| 5660 | 9343 | 1 | 1.385 | 1.335 | 1.025 | 0.975 | 25 |
| 5680 | 9357 | 2 | 1.185 | 1.535 | 0.825 | 1.175 | -175 |
| 5700 | [9371] | 2 | 1.345 | 1.375 | 0.985 | 1.015 | -15 |
| 5720 | 9385 | 2 | 1.505 | 1.215 | 1.145 | 0.855 | 145 |
| 5745 | 9402 | 1 | 1.345 | 1.375 | 0.985 | 1.015 | -15 |
| 5765 | 9416 | 1 | 1.505 | 1.215 | 1.145 | 0.855 | 145 |
| 5785 | 9430 | 2 | 1.305 | 1.415 | 0.945 | 1.055 | -55 |
| 5805 | 9444 | 2 | 1.465 | 1.255 | 1.105 | 0.895 | 105 |
| 5825 | 9458 | 3 | 1.265 | 1.455 | 0.905 | 1.095 | -95 |
| 5845 | [9471] | 0 | 1.065 | 1.655 | 0.705 | 1.295 | -295 |
| 5865 | [9485] | 0 | 1.225 | 1.495 | 0.865 | 1.135 | -135 |
| 5885 | [9499] | 0 | 1.385 | 1.335 | 1.025 | 0.975 | 25 |
| 5905 | [9513] | 1 | 1.185 | 1.535 | 0.825 | 1.175 | -175 |

For each GSCN, we evaluated the guard band of an initial DL BWP matches the CORESET #0 bandwidth under all possible offsets (0/1/2/3 RBs). The results are shown in Figure 4 (the red dots), where only the initial DL BWP satisfy the 805KHz guard band requirement for 20MHz are shown. The RB offsets selected in Table 1 (connected with a green line in the figure) allow the largest guard band protection. Note that the chosen RB offset also satisfy the guardband criteria from the RAN4 way forward on sync raster [21] (i.e. inter-carrier and intra-carrier guardbands).

Figure 4: RB offset selection for 30 kHz CORESET0 for NR-U

Given a fixed CORESET #0 location within 20MHz, there is no need to indicate the coreset frequency domain location in the pdcch-ConfigSIB1 anymore. Additionally, we also agreed that SSB and CORESET #0 will be of the same SCS, and only 1 or 2 symbol CORESET #0 will be supported for NR-U. As a result, the 4 MSB of pdcch-ConfigSIB1 can be reinterpreted using the following tables for NR-U.

Table 2. Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30,30} kHz for unlicensed band operation

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Q | Offset (RBs) |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 48 | 1 | 1 | See Table 1 |
| 1 | 1 | 48 | 2 | 1 | See Table 1 |
| 2 | 1 | 48 | 1 | 2 | See Table 1 |
| 3 | 1 | 48 | 2 | 2 | See Table 1 |
| 4 | 1 | 48 | 1 | 4 | See Table 1 |
| 5 | 1 | 48 | 2 | 4 | See Table 1 |
| 6 | 1 | 48 | 1 | 8 | See Table 1 |
| 7 | 1 | 48 | 2 | 8 | See Table 1 |
| 8-15 | Reserved |  |  |  |  |

Table 3. Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15,15} kHz for unlicensed band operation

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Q | Offset (RBs) |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | 1 | See Table 1 |
| 1 | 1 | 96 | 2 | 1 | See Table 1 |
| 2 | 1 | 96 | 1 | 2 | See Table 1 |
| 3 | 1 | 96 | 2 | 2 | See Table 1 |
| 4 | 1 | 96 | 1 | 4 | See Table 1 |
| 5 | 1 | 96 | 2 | 4 | See Table 1 |
| 6 | 1 | 96 | 1 | 8 | See Table 1 |
| 7 | 1 | 96 | 2 | 8 | See Table 1 |
| 8-15 | Reserved |  |  |  |  |

## 2.3	CSI-RS in DRS

P-CSI-RS and A-CSI-RS are reasonable candidates for multiplexing CSI-RS in DRS. To the extent to which the SP-CSI-RS trigger is outside DRS (and this less likely to impact DRS latency), it can be considered equivalent to P-CSI-RS in this paragraph.

A-CSI-RS requires a Format 0_1 trigger, which could fit in DRS, at the expense of some impact on the capacity of common (S-RNTI, P-RNTI) PDSCH allocation for RMSI and Paging, and is also subject to CSI reporting requirements soon after DRS.

Up to 64 P-CSI-RS can be configured at DRS periodicity, at fixed offsets in the SSB candidate slots. Since there are up to eight such SSB candidates, there is no spec restriction for broadcasting P-CSI-RS. In Rel-15, RMSI does not rate match around P-CSI-RS, likely because there is sufficient flexibility to avoid overlap between CSI-RS and RMSI. In light of the tight timeline remaining for Rel-16, we propose that the restriction be maintained for NR-U, and possibly revisited in future optimizations.

- FDM: possible with SSB and – if PDSCH (RMSI, Paging) allocation does not exceed 24 PRBs – RMSI and Paging as well

- TDM: possible, at the expense of some reduction of available PDSCH capacity in the SSB slot.

Proposal 5. No further enhancement of common PDSCH rate matching around CSI-RS is needed in NR-U Rel-16. Further optimisations can be considered beyond Rel-16 NR-U

# PRACH

The following agreements were made in RAN1 AH1901 [6-7]

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |

The following was agreed in RAN1 #96 [8-9].

Agreement:

For PRACH evaluations, following metrics should be provided:

| Parameter | Value | Notes |
| --- | --- | --- |
| Scheme |  | Eg. Alt4-ZC139x2 |
| SCS |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  | Eg. 139, |
| # of repetition (R) |  | If repetition of sequence is used in freq domain |
| N_cs |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) |  | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency occupancy (MHz) |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  | Np= -174+10*log10(SCS*L_RA*R)+NF NF=-5dB |
| SNR (dB) |  | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX-SNR-Np |
| N_FDM |  | # of ROs in 20MHz |
| Capacity |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

The following was agreed in RAN1 96 bis [10-11].

Agreement:

PRB/RE-interlaced PRACH is not considered further. Consider the following alternatives should be studied further as options.

- Alt 1: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers, with repetitions in frequency

  - FFS: Guard bands between repetitions

  - FFS: Number of repetitions

  - FFS: Whether repetitions are constrained to be contiguous in frequency or not

- Alt 2: A single PRACH sequence mapped to contiguous PRBs according to one of the following alternatives

  - Alt 2.1: ZC sequence with longer length than 139

  - Alt 2.2: New sequence with longer length than 139

The following was agreed in RAN1 97 [12].

Agreement:

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA= [571, 1151]

- 30 kHz: Choose one of L_RA= [283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

The following was agreed in RAN1 98 [13].

Agreement:

No new PRACH formats are specified for operation in unlicensed spectrum.

## 3.1 PRACH channel access

In NR, multiple RACH occasions (RO) are configured back-to-back. NR-U UEs are expected to perform LBT before RACH transmission. To maintain the availability of all ROs, it is imperative that a UE attempting RACH transmission should not be blocked (due to LBT failure) by other UEs from the same cell using the previous RO. This blocking issue will be more prominent where multiple RACH periods are blocked by transmissions of neighbouring nodes in a loaded system. In NR different UEs would use these different RACH periods but in NR-U due to entire RACH period being blocked, all UEs end up trying RACH in the same RACH period when medium becomes free. Hence the chances of UEs trying to transmit RACH in back to back RACH occasions becomes high. For example, in NR UE1 tries PRACH in period 1, UE 2 tries in period 2, UE 3 tries in period 3. But in NR-U, it is possible that periods 1 to 3 are blocked by transmissions of other nodes. In this scenario all the UEs will try period 4. Within period 4, for all the UEs to be able to transmit PRACH successfully, all of their LBTs must pass.

One of the solutions could be to introduce gaps between successive ROs to allow UEs to perform UL LBT without any interference from other serving cell UEs as is shown in Figure 5.

Figure 5. PRACH occasions with LBT gaps

The gaps can be created in multiple ways. One simple method would be to puncture the PRACH signals by 1 or 2 symbols in the beginning. In this case, we can reuse the same PRACH configuration tables (Table 6.3.3.2-3 and Table 6.3.3.2-4 in [14]). If the UE does not pass LBT in the first symbol due to an ongoing PRACH transmission from another user, it tries LBT again in the next symbol and if that is successful transmits a PRACH signal with the first symbol punctured. The disadvantage of this method is that the gNB has to perform blind detection for all possible PRACH starting symbols. Alternatively, only the alternate ROs can be considered valid for NR-U. For example, with format A1 if there are 6 ROs, only RO# 0, 2, 4 are valid and RO# 1, 3, 5 are invalid. This method is easy to implement with minimal modifications to the specifications. However, this leads to more than required wastage of PRACH opportunities.

Keeping in mind the limited available time in order to have no RRC impact, simpler solution is to use even ROs. RAN1 specification have to include that only even RO indices within a slot are valid for NR-U.

Figure 6: LBT gap options for NR-U

Proposal 6: Introduce gaps between consecutive TDM-ed RACH occasions for UL LBT operation.

- The size of gaps shall match the maximum expected UL LBT duration (e.g. at least larger than that obtained by assuming the largest contention window and no other interferers)

- Gaps should be in units of symbols for easier alignment

- Minimum gap required is 1 or 2 symbols for 15 or 30 kHz SCS respectively

- Gaps can be introduced in the following way:

  - Use only the even Rach Occasions within a slot

## 3.2	PRACH numerology and bandwidth

According to agreement made in RAN1 #96 Bis, PRACH will only be time domain multiplexed with other UL channels. Since there is no frequency domain multiplexing of PRACH with PUCCH/PUSCH for SCS= 60 kHz, there is no requirement of supporting PRACH with 60 kHz SCS. Hence PRACH waveforms with 15 and 30 kHz SCS will be sufficient.

Proposal 7: Only 15 and 30 kHz SCS will be supported for NR-U PRACH.

## 3.3	PRACH simulation results

In this section, we provide PRACH simulation results with 15 kHz and 30 kHz SCS. We consider the scenario with 20 MHz BW, where there are 106 PRBs with 15 kHz SCS and 51 PRBs with 30 kHz SCS.

In general, we compare the following alternatives:

- Rel-15 PRACH: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers

- Rel-15 PRACH repeated to occupy half the BW: Legacy NR PRACH sequence of length 139 mapped to 12 contiguous RBs:

  - {0-11, 12-23} for 30 kHz and

  - {0-11, 12-23, 24-35, 36-47} for 15 kHz SCS.

- Rel-15 PRACH repeated to occupy the entire BW: Legacy NR PRACH sequence of length 139 mapped to 12 contiguous RBs:

  - {0-11, 12-23, 24-35, 36-47} for 30 kHz and

  - {0-11, 12-23, 24-35, 36-47, 48-59, 60-71, 72-83, 84-95} for 15 kHz SCS

- Wideband ZC sequence with longer length mapped to contiguous PRBs.

  - L_RA= 571 for 30 kHz SCS, L_RA= 1151 for 15 kHz SCS, (Full band contiguous PRACH)

  - L_RA= 283 for 30 kHz SCS, L_RA= 571 for 15 kHz SCS, (Half band contiguous PRACH)

For 139 length PRACH sequence (L_RA=139), 64 preambles are generated using NCS =17, and u = [21   118 22  117 23  116 24  115],  values from Table 6.3.3.1-4 and 6.3.3.1-5 in [13]. For case 4 with 30 kHz SCS (L_RA=571), the 64 PRACH sequences are generated using NCS =41, and u = [1 570 2 569 3 ]. For case 4 with 30 kHz SCS (L_RA= 283), the 64 PRACH sequences are generated using NCS =34, and u = [1 282 2 281 3 280 4 279].

For case 4 with 15 kHz SCS (L_RA= 1151) the 64 PRACH sequences are generated using NCS =50, and u = [1 1150 2 ]. For case 4 with 15 kHz SCS (L_RA= 571) the 64 PRACH sequences are generated using NCS =46, and u = [1 570 2 569 3 568].

Note that, the 80% BW occupancy OCB constraint is not met in alternative 1, 2, and 4b, but the constraint is satisfied for alternatives 3 and 4a. Alternatives 2 and 3 have large PAPR/CM compared to contiguous sequences. In these cases, PAPR reduction techniques have been applied on the repetitions of ZC sequences. For case 2, phase ramp of π radian per subcarrier is applied to the second ZC sequence. For case 3, random scrambling is applied on top of the repetitions of the ZC sequences. Details of the PAPR/CM reduction study is provided in Appendix A.3.

The PRACH sequences are detected by correlating the received tones with candidate sequences and thus obtaining a set of raw frequency domain channel estimates. We perform IFFT on this raw channel estimate vector and sum the power delay profile across receive antennas and frequency repetitions (if any) and OFDM symbols. We then find the correct timing from the peak location in the power delay profile. One particular PRACH sequence is declared to be detected if its correlation energy is greater than a pre-computed threshold and the timing is within the maximum value (i.e., 50% of normal CP length).  The simulation results are generated using TDL-C channel having delay scaling of 100ns, using 64 PRACH sequences of format A1 without any interference and SCS of 30 kHz and 15 kHz.

We compare the maximum coupling loss (MCL) metric for all the frequency allocation schemes using SCS = 30 kHz in Table 4. Note that, the P_TX computation as per the agreement (min(P_max, 23-Backoff)) is pessimistic as it does not allow even the fullband ZC sequence transmission to achieve 23 dBm transmit power. Hence we also present P_TX and MCL numbers with an updated formula min(P_max, 23+X-Backoff)), X =2.

Table 4. MCL values for different frequency allocation schemes for SCS=30kHz

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scheme | Rel 15, ZC-139 | ZC-139x2 | ZC-139x4 | ZC-571 | ZC-283 |
| SCS | 30 | 30 | 30 | 30 | 30 |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 571 | 283 |
| # of repetition (R) | 1 | 2 | 4 | 1 | 1 |
| N_cs | 17 | 17 | 17 | 41 | 34 |
| # of RBs used for one RO (N_RB) | 12 | 24 | 48 | 48 | 24 |
| # of interlaces used by one RO (N_interlace) | N/A | N/A | N/A | N/A | N/A |
| RACH frequency occupancy (MHz) | 4.17 | 8.34 | 16.68 | 17.13 | 8.49 |
| Noise level, Np (dBm) | -102.8 | -99.8 | -96.8 | -96.66 | -99.7 |
| SNR (dB) | -4.81 | -8.69 | -12.52 | -11.89 | -8.68 |
| P_max (dBm) | 16.2 | 19.2 | 22.2 | 22.33 | 19.28 |
| Backoff (dB) | 2.34 | 3.04 | 4.27 | 2.34 | 2.34 |
| P_TX (dBm) | 16.2 | 19.2 | 18.73 | 20.66 | 19.28 |
| P_TX with updated formula | 16.2 | 19.2 | 20.73 | 22.33 | 19.28 |
| MCL (dB) | 123.8 | 127.69 | 128.05 | 129.21 | 127.66 |
| MCL with updated formula | 123.8 | 127.69 | 130.05 | 130.88 | 127.66 |
| N_FDM | 4 | 2 | 1 | 1 | 2 |
| Capacity | 4416 | 2208 | 1104 | 7410 | 4512 |

The MCL values for the five frequency allocation schemes using SCS =15 kHz are shown in Table 5.

Table 5. MCL values for different frequency allocation schemes for SCS=15 kHz

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scheme | Rel 15, ZC-139 | ZC-139x4 | ZC-139x8 | ZC-1151 | ZC-571 |
| SCS | 15 | 15 | 15 | 15 | 15 |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 1151 | 571 |
| # of repetition (R) | 1 | 4 | 4 | 1 | 1 |
| N_cs | 17 | 17 | 17 | 50 | 46 |
| # of RBs used for one RO (N_RB) | 12 | 24 | 96 | 96 | 48 |
| # of interlaces used by one RO (N_interlace) | N/A | N/A | N/A | N/A | N/A |
| RACH frequency occupancy (MHz) | 2.085 | 8.34 | 16.68 | 17.265 | 8.565 |
| Noise level, Np (dBm) | -105.81 | -99.8 | -96.79 | -96.63 | -99.67 |
| SNR (dB) | -4.63 | -11.48 | -14.86 | -14.97 | -11.65 |
| P_max (dBm) | 13.19 | 19.2 | 22.22 | 22.37 | 19.32 |
| Backoff (dB) | 2.34 | 4.29 | 4.29 | 2.34 | 2.34 |
| P_TX (dBm) | 13.19 | 18.71 | 18.71 | 20.66 | 19.32 |
| P_TX with updated formula | 13.19 | 20.71 | 20.71 | 22.37 | 19.32 |
| MCL (dB) | 123.63 | 129.99 | 130.36 | 132.26 | 130.64 |
| MCL with updated formula | 123.63 | 131.99 | 132.36 | 133.97 | 130.64 |
| N_FDM | 8 | 2 | 1 | 1 | 2 |
| Capacity | 8832 | 2208 | 1104 | 26473 | 13680 |

From the simulation results, we make the following observations:

Observation 8: Long ZC sequence with contiguous allocation has the maximum capacity for both SCS =15 and 30 kHz.

Observation 9: Long ZC sequence with contiguous allocation have lower CM than repeated 139 length ZC sequences even with the application of CM reduction methods.

Observation 10: Long ZC sequence with full band contiguous allocation with L_RA=1151 and 571 have better MCL than long ZC sequence with half band contiguous allocation L_RA=571 and 283, respectively for 15/30 kHz SCS.

Observation 11: Long ZC sequence with contiguous allocation provides the best MCL value for both SCS =15 kHz and 30 kHz. Note that, the P_TX computation is pessimistic. If the P_TX formula is updated as min(P_max, 23+X-Backoff)), X =2, then Alt4 (long continuous allocation) has even better MCL gains.

Considering the MCL/capacity trade-off it can be observed that the wideband long sequence is the most preferred option.

For long ZC sequences the root sequences are chosen similar to Rel. 15 139 length ZC sequence. We consider root pairs in the form of (i, L_RA-i). The exact mapping from logical index i to sequence number u for preamble formats is given as following:

ui=yi,  i=1, 2, …LRA-2

y2k=k+1,

y2k+1=LRA-k-1.k=0,1,2,…,⌊LRA/2⌋

The whole tables for L_RA=571 and 1151 are presented in Appendix A.1.

The NCS values for both for L_RA=571 and 1151 are also computed by scaling the NCS values from 38.211 Section 6.3.3. [14] assuming the same maximum cell radius. The NCS values are presented in Appendix A.1.

Proposal 12: The root sequences for long ZC sequence are chosen as 1, L_RA-1, 2, L_RA-2, ….

Proposal 13: NCS values are chosen as follows:

- 15 kHz: NCS∈ {0, 17, 33, 50, 66, 83, 99, 108, 124, 141, 157, 190, 224, 282, 381, 571},

- 30 kHz: NCS∈ {0, 8,  16, 25, 33, 41, 49, 53, 62, 70, 78, 94, 111, 140, 189, 283}.

## 3.4 Virtual PRACH occasions

If long ZC sequences occupying full band are used for PRACH, different RACH occasions (RO’s) corresponding to different beams/SSB index cannot be frequency domain multiplexed (FDM’d). This this may increase the number of resources used for different ROs as they have to be TDMed. One efficient solution to solve this problem is to define a Virtual Rach Occasion (VRO). Virtual Rach Occasion (VRO) is a set of long PRACH sequences which occupy a certain time-frequency block. Since with long ZC sequences, the number of available orthogonal sequences are also increased, one may assign multiple sets of 64 sequences in each cell. Note that, using full band contiguous sequences increases the PRACH capacity significantly as seen in Table 4 and Table 5. Each such set of 64 sequences then becomes a VRO. The number of VROs can be kept same as the number of FDM’d ROs before using 139 length sequence. Thus, there are no specification changes compared to NR Rel. 15. For example, in a 20 MHz BW using 30 kHz SCS, 4 ROs can be FDM’d in one symbol using 139 length PRACH sequence. Instead we define 4 VROs, i.e. 4 sets each containing 64 sequences of length 571.

Each VRO will have a different set of root indices configured by the gNB, which may or may not be exclusive. The 0th VRO will have a starting root index configured. Root index is incremented after all cyclic shifts are exhausted. This process is continued till 64 sequences corresponding to 0th VRO are selected. For the 1st VRO, one can start from the next cyclic shift of the same root index or move to the next root index and start populating PRACH sequences. The VRO to RO mapping is shown in Figure 7.

Figure 7. Virtual Rach Occasions

Proposal 14: SSB to RO mappings with frequency domain multiplexing are replaced by SSB to VRO mappings, where VROs are a set of 64 long sequences.

Each VRO will have different set of root indices configured by gNB

### Specification impact for supporting Virtual Rach Occasion

From 38.213 Section 8.1 [15],

“A UE is provided a number N of SS/PBCH blocks associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. If N<1, one SS/PBCH block is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles and is an integer multiple of N. “

By default, totalNumberOfRA-Preambles is set to 64. In NR-U, for using L_RA=1151 and L_RA=571 for 15 and 30 kHz SCS, totalNumberOfRA-Preambles has to be set to 512 and 256 resprectively. The specification language could be modified as:

“A UE is provided a number N of SS/PBCH blocks associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. If N<64/Npreambletotal, one SS/PBCH block is mapped to 64/N.Npreambletotalconsecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block per valid PRACH occasion start from preamble index 0. If 1>N≥64/Npreambletotal, R contention based preambles per SS/PBCH block per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles and is an integer multiple of N “

Thus, the wideband ZC sequence with virtual Rach occasions can be supported with minimal changes in specification.

## 3.5 Advantages of full-band ZC with VRO over half-band ZC sequence

In RAN1 #98, two long ZC sequence options were discussed: full-band and half-band. The advantages of using full-band ZC sequence is as follows:

- Full band ZC sequence (L_RA= 1151/571 for 15/30 kHz respectively) has better MCL compared to half-band ZC sequence (L_RA= 571/283 for 15/30 kHz respectively) as observed in section 3.3.

- It is easier to support only one length for PRACH sequence in specification compared to supporting sequences of two lengths

- Long ZC with virtual Rach Occasion can provide the same FDM as Rel. 15 139 length sequence, which is greater than FDMs possible with half-band ZC sequence.

Hence, we observe that supporting half-band sequences along with full-band sequence increases specification effort without giving any MCL/capacity benefits.

Hence, we make the following proposal:

Proposal 15: Long ZC sequence with contiguous frequency allocation should be used for NRU PRACH. Following ZC sequence lengths are supported:

- For 15 kHz: L_RA=1151

- For 30 kHz: L_RA=571.

# Summary

In this section, we summarize the initial access signals and channels for NR-U in sub-7Ghz band along with the performance evaluation. We have the following observations and proposals:

Observation 1. Legacy Type0-PDCCH time domain monitoring configuration has multiple options to support both DRS configurations where RMSI PDSCH shares the same slot as SSB, as well as DRS configurations where RMSI PDSCH uses a different slot than SSB

Proposal 2. RMSI PDSCH rate matching around SSB is not supported in NR-U.

Error! Reference source not found.Proposal 3. 30KHz SCS CORESET #0 location within each 20MHz LBT bandwidth is fixed as in Table 1 (3rd column), and can be captured by a per 20MHz LBT bandwidth RB level offset from SSB on sync raster to coreset #0.

Proposal 4. For 15KHz SCS, CORESET #0 location can overlap with 30KHz CORESET #0 defined (align at the center of the lowest RE).

Proposal 5. No further enhancement of common PDSCH rate matching around CSI-RS is needed in NR-U Rel-16. Further optimisations can be considered beyond Rel-16 NR-U

Proposal 6: Introduce gaps between consecutive TDM-ed RACH occasions for UL LBT operation.

- The size of gaps shall match the maximum expected UL LBT duration (e.g. at least larger than that obtained by assuming the largest contention window and no other interferers)

- Gaps should be in units of symbols for easier alignment

- Minimum gap required is 1 or 2 symbols for 15 or 30 kHz SCS respectively

- Gaps can be introduced in the following way:

  - Use only the even Rach Occasions within a slot

Proposal 7: Only 15 and 30 kHz SCS will be supported for NR-U PRACH.

Observation 8: Long ZC sequence with contiguous allocation has the maximum capacity for both SCS =15 and 30 kHz.

Observation 9: Long ZC sequence with contiguous allocation have lower CM than repeated 139 length ZC sequences even with the application of CM reduction methods.

Observation 10: Long ZC sequence with full band contiguous allocation with L_RA=1151 and 571 have better MCL than long ZC sequence with half band contiguous allocation L_RA=571 and 283, respectively for 15/30 kHz SCS.

Observation 11: Long ZC sequence with contiguous allocation provides the best MCL value for both SCS =15 kHz and 30 kHz. Note that, the P_TX computation is pessimistic. If the P_TX formula is updated as min(P_max, 23+X-Backoff)), X =2, then Alt4 (long continuous allocation) has even better MCL gains.

Proposal 12: The root sequences for long ZC sequence are chosen as 1, L_RA-1, 2, L_RA-2, ….

Proposal 13: NCS values are chosen as follows:

- 15 kHz: NCS∈ {0, 17, 33, 50, 66, 83, 99, 108, 124, 141, 157, 190, 224, 282, 381, 571},

- 30 kHz: NCS∈ {0, 8,  16, 25, 33, 41, 49, 53, 62, 70, 78, 94, 111, 140, 189, 283}.

Proposal 14: SSB to RO mappings with frequency domain multiplexing are replaced by SSB to VRO mappings, where VROs are a set of 64 long sequences.

Each VRO will have different set of root indices configured by gNB

Proposal 15: Long ZC sequence with contiguous frequency allocation should be used for NRU PRACH. Following ZC sequence lengths are supported:

- For 15 kHz: L_RA=1151

- For 30 kHz: L_RA=571.

# References

- RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#82, Sorrento, Italy, Dec. 10-13, 2018

- R1-1813457, “On UL signals and channels”, Ericsson, RAN1#95

- R1-1812915, “On UL signals and channels for NR-U”, Apple, RAN1#95

- R1-1808060, “UL PHY channels for NR unlicensed”, Huawei, HiSilicon, RAN1#94

- Draft Report of 3GPP TSG RAN WG1 #95 v0.2.0, MCC

- R1-1901419, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, AH-1901, Taipei, Taiwan Jan. 21st – Jan. 25th, 2019

- R1-1901308, “Feature lead summary of UL Signals and Channels”, Ericsson, AH-1901, Taipei, Taiwan, 21st – 25th January 2019

- R1-1903629, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1#96, Athens, Greece, February 25th – March 1st, 2019

- R1-1903404, “Feature lead summary for initial access signals and channels for NR-U”, Qualcomm, RAN1#96, Athens, Greece, February 25th – March 1st, 2019

- R1-1905801, “Chairman's notes of AI 7.2.2 Study on NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1#96bis, Xi’an, China, 8th – 12th April 2019

- R1-1905634, “Feature lead summery on initial access signals and channels for NR-U”, Qualcomm, RAN1 #96 Bis, Xian, China, April 8th- April 12th, 2019

- R1-1907846, “Chairman's notes of AI 7.2.2 Study on NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1#97, Reno, USA, 13th – 17th, May 2019

- R1-1909576, “Feature lead summary on initial access signals and channels for NR-U”, Qualcomm, RAN1 #98, Prague, CZ, 26th -30th Aug, 2019

- 3GPP TS 38.211, “NR; Physical channels and modulation”.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”

- RAN4 #92 Meeting report, Chongqing, China, 14th – 18th Oct 2019

- 3GPP 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v15.4.0.

- R1-1909241, “Other considerations for two-step RACH”, Qualcomm, RAN1 #98, Prague, CZ, 26th -30th Aug, 2019

- 3GPP 38.104, “NR; Base Station (BS) radio transmission and reception”, v16.0.0.

- R1-1911511, “Chairman’s Notes of AI 7.2.2 Study on NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1#98bis, Xi’an, China, 14th – 20th October 2019

- R4-1912982, “WF for sync raster for NR-U”, Qualcomm, RAN4#93, Reno, US, 18th – 22th Nov 2019

# Appendix

A.1 Specification changes for long wideband PRACH

Table 6 NCS for preamble formats with SCS= 15, 30 kHz

| zeroCorrelationZoneConfig | NCS value for SCS=15 kHz | NCS value for SCS=30 kHz |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 17 | 8 |
| 2 | 33 | 16 |
| 3 | 50 | 25 |
| 4 | 66 | 33 |
| 5 | 83 | 41 |
| 6 | 99 | 49 |
| 7 | 108 | 53 |
| 8 | 124 | 62 |
| 9 | 141 | 70 |
| 10 | 157 | 78 |
| 11 | 190 | 94 |
| 12 | 224 | 111 |
| 13 | 282 | 140 |
| 14 | 381 | 189 |
| 15 | 571 | 283 |

Table 7 Root index table for 30 kHz SCS, L_RA=571

| i | Sequence number u in increasing order of i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-19 | 1 | 570 | 2 | 569 | 3 | 568 | 4 | 567 | 5 | 566 | 6 | 565 | 7 | 564 | 8 | 563 | 9 | 562 | 10 | 561 |
| 20-39 | 11 | 560 | 12 | 559 | 13 | 558 | 14 | 557 | 15 | 556 | 16 | 555 | 17 | 554 | 18 | 553 | 19 | 552 | 20 | 551 |
| 40-59 | 21 | 550 | 22 | 549 | 23 | 548 | 24 | 547 | 25 | 546 | 26 | 545 | 27 | 544 | 28 | 543 | 29 | 542 | 30 | 541 |
| 60-79 | 31 | 540 | 32 | 539 | 33 | 538 | 34 | 537 | 35 | 536 | 36 | 535 | 37 | 534 | 38 | 533 | 39 | 532 | 40 | 531 |
| 80-99 | 41 | 530 | 42 | 529 | 43 | 528 | 44 | 527 | 45 | 526 | 46 | 525 | 47 | 524 | 48 | 523 | 49 | 522 | 50 | 521 |
| 100-119 | 51 | 520 | 52 | 519 | 53 | 518 | 54 | 517 | 55 | 516 | 56 | 515 | 57 | 514 | 58 | 513 | 59 | 512 | 60 | 511 |
| 120-139 | 61 | 510 | 62 | 509 | 63 | 508 | 64 | 507 | 65 | 506 | 66 | 505 | 67 | 504 | 68 | 503 | 69 | 502 | 70 | 501 |
| 140-159 | 71 | 500 | 72 | 499 | 73 | 498 | 74 | 497 | 75 | 496 | 76 | 495 | 77 | 494 | 78 | 493 | 79 | 492 | 80 | 491 |
| 160-179 | 81 | 490 | 82 | 489 | 83 | 488 | 84 | 487 | 85 | 486 | 86 | 485 | 87 | 484 | 88 | 483 | 89 | 482 | 90 | 481 |
| 180-199 | 91 | 480 | 92 | 479 | 93 | 478 | 94 | 477 | 95 | 476 | 96 | 475 | 97 | 474 | 98 | 473 | 99 | 472 | 100 | 471 |
| 200-219 | 101 | 470 | 102 | 469 | 103 | 468 | 104 | 467 | 105 | 466 | 106 | 465 | 107 | 464 | 108 | 463 | 109 | 462 | 110 | 461 |
| 220-239 | 111 | 460 | 112 | 459 | 113 | 458 | 114 | 457 | 115 | 456 | 116 | 455 | 117 | 454 | 118 | 453 | 119 | 452 | 120 | 451 |
| 240-259 | 121 | 450 | 122 | 449 | 123 | 448 | 124 | 447 | 125 | 446 | 126 | 445 | 127 | 444 | 128 | 443 | 129 | 442 | 130 | 441 |
| 260-279 | 131 | 440 | 132 | 439 | 133 | 438 | 134 | 437 | 135 | 436 | 136 | 435 | 137 | 434 | 138 | 433 | 139 | 432 | 140 | 431 |
| 280-299 | 141 | 430 | 142 | 429 | 143 | 428 | 144 | 427 | 145 | 426 | 146 | 425 | 147 | 424 | 148 | 423 | 149 | 422 | 150 | 421 |
| 300-319 | 151 | 420 | 152 | 419 | 153 | 418 | 154 | 417 | 155 | 416 | 156 | 415 | 157 | 414 | 158 | 413 | 159 | 412 | 160 | 411 |
| 320-339 | 161 | 410 | 162 | 409 | 163 | 408 | 164 | 407 | 165 | 406 | 166 | 405 | 167 | 404 | 168 | 403 | 169 | 402 | 170 | 401 |
| 340-359 | 171 | 400 | 172 | 399 | 173 | 398 | 174 | 397 | 175 | 396 | 176 | 395 | 177 | 394 | 178 | 393 | 179 | 392 | 180 | 391 |
| 360-379 | 181 | 390 | 182 | 389 | 183 | 388 | 184 | 387 | 185 | 386 | 186 | 385 | 187 | 384 | 188 | 383 | 189 | 382 | 190 | 381 |
| 380-399 | 191 | 380 | 192 | 379 | 193 | 378 | 194 | 377 | 195 | 376 | 196 | 375 | 197 | 374 | 198 | 373 | 199 | 372 | 200 | 371 |
| 400-419 | 201 | 370 | 202 | 369 | 203 | 368 | 204 | 367 | 205 | 366 | 206 | 365 | 207 | 364 | 208 | 363 | 209 | 362 | 210 | 361 |
| 420-439 | 211 | 360 | 212 | 359 | 213 | 358 | 214 | 357 | 215 | 356 | 216 | 355 | 217 | 354 | 218 | 353 | 219 | 352 | 220 | 351 |
| 440-459 | 221 | 350 | 222 | 349 | 223 | 348 | 224 | 347 | 225 | 346 | 226 | 345 | 227 | 344 | 228 | 343 | 229 | 342 | 230 | 341 |
| 460-479 | 231 | 340 | 232 | 339 | 233 | 338 | 234 | 337 | 235 | 336 | 236 | 335 | 237 | 334 | 238 | 333 | 239 | 332 | 240 | 331 |
| 480-499 | 241 | 330 | 242 | 329 | 243 | 328 | 244 | 327 | 245 | 326 | 246 | 325 | 247 | 324 | 248 | 323 | 249 | 322 | 250 | 321 |
| 500-519 | 251 | 320 | 252 | 319 | 253 | 318 | 254 | 317 | 255 | 316 | 256 | 315 | 257 | 314 | 258 | 313 | 259 | 312 | 260 | 311 |
| 520-539 | 261 | 310 | 262 | 309 | 263 | 308 | 264 | 307 | 265 | 306 | 266 | 305 | 267 | 304 | 268 | 303 | 269 | 302 | 270 | 301 |
| 540-559 | 271 | 300 | 272 | 299 | 273 | 298 | 274 | 297 | 275 | 296 | 276 | 295 | 277 | 294 | 278 | 293 | 279 | 292 | 280 | 291 |
| 560-569 | 281 | 290 | 282 | 289 | 283 | 288 | 284 | 287 | 285 | 286 | - | - | - | - | - | - | - | - | - | - |

Table 8 Root index table for 15 kHz SCS, L_RA=1151

| i | Sequence number u in increasing order of i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-19 | 1 | 1150 | 2 | 1149 | 3 | 1148 | 4 | 1147 | 5 | 1146 | 6 | 1145 | 7 | 1144 | 8 | 1143 | 9 | 1142 | 10 | 1141 |
| 20-39 | 11 | 1140 | 12 | 1139 | 13 | 1138 | 14 | 1137 | 15 | 1136 | 16 | 1135 | 17 | 1134 | 18 | 1133 | 19 | 1132 | 20 | 1131 |
| 40-59 | 21 | 1130 | 22 | 1129 | 23 | 1128 | 24 | 1127 | 25 | 1126 | 26 | 1125 | 27 | 1124 | 28 | 1123 | 29 | 1122 | 30 | 1121 |
| 60-79 | 31 | 1120 | 32 | 1119 | 33 | 1118 | 34 | 1117 | 35 | 1116 | 36 | 1115 | 37 | 1114 | 38 | 1113 | 39 | 1112 | 40 | 1111 |
| 80-99 | 41 | 1110 | 42 | 1109 | 43 | 1108 | 44 | 1107 | 45 | 1106 | 46 | 1105 | 47 | 1104 | 48 | 1103 | 49 | 1102 | 50 | 1101 |
| 100-119 | 51 | 1100 | 52 | 1099 | 53 | 1098 | 54 | 1097 | 55 | 1096 | 56 | 1095 | 57 | 1094 | 58 | 1093 | 59 | 1092 | 60 | 1091 |
| 120-139 | 61 | 1090 | 62 | 1089 | 63 | 1088 | 64 | 1087 | 65 | 1086 | 66 | 1085 | 67 | 1084 | 68 | 1083 | 69 | 1082 | 70 | 1081 |
| 140-159 | 71 | 1080 | 72 | 1079 | 73 | 1078 | 74 | 1077 | 75 | 1076 | 76 | 1075 | 77 | 1074 | 78 | 1073 | 79 | 1072 | 80 | 1071 |
| 160-179 | 81 | 1070 | 82 | 1069 | 83 | 1068 | 84 | 1067 | 85 | 1066 | 86 | 1065 | 87 | 1064 | 88 | 1063 | 89 | 1062 | 90 | 1061 |
| 180-199 | 91 | 1060 | 92 | 1059 | 93 | 1058 | 94 | 1057 | 95 | 1056 | 96 | 1055 | 97 | 1054 | 98 | 1053 | 99 | 1052 | 100 | 1051 |
| 200-219 | 101 | 1050 | 102 | 1049 | 103 | 1048 | 104 | 1047 | 105 | 1046 | 106 | 1045 | 107 | 1044 | 108 | 1043 | 109 | 1042 | 110 | 1041 |
| 220-239 | 111 | 1040 | 112 | 1039 | 113 | 1038 | 114 | 1037 | 115 | 1036 | 116 | 1035 | 117 | 1034 | 118 | 1033 | 119 | 1032 | 120 | 1031 |
| 240-259 | 121 | 1030 | 122 | 1029 | 123 | 1028 | 124 | 1027 | 125 | 1026 | 126 | 1025 | 127 | 1024 | 128 | 1023 | 129 | 1022 | 130 | 1021 |
| 260-279 | 131 | 1020 | 132 | 1019 | 133 | 1018 | 134 | 1017 | 135 | 1016 | 136 | 1015 | 137 | 1014 | 138 | 1013 | 139 | 1012 | 140 | 1011 |
| 280-299 | 141 | 1010 | 142 | 1009 | 143 | 1008 | 144 | 1007 | 145 | 1006 | 146 | 1005 | 147 | 1004 | 148 | 1003 | 149 | 1002 | 150 | 1001 |
| 300-319 | 151 | 1000 | 152 | 999 | 153 | 998 | 154 | 997 | 155 | 996 | 156 | 995 | 157 | 994 | 158 | 993 | 159 | 992 | 160 | 991 |
| 320-339 | 161 | 990 | 162 | 989 | 163 | 988 | 164 | 987 | 165 | 986 | 166 | 985 | 167 | 984 | 168 | 983 | 169 | 982 | 170 | 981 |
| 340-359 | 171 | 980 | 172 | 979 | 173 | 978 | 174 | 977 | 175 | 976 | 176 | 975 | 177 | 974 | 178 | 973 | 179 | 972 | 180 | 971 |
| 360-379 | 181 | 970 | 182 | 969 | 183 | 968 | 184 | 967 | 185 | 966 | 186 | 965 | 187 | 964 | 188 | 963 | 189 | 962 | 190 | 961 |
| 380-399 | 191 | 960 | 192 | 959 | 193 | 958 | 194 | 957 | 195 | 956 | 196 | 955 | 197 | 954 | 198 | 953 | 199 | 952 | 200 | 951 |
| 400-419 | 201 | 950 | 202 | 949 | 203 | 948 | 204 | 947 | 205 | 946 | 206 | 945 | 207 | 944 | 208 | 943 | 209 | 942 | 210 | 941 |
| 420-439 | 211 | 940 | 212 | 939 | 213 | 938 | 214 | 937 | 215 | 936 | 216 | 935 | 217 | 934 | 218 | 933 | 219 | 932 | 220 | 931 |
| 440-459 | 221 | 930 | 222 | 929 | 223 | 928 | 224 | 927 | 225 | 926 | 226 | 925 | 227 | 924 | 228 | 923 | 229 | 922 | 230 | 921 |
| 460-479 | 231 | 920 | 232 | 919 | 233 | 918 | 234 | 917 | 235 | 916 | 236 | 915 | 237 | 914 | 238 | 913 | 239 | 912 | 240 | 911 |
| 480-499 | 241 | 910 | 242 | 909 | 243 | 908 | 244 | 907 | 245 | 906 | 246 | 905 | 247 | 904 | 248 | 903 | 249 | 902 | 250 | 901 |
| 500-519 | 251 | 900 | 252 | 899 | 253 | 898 | 254 | 897 | 255 | 896 | 256 | 895 | 257 | 894 | 258 | 893 | 259 | 892 | 260 | 891 |
| 520-539 | 261 | 890 | 262 | 889 | 263 | 888 | 264 | 887 | 265 | 886 | 266 | 885 | 267 | 884 | 268 | 883 | 269 | 882 | 270 | 881 |
| 540-559 | 271 | 880 | 272 | 879 | 273 | 878 | 274 | 877 | 275 | 876 | 276 | 875 | 277 | 874 | 278 | 873 | 279 | 872 | 280 | 871 |
| 560-579 | 281 | 870 | 282 | 869 | 283 | 868 | 284 | 867 | 285 | 866 | 286 | 865 | 287 | 864 | 288 | 863 | 289 | 862 | 290 | 861 |
| 580-599 | 291 | 860 | 292 | 859 | 293 | 858 | 294 | 857 | 295 | 856 | 296 | 855 | 297 | 854 | 298 | 853 | 299 | 852 | 300 | 851 |
| 600-619 | 301 | 850 | 302 | 849 | 303 | 848 | 304 | 847 | 305 | 846 | 306 | 845 | 307 | 844 | 308 | 843 | 309 | 842 | 310 | 841 |
| 620-639 | 311 | 840 | 312 | 839 | 313 | 838 | 314 | 837 | 315 | 836 | 316 | 835 | 317 | 834 | 318 | 833 | 319 | 832 | 320 | 831 |
| 640-659 | 321 | 830 | 322 | 829 | 323 | 828 | 324 | 827 | 325 | 826 | 326 | 825 | 327 | 824 | 328 | 823 | 329 | 822 | 330 | 821 |
| 660-679 | 331 | 820 | 332 | 819 | 333 | 818 | 334 | 817 | 335 | 816 | 336 | 815 | 337 | 814 | 338 | 813 | 339 | 812 | 340 | 811 |
| 680-699 | 341 | 810 | 342 | 809 | 343 | 808 | 344 | 807 | 345 | 806 | 346 | 805 | 347 | 804 | 348 | 803 | 349 | 802 | 350 | 801 |
| 700-719 | 351 | 800 | 352 | 799 | 353 | 798 | 354 | 797 | 355 | 796 | 356 | 795 | 357 | 794 | 358 | 793 | 359 | 792 | 360 | 791 |
| 720-739 | 361 | 790 | 362 | 789 | 363 | 788 | 364 | 787 | 365 | 786 | 366 | 785 | 367 | 784 | 368 | 783 | 369 | 782 | 370 | 781 |
| 740-759 | 371 | 780 | 372 | 779 | 373 | 778 | 374 | 777 | 375 | 776 | 376 | 775 | 377 | 774 | 378 | 773 | 379 | 772 | 380 | 771 |
| 760-779 | 381 | 770 | 382 | 769 | 383 | 768 | 384 | 767 | 385 | 766 | 386 | 765 | 387 | 764 | 388 | 763 | 389 | 762 | 390 | 761 |
| 780-799 | 391 | 760 | 392 | 759 | 393 | 758 | 394 | 757 | 395 | 756 | 396 | 755 | 397 | 754 | 398 | 753 | 399 | 752 | 400 | 751 |
| 800-819 | 401 | 750 | 402 | 749 | 403 | 748 | 404 | 747 | 405 | 746 | 406 | 745 | 407 | 744 | 408 | 743 | 409 | 742 | 410 | 741 |
| 820-839 | 411 | 740 | 412 | 739 | 413 | 738 | 414 | 737 | 415 | 736 | 416 | 735 | 417 | 734 | 418 | 733 | 419 | 732 | 420 | 731 |
| 840-859 | 421 | 730 | 422 | 729 | 423 | 728 | 424 | 727 | 425 | 726 | 426 | 725 | 427 | 724 | 428 | 723 | 429 | 722 | 430 | 721 |
| 860-879 | 431 | 720 | 432 | 719 | 433 | 718 | 434 | 717 | 435 | 716 | 436 | 715 | 437 | 714 | 438 | 713 | 439 | 712 | 440 | 711 |
| 880-899 | 441 | 710 | 442 | 709 | 443 | 708 | 444 | 707 | 445 | 706 | 446 | 705 | 447 | 704 | 448 | 703 | 449 | 702 | 450 | 701 |
| 900-919 | 451 | 700 | 452 | 699 | 453 | 698 | 454 | 697 | 455 | 696 | 456 | 695 | 457 | 694 | 458 | 693 | 459 | 692 | 460 | 691 |
| 920-939 | 461 | 690 | 462 | 689 | 463 | 688 | 464 | 687 | 465 | 686 | 466 | 685 | 467 | 684 | 468 | 683 | 469 | 682 | 470 | 681 |
| 940-959 | 471 | 680 | 472 | 679 | 473 | 678 | 474 | 677 | 475 | 676 | 476 | 675 | 477 | 674 | 478 | 673 | 479 | 672 | 480 | 671 |
| 960-979 | 481 | 670 | 482 | 669 | 483 | 668 | 484 | 667 | 485 | 666 | 486 | 665 | 487 | 664 | 488 | 663 | 489 | 662 | 490 | 661 |
| 980-999 | 491 | 660 | 492 | 659 | 493 | 658 | 494 | 657 | 495 | 656 | 496 | 655 | 497 | 654 | 498 | 653 | 499 | 652 | 500 | 651 |
| 1000-1019 | 501 | 650 | 502 | 649 | 503 | 648 | 504 | 647 | 505 | 646 | 506 | 645 | 507 | 644 | 508 | 643 | 509 | 642 | 510 | 641 |
| 1020-1039 | 511 | 640 | 512 | 639 | 513 | 638 | 514 | 637 | 515 | 636 | 516 | 635 | 517 | 634 | 518 | 633 | 519 | 632 | 520 | 631 |
| 1040-1059 | 521 | 630 | 522 | 629 | 523 | 628 | 524 | 627 | 525 | 626 | 526 | 625 | 527 | 624 | 528 | 623 | 529 | 622 | 530 | 621 |
| 1060-1079 | 531 | 620 | 532 | 619 | 533 | 618 | 534 | 617 | 535 | 616 | 536 | 615 | 537 | 614 | 538 | 613 | 539 | 612 | 540 | 611 |
| 1080-1099 | 541 | 610 | 542 | 609 | 543 | 608 | 544 | 607 | 545 | 606 | 546 | 605 | 547 | 604 | 548 | 603 | 549 | 602 | 550 | 601 |
| 1100-1119 | 551 | 600 | 552 | 599 | 553 | 598 | 554 | 597 | 555 | 596 | 556 | 595 | 557 | 594 | 558 | 593 | 559 | 592 | 560 | 591 |
| 1120-1139 | 561 | 590 | 562 | 589 | 563 | 588 | 564 | 587 | 565 | 586 | 566 | 585 | 567 | 584 | 568 | 583 | 569 | 582 | 570 | 581 |
| 1140-1149 | 571 | 580 | 572 | 579 | 573 | 578 | 574 | 577 | 575 | 576 | - | - | - | - | - | - | - | - | - | - |

A.2 PAPR/CM reduction

The PAPR/CM in the PRACH frequency allocation schemes with repetitions can be reduced by using several methods like scrambling, phase ramp, using different PRACH sequences etc. The 95% CCDF values of CM with different methods using 30 kHz SCS are compared in Table 9. Note that the corresponding value for a Rel. 15 ZC 139 length sequence is 2.34 dB.

Table 9. CM comparison for 30 kHz SCS

| Methods | ZC 139x2, 2 repetitions | ZC 139x4, 4 repetitions |
| --- | --- | --- |
| No PAPR reduction | 4.88 | 8.48 |
| Random scrambling | 3.95 | 4.27 |
| π phase ramp | 3.04 | 6.32 |
| π/2 phase ramp | 3.62 | 5.53 |
| π/4 phase ramp | 3.48 | 4.81 |