3GPP TSG RAN WG1 Meeting #99	                                                                          R1-1912941

Reno, USA

Nov. 18 – Nov. 22, 2019

Agenda item:	7.2.2.2.4

Source:	Qualcomm Incorporated

Title:	Enhancement to configured grants in NR unlicensed

Document for:	Discussion and Decision

# Introduction

In RAN#82 a new WI [1] for NR based access to unlicensed spectrum was approved following the closure of the corresponding SI [2,3]. The agreements and conclusions of the SI are captured in TR38.889 [4]. The objectives of the WI include specifying the following for physical layer procedures:

- Configured Grant operation: NR Type-1 and Type-2 configured grant mechanisms are the baseline for NR-U operation with modifications in line with agreements during the study phase (NR-U TR section 7.2.1.3.4). (RAN1)

In RAN#84 the following guidance was provided on prioritization of work items for configured grant operation in NR-U.

Essential

- DFI design, including content and CG PUSCH to DFI  timing

- UCI design, including content and multiplexing

Optimizations

- Ending symbol flexibility

- CBG based transmission with CG resource

In this contribution, we provide our views on the NR configured grant (CG) operation in unlicensed spectrum.

# Enhancements to configured grant for NR unlicensed

## 2.1	DFI

DFI provides the HARQ-ACK feedback information for all HARQ IDs. Since this information is useful for contention window update at UE independent of whether CG is configured or not, it needs to be clarified if the DFI can be enabled only when CG is enabled or can it be independently enabled, Furthermore, if it is only enabled when CG is enabled, can it be sent when CG transmissions are yet to be activated. To keep the design simple, we propose that CG-DFI be only considered valid when CG is enabled and activated.

Proposal : For type 1 configured grant, UE assumes DFI is only present when CG is configured.  For type 2 configured grant, UE assumes DFI is only present when CG is configured, and UE is in activated state for configured grant transmissions.

The following was agreed on timing between PUSCH and DFI where ACK feedback is considered valid.

- RRC configured minimum duration, D, from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH

  - Note: UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI.

  - FFS: the definition of minimum duration for the case of slot aggregation

On the FFS point above, we don’t see a need to treat PUSCH with slot aggregation differently from any other PUSCH.at least from the perspective of DFI including A/N information.

Proposal : For the case with PUSCH slot aggregation, UE assumes HARQ-ACK is valid and corresponds to all the repeated PUSCHs sent as part of the slot aggregated PUSCHs whose  transmission ends before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI and D is the RRC configured minimum duration.

Since the DFI may include feedback for a subset of the repeated PUSCH transmitted by the UE, UE should wait to receive feedback for all the repeated PUSCH for triggering retransmission based on reception of NACK in DFI.

Proposal : For PUSCH sent with slot aggregation, UE should not retransmit a PUSCH based on reception of NACK in DFI if the DFI only included valid ACK feedback for subset of the repeated PUSCHs transmitted by the UE

In FeLAA, the DFI was allowed to contain ACK for a particular transmission of an HARQ ID only once. If UE did not detect the DFI and gNB resends the DFI, it has to set the A/N feedback to NACK for that HARQ ID. This was to ensure situations such as that shown in Figure 1 do not occur.

Figure 1: Issue if ACK is allowed to be repeated without including NDI in the DFI across multiple DFI

However, DFI transmission by gNB may not be decodable at UE in many cases and having the ability to resend the DFI will be very beneficial to the overall system operation. One way to enable this without causing the issue depicted in  Figure 1 is to also include the NDI bit for each of the HARQ ID for which the HARQ feedback is being sent.

Proposal : DFI should also include an NDI bit for each HARQ ID.

To provide more opportunities for channel access, the DFI design should allow gNB to send DFI corresponding to one carrier on a different carrier. Also, since the useful DFI payload corresponding to one carrier may be small (e.g. only 16 bits for the main HARQ A/N feedback) but the DCI size might be much larger (based on agreement that PDCCH blind decoding is not increased due to DFI DCI size), some methods to make more efficient use of the space in the DCI should be considered. A simple method could be to include the HARQ A/N information of multiple carriers in the same DCI.

Proposal : DFI DCI includes information on carrier(s) for which feedback is included

Proposal : A single DFI DCI can include HARQ A/N feedback for more than one carrier

The RNTI to use for DFI is yet to be agreed on. We prefer using CS-RNTI and Format 0_0/0_1. We propose to use NDI=0 and LSB of RV =1 to distinguish the DFI from activation/deactivation DCI (which use NDI=0, HARQ_ID=0, RV=0, and for deactivation all 1 value for MCS and FDRA) and UL grant (which uses NDI=1) while maximizing number of DCI bits available for the DFI payload.

Proposal : DFI is sent using DCI Format 0_0/0_1 with CS-RNTI

Proposal : To distinguish DFI from CG activation/deactivation and UL grant, NDI is set to 0 and LSB of RV is set to 1

## 2.2	UCI Payload

The following was agreed for UCI payload

- CG-UCI should at least include the following information:

  - HARQ ID

  - NDI

  - RV

  - COT sharing information, FFS details

  - FFS: other information including UE ID

- When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

  - The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

  - FFS:

    - How the duration is signalled

    - Whether the UE should signal continued use of the COT for its own transmissions

    - LBT priority class

To signal the duration of the channel occupancy, UE may signal expected ending symbol as well as duration of the remaining COT available for gNB in units of number of symbols. We may limit the ending symbol to slot boundaries/symbol ending before slot boundaries – X us (for LBT gap / switching time etc). We can provide full flexibility to signal remaining duration of the COT (e.g. 7 bits to signal up to ~4.5ms for 30Khz SCS). However, to reduce the overhead, we could consider subsampling the set of values.

Proposal : CG-UCI includes information on

- Remaining CO duration from end of the PUSCH

  - This can be replaced with a single bit indicator to enable/disable COT sharing when COT sharing ED threshold is not configured

- Offset information indicating where the UE transmissions end

- RRC configures the max duration to report and the granularity (e..g symbol level, half slot level, slot level)

Priority class information is needed so that gNB only includes corresponding or higher priority class data in its transmission when sharing the UE acquired COT. Note that gNB may not detect all UE transmissions so it may not always be able to figure out the LBT priority class from the COT duration related signalling.

Proposal : CAPC is included as part of the COT sharing information in the CG-UCI when COT sharing ED threshold is configured

Configured grant transmissions may involve scheduling multiple UEs on overlapping resources including possibly with same DMRS sequence/resource. Including UE-ID in the UCI payload provides a robust and reliable way for gNB to determine which UE is transmitting in such cases. An alternate solution has been proposed that relies on using orthogonal DMRS sequences to different UEs. However, configuring multiple orthogonal DMRS sequences may result in higher DMRS overhead and may not always be feasible such as when also supporting higher rank transmissions on CG PUSCH resources, we hence prefer being able to configure inclusion of UE-ID in the CG-UCI.

Proposal : Inclusion of UE-ID in the CG-UCI in supported. The presence/absence of UE-ID in CG-UCI can be RRC configured.

## 2.3	UCI Multiplexing

UCI multiplexing when CG-UCI and HARQ-ACK feedback are required to be present on a CG-PUSCH is still FFS. In previous meetings the following options were discussed for this case

- Alt 1: Drop CG-PUSCH

- Alt 2: Drop CSI-part 2

- Alt 3: Drop CSI part 2 with CG-UCI indicating information on other included UCIs (e.g. the number of A/N bits that are included)

- Alt 4; Jointly code HARQ-ACK payload with CG-UCI

In scheduled PUSCH, the DAI that is included in UL grant can be used to confirm the number of A/N bits to multiplex in PUSCH. No such mechanism exists for CG-PUSCH. Hence errors in determining the number of A/N bits at UE lead to potential decoding failure of the CG-PUSCH. Alt 3 is the only alternative that resolves this issue. However, given that gNB can easily avoid such situations where HAQ-ACK feedback overlaps with CG-PUSCH and impact of delaying the CG-PUSCH transmission to next occasion is expected to be minor, we propose to keep the design simple by using Alt 1 above.

Proposal : If HARQ ACK feedback is scheduled on a configured grant resource, configured grant PUSCH is skipped on that resource.

## 2.4	Design details for use of CP extension

The following was agreed in RAN1#98bis:

The starting time offset applied by a UE at the beginning of a transmitted burst with a CG resource at the start of the transmission burst, is RRC configured and defined as the length of a CP extension of the first symbol that is located before the configured resource

- Regardless of SCS, the CP extension is up to 72 micro seconds with a granularity of 9 micro seconds

The CP extension duration can span multiple symbols. Not all UEs may be capable of performing such extensions. We hence make the following proposals:

Proposal : UE can indicate a capability to support CP extension/ amount of CP extension it can support

- UE is only expected to be configured with starting point offsets within its capability

- When UE picks the starting point offset randomly, it does so from among the offsets that are within its capability

Use of CP extension results in UE starting the PUSCH preparation for NR-U earlier than what it would do for NR. The timelines for including the UCI need to be relaxed taking the CP extension duration into account.

Proposal : UE is only required to include the UCI (such as A/N, CSI-part1, CSI-part2) in CG-PUSCH if the UCI is scheduled Np+ ceil(Toffset / Ts) symbols prior to the CG-PUSCH resource where Np is the previous capability of UE to include the UCI not considering any cyclic prefix extension

- UE should drop CG-PUSCH and transmit the UCI in case the UCI timeline of Np can be met but the new timeline cannot be met

- If only a single offset is configured Toffset = the configured offset

- If UE randomly selects the offset Toffset = max of all allowed offsets

## 2.5	Repetition Configuration

NR-U supports configuration of repetitions for CG transmissions to improve coverage. This feature should also be supported for NR-U for the same reasons. However, given that we include the HARQ ID, RV, and NDI in the CG-UCI, there is no need to restrict the repetitions to be on back to back slots. We can allow UE to choose where it performs the repetitions. For example, it may choose to cycle over all HARQ IDs before performing a repetition for a particular HARQ ID or it may choose to do all repetitions of a HARQ ID before moving to the next HARQ ID. The repetition feature implemented in such a way provides UE an alternative to retransmit a packet without receiving DFI and without waiting for CG retransmission timer expiry. This can also help to reduce the DFI overhead as gNB can get UE to retransmit UL transmissions that it wasn’t able to decode without sending a DFI.

Proposal : NR-U should continue to support configuration of repetitions for CG PUSCH. However, unlike NR where the repetitions of the same TB are on back to back slots, for NR-U, the UE should be given some flexibility on where it chooses to perform the repetitions.

## 2.6	Time domain resource allocation

The following agreements were made so far in relation to the resource allocation.

- For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

A few further details need to be agreed with respect to the above agreement

- As discussed in previous sub-section, NR configured grant already supports configuring a repK factor which refers to the number of repetitions of PUSCH on back to back slots. In NR-U we should make this more flexible and allow UE to choose where to perform the repetitions. We can consider two options when repetitions are also configured. Let N represent the number of slots configured through the new parameter.

  - Opt1: Number of allocated slots = N * repK

  - Opt2: Number of allocated slots = N

    - i.e. independent of repK

Our preference is to use option 2.

- The timeDomainAllocation field was used to derive the starting and ending positions within a slot. However, when we have multiple back to back slots, the timeDomainAllocation needs to be reinterpreted as starting symbol applying to the first slot and ending symbol applying to the last slot of the burst of slot. Furthermore, there is no need to have the end symbol > start symbol anymore.

- Regarding the FFS on mini-slot, we propose the following. UE can be configured with overlapping mini-slot and full slot configurations and UE can choose which configuration it wishes to use on any slot. To indicate the mini-slot configurations, a bitmap is provided spanning a single slot which provides the allowed starting points. The starting point of the next mini-slot / start of next slot provides the ending point of previous mini-slot. As another alternative, the UE may be allowed to choose the ending point for the mini-slot to be any of the subsequent starting points in the slot/ start of the next slot. Such flexibility allows UE to use the system more efficiently. For example if UE is configured with 4 mini-slots in a slot (0-3,4-6,7-10,11-13), however, it has very few HARQ IDs available (some CG-HARQ IDs got used for scheduled UL or are blocked waiting for retx timer etc), it can choose to switch to two mini-slot config mode (0-6,7-13)

Proposal : Number of allocated slots is a M bit number which directly indicates the number of allocated slots from 1 to 2M. We propose M = 4/5/6 for 15/30/60 kHz SCS respectively.

Proposal : timeDomainAllocation field is enhanced to indicate one start symbol value providing start symbol for the first slot of any burst and one end symbol value that applies to last slot of any burst, where a burst comprises a set of contiguously allocated slots.

- For all slots in between the first slot and last slot in any burst, all symbols are allocated.

Proposal : The following enhancements are considered to support mini-slot operation on configured grant

UE can be configured with overlapping mini-slot and full slot configurations and UE can choose which configuration it wishes to use on any slot.

A symbol level bitmap is used to indicate the allowed starting points within a slot. The same configuration applies for every allocated slot.

  - For the ending symbol, UE can choose from any symbol immediately preceding a subsequent starting point in the slot and the last symbol of the slot

  - For the first (last) slot, the starting (ending) symbol specified through timeDomainAllocation is also an allowed starting (ending) point

## 2.7	Ending Symbol

Since UE may monitor PDCCH at slot boundaries, it may be useful to provide flexibility in the ending symbol used by UE to facilitate COT sharing. For example, UE may leave a gap of a few symbols in the last UL PUSCH it transmits so gNB can use 25us cat-2 LBT and still start transmission at slot boundary.

Proposal : When a full slot is allocated to the UE for CG-PUSCH, UE may select ending symbols from symbols {12,13} for SCS of 15/30kHz and from symbols {11,12,13} for SCS of 60kHz. Ending symbol used by the UE is indicated in the CG-UCI.

- FFS Whether this flexibility is RRC configured

## 2.7	TB size determination

Since the number of time domain symbols can vary across different CG resources (mini-slot, full slot, partial slot etc) the gNB and UE may not be in sync on the TB size especially for retransmissions on CG-PUSCH. Some mechanisms need to be introduced to ensure that gNB and UE are in sync on the TB size being used for any transmission. One mechanism could be for gNB to include information on the set of allowed number of time domain symbols to be used just for TB size computation as part of the configured grant configuration. UE can then include an index which points to the number of time domain symbols being used for the TB size computation. For the first transmission of a TB, UE may be mandated to use the number of time domain symbols corresponding to the resource being used.

Proposal : Introduce mechanisms to ensure TB size determination is matching at gNB and UE for CG transmissions.

## 2.8	Grant based retransmission of configured grant transmissions

To determine if a grant corresponding to a CG-PUSCH transmission is for new TB or a retransmission request, in LTE-LAA based approach, the UE would compare NDI in the CG-UCI with the NDI in the grant. In NR Rel-15, a retransmission of CG transmission is performed when a grant is received using CS-RNTI (with NDI hard coded to 1) and transmission of new TB is done when a grant is received using C-RNTI. Note that there was no CG-UCI for configured grant in NR Rel-15. Also note that RAN2 has made the following agreement:

- CS-RNTI is used for scheduled retransmission, and C-RNTI is used for new transmission, similar to NR CG. To be confirmed by RAN1.

RAN1 should also confirm the above agreement. In addition, in NR-U a more robust mechanism can be made by combining both the NR and LTE-LAA approaches. UE performs a retransmission only if the NDI is not-toggled and the grant is sent using CS-RNTI.

Note that RAN2 has already agreed to reuse configured grant timer functionality to limit the number of autonomous retransmission attempts for a configured grant Transport Block (TB), which implies that UE clears the HARQ buffer and performs new TB transmission after configured grant timer expiry. Configured grant timer is started at the first transmission of a TB within the CG resource (i.e. no impact on configured grant timer due to autonomous CG retransmissions).

Given that gNB may miss the initial CG transmission by UE due to hidden node issue, mismatch can occur between the gNB and the UE about the start instance of configured grant timer. This is depicted in Figure 2. Note that such mismatch can lead to many issues, for e.g. gNB may transmit a retransmission request for a HARQ TB which has already been flushed by UE, which leads to UE misinterpreting the transmitted control information and unnecessary retransmission of the new TB. Hence, it is preferable that NDI matching (with the previous CG HARQ transmission) should be performed by UE even for the case of CS-RNTI based UL grants.

Observation 1: NDI matching (with the previous CG HARQ transmission) should be performed by UE even for the case of CS-RNTI based UL grants

Figure 2 Mismatch of CG timer between UE and gNB

For CS-RNTI in NR Rel-15, NDI bit = 0 and HARQ ID=0 is the condition used to differentiate activation/deactivation with a retransmission grant (which always uses NDI bit=1). If we now need to indicate the NDI through the CS-RNTI, the mechanism to differentiate activation/deactivation DCI with retransmission grant needs to be updated.

Proposal : UE performs retransmission of CG transmission only if the grant is received using CS-RNTI and NDI is not toggled

- FFS: Updates to mechanisms to differentiate CG activation / deactivation with CG retransmission grant

# Summary

The proposals made in this contribution are summarized below:

Proposal 1: For type 1 configured grant, UE assumes DFI is only present when CG is configured.  For type 2 configured grant, UE assumes DFI is only present when CG is configured, and UE is in activated state for configured grant transmissions.

Proposal 2: For the case with PUSCH slot aggregation, UE assumes HARQ-ACK is valid and corresponds to all the repeated PUSCHs sent as part of the slot aggregated PUSCHs whose  transmission ends before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI and D is the RRC configured minimum duration.

Proposal 3: For PUSCH sent with slot aggregation, UE should not retransmit a PUSCH based on reception of NACK in DFI if the DFI only included valid ACK feedback for subset of the repeated PUSCHs transmitted by the UE

Proposal 4: DFI should also include an NDI bit for each HARQ ID.

Proposal 5: DFI DCI includes information on carrier(s) for which feedback is included

Proposal 6: A single DFI DCI can include HARQ A/N feedback for more than one carrier

Proposal 7: DFI is sent using DCI Format 0_0/0_1 with CS-RNTI

Proposal 8: To distinguish DFI from CG activation/deactivation and UL grant, NDI is set to 0 and LSB of RV is set to 1

Proposal 9: CG-UCI includes information on

- Remaining CO duration from end of the PUSCH

  - This can be replaced with a single bit indicator to enable/disable COT sharing when COT sharing ED threshold is not configured

- Offset information indicating where the UE transmissions end

- RRC configures the max duration to report and the granularity (e..g symbol level, half slot level, slot level)

Proposal 10: CAPC is included as part of the COT sharing information in the CG-UCI when COT sharing ED threshold is configured

Proposal 11: Inclusion of UE-ID in the CG-UCI in supported. The presence/absence of UE-ID in CG-UCI can be RRC configured.

Proposal 12: If HARQ ACK feedback is scheduled on a configured grant resource, configured grant PUSCH is skipped on that resource.

Proposal 13: UE can indicate a capability to support CP extension/ amount of CP extension it can support

- UE is only expected to be configured with starting point offsets within its capability

- When UE picks the starting point offset randomly, it does so from among the offsets that are within its capability

Proposal 14: UE is only required to include the UCI (such as A/N, CSI-part1, CSI-part2) in CG-PUSCH if the UCI is scheduled Np+ ceil(Toffset / Ts) symbols prior to the CG-PUSCH resource where Np is the previous capability of UE to include the UCI not considering any cyclic prefix extension

- UE should drop CG-PUSCH and transmit the UCI in case the UCI timeline of Np can be met but the new timeline cannot be met

- If only a single offset is configured Toffset = the configured offset

- If UE randomly selects the offset Toffset = max of all allowed offsets

Proposal 15: NR-U should continue to support configuration of repetitions for CG PUSCH. However, unlike NR where the repetitions of the same TB are on back to back slots, for NR-U, the UE should be given some flexibility on where it chooses to perform the repetitions.

Proposal 16: Number of allocated slots is a M bit number which directly indicates the number of allocated slots from 1 to 2M. We propose M = 4/5/6 for 15/30/60 kHz SCS respectively.

Proposal 17: timeDomainAllocation field is enhanced to indicate one start symbol value providing start symbol for the first slot of any burst and one end symbol value that applies to last slot of any burst, where a burst comprises a set of contiguously allocated slots.

- For all slots in between the first slot and last slot in any burst, all symbols are allocated.

Proposal 18: The following enhancements are considered to support mini-slot operation on configured grant

UE can be configured with overlapping mini-slot and full slot configurations and UE can choose which configuration it wishes to use on any slot.

A symbol level bitmap is used to indicate the allowed starting points within a slot. The same configuration applies for every allocated slot.

  - For the ending symbol, UE can choose from any symbol immediately preceding a subsequent starting point in the slot and the last symbol of the slot

  - For the first (last) slot, the starting (ending) symbol specified through timeDomainAllocation is also an allowed starting (ending) point

Proposal 19: When a full slot is allocated to the UE for CG-PUSCH, UE may select ending symbols from symbols {12,13} for SCS of 15/30kHz and from symbols {11,12,13} for SCS of 60kHz. Ending symbol used by the UE is indicated in the CG-UCI.

- FFS Whether this flexibility is RRC configured

Proposal 20: Introduce mechanisms to ensure TB size determination is matching at gNB and UE for CG transmissions.

Observation 1: NDI matching (with the previous CG HARQ transmission) should be performed by UE even for the case of CS-RNTI based UL grants

Proposal 21: UE performs retransmission of CG transmission only if the grant is received using CS-RNTI and NDI is not toggled

- FFS: Updates to mechanisms to differentiate CG activation / deactivation with CG retransmission grant

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#82, Sorrento, Italy, Dec. 10-13, 2018.

[2] RP-172021, “Revised SID on NR-based Access to Unlicensed Spectrum, Qualcomm Incorporated”, RAN#77, Sapporo, Japan, September 11 - 14, 2017.

[3] RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#80, La Jolla, USA, June 11-14, 2018.

[4] TR 38.889, “Study on NR-based Access to Unlicensed Spectrum”, RAN#82, Dec. 2018.

# Previous Agreements

Below is a list of previous agreements on this topic:

RAN1 #93, May 2018

Agreement:

- The following modifications to the configured grant procedures are beneficial

  - Removing dependencies of HARQ process information to the timing

    - Introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID

  - Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

  - Increased flexibility on time domain resource allocation for the configured grant transmissions

  - Supporting retransmissions without explicit UL grant

RAN1 #94, August 2018

Agreement:

Allowing consecutive configured grant resources in time without any gaps in between the resources and non-consecutive configured grant resources (not necessarily periodic) with gaps in between the resources is beneficial and should be considered for NR in unlicensed spectrum

Conclusion:

There is no necessity to exclude Type-1 or Type-2 configured grant mechanism for operation of NR in unlicensed spectrum.

Agreement:

UE selects the HARQ process ID from an RRC configured set of HARQ IDs for NR-unlicensed configured grant transmission.

Agreement:

It is identified to be beneficial to support DFI to include pending HARQ ACK feedback for prior configured grant transmissions from the same UE.

- FFS: DFI to include HARQ ACK feedback for scheduled UL transmissions using HARQ IDs configured for NR-unlicensed configured grant transmission.

Agreement:

- Retransmission via same configured grant resource is supported for a HARQ process that was initially transmitted via configured grant resource.

- Retransmission via resource scheduled by UL grant is supported for a HARQ process that was initially transmitted via configured grant resource.

Agreement:

UE may autonomously initiate retransmission for a HARQ process that was initially transmitted via configured grant mechanism for NR-unlicensed when one of the following conditions is met:

- Reception of NACK feedback via DFI for the corresponding HARQ process

- FFS: No reception of feedback from gNB upon the timer expiration.

  - To introduce a new timer or reuse configuredGrantTimer.

Agreement:

It is identified to be beneficial to consider UE multiplexing and collision avoidance mechanisms between configured grant transmissions and between configured grant and scheduled grant transmissions.

- FFS: detailed mechanism.

Agreement:

NR-unlicensed configured grant transmission is not allowed during the time when it overlaps with occasions configured for potential NR-U DRS of the serving cell irrespective of the configured time domain resource for configured grant transmission.

RAN1 #95, November 2018

Agreement:

Adopt the following TP for the TR:

As for potential solutions to providing flexibility on time domain resource allocation, bitmap based approach and NR Rel-15 based time domain resource allocation approach, which includes {periodicity, offset in the frame, start symbol and length of PUSCH and K-repetition signaling}, are identified as potential candidates. Additional aspects such as finer granularity of resource allocation, and multiple resources within a period may be considered for enhancing flexibility on time domain resource allocation.

Agreement:

It was identified that CBG based retransmissions for configured grant based transmissions is beneficial. Details on which CBG related control information is transmitted as part of DFI and UCI, and how such control information is conveyed through DFI and UCI can be determined when standards are developed.

Agreement:

It was identified that collision avoidance between configured grant and scheduled grant based transmission can be achieved by management of starting point of the transmission for configured grant and scheduled grant based transmission. Further details on the management of the starting point of the transmission can be determined when standards are developed.

Agreement:

It was identified that the resources utilized by the UCI, and multiplexing of UCI and data information of PUSCH require consideration of DMRS placement and starting and ending symbols of the configured grant based transmissions. Details on multiplexing UCI and data information of configured grant PUSCH can be determined when standards are developed.

Agreement:

Adopt the following text for the TR:

- Additional information fields can be considered to be included in the UCI, e.g. UE-ID, COT sharing information, PUSCH duration, etc.

- It was identified that it is problematic for the UE to assume ACK in absence of reception of feedback, which may include explicit feedback or feedback in the form of uplink grants. It was additionally identified that assuming NACK upon timer expiration can be a candidate solution to avoid LBT impact on reception of feedback. It was also identified that possible conflicts, with respect to NDI and RNTI for the same HARQ process, between configured grant transmission and scheduled grant transmission may have to be addressed. Details can be determined when specification are developed.

Agreement:

Adopt the following text proposal to the TR:

- It was identified that sharing resources with gNB within COT(s) that is acquired by UE(s) as part of configured grant based transmissions should be supported. It was also identified that allowing configured grant based transmissions within a gNB acquired COT should be supported. Details of identification of situations when COT(s) sharing is possible and the details of potential resource sharing mechanisms and rules can be determined when specifications are developed.

RAN1 #AH-1901, Jan 2019

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Conclusion:

The following aspects should be discussed further as part of the channel access discussions

- Contention window adjustment

- Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

RAN1 #96, Feb 2019

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

- The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

- FFS:

  - How the duration is signalled

  - Whether the UE should signal continued use of the COT for its own transmissions

  - LBT priority class

RAN1 #96bis, Apr 2019

Agreement:

Select from the following additional options for type 1 and type 2 configured grant time domain resource allocation mechanism in NR by RAN1#97

- Option 1: A bitmap to selectively enable or disable configured UL transmission opportunities as per NR Rel-15 configurations.

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

- Option 2: A mechanism based on multiple NR Rel-15 configurations

  - FFS: Whether any further enhancement is needed to Rel-16 beyond what is being considered in the URLLC WI

- Option 3: Configuration in addition to the Rel-15 baseline of one or more of the following aspects:

  - Multiple offsets within an active configuration

  - Duration of transmission for an offset

- Option 4: A bitmap to configure UL transmission opportunies to replace current time domain resource configuration

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

  - Note: This is importing LAA AUL functionality into NR

RAN1 #97, May 2019

Agreement:

For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

Agreement:

UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

RAN1 #98, Aug. 2019

Agreement:

- For DFI design for configured grants, support at least the following

- DFI including at least TB level HARQ-ACK bitmap for all UL HARQ processes

- Note: Total number of HARQ processes is as defined in Rel-15

- FFS: CBG level HARQ-ACK feedback, if supported

- RRC configured minimum duration, D, from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH

  - Note: UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI.

  - FFS: the definition of minimum duration for the case of slot aggregation

- UE blind decoding complexity shall not be increased due to DFI size

Agreement:

The number of separately encoded UCIs multiplexed in a PUSCH transmitted using a configured grant is not changed from Rel-15 (maximum of 3).

Working assumption:

CG-UCI is included in every CG-PUSCH transmission

RAN1 #98bis, Oct. 2019

Agreement:

The starting time offset applied by a UE at the beginning of a transmitted burst with a CG resource at the start of the transmission burst, is RRC configured and defined as the length of a CP extension of the first symbol that is located before the configured resource

- Regardless of SCS, the CP extension is up to 72 micro seconds with a granularity of 9 micro seconds

Agreement:

CG-UCI is mapped as per Rel-15 rules with CG-UCI having the highest priority (CG-UCI is mapped on the symbols starting after first DMRS symbol)

Agreement:

To determine the number of REs used for CG-UCI, the mechanism of beta-offset in Rel-15 NR for HARQ-ACK on CG-PUSCH is reused.

- A new RRC parameter to configure the beta-offset for CG-UCI is defined. FFS: Value range

Agreement:

CG-UCI is included in every CG-PUSCH transmission (confirms working assumption from RAN1#98)

Agreement:

- CG-UCI, CSI-part1, CSI-part 2 can be sent on CG-PUSCH at least when CG-UCI and HARQ-ACK feedback is not multiplexed on a CG-PUSCH