3GPP TSG RAN WG1 #99			R1-1912955

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.5.2

Source: 	Qualcomm Incorporated

Title: 	Physical Layer Procedures for NTN

Document for:	Discussion/Decision

# Introduction

In RAN meeting #98, an agreement was reach on UL transmission timing [1].  Specifically:

Agreement:

For UL transmission timing, introduce an offset Koffset  for NR NTN.

- For UL HARQ-ACK on PUCCH, where HARQ ACK on PUCCH is transmitted on slot n + K1 + Koffset when a scheduling DCI is received in slot n.

- For UL transmission on PUSCH, where PUSCH is transmitted on slot n∙2μPUSCH-μPDCCH+K2+Koffset when a scheduling DCI is received in slot n.

- For CSI transmission on PUSCH, where CSI on PUSCH is transmitted on slot n +K+Koffset, when the DCI with CSI request is received in slot n and K is selected by the DCI.

- For a CSI report in uplink slot n’, the CSI reference resource is given in downlink slot n-nCSI_ref-Koffset, where  and nCSI_ref is as defined in 38.214.

- Koffset  is per beam or per-cell

  - FFS: Whether Koffset is derived from broadcast information or is signaled by higher layers

- The focus of the Rel-16 study will be on pedestrian and on board vehicle usage-scenarios.

- The scope of the release 16 study item will be limited to key issues and solutions associated with transparent GEO satellite and LEO based non-terrestrial access network (moving beam on earth).

In RAN #98bis [2], additional cases where timing is affected by large round trip delay are identified and the following agreements were made accordingly:

Agreement:

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot n for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot n + K2 +Δ+Koffset.

Agreement:

When the HARQ-ACK corresponding to a PDSCH carrying a MAC-CE command is transmitted in slot n, the corresponding action and the UE assumption on the downlink configuration indicated by the MAC-CE command shall be applied starting from the first slot that is after slot n+XNslotsubframe,µ+Koffsetn+3Nslotsubframe,µ+Koffset (X can be determined when specifications are developed).

Agreement:

If a UE receives a DCI triggering aperiodic SRS in slot n, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot ⌊n∙2μSRSμPDCCH⌋+k+Koffset.

Agreement:

Koffset may take different values for each of the identified timing relationships that need to be modified for NTN.

In this contribution, we discuss enhancements of several areas in NR physical layer procedures for better support of NTN. These include CSI feedback and beam management.

# CSI Feedback

Due to limited number of retransmissions and long delay between ACK/NACK feedback and its associated transmission, network has limited or no scheduling flexibility to adjust the MCS to the channel quality during the retransmissions. In addition, satellite communication channels tend to be mostly line of sight and much stable. As a result, network is required and can be more efficient on channel usage based on fine granularity of MCS. The above is particularly true for GEO satellites. Considering that NR defines 15 CQI levels but up to 29 MCS for PDSCH, more than 15 CQI levels should be considered for NTN.

Proposal 1: Consider increased CQI levels for NTN.

In NTN, depending on deployment scenarios and possibly UE capability, different maximal number of retransmissions may be applied. As a result, different target BLER per transmission is needed.

Proposal 2: Consider additional target BLER other than 1e-1 and 1e-5 for CQI reporting for NTN.

# Beam Management CSI-RS

In NR, all CSI-RS resources in a beam management CSI-RS resource set are transmitted in the same frequency. By doing this, beam measurements can be carried out by UEs without frequency retuning. However, some satellite implementation may only support a fixed relationship between beams and bandwidth parts (BWPs) while others may not have this limitation. As shown in the figure below, CSI-RS resources associated with different beams may be transmitted in different BWPs or the same BWPs, depending on satellite implementation.

For satellites that lack of frequency tunability of beams, it is still beneficial to group some or all the beams in one cell so that UE movement from one beam to another can be treated as beam switch instead of cell handover. Hence it is important to consider BM CSI-RS for these satellites.

Figure 1. Example BM CSI-RS arrangements.

Proposal 3: Consider additional BM CSI-RS configurations to support different satellite implementation needs.

# Conclusions

In this contribution, we have discussed several aspects in physical layer procedures and identified enhancements needed for CQI feedback, beam management CSI-RS, and UL frequency control with following proposals.

Proposal 1: Consider increased CQI levels for NTN.

Proposal 2: Consider additional target BLER other than 1e-1 and 1e-5 for CQI reporting for NTN.

Proposal 3: Consider additional BM CSI-RS configurations to support different satellite implementation needs.

# References

- Chairman’s notes, 3GPP RAN1 #98, Aug. 2019.

- Chairman’s notes, 3GPP RAN1 #98 bis, Oct. 2019.