3GPP TSG RAN WG1 #99			 R1-1912969

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.8.4

Source: 	Qualcomm Incorporated

Title: 	Full Tx power for UL transmissions

Document for:	Discussion/Decision

# Introduction

In RAN #80 meeting [1], the following WI was approved as one of work items for NR MIMO Enhancements.

  - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

In RAN1 #94bis meeting [2], the following agreements were made on this WI:

Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95:

Option 1: Refinement/adjustment of UL codebook is supported

•             1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

•             1-2: Introduce additional scaling factor for uplink codebook

Option 2: UE transparently apply a small cyclic or linear delay

Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

•             Note: Full rated PA refers to a PA having power not lower than that of the power class

Option 4: Up to UE implementation (no specification impact)

In RAN1 #95 meeting [3], the following agreements were made on this WI:

Agreement 1: Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

-	This specification support is a UE optional feature

-	FFS: Whether this applies for the entire codebook or subset of codebook

Agreement 2: Full TX power UL transmission, one additional option (option 5) is added

Option5: For the precoders with 0 entries, the linear value PPUSCH,b,f,c(i,j,qd,l)PPUSCH,b,f,c(i,j,qd,l) of a PUSCH transmission power is scaled by a ratio Rel-16.  The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

- UE is required to maintain consistent Rel-16αRel-16αRel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

Agreement 3: Full TX power UL transmission, option 4 is updated as follows

Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL (no specification impact)

For guidance in future RAN1 discussions:

Understanding of antenna virtualization for ease of discussion (not for specification):

-	A UE forms an antenna port by transmitting on one or more TX chains (each with a power amplifier)

In RAN1 AH-1901 meeting [4], the following agreement was made in this WI:

Agreement: Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified, or a subset will be specified

FFS: UE capability signaling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

Send LS to RAN4 to provide their view on PC 2 applicability of the new UE capability (Rakesh, vivo).

In Ran1 96 meeting, the following agreements were made

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

Agreement

Down select among the following two alternatives by RAN1#96bis. As part of UE capabilities signalled the following is included:

Alt1: UE capability signaling of supported one or group of TPMI precoder(s)

Alt3: UE capability signaling of power scaling schemes for full uplink power transmission

- Note: This does not imply any restriction on UE antenna virtualization

- FFS: Whether full uplink power transmission needs to be supported for all precoders

In RAN1 96bis, the following agreements were made

Agreement

For the the 2TX and 4TX case, the linear value of power after power scaling, is divided equally among the non-zero PUSCH ports

- The above applies for the cases including when UE transmitting at P_c_max

Agreement

Supported UE capabilities and supported scheme for UE capability 1

- Option 3

- FFS: Whether to additionally support Option 1-2

Agreement

Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- FFS: For UE capability 1, if any other information is necessary

- For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed

  - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced

  - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission

UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1. The signalling of above information does not imply any specific UE PA architecture implementation.

In RAN1 97, the follow working assumption were made:

Working Assumption

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - FFS: Additional support of antenna selection TPMI precoders

  - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

    - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UEcap3, for SRS resource with more than 1 ports,

    - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects

    - The following cases are not precluded

      - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1]

    - FFS: number of SRS resources supported

      - 2

      - 3

    - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power

  - FFS: any rules for spatial filter update for the SRS resources with different number ports

Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD

Note: For single port, there is no SRI and TPMI

Note: Support of Mode 1, Mode 2 have separate UE capability

In RAN1 98, the follow agreements were made:

Agreement

For mode 1, 2Tx non-coherent UE, the new codebook subset at least includes rank=1 TPMI=2 defined in Rel-15 which can be used for UL full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset at least includes, rank 1 TPMI= 13 defined in Rel-15 which can be used for UL full power transmission

- FFS for the case that part of ports can deliver full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset

- at least includes, rank 2 TPMI=6 defined in Rel-15

- at least includes, rank 3 TPMI=1 defined in Rel-15

Agreement

For mode 2, in case of non-coherent with 2 ports, support following TPMI indication for rank 1 which support UL full power transmission:

- Rank 1: support {TPMI=0} and {TPMI=1}

- FFS: Details on UE capability signalling

Agreement

For a capability 1 UE working with full power operations, for PUSCH power control, power scaling factor is fixed to 1

In RAN1 98bis, the follow agreements were made:

Agreement

- Support RRC configuration to operate in Mode1 or Mode2 subject to UE capability

  - For UE capabilty-2 and-3, gNB can configure a UE to operate in Mode 1 or Mode 2 subject to UE capability

    - Note : if UE only supports Mode 1 gNB cannot configure this UE to operate in Mode 2, if UE only supports Mode 2 gNB cannot configure this UE to operate in Mode 1

  - FFS: UE capability signaling discussion

  - Note: capability-1 UE can be configured with RRC parameter “ULFPTx” to deliver UL full power has been agreed, exact parameter name is up to RAN2

- If gNB does not configure UE for Rel-16 full power UL transmission, Rel-16 UEs operate in Rel-15 behavior

Agreement

For 2Tx in mode 1,

- For rank=1, TPMI=2, TPMI=0, TPMI=1 are included in new codebook subset for non-coherent UEs with power scaling defined as in [38.213] Rel-15

- For rank=2, TPMI=0 is included in the new codebook subset

Agreement

For Mode2,

- Power scaling factor is equal to 1 for the reported TPMI precoders that supports full power Tx

- for the other TPMI precoders, if only one SRS resource is configured, the power scaling factor is determined by #non-zero-PUSCH-port divided by #SRS-ports

- for the other TPMI precoders, the power scaling factor is determined by #non-zero PUSCH port/#SRS ports in the SRS resource indicated by SRI

Agreement

For Mode 1 4TX, for non-full power uplink transmission, antenna selection precoders are included in the new codebook subset following Rel-15 power scaling factor

- FFS: Whether to include antenna selection precoders for full power uplink transmission

Agreement

For full power uplink transmission Mode 1, 4TX partial-coherent, the new codebook subset includes

- Rank1(CP-OFDM): TPMI = 12,13,14,15

- Rank1(DFT-s-OFDM): TPMI = 12,13,14,15

  - FFS: TPMI=16, 17, 18, 19

- FFS: Whether clarification on which port pairs are coherent is needed

With the above-mentioned agreements and working assumptions, the major open issues on this topic are closed. In this contribution, we address the remaining issues in UL full power Tx.

In this contribution, Section 2 discusses the necessity to extend the UE capabilities defined for power class 3 to other power classes. The way to signal UE capabilities to support UL full power transmissions is discussed in Section 3. The remaining issue for mode 1 and mode 2 are addressed respective in Section 4 and Section 5. RRC signaling for full power is discussed in Section 6. Conclusions are drawn in Section 7.

# Extend the UE capabilities to other power class UEs

In RAN1 AH-1901 meeting [4], the 3 types of UE capabilities are agreed for power class 3 UEs. We additionally note that these three capabilities are useful for UEs of all power classes, and must not be restricted to only power class 3 UEs.

With the above in mind, we propose the following:

Proposal 1: Extend the definitions of the three full power capabilities to UEs of any power class.

# Signalling of UE capability to support UL full power transmissions

Due to the existence of 3 UE capability types and the two modes for capability 2 and 3 UEs, the UE capable signalling for full power needs to follow a hierarchy structure. The top level signalling can be just 1 bit to signal UE can support full power with all precoders. This signalling is sufficient for capability 1 UE and it needs no more additional lower level signalling for full power capability. For capability 2 or 3 UEs, they need to further signal they can support which mode, i.e., mode 1 only, mode 2 only, both, or none of the two modes. For UEs supports mode 1 only, no additional capability signalling is needed. NW knows that full power can be delivered with the introduced “new codebookSubset” while full power cannot be guaranteed with the “old antenna selection codebook”. For UEs supports mode 2, it was agreed in RAN1 #97 that further UE capability signalling per TPMI group is introduced. Our view is that RAN1 should aim to minimize the signalling overhead for mode 2.

Based on the above discussion, therefore, we make the follow proposal.

Proposal 2: A single bit (level 1) is used to indicate that a UE supports full power by setting power scaling factor in power control equals to 1 always for all precoders or not.

- For a capability 1 UE, no additional full power capability signaling is needed.

- For a capability 2 or 3 UE, two more bits (level 2) to signal support mode 1 only, mode 2 only, both mode 1 and 2, none of mode 1 or 2.

  - For a UE supports mode 1 only or none of mode 1 or 2, no additional full power capability signaling is needed.

  - For a UE supports mode 2 only or both mode 1 and 2, RAN1 strive to minimize the number of additional bits (level 3) for full power capability signaling.

Note: the above UE full power capability signaling is per feature set per component carrier (per CC per band per band combination).

# Remaining issue for UE capability 2 and 3 in mode 1

For mode 1, an opening issue is whether to further include rank 1 TPMIs {16,17,18,19} for DFT-S-OFDM in the new codebook subset for 4 Tx partial coherent UEs, in addition to agreed rank 1 TPMIs {12,13,14,15}. We believe inclusion of  rank 1 TPMIs {16,17,18,19} is beneficial hence make the following proposal.

Proposal 3: For mode 1, 4Tx partial-coherent UE, the new codebook subset includes, rank 1 TPMI = {12,13,14,15, 16, 17, 18, 19} defined in Rel-15 which can be used for UL full power transmission with transform precoding enabled.

# Remaining issue for UE capability 2 and 3 in mode 2

For mode 2, one open issue is how to group TPMIs to reduce the UE capability signalling overhead, as the signalling has to be per CC per band per band combination.

Obviously, we need to design the grouping for 2 Tx and 4 Tx UEs separately. For 2 Tx UE, the design is trivial. Therefore, we can focus on the design for 4 Tx UEs. For a UE with 4 Tx, there are 12 rank 1 precoders (0-11 in Table 6.3.1.5-3 in 38.211), 7 rank 2 precoders (0-6 in Table 6.3.1.5-5 in 38.211), and 1 rank 3 precoder (0 in Table 6.3.1.5-6 in 38.211). If we signal full power capability per precoder, there are 20 bits needed per band in each band-band combination, which is too much overhead. Therefore, we must aim to reduce the bitmap length for overhead reduction.

First, we observe that the full power capability with multi-layer precoders can be inferred from single layer precoders. For example, if precoder  can support full power, so can . Another example, if precoder  can support full power, so can  or . Therefore, we can put {, } into a same TPMI group. Similarly, we can group {,,} into a single group.

Based on this observation, we make the following proposal.

Proposal 4: For a capability 2 or 3 UE supports mode 2, no need to signal full power capability for a multi-layer precoders as it can be implicitly derived based on signaling for single layer precoders.

Based on the idea to utilizing single layer precoder to infer the full power capability of multi-layer precoder and group them into the same group, by exhaustive search of the Rel-15 PUSCH codebook, we list potentially 11 groups of precoders.

Group 1: {121000},

Group 2: {120100},

Group 3: {120010},

Group 4: {120001},

Group 5: {121010,1210-10,1210j0,1210-j0,1210000100},

Group 6: {120101,12010-1,12010j,12010-j,1200100001},

Group 7: 1210010000,

Group 8: 1210000001

Group 9: 1200100100

Group 10: 1200001001

Group 11: 12100010001000

Group 12: 1111 and all the rest precoders with no zero-power antenna port

One should notice that with above grouping, 11 bits (excluding the last group) is needed per CC per band per band combination if a UE signals full power capability per precoder. There is a strong motivation to reduce capability signalling overhead.

To reduce the overhead, we can consider two types of UE. Type A UE is an advanced UE which can dynamically switch the linkage between PA and antenna ports. The concept of PA switch is illustrated in Figure 1, with an example of two Tx UE.  The PA with 23dBm max power is switch between physical antenna 0 and 1 to create PUSCH port 0 and port 1. The cable loss may need to be calibrated between 23dBm PA to physical antenna 1 and 20dBm PA to physical antenna 1, such that the PA switch has no huge impact to channel amplitude change between SRS sounding and PUSCH. The calibration of cable loss can be done within certain power tolerance margin referring to either “relative power tolerance” or “aggregated power tolerance” as specified in 38.101-1 subclause 6.3.4. The absolute phase change due to PA switch is fine, because anyhow UE is not required to maintain absolute phase continuity between SRS sounding and PUSCH transmission, irrespective the UE antennas are coherent or not. Even for a coherent UE, the phase coherence is referring to the relative phase coherence among the multiple antennas between SRS sounding and PUSCH transmission. The absolute phase could change with a common phase difference (cross multiple antennas) between SRS sounding and PUSCH transmission. For the 4Tx non-coherent precoders or the 2Tx non-coherent precoders, PA switch can be applied to support full power transmission, subject to UE capability. Moreover, PA switch can be applied to support full power transmission for all precoders when the switch is not too frequent.

Figure 1: PA and physical antenna switch for type A 2Tx UEs

The principle of PA to physical antenna switch can be applied to 4 Tx UEs, except the following case where PA switch is not doable between set {121010,1210-10,1210j0,1210-j0} and set {120101,12010-1,12010j,12010-j}. The reason can be illustrated in the following Figure 2. As the relative phase difference between SRS port 1&3 is delta_2, while the relative phase different between PUSCH port 1&3 is delta_1 due to PA switch, phase coherence from SRS sounding to PUSCH transmission from port 1&3 for partial coherent 4 Tx UE is not maintained due to PA switch. Therefore, dynamic PA switch cannot apply to 4 Tx partial coherent precoders. But it still can be applied to 4 Tx non-coherent precoders, i.e., for Type A UE, if it can support full power with 1000, it can also support full power with  0100, 0010, 0001.

Figure 2: PA and physical antenna switch for type A 4Tx UEs (partial coherent)

For partial coherent precoders, if a UE can support full power with 121010, it should also able to support full power with {1210-10,1210j0,1210-j0}.  If a UE can support full power with 120101, it should be able to support full power with {12010-1,12010j,12010-j}. The only constraint for partial coherent UE is that it support set {121010,1210-10,1210j0,1210-j0} with full power does not automatically mean it can support set {120101,12010-1,12010j,12010-j} as well.

For rank 2 precoders, since phase coherence is not required between two layers, it can support PA switch for type A UEs.

Based on the above reasoning, we can partition the precoders into the following 4 groups

Table 1: groups of TPMIs for type A UEs

| Group 1 | {121000,120100,120010,120001}, for type A UE with e.g. {23, 17, 17, 17}dBm PAs |
| --- | --- |
| Group 2 | {121010,1210-10,1210j0,1210-j0,1210000100,1200100001,1210010000,1210000001, 1200100100,1200001001}, for type A UE with e.g. {20, 17, 20, 17}dBm PAs |
| Group 3 | {120101,12010-1,12010j,12010-j,1210000100,1200100001,1210010000,1210000001, 1200100100,1200001001 }, for type A UE with e.g. {17, 20, 17, 20}dBm PAs |
| Group 4 | 12100010001000, for type A UE with e.g. {18.3,18.3,18.3,17}dBm PAs |

Another UE, which is type B UE, is a UE cannot dynamically switch connection between PA and antenna ports. To align the number of bits used for capability signaling, for type B UE, we also propose use 4-bit bitmap to signal full power capability for precoders with one, two, and three ports respectively with the following grouped precoders

Table 2: groups of TPMIs for type B UEs

| Group 1 | {121000}, for type B UE with e.g. {23, 17, 17, 17}dBm PAs |
| --- | --- |
| Group 2 | {121010,1210-10,1210j0,1210-j0,1210000100} for type B UE with e.g. {20, 17, 20, 17}dBm PAs |
| Group 3 | { 120101,12010-1,12010j,12010-j,1200100001 }, for type B UE with e.g. {17, 20, 17, 20}dBm PAs |
| Group 4 | 12100010001000, for type B UE with e.g. {18.3,18.3,18.3,17}dBm PAs |

For type B UE, the advantage of this scheme is the reduced signaling overhead and aligned number of bits between type A and type B UEs. The limitation of this scheme is that type B UE cannot guarantee to deliver full power with all precoders. But given that the goal of this Rel-16 WI is already achieve by allow eNB to use these precoders to request full power UL transmission, extending type B UE to support full power with other precoders can be considered as a secondary optimization for later releases. If significant performance degradation is identified due to not using full set of precoders, a 11-bit bitmap can be used to signal full power capability for type B UEs. But so far, we don’t prefer to go with this route unless significant performance loss is demonstrated.

Based on the above discussion, we have the following proposal for 4 Tx UEs.

Proposal 5: For a capability 2 or 3 UE supports mode 2, if the UE is with 4 Tx, 4 groups of precoders are supported with 5 bits are used for (L3) full power capability signaling per feature set per component carrier (per CC per band per band combination).

- 4 bits(LSB) bitmap to indicate support full power with group 1, 2, 3, 4 or not.

- 1 bit(MSB) to indicate the grouping is following Table 1 or Table 2 as in section 5.

We can have the similar signaling for 2 Tx UEs. Note that in this case there are only two TPMIs of interest 10 and 01. Without any loss of functionality, it can be mandated that a 2 Tx UE indicating support for mode 2 supports full power using precoder 10. This simplifies signaling and in fact requires no additional signaling for such UEs. If necessary, an additional bit signaling can indicate if this UE can also support full power on the 01 precoder. Thus 10 becomes the default precoder support full power transmission.

Proposal 6: For a capability 2 or 3 UE supports mode 2, if the UE is with 2 Tx, 2 bits (level 3) are used for full power capability signaling per feature set per component carrier (per CC per band per band combination).

- 1 bit(LSB) to indicate support full power with precoder 10 or not.

- 1 bit(MSB) to indicate if gNB can infer full power support from the signaled precoders to other precoders with the same number of nonzero ports but different nonzero port indices.

For mode 2, an additional aspect to consider for 4 TX UEs is on the number of ports the new SRS resources can have. Since a 4 TX UE may be able to virtualize its 4 transmit chains to form 2 full power virtual ports, as in the case of a power class 3 UE with four 20 dBm PAs, NW may configure 4 SRS resources with {Res1 for 4 SRS ports(spatial relation A), Res 2 for 4 SRS ports(spatial relation B), Res 3 for 2 SRS ports, Res 4 for single SRS port}. For a UE with four 17dBm PAs, the full power can only be achieved with virtualizing 4 PAs into one port. Therefore, NW has no incentive to configure 2 ports SRS for this UE. Two SRS resources such as {Res1 for 4 SRS ports, Res2 for 1 SRS port} is enough for this UE. The UE’s ability to synthesize two virtual ports each with full power can be signalled to NW to help NW with better SRS resource configuration.

Therefore, based on the above discussion, we have the following proposal.

Proposal 7: For a 4 Tx UE that support mode 2, one additional bit of capability signaling is introduced to indicate whether the UE is able to synthesize 1 or 2 virtual ports that can independently transmit at full power.

- If this bit is set to ‘0’, then the UE is capable of synthesizing only 1 virtual full power port, and if this bit is set to ‘1’, the UE is capable of synthesizing 2 virtual full power ports.

# RRC signalling for UL full power

As an initial step, two RRC parameters “ULFPTx” and “ULFPTxModes” are agreed to enable Rel-16 full power feature for a UE and furthermore configure the UE to operate in mode 0, 1, or 2 to deliver full power. However, the granularity of the two parameters are still FFS, i.e. “per CC or UL BWP (TBD in RAN1#99)”. In this section, this issue is addressed.

We propose that these two parameters being signalled for each uplink BWP. SRS configurations for the UE, including parameters such as nrofSRS-Ports (contained in srs-Config) are already signalled at per uplink BWP level. Therefore, we think the RRC configuration for full power features must be specified at the same level. The reason is following. A gNB can configure a 4 Tx UE with four 20dBm PAs operates in UL BWP 1 with 4 SRS ports, and in UL BWP 2 with 2 SRS ports (for power saving purpose for example). The UE can work in UL BWP 1 with mode 1 or mode 2. But in UL BWP 2, it can act as 2 Tx capability 1 UE with power scaling factor set to 1 always.

With these considerations in mind, we make the following proposal:

Proposal 8: The two RRC parameters “ULFPTx” and “ULFPTxModes” are configured on per UL BWP.

# Conclusions

In summary, we have the following proposals to address the remaining issues for full power uplink transmissions for Rel-16 MIMO enhancements.

Proposal 1: Extend the definitions of the three full power capabilities to UEs of any power class.

Proposal 2: A single bit (level 1) is used to indicate that a UE supports full power by setting power scaling factor in power control equals to 1 always for all precoders or not.

- For a capability 1 UE, no additional full power capability signaling is needed.

- For a capability 2 or 3 UE, two more bits (level 2) to signal support mode 1 only, mode 2 only, both mode 1 and 2, none of mode 1 or 2.

  - For a UE supports mode 1 only or none of mode 1 or 2, no additional full power capability signaling is needed.

  - For a UE supports mode 2 only or both mode 1 and 2, RAN1 strive to minimize the number of additional bits (level 3) for full power capability signaling.

Note: the above UE full power capability signaling is per feature set per component carrier (per CC per band per band combination).

Proposal 3: For mode 1, 4Tx partial-coherent UE, the new codebook subset includes, rank 1 TPMI = {12,13,14,15, 16, 17, 18, 19} defined in Rel-15 which can be used for UL full power transmission with transform precoding enabled.

Proposal 4: For a capability 2 or 3 UE supports mode 2, no need to signal full power capability for a multi-layer precoders as it can be implicitly derived based on signaling for single layer precoders.

Proposal 5: For a capability 2 or 3 UE supports mode 2, if the UE is with 4 Tx, 4 groups of precoders are supported with 5 bits are used for (L3) full power capability signaling per feature set per component carrier (per CC per band per band combination).

- 4 bits(LSB) bitmap to indicate support full power with group 1, 2, 3, 4 or not.

- 1 bit(MSB) to indicate the grouping is following Table 1 or Table 2 as in section 5.

Proposal 6: For a capability 2 or 3 UE supports mode 2, if the UE is with 2 Tx, 2 bits (level 3) are used for full power capability signaling per feature set per component carrier (per CC per band per band combination).

- 1 bit(LSB) to indicate support full power with precoder 10 or not.

- 1 bit(MSB) to indicate if gNB can infer full power support from the signaled precoders to other precoders with the same number of nonzero ports but different nonzero port indices.

Proposal 7: For a 4 Tx UE that support mode 2, one additional bit of capability signaling is introduced to indicate whether the UE is able to synthesize 1 or 2 virtual ports that can independently transmit at full power.

- If this bit is set to ‘0’, then the UE is capable of synthesizing only 1 virtual full power port, and if this bit is set to ‘1’, the UE is capable of synthesizing 2 virtual full power ports.

Proposal 8: The two RRC parameters “ULFPTx” and “ULFPTxModes” are configured on per UL BWP.

# References

- 3GPP TSG RAN Meeting #80, RP-181453 “WI Proposal on NR MIMO Enhancements,” La Jolla, USA, June 11 – 14, 2018.

- 3GPP TSG RAN WG1 Meeting #94b, “RAN1 Chairman’s notes,” Chengdu, China, October 8th – 12th, 2018.

- 3GPP TSG RAN WG1 Meeting #95, “RAN1 Chairman’s notes,” Spokane, USA, November 21-25, 2018.

- 3GPP TSG RAN WG1 Meeting AH 1901, “RAN1 Chairman’s notes,” Taipei, Taiwan, 21st – 25th January, 2019.