3GPP TSG-RAN WG1 #99	R1-1912979

November 18th – 22nd, 2019

Reno, USA

Agenda item:	7.2.13.2

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for EN-DC Single-Tx TDM Operation

Document for:	Discussion/Decision

# Introduction

Various agreements have been made for Single Tx Switched Uplink Solution for EN-DC [1]. In this contribution, we discuss remaining issues and present our proposals.

# Remaining UE behaviors for UL transmissions

Following is a set of agreements related to Rel.16 UE behaviors when the UE is configured with tdm-PatternConfig:

|  | Type 1 UE (UE w dynamic power sharing capability) | Type 2 UE (UE w/o dynamic power sharing capability) |
| --- | --- | --- |
| HARQ-ACK feedback | HARQ-ACK timing is determined based on DL-reference UL-DL configuration and HARQ-offset - For TDD-PCell; - As for DL-ref UL-DL config, only #2, #4, #5 are allowed - For TDD-PCell itself and TDD-SCell with the same UL-DL config, timing follows Table 10.1.3.1-1 in 36.213 - For TDD-SCell with different UL-DL config, timing follows Table 10.1.3A-1 in 36.213 - For FDD-PCell, - Same as in Rel.15 |  |
| Scheduled PUSCH | Timing is determined based on the serving cell frame structure (and SIB1 UL-DL configuration in case TDD) |  |
|  | All UL subframes can be scheduled for LTE - For single-Tx case, NR transmission is dropped if any collision with LTE | Only UL subframes that are used for HARQ-ACK feedback can be scheduled for LTE |
| PRACH | With LTE TDD PCell and (as a working assumption) with LTE FDD PCell, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config |  |
|  | if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized | the UE is not required to support transmission of LTE PRACH which does not coincide with the configured HARQ-ACK transmission occasions |

Collision handling so far has been concluded on LTE UL transmissions for (1) dynamically scheduled PUSCH and (2) PRACH, while others are still open.

For type 2 UE (i.e., UE without dynamic power sharing capability), the conclusion so far is to use UL subframes designated for HARQ-ACK feedback for both (1) and (2). It is straightforward to apply the same rule for any other LTE UL transmissions.

For type 1 UE (i.e., UE with dynamic power sharing capability), LTE PUSCH can be scheduled in UL subframes not designated for HARQ-ACK feedback. Since aperiodic SRS is also dyinamically granted by DCI, aperiodic SRS should be able to be scheduled in the UL subframes not designated for HARQ-ACK feedback, as for LTE PUSCH. For semi-statically configured LTE UL transmissions (e.g., SR/periodic CSI on PUCCH, periodic SRS, semi-perspstent PUSCH), following two options can be considered:

Opt.1: Semi-statically configured LTE UL transmissions are transmitted only in UL subframes designated for HARQ-ACK feedback. For single-Tx case, in the other UL subframes, the collision between LTE and NR transmissions is treated as an error case.

Opt.2: Semi-statically configured LTE UL transmissions are transmitted even in UL subframes not designated for HARQ-ACK feedback. For single-Tx case, in the other UL subframes, if an LTE UL transmission collides with an NR UL transmission, the NR UL transmission is dropped.

With Option 1, LTE UL transmissions in the other UL subframes are only scheduled by DCI. Since LTE-NR collision in these UL subframes is avoidable by eNB-gNB scheduler coordination, the collision can be treated as an error case. Option 2, on the other hand, should not handle the LTE-NR collision as such, since collision could occur in general. Rather, NR UL transmissions should be dropped in case of the collision. Compareing the two options, Option 1 increases the UL transmission opportunities for NR side while Option 2 could drop NR UL transmissions periodically due to LTE UL transmissions. Therefore we can adopt Option 1.

Proposal: For a UE configured with tdm-PatternConfig in Rel.16:

- For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback.

- For type 1 UE (i.e., UE with dynamic power sharing capability), any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback.

- For type 1 UE (i.e., UE with dynamic power sharing capability):

  - Semi-statically configured LTE UL transmissions are transmitted only in UL subframes designated for HARQ-ACK feedback. For single-Tx case, in the other UL subframes, the collision between LTE (except LTE PRACH) and NR transmissions is treated as an error case.

# Conclusion

In this paper, we discussed some design details related to the remaining open issues and proposed the following.

Proposal: For a UE configured with tdm-PatternConfig in Rel.16:

- For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback.

- For type 1 UE (i.e., UE with dynamic power sharing capability), any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback.

- For type 1 UE (i.e., UE with dynamic power sharing capability):

  - Semi-statically configured LTE UL transmissions are transmitted only in UL subframes designated for HARQ-ACK feedback. For single-Tx case, in the other UL subframes, the collision between LTE (except LTE PRACH) and NR transmissions is treated as an error case.

# Reference

[1] R1-1911737	RAN1 agreements for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)	Nokia, Nokia Shanghai Bell