3GPP TSG-RAN WG1 Meeting #99 	R1-1913009

# Reno, U.S.A.,  November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PT-RS port association to DM-RS port |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing text cannot fully reflect old update for PUSCH/DM-RS precedure in TS38.211/212. Based on the updated specification, PTRS port should be associated to DMRS port but one paragraph implemented incorrectly by associating PT-RS with PUSCH layer(s). Thus, it is clarified as DMRS port instead of layer. TRI in the text is not defined in any of the specification, and should be corrected with number of layers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the procedure for association of PTRS and DMRS ports, - PT-RS port to be assoicated with DM-RS port not with layer - Clarify the procedure to determine the actual number of PT-RS ports - Replace TRI by number of layers (TRI doesn’t exist in any specification) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity to reader of the specification due to misalignment with the related text in the other specification (e.g. TS38.211 and TS38.212) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR is intended for clarifying the ambiguity in the existing text with the alignement with the related text in the other specification. No functional change exists. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant.

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

When transform precoding is not enabled and if a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively.

-	if either or both higher layer parameters timeDensity and/or frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS and scheduled bandwidth in a corresponding bandwidth part as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively,

-	if the higher layer parameter timeDensity is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity is not configured, the UE shall assume KPT-RS = 2.

-	if none of the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume LPT-RS = 1 and KPT-RS = 2.

Table 6.2.3.1-1: Time density of PT-RS as a function of scheduled MCS

| Scheduled MCS | Time density() |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

Table 6.2.3.1-2: Frequency density of PT-RS as a function of scheduled bandwidth

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 is used and 0-28 when MCS Table 5.1.3.1-2 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 is used and 28 when MCS Table 5.1.3.1-2 is used. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 1-276.

If the higher layer parameter PTRS-UplinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-1 is disabled. If the higher layer parameter frequencyDensity in PTRS-UplinkConfig indicates that the frequency density thresholds NRB,i = NRB,i+1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-2 is disabled.

If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, indicates that are configured as 'PT-RS not present', the UE shall assume that PT-RS is not present.

When a UE is scheduled to transmit PUSCH with allocation duration of 2 symbols or less, and if LPT-RS is set to 2 or 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH with allocation duration of 4 symbols or less, and if LPT-RS is set to 4, the UE shall not transmit PT-RS.

When a UE is scheduled to transmit PUSCH for retransmission, if the UE is scheduled with IMCS > V, where V = 28 for MCS table 1 and V = 27 for MCS table 2, respectively, the MCS for PT-RS time-density determination is obtained from the DCI for the same transport block in the initial transmission, which is smaller than or equal to V.

The maximum number of configured PT-RS ports is given by the higher layer parameter maxNrofPorts in PTRS-UplinkConfig. The UE is not expected to be configured with a larger number of UL PT-RS ports than it has reported need for.

If a UE has reported the capability of supporting full-coherent UL transmission, the UE shall expect the number of UL PT-RS ports to be configured as one if UL-PTRS is configured.

For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1.

The UE does not expect to be indicated PT-RS association to DM-RS port which is not scheduled.

For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s). A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers in DCI format 0_1:

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual number of UL PT-RS port(s) and the associated DM-RS port(s) are derived from indicated TPMI as:

-	DM-RS port(s) transmitted on antenna port 1000 and/or 1002 in indicated TPMI share PT-RS port 0, and DM-RS port(s) transmitted on antenna port 1001 and/or 1003 in indicated TPMI share PT-RS port 1.

-	if all scheduled DM-RS port(s) share the same PT-RS port, the UE shall assume one PT-RS port, and the PT-RS port is associated with one of DM-RS ports indicated by PTRS-DMRS association field in DCI format 0_1.

-    otherwise, UE shall assume two PT-RS ports, and each PT-RS port is associated with one of scheduled DM-RS ports shares the PT-RS port indicated by PTRS-DMRS association field in DCI format 0_1.

When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

-	If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE  is given by , where  is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor  specified in subclause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the Precoding Information and Number of Layers field in DCI.

-	The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH.

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |