3GPP TSG RAN WG1 #99						  R1-1913071

Reno, USA, November 18th – 22nd, 2019

Source:	WILUS Inc.

Title:	Remaining Issues on SPS PDSCH for NR URLLC

Agenda item:	7.2.6.7

Document for:	Discussion/Decision

- Introduction

In RAN#83 plenary meeting, the WID on support of NR industrial internet of things (IoT) [1] was approved. Regarding SPS PDSCH enhancements, the following scope is defined in [1]

- The detailed objectives for NR TSC-related enhancements include:

- Specify enhancements to satisfy QoS for wireless Ethernet when using TSC traffic patterns, including

  - Support for multiple simultaneous active semi-persistent scheduling (SPS) configurations for a given BWP of a UE. [RAN2, RAN1].

  - Support for shorter SPS periodicities than the existing ones [RAN2, RAN1].

- Address support for TSC message periodicities with non-integer multiple of NR supported CG/SPS periodicities, as captured in TR 38.825, section 6.5.2. [RAN2, RAN1].

In the last RAN1#99 meeting, RAN1 made the following agreements. [2]

Agreements: Confirm the following working assumption: Working assumption: Support joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell - Reusing the joint release mechanism as that defined for UL type 2 CG. Agreements: For cases where only HARQ-ACK feedback for SPS PDSCHs shall be reported (i.e. no dynamic PDSCH HARQ-ACK), PUCCH formats 2/3/4 are applicable in addition to PUCCH formats 0/1. Agreements: For cases where HARQ-ACK feedback for SPS PDSCH is multiplexed with HARQ-ACK feedback for dynamic scheduled PDSCH, the PUCCH resource to be used is determined by reusing rel-15 mechanism. Agreements: For cases where only HARQ-ACK feedback for SPS PDSCHs shall be reported (i.e. no dynamic PDSCH HARQ-ACK), RAN1 down-selects the following options: - Option 1: Multiple PUCCH resources are configured common for all SPS configurations (similar to multi-CSI-PUCCH-ResourceList) per HARQ-ACK codebook. The actual PUCCH resource to be used among PUCCH resources is determined based on HARQ-ACK payload size - FFS: Number of maximum PUCCH resources - FFS details (threshold for determining PUCCH resource) - Option 2: Multiple PUCCH resource sets are configured common for all SPS configurations per HARQ-ACK codebook. The PUCCH resource set to be used is determined based on HARQ-ACK payload size. - FFS whether or not to configure PUCCH resource sets separately from PUCCH resource set for dynamic-scheduled PDSCH - FFS whether to configure separate payload range - The actual PUCCH resource to be used among PUCCH resources in the chosen PUCCH resource set is determined by reusing rel-15 HARQ-ACK PUCCH resource determination mechanism for dynamic PDSCH based on the latest activation DCI Agreements: For cases where only HARQ-ACK feedback for SPS PDSCHs without associated DL assignment shall be reported (i.e. no dynamic PDSCH HARQ-ACK), - Multiple PUCCH resources are configured common for all SPS configurations per HARQ-ACK codebook. The actual PUCCH resource to be used among PUCCH resources is determined based on HARQ-ACK payload size - Number of PUCCH resources is up to 4 - FFS details (e.g., threshold for determining PUCCH resource) Agreements: For cases where only HARQ-ACK feedback for SPS PDSCHs without associated DL assignment shall be reported (i.e. no dynamic PDSCH HARQ-ACK), PUCCH resource i is selected if HARQ-ACK payload size (not including CRC) is in the range of {Ni,min, …, Ni,max} bits, where the number of PUCCH resources in the selection is from 0 up to 3. - N0,min=1, N0,max=2 - For i≠0 - Ni,max is configured by RRC; if not configured, Ni,max is 1706. - Ni,min is equal to Ni-1,max+1 Note: The above mechanism is equivalent to rel-15 procedure when a single PUCCH resource is configured per PUCCH resource set. Agreements: For cases where only HARQ-ACK feedback for SPS PDSCHs without associated DL assignment shall be reported (i.e. no dynamic PDSCH HARQ-ACK), the number of PRBs for the PUCCH transmission is determined by reusing rel-15 mechanism in Subclause 9.2.3 (UE procedure for reporting HARQ-ACK) of 38.213. - The maximum code rate per PUCCH format is reused from the parameter associated with the identified HARQ-ACK codebook for SPS PDSCH

In this contribution, we discuss remaining issues on HARQ-ACK transmission and codebook construction for multiple SPS configurations.

- SPS Periodicity Less Than DL/UL Configuration Periodicity

In Rel-15, minimum periodicity of SPS PDSCH configuration is 10ms and maximum periodicity of DL/UL configuration is 10ms. Based on this condition, every SPS PDSCH reception can have appropriate PUCCH resource for its HARQ-ACK transmission. In other words, it can guarantee that the PUCCH resource of SPS PDSCH reception overlaps with semi-static flexible or uplink symbol. In the RAN1#98 meeting, RAN1 agreed to support SPS periodicity of no less than 1 slot. If the SPS periodicity become shorter than the DL/UL configuration periodicity, it is necessary to discuss how to transmit HARQ-ACK information for the SPS PDSCHs. When a SPS PDSCH reception is scheduled in slot n, the HARQ-ACK information of the SPS PDSCH reception is transmitted in a slot n+k, where k is a PDSCH-to-HARQ_feedback timing indicator value in an activating DCI. A UE cannot transmit HARQ-ACK information whenever the configured PUCCH resource in slot n+k is DL symbol(s) depending on DL/UL configuration in an unpaired spectrum. Figure 1 illustrates HARQ-ACK transmission for SPS PDSCH receptions in the unpaired spectrum, where DL/UL configuration is DDDDDDDDUU and SPS periodicity (P in Figure 1) is 2 slots and PDSCH-to-HARQ_feedback timing indicator (k in Figure 1) is 3 slots. As shown in Figure 1, the HARQ-ACK information of the 4th SPS PDSCH can be transmitted while the HARQ-ACK information of the first three SPS PDSCHs cannot be transmitted since the corresponding PUCCH is overlapped with DL symbols. If a UE follows the Rel-15 rule, i.e., dropping the PUCCH on DL symbols, then gNB cannot retransmit the first three SPS PDSCHs. Overall, a SPS PDSCH configuration depends on DL/UL configuration.

- Observation 1: For TDD, if the SPS periodicity becomes smaller than the DL/UL configuration periodicity, a UE may not transmit HARQ-ACK information for a SPS PDSCH reception depending on DL/UL configurations.

Thus, it is needed to define a new rule to transmit HARQ-ACK information of multiple SPS PDSCH receptions in a single PUCCH according to of DL/UL configurations.

Figure 1. HARQ-ACK transmission for shorter SPS periodicity in TDD case

To address this issue, several options can be considered. First, multiple numbers of PDSCH-to-HARQ_feedback timing indicator values can be introduced. For example, in Figure 1, PDSCH-to-HARQ_feedback timing indicator value for the first SPS PDSCH is indicated as 9 slots, and PDSCH-to-HARQ_feedback timing indicator value for the second SPS PDSCH is indicated as 7 slots, so on. This way can provide full flexibility but results in DCI overhead increase. The second option is to defer HARQ-ACK information until the valid PUCCH resource is found. For example, in Figure 1, the HARQ-ACK information for the first three SPS PDSCHs is deferred and is transmitted on the valid PUCCH resource of the 4th SPS PDSCH reception. The merit of the second option is to multiplex HARQ-ACK information of multiple SPS PDSCH receptions without additional signalling. Since the valid PUCCH resource can be determined by both the DL/UL configuration and the activation DCI, there would be no ambiguity to determine the valid PUCCH resource.

- Proposal 1: We propose to support deferring HARQ-ACK information for SPS PDSCH receptions until the valid PUCCH resource for transmitting HARQ-ACKs of SPS PDSCH receptions is found.

- HARQ-ACK for Multiple SPS PDSCH Configurations

In Rel-15, a single SPS PDSCH configuration is supported, but in Rel-16 URLLL, up to 8 or 16 SPS PDSCH configuration can be configured on a BWP in a cell. If more than one SPS PDSCH configurations are activated, we need to define how to generate HARQ-ACK codebook for the more than one SPS PDSCHs.

Semi-static HARQ-ACK codebook

In case of semi-static HARQ-ACK codebook, a PDSCH occasion for a SPS PDSCH is already defined in Rel-15 and thus HARQ-ACK information for SPS PDSCHs can be included in the semi-static HARQ-ACK codebook by extending the construction rule to cover multiple SPS PDSCHs. However, since it was agreed in the last RAN1#98bis meeting to support joint release of multiple SPS PDSCH configurations, we need to further discuss how to transmit HARQ-ACK information for joint release of SPS PDSCH configurations. In Rel-15, the HARQ-ACK position in the semi-static HARQ-ACK codebook corresponding to a SPS PDSCH release is the same as the position corresponding to SPS PDSCH reception. In case of joint SPS release, there are multiple corresponding SPS PDSCH receptions. If HARQ-ACK bits for corresponding SPS PDSCH receptions are used to indicate ACK/NACK for reception of joint SPS release, HARQ-ACK for different PDSCHs sharing the same PDSCH occasions cannot be transmitted. To resolve such a problem, the possible approaches can be considered:

- Alt 1. a location in the semi-static HARQ-ACK codebook for HARQ-ACK information corresponding to a joint SPS PDSCH release is the same as a location corresponding to SPS PDSCH reception with lowest ID.

- Alt 2. a location in the semi-static HARQ-ACK codebook for HARQ-ACK information corresponding to a joint SPS PDSCH release is indicated by TDRA field in SPS PDSCH release.

In Alt 1, one SPS PDSCH is selected from multiple SPS PDSCH released by joint SPS release. For simplicity, lowest SPS PDSCH ID can be used. Alt 2 provides more flexibility by indicating the location in TDRA field. However, the TDRA field size can be configured to 0 bit so that it is not a complete solution. Therefore, we propose

- Proposal 2: In case of semi-static HARQ-ACK codebook, a location in the semi-static HARQ-ACK codebook for HARQ-ACK information corresponding to a joint SPS PDSCH release is same as a location corresponding to SPS PDSCH reception with lowest ID.

Dynamic HARQ-ACK codebook

In Rel-15, in case of dynamic HARQ-ACK codebook, first HARQ-ACK bits for dynamic grant PDSCHs are generated according to counter-DAI or total DAI and then single HARQ-ACK bit for SPS PDSCH is added if exists. As a direct extension of Rel-15 dynamic HARQ-ACK codebook, generating HARQ-ACK bits for more than one SPS PDSCHs can be considered. To generate HARQ-ACK bits for more than one SPS PDSCHs, it is necessary to discuss the bit ordering for the SPS PDSCHs as follows:

- Opt 1. Ascending order of SPS PDSCH configuration ID

- Opt 2. Ascending order of time domain resource assignment of SPS PDSCHs

- Opt 3. Ascending order of HARQ process number of SPS PDSCHs

In option 1, SPS PDSCH configuration ID is used to determine bit ordering. If SCS of UL carrier is lower than that of DL carrier (e.g. DL: 30kHz and UL: 15kHz) and periodicity of SPS PDSCH configuration is 1 slot, there are more than one SPS PDSCH receptions with the same ID, which are multiplexed into the same HARQ-ACK codebook, as shown in Figure 2. In option 2, bit ordering is determined by time domain resource assignment of SPS PDSCHs. If two SPS PDSCH receptions starts the same symbol and ends the same symbol (i.e. two SPS PDSCH receptions are overlapped in the same symbol), it is not possible to determine bit ordering. The last option is to use HARQ process number of SPS PDSCHs. Even if there are more than one SPS PDSCH receptions corresponding to a HARQ-ACK codebook, their HARQ process number may be different. If two SPS PDSCH receptions have the same HARQ process number, a UE combines two PDSCH receptions and generate one HARQ-ACK. Thus, considering one HARQ-ACK for one HARQ process number, bit order of SPS PDSCHs can be uniquely determined.

Figure 2. SPS PDSCH examples

Note that when SPS configurations among multiple cells are activated, the cell index can be additionally used to determine bit ordering.

- Proposal 3: In case of dynamic HARQ-ACK codebook, bit ordering of SPS PDSCH receptions should be determined. The potential solutions are

  - Opt 1. Ascending order of SPS PDSCH configuration ID

  - Opt 2. Ascending order of time domain resource assignment of SPS PDSCHs

  - Opt 3. Ascending order of HARQ process number

- Conclusion

In this contribution, we discussed the HARQ-ACK transmission and codebook construction rule for multiple SPS configurations.

- Observation 1: For TDD, if the SPS periodicity becomes smaller, a UE may not transmit HARQ-ACK information for a SPS PDSCH reception depending on DL/UL configurations.

- Proposal 1: For TDD, we propose to support deferring HARQ-ACK information for SPS PDSCH receptions until the valid PUCCH resource for transmitting HARQ-ACKs of SPS PDSCH receptions is found.

- Proposal 2: In case of semi-static HARQ-ACK codebook, a location in the semi-static HARQ-ACK codebook for HARQ-ACK information corresponding to a joint SPS PDSCH release is same as for a corresponding SPS PDSCH reception with lowest ID.

- Proposal 3: In case of dynamic HARQ-ACK codebook, bit ordering of SPS PDSCH receptions should be determined. The potential solutions are

  - Opt 1. Ascending order of SPS PDSCH configuration ID

  - Opt 2. Ascending order of time domain resource assignment of SPS PDSCHs

  - Opt 3. Ascending order of HARQ process number

References

- RP-190728, “New WID: Support of NR Industrial Internet of Things (IoT),” RAN#83

- RAN1 Chairman’s Notes, RAN WG1 Meeting #98bis