3GPP TSG RAN WG1 #99		R1-1913096

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	6.2.1.7

Source: 	Sony

Title:	Summary of the use of RSS for measurement improvements

Document for:	Discussion / decision

# Introduction

In RAN1#98bis, we made the following agreements:

Agreement

The RSS Frequency Location function is as follows:

- Possible RSS Frequency Locations can only be within legacy Rel-13 narrowbands

- A RSS Frequency Location does not span two narrowbands.

- In each legacy narrowband, there are 3 non-overlapping RSS Frequency Locations

- Network can configure a subset of narrowbands to contain possible RSS

  - The subset of narrowbands is common across all cells in the network

  - The total number of selected narrowbands that can contain possible RSS is NNB

- The RSS Frequency Location function:

  - IRSS = PCID MOD (3NNB)

  - Where IRSS is the index of possible RSS Frequency Location starting with the lowest location

Agreement

- For the configuration of the NNB narrowbands, following is supported

  - A bitmap to indicate the NNB narrowbands. The narrowbands belonging to the central 6 PRBs are excluded.

    - By default, all narrowbands, except for the narrowbands belonging to the central 6 PRBs, are selected

- A one-bit indicator indicating RSS colocation (time and frequency domain) in all cells

Agreement

For the RSS Time Offset ORSS, the RSS time offset is distributed across MRSS as a function of PCID.  The RSS Time Offset function is:

- ORSS = PCID/(3NNB) MOD MRSS

- NOTE: Actual Time Offset (in SFN radio frames) = ORSS × GRSS

Where, the granularity of each unit of GRSS = PRSS / (10 MRSS),  where GRSS is configurable and is common across all cells in the network

- FFS: value for GRSS

Agreement

For each neighbor cell in the Neighbour Cell List:

  - Use 3 bits to signal a RSS Power Bias relative to Q_offset, where 1 state is used to indicate that RSS is not used for that neighbor cell. FSS the range

Agreement

Use RSS of neighbour cells for measurement improvement in Connected Mode:

- Signalling of RSS parameters for UE reported neighbour cells that are NOT in the Neighbour Cell List

- FFS: Handling of potential mismatch of UE measurement gaps and RSS periodicity

Agreement

Introduce a time shift RSS, within two consecutive ORSS steps, so that the actual time offset can be shifted by RSS radio frames, i.e.:

  - The Actual Time Offset = (ORSS × GRSS) +  RSS

  - The value RSS can be determined by the UE from the ORSS of the serving cell

The full set of agreements related to this work item objective are included at the end of this document (section 5).

This contribution summarises the remaining issues raised in [1] – [7] on the use of RSS for measurement improvements.

# Discussions

For reference, the following are the agreed RSS parameters required for neighbor cell measurement:

Table 1: RSS parameters for neighbour cell measurements

| RSS Parameter | Number of Bits | Comments |
| --- | --- | --- |
| Periodicity, PRSS | Not signalled | Common across network |
| Duration | Not signalled | Blind decoded by UE |
| Frequency Location | Max 16 bits (20 MHz system bandwidth). Common across network. | Bitmap of NNB narrowbands is signalled. UE derives location from NNB and PCID |
| Time Offset | FFS bits. | Granularity GRSS is signalled. UE derives offset from NNB, GRSS & PRSS (RSS periodicity). GRSS is common across network |
| Power Bias | 3 bits per neighbour cell | Power bias relative to Qoffset. Values are FFS. |

## RSS Time Offset Granularity

The RSS Time Offset is a function of the PCID and a signalled RSS Time Offset Granularity GRSS, where the GRSS values are FFS.

Four companies [1], [2], [3], [4] proposed that the range of GRSS values depend on the RSS periodicity PRSS, where in [4], it is further proposed that GRSS also depends on the periodicity of the measurement gap (MG).  One company [5] proposed a GRSS range that is independent of PRSS.  Another company [6] proposed that the network signals the number of RSS Time Offset stops MRSS instead of GRSS since the UE can work out the GRSS from PRSS & MRSS.  Table 2 summarises the proposed GRSS values.

Table 2: GRSS values & number of signalling bits

| Company | PRSS=160ms | PRSS =320ms | PRSS =640ms | PRSS =1280ms | Number of Bits |
| --- | --- | --- | --- | --- | --- |
| Sony [1] | {1, 2, 4, 8, 16} | {1, 2, 4, 8, 16, 32} | {2, 4, 8, 16, 32, 64} | {4, 8, 16, 32, 64, 128} | 3 |
| E/// [2] | {1, 2, 4, 8} | {1, 2, 4, 8} | {2, 4, 8, 16} | {4, 8, 16, 32} | 2 |
| QC [3] | {1, 4, 8, 16} | {1, 4, 8, 32} | {2, 4, 8, 64} | {4, 8, 16, 128} | 2 |
| HW [4] | {4, 8, 16} for MG=40ms {8, 16} for MG =80ms | {4, 8, 16, 32} for MG=40ms {8, 16, 32} for MG =80ms | {4, 8, 16, 32, 64} for MG=40ms {8, 16, 32, 64} for MG =80ms | {4, 8, 16, 32, 64, 128} for MG=40ms {8, 16, 32, 64, 128} for MG =80ms | 1 to 3 |
| ZTE [5] | {4, 8 16} |  |  |  | 2 |
| Nokia [6] | MRSS = {1, 2, 4, 8, 16, 32, 64} |  |  |  | 3 |

Since majority companies proposed that the range of GRSS values depends on the PRSS, we can use this approach. It is noted that Sony [1] range of GRSS values cover all the values proposed by other companies [2], [3], [4], [5] and that the total number of bits used to cover all these values is just 3 bits, i.e. at most 1 bit more than the most of the other proposals.  It should be noted that only 3 bits are used even though the exact GRSS values are different for different PRSS.  Huawei noted the number of GRSS can be reduced for smaller periodicities, e.g. {4, 8, 16} for 160 ms and {4, 8, 16, 32} for 320 ms, which uses a reduced 2 bits signalling for these periodicities.  The feature lead proposes to use the most extensive range of values for GRSS to give full flexibility to the network at the cost of only 1 extra bit.

Proposal 1: RSS Time Offset granularity GRSS is signalled using a total of 3 bits, where the values of GRSS depends on the RSS periodicity PRSS as follows:

- GRSS = {1, 2, 4, 8, 16} frames for PRSS = 160 ms

- GRSS = {1, 2, 4, 8, 16, 32} frames for PRSS = 320 ms

- GRSS = {2, 4, 8, 16, 32, 64} frames for PRSS = 640 ms

- GRSS = {4, 8, 16, 32, 64, 128} frames for PRSS = 1280 ms

## RSS Power Bias

The RSS Power Bias is agreed to be signalled using 3 bits per neighbour cell, where one of the state indicates that RSS is not used for measurement for that neighbour cell.  Since there number of possible values is limited, the range of RSS Power Bias need to balance between being able to cover full range of possible RSS Power Bias against having finer granularity between different RSS Power Bias values.  Three companies [1], [2], [3] made proposals on the range of RSS Power Bias and they are as follows:

- Option 1: {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}: Sony [1], E/// [2]

- Option 2: {-4, -3, -1.5, 0, 1.5, 3, 6, RSS Not Used}: QC [3]

Option 1 has wider range but coarser granularity of RSS Power Bias values while Option 2 has finer granularity but narrower range of RSS Power Bias values.  Since there is a slight majority for Option 1, i.e. preference for a wider range of RSS Power Bias, it is proposed to use the values in Option 1.  LG suggested to have a higher minimum RSS Power Bias value.  We can consider the following proposal as a start and decide online whether a higher minimum value is required.

Proposal 2: The RSS Power Bias values are {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}, where “RSS Not Used” indicates that the RSS of that neighbour cell is not used for measurements.

## RSS Measurement for Neighbour Cells not in Neighbour Cell List

Three companies [1], [2], [3] discussed on the use RSS of neighbour cells NOT in the Neighbour Cell List for measurements in Idle Mode.  The proposals are as follows:

- Option A: Signal a Default RSS Power Bias from set {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}, where “RSS Not Used” indicates the RSS for neighbour cells not in Neighbour Cell List is not used for measurement and assume Qoffset = 0: Sony [1]

- Option B: The UE calculate an RSS Power Bias based on whether the PresenceAntennaPort1 IE is set as follows: E///[2]

  - If PresenceAntennaPort1 is set, derive 2 power biases, one for 2 CRS port and another for 4 CRS port and then take the average of these 2 power biases

  - If PresenceAntennaPort1 is NOT set, derive 3 power biases, for 1 CRS port, 2 CRS port and 4 CRS port and then take the average of these 3 power biases

- Option C: Up to UE implementation: QC [3], LG

It should be noted that RSS for neighbour cells not in the Neighbour Cell List may not be suitable for measurement or in some scenario, RSS are not configured for majority of the neighbour cells not in the Neighbour Cell List.  Even if we use Option C, it may be useful to indicate to the UE whether it can attempt to use RSS of neighbour cells that are NOT in the Neighbour Cell List.  Hence it is proposed to at least provide such an indication.

Proposal 3: Introduce an indicator to indicate whether RSS of neighbour cells that are NOT in the Neighbour Cell List can be used for measurements.

It is noted in [3] that the Number of CRS Ports alone does not give sufficient information for UE to use RSS for RSRP measurements.  Hence, Option B may need to further assume values for parameter PB and RSS Power Bosst to measure RSRP based on RSS.  It is proposed to discuss and consider one of these proposed options:

Proposal 4: Select one of the following options in RAN1#99, for using RSS of neighbour cells that are NOT in the Neighbour Cell List for measurements in Idle Mode:

- Option A: Signal a Default RSS Power Bias from set {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}, where “RSS Not Used” indicates the RSS for neighbour cells not in Neighbour Cell List is not used for measurement and assume Qoffset = 0

- Option B: The UE calculate an RSS Power Bias based on whether the PresenceAntennaPort1 IE is set as follows:

  - If PresenceAntennaPort1 is set, derive 2 power biases, one for 2 CRS port and another for 4 CRS port and then take the average of these 2 power biases

  - If PresenceAntennaPort1 is NOT set, derive 3 power biases, for 1 CRS port, 2 CRS port and 4 CRS port and then take the average of these 3 power biases

- Option C: Up to UE implementation

## RSS Duration

The RSS duration of a cell can be {8, 16, 32, 40} subframes.  There was no agreement in previous meetings on whether to signal the RSS duration for each cell in the Neighbour Cell List.  It should be noted that without any signalling the UE will blind detect the RSS duration of each neighbour cell, which may lead to increased UE complexity.  Two companies [2], [3] addressed this issue with the following proposals:

- The RSS duration is not signalled: E/// [2]

- The RSS duration {8, 16, 32, 40} subframes, is signalled using 2 bits for each neighbour cell in the Neighbour Cell List: QC [3], LG

There isn’t sufficient discussion on RSS duration and it is proposed that RAN1 decide whether to signal the RSS duration or not.

Proposal 5: Discuss and decide whether to signal the RSS duration with values {8, 16, 32, 40} subframes, using 2 bits for each neighbour cell in the Neighbour Cell List.

## RSS & Measurement Gap Alignment

In RAN1#98bis, it is recognised that in Connected Mode, there may not be any RSS transmission in some of these Measurement Gaps since the Measurement Gaps’ periodicities are {40, 80} ms.  Five companies [2], [4], [5], [6], [7] provided their views as follow:

- Introduce shorter RSS periodicities, ≤ 80ms or longer Measurement Gap periodicities ≥160 ms: LG [7]

- No additional RSS or Measurement Gap values: E/// [2], HW [4], ZTE [5], Nokia [6]

  - It is noted that some of the RSS Time Offset Granularity values GRSS are sufficient to provide some alignment between RSS and the Measurement Gaps: E/// [2], HW [4]

It is therefore proposed that no further optimisation is required to align RSS and Measurement Gaps.

Proposal 6: No further optimisation is required to align RSS with Measurement Gaps in Connected Mode.

# Conclusion

Based on the contributions [1] – [7], the feature lead proposes the following:

Proposal 1: RSS Time Offset granularity GRSS is signalled using a total of 3 bits, where the values of GRSS depends on the RSS periodicity PRSS as follows:

- GRSS = {1, 2, 4, 8, 16} frames for PRSS = 160 ms

- GRSS = {1, 2, 4, 8, 16, 32} frames for PRSS = 320 ms

- GRSS = {2, 4, 8, 16, 32, 64} frames for PRSS = 640 ms

- GRSS = {4, 8, 16, 32, 64, 128} frames for PRSS = 1280 ms

Proposal 2: The RSS Power Bias values are {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}, where “RSS Not Used” indicates that the RSS of that neighbour cell is not used for measurements.

Proposal 3: Introduce an indicator to indicate whether RSS of neighbour cells that are NOT in the Neighbour Cell List can be used for measurements.

Proposal 4: Select one of the following options in RAN1#99, for using RSS of neighbour cells that are NOT in the Neighbour Cell List for measurements in Idle Mode:

- Option A: Signal a Default RSS Power Bias from set {-6 dB, -3 dB, 0 dB, 3 dB, 6 dB, 9 dB, 12 dB, RSS Not Used}, where “RSS Not Used” indicates the RSS for neighbour cells not in Neighbour Cell List is not used for measurement and assume Qoffset = 0

- Option B: The UE calculate an RSS Power Bias based on whether the PresenceAntennaPort1 IE is set as follows:

  - If PresenceAntennaPort1 is set, derive 2 power biases, one for 2 CRS port and another for 4 CRS port and then take the average of these 2 power biases

  - If PresenceAntennaPort1 is NOT set, derive 3 power biases, for 1 CRS port, 2 CRS port and 4 CRS port and then take the average of these 3 power biases

- Option C: Up to UE implementation

Proposal 5: Discuss and decide whether to signal the RSS duration with values {8, 16, 32, 40} subframes, using 2 bits for each neighbour cell in the Neighbour Cell List.

Proposal 6: No further optimisation is required to align RSS with Measurement Gaps in Connected Mode.

# References

[1] R1-1912335, “Remaining issues in using RSS for neighbour cell measurements,” Sony, RAN1#99

[2] R1-1911836, “Use of RSS for measurement improvements in LTE-MTC,” Ericsson, RAN1#99

[3] R1-1912683, “Measurements based on RSS,” Qualcomm Incorporated, RAN1#99

[4] R1-1911923, “Considerations on RSS for measurement improvements,” Huawei, HiSilicon, RAN1#99

[5] R1-1912415, “Remaining issues on use of RSS for measurement improvement,” ZTE, RAN1#99

[6] R1-1911986, “Use of RSS for measurement improvements,” Nokia, Nokia Shanghai Bell, RAN1#99

[7] R1-1912381, “Use of RSS for measurement improvements,” LG Electronics, RAN1#99

# Past Agreements

RAN1#94:

R1-1809503 Summary on the use of RSS for measurement improvements Sony Agreement From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility - RAN1 to identify the related parameters in the next RAN1 meeting

RAN1#94bis:

R1-1811641 Summary on the use of RSS for measurement improvements Sony Agreement In using RSS for measuring RSRP of neighbour cells, the following parameters are signalled to the UE for each cell: - ce-rss-periodicity-config: RSS periodicity {160, 320, 640, 1280} ms - ce-rss-duration-config : RSS duration {8, 16, 32, 40} subframes - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames - ce-rss-powerBoost-config : RSS power offset relative to LTE CRS {0, 3, 4.8, 6} dB Agreement The parameters related in the use of RSS for measuring RSRP of cells are signalled in the SI. Agreement Send an LS to RAN4: - Request to evaluate the benefits and feasilibity of using RSS for RSRP measurements compared to using LTE CRS - For measurement purposes, assess the benefits and feasibility of linking RSS to CRS LS is endorsed in R1-1811934

RAN1#96:

R1-1901486 LS on RSS based measurements signalling RAN2, Huawei R1-1903521 Draft Reply LS on RSS based measurements signaling Ericsson Agreement The draft LS is approved in R1-1903659 with the following modified text: - RSS periodicity can be considered as a carrier specific parameter, hence it may be omitted from neighbor cell RSS configuration signaling. - RAN1 expects most cells to have common or similar configurations for most parameters. Parameter commonality or similarity between serving cell and neighbor cell(s) may be exploited to reduce the required number of bits. - RAN1 will study whether and how to reduce the overhead from the two most expensive parameters, time offset and frequency location. Depending on the study, this could be done, e.g., by making one or both of them carrier specific or by reducing their resolution.

RAN1#96bis:

R1-1905459 Summary of the use of RSS for measurement improvements Sony R1-1905584 Follow up summary of the use of RSS for measurement improvements Sony Agreement Prioritize future RAN1 work on the following neighbor cells’ RSS parameters when reducing the number of signaling bits: - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames For further discussion until RAN1#97 Further evaluate which RSS parameters are likely to be common and/or similar between camped cell and neighbor cell(s). For those RSS parameters that are not common and/or similar discuss how they are signaled. - Including whether to send LS to RAN2 on RAN1 conclusion on which RSS parameters are likely to be common and/or similar. Agreement Support the indication of the number of CRS ports for each neighbor cell in signaling the neighbor cells’ RSS parameters. For further study Consider until RAN1#97 the following options for reducing the signaling bits of the neighbor cells’ RSS frequency location and RSS time offset: - Option 1: Reduce the signalling resolution; and then UE searches for exact RSS location - The reduced signalling resolution is FFS - Option 2: Restrict the RSS locations by reducing the number of possible RSS locations - The exact restriction method is FFS Either Option 1 or 2 can be complemented by other methods.

RAN1#97:

R1-1907071 Summary of the use of RSS for measurement improvements Sony Working Assumption Restrict the RSS locations by reducing the number of possible RSS locations Agreement Down select and/or combine from the following in RAN1#98 for overhead reduction of the RSS time offset & RSS frequency location: - Method 1: Reduce the number of possible RSS time offsets and RSS frequency locations - Method 2: The RSS time offset & RSS frequency location is a function of Cell ID, i.e. no signaling bits required - Method 3: The restricted RSS locations are dependent upon the Cell ID - Method 4: The RSS time offset & RSS frequency location is carrier specific - Method 5: The RSS frequency location of neighbor cells are within a frequency block relative to the RSS frequency location of the serving cell, e.g. the frequency block can be a narrowband Other methods are not precluded For further discussion Introduce an indicator to indicate whether the RSS locations are from a restricted smaller set of possible RSS locations or from the full set of possible RSS locations for each neighbour cell. The exact signalling method is up to RAN2. For further discussion Study the impact of the UE assuming that the number of CRS ports is carrier specific.

RAN1#98:

R1-1908936 Summary of the use of RSS for measurement improvements Sony Agreement Confirm the following working assumption: - Restrict the RSS locations by reducing the number of possible RSS locations Agreement Methods to restrict the RSS locations: - The RSS time offset(s) & RSS frequency location(s) are a function of Cell ID - The Cell ID function is FFS For further study: - Consider reducing the overhead in signaling the number of CRS ports of the neighbours in using neighbour cells’ RSS for measurement improvements. - Consider ways to provide RSS parameters for UE detected neighbour cells.

RAN1#98bis:

R1-1910542 Summary of the use of RSS for measurement improvements Sony R1-1911388 Follow Up Summary of the use of RSS for measurement improvements Sony Agreement The RSS Frequency Location function is as follows: - Possible RSS Frequency Locations can only be within legacy Rel-13 narrowbands - A RSS Frequency Location does not span two narrowbands. - In each legacy narrowband, there are 3 non-overlapping RSS Frequency Locations - Network can configure a subset of narrowbands to contain possible RSS - The subset of narrowbands is common across all cells in the network - The total number of selected narrowbands that can contain possible RSS is NNB - The RSS Frequency Location function: - IRSS = PCID MOD (3NNB) - Where IRSS is the index of possible RSS Frequency Location starting with the lowest location Agreement - For the configuration of the NNB narrowbands, following is supported - A bitmap to indicate the NNB narrowbands. The narrowbands belonging to the central 6 PRBs are excluded. - By default, all narrowbands, except for the narrowbands belonging to the central 6 PRBs, are selected - A one-bit indicator indicating RSS colocation (time and frequency domain) in all cells Agreement For the RSS Time Offset ORSS, the RSS time offset is distributed across MRSS as a function of PCID. The RSS Time Offset function is: - ORSS = PCID/(3NNB) MOD MRSS - NOTE: Actual Time Offset (in SFN radio frames) = ORSS × GRSS Where, the granularity of each unit of GRSS = PRSS / (10 MRSS), where GRSS is configurable and is common across all cells in the network - FFS: value for GRSS Agreement For each neighbor cell in the Neighbour Cell List: - Use 3 bits to signal a RSS Power Bias relative to Q_offset, where 1 state is used to indicate that RSS is not used for that neighbor cell. FSS the range Agreement Use RSS of neighbour cells for measurement improvement in Connected Mode: - Signalling of RSS parameters for UE reported neighbour cells that are NOT in the Neighbour Cell List - FFS: Handling of potential mismatch of UE measurement gaps and RSS periodicity Agreement Introduce a time shift RSS, within two consecutive ORSS steps, so that the actual time offset can be shifted by RSS radio frames, i.e.: - The Actual Time Offset = (ORSS × GRSS) + RSS - The value RSS can be determined by the UE from the ORSS of the serving cell