3GPP TSG-RAN WG1 #99	R1-1913099

Reno, USA, November 18th – 22th, 2019

Agenda Item:	7.2.2.2.5

Source:	Panasonic

Title:	Wideband operation in NR unlicensed

Document for:	Discussion, Decision

# Introduction

- In this paper, we discuss the remaining issues for wideband operation, including relationship between PDCCH and LBT bandwidth, and in-carrier guard band.

# PDCCH transmission

In RAN1#98, some progress has been made regarding the relationship between PDCCH/CORESET configuration and LBT bandwidth for wideband operation. Specifically, the following conclusion/agreement were made.

Conclusion: The following are unchanged from Rel-15 for PDCCH. - The maximum number of monitored PDCCH candidates per slot and per serving cell. - The maximum number of non-overlapped CCEs per slot and per serving cell. - CCE-to-REG mapping rule and hashing function. Agreement: For CORESET configuration in a serving cell with carrier bandwidth greater than LBT bandwidth, - For the case where a CORESET is confined within a LBT bandwidth, the search space set configuration associated with the CORESET can have multiple monitoring locations in the frequency domain (per LBT bandwidth) - Send an LS to RAN2 informing them of this agreement and providing clarifications on the above if necessary - Note: For scenarios in which gNB transmits PDCCH/PDSCH on a single BWP if CCA is successful at gNB for the whole BWP, CORESET(s) need not all be confined within an LBT bandwidth, and no specification impact is foreseen

For the agreement, the motivation is to ensure the number of CORESETs does not exceed the limit (such as 3 per BWP for Rel-15), but at the same time to enable UE to monitor multiple LBT bandwidths (potentially, larger than 3) for PDCCH. Therefore, new configuration has been introduced to support duplicating one search space set into multiple frequency domain monitoring locations. The above intention becomes clearer by the follow-up agreement in RAN1#98bis, as shown below.

Agreement: For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth): - PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET. - Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET. - CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain. - Include this and the prior agreement on this issue in an LS to RAN2

The agreement is particularly useful before the DL burst is detected. After DL burst is detected, which LBT bandwidth(s) are available to the UE is indicated by GC-PDCCH according to the following agreement made in RAN1#98 as well.

Agreement: Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

In [1], we propose that before DL burst is detected, UE only performs 1 BD for GC-PDCCH or UE-specific PDCCH (which have aligned sizes) for every two symbols, consuming 7 BD budget per slot per LBT bandwidth. To increase the detection probability, such search space can be configured to be monitored over all LBT bandwidths. For a 80 MHz wideband carrier with 4 LBT bandwidths, the total BD consumption would be 7x4=28 BDs. Once DL burst is detected and UE is informed via GC-PDCCH regarding the set of available LBT bandwidths, PDCCH candidates are only monitored over the available bandwidths. It would be better that the BD budget over the unavailable LBT bandwidths can be reallocated to the available bandwidths in order to provide more scheduling flexibility. For example, if only LBT bandwidth #1 is available, total BD budget such as 44 can be allocated only to LBT bandwidth #1 (and at beginning of the slot for slot-based scheduling). If LBT bandwidths #1 and #2 are available, total BD budget 44 are shared among these two bandwidths. As the number of BD/CCE limitation is specified within a slot, the change of the number of BDs per LBT bandwidth is applied from the slot boundary after available LBT bandwidth is known.

Instead of monitoring PDCCH candidates only over the available bandwidth, another possibility is to let UE keep monitoring the unavailable LBT bandwidth at the mini-slot level, e.g. every two symbols in order for UE to use bandwidth as soon as available. Even for this option, the remaining BD budget can still be allocated among the available bandwidths.

Proposal 1: The number of BDs per LBT bandwidth can be varied from slot boundary depending on the number of available LBT bandwidths after DL burst is detected.

To realize such dynamic BD budget allocation, one way is to have multiple search space set configurations for each LBT bandwidth, and to activate/de-activate some configurations depending on the number of available LBT bandwidths. The drawback of such method is that it requires additional signalling to select which configurations to (de)activate. Moreover, the number of configurations may also be unnecessarily increased, resulting in increased RRC configuration overhead.

Alternatively and preferably, one search space set can be configured to have multiple frequency monitoring locations, and the overbooking of PDCCH candidates can be used to enable dynamic BD budget allocation. Overbooking is useful because the availability of LBT bandwidths are unknown at the time when multiple frequency monitoring locations are configured. Once the set of LBT bandwidths to be monitored is determined (e.g. based on indication of GC-PDCCH), the overbooked PDCCH candidates can be dropped to meet the BD/CCE limit.

Consider an example where two search space sets are configured. Set #1 (CSS) has 7 BDs, and set #2 (USS) has 15 BD candidates, in a cell with carrier bandwidth of 80 MHz (i.e. 4 LBT bandwidths). If overbooking is not allowed, on which set of LBT bandwidths the search space sets can be monitored should be decided by configuration. In this example of 2 sets, only two LBT bandwidths can be included if both sets are monitored, in order to meet 44 BD limit. However, the selected LBT bandwidths by the configuration are not necessarily free from the LBT procedure later. If they are not available after LBT procedure, no PDCCH candidate can be monitored.

On contrary, with overbooking, it is possible to monitor PDCCH candidate no matter which set of LBT bandwidths are free from LBT. This can be explained using the following Figure 1.

Figure 1. Example of overbooking during configuration and dropping during monitoring

As shown in Figure 1(a), both search space sets #1 and #2 are configured to be monitored over all LBT bandwidths, resulting in (7+15)x4 = 88 BDs (> 44 limit). However, depending on the bandwidth availability, some PDCCH candidates can be dropped to meet the BD limit. In Figure 1(b), only LBT bandwidth #4 is available, therefore, the resulting monitoring effort is 22 BD. On the other hand, as shown in Figure 1(c), all 4 LBT bandwidths need to be monitored (This could be either because all LBT bandwidths are available or because UE has not received GC-PDCCH indicating bandwidth availability, e.g. before DL burst detection). To meet the BD budget requirement, one option is to apply Rel-15 dropping rule but only within each active BWP, i.e., SS set with higher ID is dropped, which is the case in Figure 1(c). Another option is to drop low-priority LBT bandwidths. For example, all PDCCH candidates (from both SS sets) in non-prioritized LBT bandwidths are dropped.

Proposal 2: In case of search space set configuration associated with the CORESET has multiple monitoring locations in the frequency domain, overbooking of PDCCH candidates should be allowed.

- Dropping rules are applied to ensure the BD/CCE limits are met for the set of actually monitored LBT bandwidths.

- PDCCH candidates in non-prioritized LBT bandwidths are dropped, and/or SS set with higher ID is dropped

# In-carrier guard band

In-carrier (or called intra-carrier) guard band is defined as guard band between LBT bandwidths in a carrier, different from guard band at the edge of the carrier. Based on the RAN4 agreement [2], in-carrier guard-band is needed at least for a gNB to operate with Option 3 (Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB).

In previous meeting, RAN1#98bis, RAN1 has progressed the issue with the following agreement.

Agreement: The intra-carrier guard bands on a carrier can be semi-statically adjusted with an RB level granularity. The RAN4 minimum guard band requirements are used as the guard bands when no semi-static adjustment is applied. - The guard bands adjustments do not affect the already agreed restrictions on PUCCH resource allocation. - FFS: Whether and how to handle the case where the intra-carrier guard bands are part of a resource allocation

The remaining issue is captured in the FFS point, and will be discussed in this section.

One option is to have static guard bands at the edges of each LBT bandwidth, regardless of whether one PDSCH is scheduled across multiple contiguous LBT bandwidths. Figure 2 illustrates one such example. Then the frequency resource for allocation is defined by excluding the in-carrier guard band. There are two issues for this option. The first one is that the frequency resource utilization is compromised because those static guard bands can never be scheduled by gNB. The second issue is that if semi-static adjustment of in-carrier guard band is not configured, the amount of in-carrier guard band will be defined by RAN4 (as minimum guard band requirement) per the above agreement. By coupling the decision of resource allocation design (e.g. the starting point, and granularity of the frequency resource unit) in RAN1 with the amount of in-carrier guard band by RAN4 would not help the two working groups to perform their designs in parallel. Instead, it is preferable for RAN1 to come up with the resource allocation design which is applicable for any value of in-carrier guard band which may be defined later by RAN4.

Another option to configure dynamic in-carrier guard band only at the outer edges of the union of LBT bandwidths acquired by the gNB. Please see Figure 3 for an example. For this option, when configuring the LBT bandwidths, the frequency resources for each LBT bandwidth need not consider the presence of any in-carrier guard band. In other words, the frequency range for each LBT bandwidth can include the PRBs which could be potentially used for in-carrier guard band. Then the frequency domain occupancy indication (e.g. bitmap indication as agreed in RAN1#98) can be used as a guard band enabler. Once UE knows which LBT bandwidths are available and unavailable, the presence of the guard band will be known to the UE. In such way, the spectrum efficiency is improved. This option can also support that the frequency resource allocation can be designed without considering the existence of the in-carrier guard band. Once in-carrier guard band is needed, a pre-configured amount will be activated by frequency domain occupancy indication, and those REs will not be used for data mapping. Correspondingly, UE can correctly de-map the transmitted data by jointly considering the frequency domain resource allocation and frequency domain occupancy structure.

Proposal 3: The PRBs configured for intra-carrier guard band can be part of frequency resource allocation of PDCCH. Frequency domain occupancy indication in GC-PDCCH is used for the dynamic indication of the existence of intra-carrier guard band.

Figure 2. Static intra-carrier guard band 			  Figure 3. Dynamic intra-carrier guard band

# Conclusion

- Based on the discussion, we have the following proposal:

Proposal 1: The number of BDs per LBT bandwidth can be varied from slot boundary depending on the number of available LBT bandwidths after DL burst is detected.

Proposal 2: In case of search space set configuration associated with the CORESET has multiple monitoring locations in the frequency domain, overbooking of PDCCH candidates should be allowed.

- Dropping rules are applied to ensure the BD/CCE limits are met for the set of actually monitored LBT bandwidths.

- PDCCH candidates in non-prioritized LBT bandwidths are dropped, and/or SS set with higher ID is dropped

Proposal 3: The PRBs configured for intra-carrier guard band can be part of frequency resource allocation of PDCCH. Frequency domain occupancy indication in GC-PDCCH is used for the dynamic indication of the existence of intra-carrier guard band.

# Reference

[1] R1-1908423, DL signals and channels for NR-U, Panasonic, RAN1#98

[2] R1-1905930 (R4-1905209), LS reply on wideband carrier operation for NR-U, RAN4#90bis