3GPP TSG RAN WG1 Meeting #99	R1-1913145

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	6.2.2.2

Source: 	Sony

Title:	Remaining issues on preconfigured UL resources for NB-IoT

Document for:	Discussion and Decision

# Introduction

One of the objectives in NB-IoT release 16 is to improve UL transmission efficiency and/or UE power consumption by supporting transmission in preconfigured uplink resources (PUR) in idle and/or connected mode [1]. In RAN1 meeting #98bis [2], we discussed issues related to PUR L1 ACK for an application layer response message (for NB-IoT and eMTC). The outcome was sending an LS to RAN2 to ask the feasibility and benefit of the above PUR L1 ACK indication.

In this contribution, we discuss our view on L1 ACK for an application later response message for NB-IoT which is also applicable to eMTC.

# Discussion

The transmission in preconfigured uplink resource can be beneficial for the NB-IoT/eMTC UE. Some of the IoT applications may have periodic and predetermined uplink data transmission depending on the application. The overall application may be hosted in a cloud server, where the cloud server processes the application data that was sent in the uplink from the UE. In many cases, the UE may expect a response from the application layer (e.g. from the cloud server itself). Depending on the application, the response can be periodic or aperiodic.

According to the legacy procedure, when the radio network has received downlink data from the application layer (e.g. cloud server), the network will transmit paging to the UE. The UE will accordingly respond to the paging with contention based random access where a random access preamble is initially transmitted by the UE according to the allocated random access resources. The preamble will be responded to according to the random access response message from the network. The UE continues with an RRC connection setup message, and after the RRC connection setup complete message has been received, the downlink data can be communicated. In total, the legacy functionality can consume a lot of signaling overhead and consume a significant amount of energy in relation to the payload size. Furthermore, the paging interval can be relatively long, especially in an eDRX scenario. Hence, the application layer response may be significantly delayed. Here, we observe the benefit of having an application layer response together with PUR operation.

In order to support the application layer response, the UE needs an additional search space window at a certain time (also known as non-zero gap) after the reception of L1-ACK of PUR transmission. Within that search space window, the UE monitor NPDCCH for possible NPDSCH transmission.

Proposal 1: Support that L1-ACK indicates additional PUR search space for application layer response purpose.

Furthermore, the additional PUR search space can be used in one of two possible scenarios. In the first approach, the additional PUR search space indicates the NPDSCH (through allocation by NPDCCH) that contains the application layer response information to the UE. In the second approach, the additional search space contains a paging indication.  This is shown in Figure 1, where the additional PUR SS starts after a non-zero gap TGAP.  The PUR SS monitoring period is DRX to reduce power consumption.  The eNB can send a paging indication if there is an application layer response to the UE. Subsequently, if the UE receives a paging indication, it can be moved to connected mode and receive the application layer response. In order to support the above possible scenarios, the UE must be preconfigured on which scenario is applied for handling the additional PUR search space.

Figure 1: Sub-PO operation in connection to PUR operation

Proposal 2: The UE can be configured to receive in an additional PUR search space either (1) an NPDCCH to indicate NPDSCH (i.e. application layer response) or to receive a paging indication.

# Conclusions

In this contribution, the following proposals are made:

Proposal 1: Support that L1-ACK indicates additional PUR search space for application layer response purpose.

Proposal 2: The UE can be configured to receive in an additional PUR search space either (1) an NPDCCH to indicate NPDSCH (i.e. application layer response) or to receive a paging indication.

# References

- RP-191576 Revised WID: NB-IoT enhancements for LTE, June 2019

- RAN1 #98bis Chairman notes, Chongqing, China, October 2019