3GPP TSG-RAN WG1 Meeting #99	R1-1913191

# Reno, USA, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 10.11.2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7 Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more PSSCH resource configuration(s). A PSSCH resource configuration can be for transmission of PSSCH, as described in Subclause 7.1, or for reception of PSSCH, as described in Subclause 7.2 and can be associated with either sidelink transmission mode 1 or sidelink transmission mode 2.

## 7.1	UE procedure for transmitting the physical sidelink shared channel

[Each PSSCH transmission is associated with an PSCCH transmission].

That PSCCH transmission carries the 1st stage of the SCI associated with the PSSCH transmission; the 2nd stage of the associated SCI is carried within the resource of the PSSCH.

If the UE transmits SCI format [TBD] on PSCCH according to a PSCCH resource configuration in slot n and PSCCH resource m, then for the associated PSSCH transmission in the same slot

-	one transport block is transmitted with up to two layers;

-	 [The number of layers (ʋ ) is determined according to the TBD field in the SCI]

-	The set of consecutive symbols within the slot for transmission of the PSSCH is determined according to subclause 7.1.2.1;

-	The set of contiguous resource blocks for transmission of the PSSCH is determined according to subclause 7.1.2.2;

Transform precoding is not supported for PSSCH transmission.

### 7.1.1	Transmission schemes

[Only one transmission scheme is defined for the PSSCH and is used for all PSSCH transmissions.]

PSSCH transmission is performed with up to two antenna ports, antenna ports [1000-1001] as defined in subclause [8.2.4] of [4, TS 38.211].

### 7.1.2	Resource allocation

#### 7.1.2.1	Resource allocation in time domain

[ The UE shall transmit the PSSCH in the same slot as the associated PSCCH. ]

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are configured as not available for sidelink.

-	The UE shall not transmit PSSCH in symbols which are configured for use by PSFCH format TBD, if configured.

-	 [ The UE shall not transmit PSSCH in the last symbol of the slot, or the symbol immediately preceding the symbols which are configured for use by PSFCH format TBD, if configured. ]

#### 7.1.2.2	Resource allocation in frequency domain

The resource allocation unit in the frequency domain is the sub-channel.

[ The sub-channel assignment for sidelink transmission is determined using the TBD field in the associated SCI and the PSCCH resource index on which the associated 1st stage SCI is transmitted. ]

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

### 7.1.3	Modulation order, target code rate, redundancy version and transport block size determination

#### 7.1.3.1	Modulation order and target code rate determination

[IMCS  is given by the field [ MCS ] in SCI format [ TBD ].

The MCS table is determined as follows: one out of Table 5.1.3.1-1, Table 5.1.3.1-2, Table 5.1.3.1-3 according to resource pool configuration [ or field [TBD] in the SCI or higher layer indication].

The UE shall use IMCS and the MCS table determined according to the previous step to determine the modulation order (Qm) and Target code rate (R) used in the physical sidelink shared channel.]

#### 7.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

## 7.2	UE procedure for transmitting sidelink reference signals

### 7.2.1	CSI-RS transmission procedure

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter [ SlCsiReporting ];

-	when enabled, if the corresponding SCI by the UE triggers the SL CQI/RI reporting

### 7.2.2	PSSCH DM-RS transmission procedure

The UE selects the DM-RS time domain pattern out of the patterns configured for the resource pool on which the PSSCH is to be transmitted. [The selected DR-MS time domain pattern is indicated by the SCI format [TBD] associated with the PSSCH transmission.]

### 7.2.3	PT-RS transmission procedure

## 7.3	UE procedure for receiving the physical sidelink shared channel

[ For sidelink transmission mode 1, a UE upon detection of SCI format TBD on PSCCH can decode PSSCH according to the detected SCI format TBD, and associated PSSCH resource configuration configured by higher layers.]

For sidelink transmission mode 2, a UE upon detection of SCI format TBD on PSCCH can decode PSSCH according to the detected SCI format TBD, and associated PSSCH resource configuration configured by higher layers.

## 7.4	UE procedure for receiving reference signals

### 7.4.1	CSI-RS reception procedure

### 7.4.2	DM-RS reception procedure for RSRP computation

The UE measures SL-RSRP based on PSSCH DM-RS.

### 7.4.3	PT-RS reception procedure

## 7.5	UE procedure for channel state information

The CSI-RS defined in Subclause TBD of [4, TS 38.211] may be used for CSI computation.

CSI consists of Channel Quality Indicator (CQI) and Rank Indicator (RI). The CQI and RI are always reported together.

The UE shall derive CQI and RI as specified in section 5.2.2, with the following changes

-	PDSCH replaced by PSSCH

## 7.6	Interaction between NR sidelink and LTE sidelink

A UE that supports short-term time-scale TDM for in-device coexistence shall:

-	drop LTE sidelink transmission if it overlaps with the higher priority NR sidelink transmission or reception

-	drop LTE sidelink reception if it overlaps with the higher priority NR sidelink transmission

-	drop NR sidelink transmission if it overlaps with the higher priority LTE sidelink transmission or reception

-	drop NR sidelink reception if it overlaps with the higher priority LTE sidelink transmission

. The priority of the NR and LTE PSSCH is indicated by the “priority” field of the corresponding SCI. The priority of the LTE PSSS/SSSS/PSBCH transmission and reception is given by higher layer parameter LTESidelinkSSBPriority. The priority of the NR sidelink SSB transmission and reception is given by higher layer parameter NRSidelinkSSBPriority. The priority of the PSFCH is the same as the priority of the corresponding PSSCH.