3GPP TSG-RAN WG1 #99	R1-1913248

Reno, USA, November 18th – 22th, 2019

Agenda Item:	6.2.1.1

Source:	Qualcomm Incorporated

Title:	Feature lead summary of 6.2.1.1 UE group MWUS

Document for:	Discussion and Decision

# 1	Introduction

The latest agreements for group NWUS in RAN1#98bis, which can be considered for group MWUS are

Agreement

The configured WUS resources associated with same gap are consecutive in time/frequency domain.

- The WUS resources in the same time resource are consecutive to each other in the frequency domain.

- When the WUS resources are configured in two time resources, the WUS resources in first time resource are consecutive to a WUS resource in the second time resource.

Agreement

- Choose supported patterns for WUS resource configuration considering the location of legacy WUS resource if Rel-15 WUS is configured.

- No more than 3 bits are used for configuration of the pattern for WUS resource

  - FFS: Considering the location of legacy WUS resource if Rel-15 WUS is configured

Agreement

For group WUS resources sharing WUS gap, per default, the number of UE groups per resource is configured for all group WUS resources. Optionally, the number of UE groups for each WUS resource can be configured individually.

Agreement

The number of UE groups per WUS resource is 1, 2, 4, or 8

Agreement

The optional eDRX configurability and configuration values regarding the number of consecutive POs a WUS is associated same as in Rel-15 legacy WU

Agreement

Design pre-defined method to allow alternating UE group to monitor different WUS resources at different POs implicitly.

- Consider both cell-specific DRX cycle and UE-specific DRX cycle.

- At least the following parameters are used in the pre-defined method

  - H_SFN of current PO

  - DRX cycle

Agreement

The group WUS resource that may coincide with legacy WUS is assigned NIDResource=0 and the other group WUS resource are using NIDResource=1,2,or 3.

- NIDResource=0, 1,2,or 3 are used to differentiate different scrambling sequences for WUS sequence.

Agreement

The following working assumption is confirmed:

To differentiate WUS sequences in different WUS resources, 2-bit MSB of scrambling initialization c_init is supported

Agreement

Regarding group WUS configuration for different gap durations (DRX, short eDRX and long eDRX), the following applies:

- If eDRX long gap is configured but no group WUS configuration for that gap is provided, the eDRX short gap configuration shall be used if configured, otherwise the DRX configuration shall be used.

- If eDRX short gap is configured but no group WUS configuration for that gap is provided, the DRX gap configuration shall be used.

In RAN1#99 meeting, we have the following agreement for NB-IoT group WUS, which can be applied to MTC group WUS as well.

Agreement

RAN2 to determine the mapping between higher layer grouping and grouping used to determine UE group per resource.

This is to summarize the issues for the 6.2.1.1 providing potential proposals on UE group MWUS based on the views expressed by companies in the contributions listed in the references.

# 2	Discussion

## 2.1	WUS resource configuration

#2.1.1 Patterns for WUS resource multiplexing associated with same gap

Agreement

- Choose supported patterns for WUS resource configuration considering the location of legacy WUS resource if Rel-15 WUS is configured.

- No more than 3 bits are used for configuration of the pattern for WUS resource

  - FFS: Considering the location of legacy WUS resource if Rel-15 WUS is configured

The WUS resource configured for Rel-16 WUS should indicate the information for UE to find the configured Rel-16 WUS resource:

- Number of WUS resources for Rel-16 (M)

- WUS resource ID, including whether WUS resource 0 is configured for Rel-16 WUS or not when Rel-15 WUS is configured

  - If legacy WUS resource 0 is not shared with Rel-16 WUS, WUS resource ID 1, 2, 3 can be configured for Rel-16 WUS.

  - Otherwise, WUS resource ID 0, 1, 2, 3 can be configured for Rel-16 WUS resources.

- Location of each WUS resource

Case 1a: Rel-15 WUS is configured

- Legacy WUS resource 0 has freqLocation indication included in Rel-15 WUS configuration.

- Based on legacy WUS freqLocation, the other WUS resource location can be predefined.

- Max 24 patterns can be configured by using up to 8 patterns (3bits) together with 3 freqLocations of legacy WUS.

- Whether WUS resource ID 0 is shared by group WUS and legacy WUS is configurable.

  - QC, Sony, LGE consider the cases (in red) that #0 is not shared by Rel-16 WUS

  - E///, Nokia, ZTE, DCM haven’t considered it yet.

- Considering UE group hopping equation, we should select contiguous WUS resource ID when M>1.

  - Selections of non-contiguous WUS resource IDs are highlighted in yellow.

- Some companies consider pattern of 3-FDM WUS resources (highlighted in blue), but other companies have concern on the power boosting.

Table 1a Companies’ views of WUS resource patterns for Case 1a

| Company | Whether ID 0 is used | M=1 | M=2 | M=3 | M=4 | Total number of patterns |
| --- | --- | --- | --- | --- | --- | --- |
| Sony | Yes | ID 0 x 3 freqLocations | (0, 1) FDM x 3 freqLocations (0, 3) TDM x 3 freqLocations | (0, 1, 2) FDM+TDM x 3 freqLocations (0, 1, 3) FDM+TDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 3 freqLocations | 24 Assuming fixed order 3; 0. 2; 1. |
|  | No | ID 1 FDM with 0 x 3 freqLocations |  | (1, 2, 3) FDM+TDM x 3 freqLocations |  |  |
| LGE | Yes | ID 0 x 2 freqLocations | (0, 1) TDM x 2 freqLocations (0, 2) FDM x 3 freqLocations |  | (0, 1, 2, 3) 2FDM+2TDM x 2 freqLocations | 16 Assuming fixed order 1; 0. 3; 2. |
|  |  | ID 1 TDM with 0 x 2 freqLocations ID 2 FDM with 0 x 2 freqLocations | (2, 3) TDM x 2 freqLocations | (1, 2, 3) FDM+TDM x 2 freqLocations |  |  |
| QC | Yes | ID 0 x 3 freqLocations | (0, 1) FDM x 3 freqLocations | (0, 1, 2) FDM+TDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 3 freqLocations | 21 Assuming fixed order 2; 0. 3; 1. |
|  | No | ID 1 FDM with 0 x 3 freqLocations | (1, 2) x 3 freqLocations | (1, 2, 3) FDM+TDM x 3 freqLocations |  |  |
| E/// | Yes |  | (0, 1) FDM x 3 freqLocations (0, 1) TDM x 3 freqLocations | (0, 1, 2) FDM+TDM x 3 freqLocations (0, 1, 2) FDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 3 freqLocations | 15 |
| Nokia | Yes | ID 0 x 3 freqLocations | (0, 1) FDM x 4 freqLocations | (0, 1, 2) FDM+TDM x 3 freqLocations; (0, 1, 2) FDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 2 freqLocations (0, 1, 2, 3) 3FDM+1TDM x 3 freqLocations | 21 |
| ZTE | Yes | ID 0 x 3 freqLocations | 2 patterns of (0, 1) FDM x 2 freqLocations (0, 3) TDM x 3 freqLocations | 2 patterns of (0, 1, 2) FDM+TDM x 2 freqLocations | 2 patterns of (0, 1, 2, 3) 2FDM+2TDM x 2 freqLocations | 18 |
| DCM | Yes | ID 0 x 3 freqLocations | (0, 1) FDM x 3 freqLocations (0, 1) TDM x 3 freqLocations | (0, 1, 2) FDM+TDM x 3 freqLocations; (0, 1, 2) FDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 3 freqLocations | 18 |
| Majority | Yes | ID 0 x 3 freqLocations | (0, 1) x 3 freqLocations FFS TDM and/or FDM with ID 0 | (0, 1, 2) FDM+TDM x 3 freqLocations FFS (0, 1, 2) FDM x 3 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 3 freqLocations | 24 |
|  | No | ID 1 x 3 freqLocations FFS TDM and/or FDM with ID 0 | (1, 2) x 3 freqLocations | (1, 2, 3) FDM+TDM x 3 freqLocations | - |  |

Majority patterns of Table 1a

|  |  | WUS resource ID order |  |  |
| --- | --- | --- | --- | --- |
| M | WUS resource ID | 3; 0. 2; 1. | 2; 0. 3; 1. | 1; 0. 3; 2. |
| 1 | 0 | - | - | - |
| 1 | 1 | FDM with ID0 |  | TDM with ID0 |
| 2 | 0, 1 | FDM |  | TDM |
| 2 | 1, 2 | TDM | TDM+FDM with ID0 |  |
| 3 | 0, 1, 2 | (1, 2) TDM | TDM+FDM |  |
| 3 | 1, 2, 3 | TDM+FDM with ID0 |  |  |
| 4 | 0, 1, 2, 3 | TDM+FDM |  |  |

Proposal 1a: If Rel-15 WUS is configured, 2-bit freqLocation of legacy WUS is used to implicitly indicate the location of the WUS resource(s) for Rel-16 WUS and 3 bits are used to select the WUS resource ID for Rel-16 WUS as one of the following entries

- WUS resource ID 0

- WUS resource ID 1

  - FFS using TDM/FDM with WUS resource ID 0

- WUS resource ID 0, 1

  - FFS using TDM/FDM

- WUS resource ID 1, 2

  - FFS using TDM/FDM

- WUS resource ID 0, 1, 2 using up to 2-FDM and up to 2-TDM

- WUS resource ID 1, 2, 3 using up to 2-FDM and up to 2-TDM

- WUS resource ID 0, 1, 2, 3 using up to 2-FDM and up to 2-TDM

- FFS WUS resource ID 0, 1, 2 using FDM

Case 1b: Rel-15 WUS is not configured

- Max 8 patterns can be configured by using up to 3bits, including different number of WUS resources (M) and WUS resource ID (m=0,…M-1) as well as resource locations.

  - WUS resource ID 0 is used for group WUS.

- For UE group hopping equation, we should select contiguous WUS resource ID when M>1.

  - Selections of non-contiguous WUS resource IDs are highlighted in yellow.

Table 1b Companies’ views of WUS resource patterns for Case 1b

| Company | M=1 | M=2 | M=3 | M=4 | Total number of patterns |
| --- | --- | --- | --- | --- | --- |
| QC | ID 0 x 2 freqLocations | (0, 1) FDM x 2 freqLocations | (0, 1, 2) FDM+TDM x 2 freqLocations | (0, 1, 2, 3) 2FDM+2TDM x 2 freqLocations | 8 Assuming fixed order 2; 0. 3; 1. |
| Nokia | ID 0 x 1 freqLocations | (0, 1) FDM x 2 freqLocations | (0, 1, 2) FDM+TDM x 1 freqLocations; | (0, 1, 2, 3) 2FDM+2TDM x 1 freqLocations | 5 |
| ZTE | ID 0 x 3 freqLocations | 2 patterns of (0, 1) FDM x 2 freqLocations (0, 3) TDM x 3 freqLocations | 2 patterns of (0, 1, 2) FDM+TDM x 2 freqLocations | 2 patterns of (0, 1, 2, 3) 2FDM+2TDM x 2 freqLocations | 18 (+2bits for resource ID 0) |

Observation: If no additional bit is introduced, only 3bits can be used to support a subset of patterns when Rel-15 WUS is configured.

Proposal 1b: If Rel-15 WUS is not configured, [1bit or 2bits] is/are used to indicate the location of WUS resource(s) for Rel-16 and 2 bits are used to select the WUS resource ID for Rel-16 WUS as one of the following entries

- WUS resource ID 0

- WUS resource ID 0, 1

  - FFS using TDM/FDM

- WUS resource ID 0, 1, 2 using up to 2-FDM and up to 2-TDM

- WUS resource ID 0, 1, 2, 3 using up to 2-FDM and up to 2-TDM

2.1.2 WUS resource location in 6-PRB

- Based on WUS resource 0, 3 possible location for up to 4 WUS resources with 2-FDM and 2-TDM are

Table 2 Companies’ views of WUS resource patterns for Case 1a and Case 1b

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations | f; 0. t. | Alt1: f. 0. t. Alt2: f. 0. t. | f. 0. t. |
| If Rel-15 WUS is configured | Ericsson, Sony, Qualcomm, Nokia, NSB (for M=2 or 3) | Alt1 only: Sony, LGE Alt2 only: Nokia, NSB (for M=2, 3, 4), Qualcomm Alt2 and Alt2: ZTE Alt1 or Alt2 based on NB location: Ericsson | Ericsson, Sony, Qualcomm, Nokia, NSB, LGE |
| If Rel-15 WUS is not configured | Qualcomm | Alt2 only: Qualcomm |  |
| Observation | No objection | No clear majority view yet | No objection |

Proposal 2a: If Rel-15 WUS is configured, predefine the time/freq location for up to 4 WUS resources with 2-FDM and 2-TDM based on the 2-bit freqLocations of WUS resource 0 as

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations if Rel-15 WUS is configured | f; 0. t. | Alt1: f. 0. t. Alt2: f. 0. t. Down-select: - Alt1 only - Alt2 only - Alt1 if NB is below center carrier; otherwise Alt2 | f. 0; t. |

If 1-bit for WUS resource location is agreed in Proposal 1b, we need to define the details:

Proposal 2b: If Rel-15 WUS is not configured, predefine the time/freq location for up to 4 WUS resources with 2-FDM and 2-TDM based on 1-bit freqLocation of WUS resource 0 as n0 or n2

#2.1.3 WUS resource ID order.

- Predefined: Sony, Qualcomm, Ericsson, Nokia, NSB, LGE, ZTE, DCM

  - Alt1: WUS resource ID 0, 1 in same time location and 0, 2 in same freq location, e.g.,

    - Qualcomm, Nokia, NSB

| f | 2 | 0 |  |
| --- | --- | --- | --- |
|  | 3 | 1 | t |

  - Alt2: WUS resource ID 0, 1 in same time location and 0, 3 in same freq location, e.g.,

    - Sony, Ericsson, Nokia, NSB

| f | 3 | 0 |  |
| --- | --- | --- | --- |
|  | 2 | 1 | t |

  - Alt3: WUS resource ID 0, 1 in same freq location and 0, 2 in same time location, e.g.,

    - LGE, Ericsson (for M=2)

| f | 1 | 0 |  |
| --- | --- | --- | --- |
|  | 3 | 2 | t |

  - FFS: ZTE, DCM

Observation: Majority view on supporting a predefined order. But need down select the alternatives.

Proposal 3: For WUS resources with 2-FDM and 2-TDM, down select the alternatives for WUS resource ID mapping order.

- Alt1: WUS resource ID 0, 1 in same time location and 0, 2 in same freq location

- Alt2: WUS resource ID 0, 1 in same time location and 0, 3 in same freq location

- Alt3: WUS resource ID 0, 1 in same freq location and 0, 2 in same time location

## 2.2	Alternating UE groups among WUS resources associated with same gap

2.2.1 Method for alternating UE groups

Alt1: Resource hopping: alternating all UE groups per WUS resource together (change WUS resource ID only)

- Ericsson, ZTE, Qualcomm, Sony

Alt2: Group hopping: alternating only min number of UE groups but keep the same number of UE groups per WUS resource (change both WUS resource ID and WUS group ID in a WUS resource)

- Huawei, HiSi, Nokia, NSB, LGE

Observation:

- If legacy WUS is not configured common WUS, Alt1 is sufficient without changing the UE groups within a WUS resource and avoid negative impact on service-based UE group mapping within a WUS resource.

- If legacy WUS is configured as common WUS, Alt1 may have potential impact on legacy WUS UE. But even by using Alt2 with constant number of UE groups per WUS resource, it does not mean the number of UEs and the paging probability for Rel-16 WUS sharing with Rel-15 WUS will be constant.

Proposal 4: When legacy WUS is configured as common WUS and the UE groups are same service type, alternate minimum UE groups and keep same number of WUS groups per WUS resource; otherwise, alternate all UE groups per WUS resource together.

2.2.1 Equations for alternating UE groups

Equations for Alt1:

- Ericsson: WUSindexUE group=WUSindex+divSFN+1024HSFNTmodulus(NWUS)

- QC: m=m0+divSFN+1024H_SFNTmin+divSFN+1024HSFNTminLCMM,TcellTminmod M

  - WUS resource ID=m if legacy WUS resource 0 is used or m+1 if legacy WUS resource 0 is not used.

- ZTE: actual UE-group MWUS resource index = mod(configured UE-group MWUS resource index + offset, number of configured UE-group MWUS resource)

offset=Mn+floor((SFN+1024*H_SFN)/256), n=mod(floor((SFN+1024*H_SFN)/32),8)

| Number of UE-group MWUS resources | M |
| --- | --- |
| 4 | 0 2 1 3 2 1 3 0 |
| 3 | 0 2 1 0 2 1 0 2 |
| 2 | 0 1 1 0 1 0 0 1 |

Equations for Alt2:

Huawei, HiSi:

- Step 1: x=NIDGroup+ MminGroups·SFN + 1024*HyperSFNTmod MtotalGroups

  - M0Groups, M1Groups, …, MK-1Groups: configured number of UE groups at each WUS resource assuming K orthogonal WUS resources are configured

  - MminGroups= min⁡(M0Groups, M1Groups…, MK-1Groups): the minimum of configured number of UE groups of each WUS resource

  - MtotalGroups=M0Groups+M1Groups+…+MK-1Groups: the sum of configured number of UE groups of each WUS resource

- Step 2: Obtain WUS resource ID based on x and the “x-to WUS resource” mapping table in Table 2

Proposal 5a: Alternating all UE groups per WUS resource is based on

m=m0+divSFN+1024H_SFNTmod M

- M number of WUS resources for group WUS with M>1

- m0 with m0=0,…M-1 is based on initial WUS resource index as m0=NIDresource if NIDresource=0 is used for group WUS or m0=NIDresource-1, otherwise.

- T is the cell-specific DRX cycle measured in radio frames

Proposal 5b: Alternating minimum UE groups is based on

g=g0+ Gmin·divSFN+1024H_SFNTmod Gtotal

- g0={0,…, Gtotal-1} is the initial UE group index

- Gm is the number of UE groups of the m-th WUS resource

- Gtotal=m=0M-1Gm  and Gmin=minm=0,…M-1Gm.

- T is the cell-specific DRX cycle measured in radio frames

2.3.2 Enable/disable alternating UE group among M WUS resources

- Explicit 1bit: Ericsson, Qualcomm (only can be enabled when M>1), Sony

- Implicit by using M>1: LGE

Observations: No strong majority view yet. But there may be a concern that alternating UE group may not be able to be disabled if using implicit way. 1-bit explicit indication would be more clear.

Proposal 6: 1bit is used to enable/disable alternating UE group among more than one WUS resources for group WUS.

## 2.3	WUS sequence design for MWUS

2.3.1 Scrambling sequence of group WUS

Agreement

The following working assumption is confirmed:

To differentiate WUS sequences in different WUS resources, 2-bit MSB of scrambling initialization c_init is supported

However, the equation for c_init is not decided yet.

Alt1: same as NB-IoT equation

- cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell+NIDResource·229

  - QC, Huawei, HiSi

Alt2: different from NB-IoT equation

- cinit_WUS=(NIDNcellNIDcell+NIDresourcePNB mod 512+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell+NIDResource·229

  - Ericsson

Observation: Slight majority on Alt1

Proposal 7: The scrambling initialization c_init for Rel-16 WUS is

- cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell+NIDResource·229

2.3.2 MWUS sequence mapping in 2PRBs

- Flip the NB-IoT WUS sequence in the second PRB

  - Yes: Ericsson, Intel

  - No: Huawei, HiSi, ZTE

  - FFS: QC

Observations: Need further study.

Proposal 8: In the second PRB pair, the MWUS sequence w(m) is mapped from the highest subcarrier in decreasing order to the lowest subcarrier such that the complete sequence per symbol is [w(0), w(1), …, w(11), w(11), …, w(1), w(0)], assuming the NB-IoT WUS sequence is [w(0), w(1), …, w(11)].

## 2.4	Other issues related with RAN2 discussion

2.4.1 UE group ID configuration

- Whether joint UE group ID across WUS resources:

  - Yes: Huawei, HiSi

  - No: DCM

  - FFS: ZTE (RAN2 decision), QC

Observations: No majority view. But even if RAN2 decide joint UE group ID, we can define the mapping rule to determine WUS resource ID and WUS group ID for WUS sequence.

Proposal 9: RAN2 to determine the mapping between higher layer grouping and grouping used to determine UE group per resource.

2.4.2 Weighting factor for UE distribution

Whether to support weighting factor configuration for different WUS resources of Rel-16 WUS UEs

- Yes: QC, LGE, Nokia, NSB

- No: ZTE, DCM, Ericsson.

Observations: No clear majority view yet. Waiting for the progress of RAN2 discussion.

2.4.3 Coverage-based UE grouping

- LGE: Configure coverage-restricted UEs to monitor last X subframes in WUS resource

Observations: No clear majority view yet. Waiting for the progress of RAN2 discussion.

# 3	Summary

Potential proposals are summarized as follows:

For WUS resource configuration:

Proposal 1a: If Rel-15 WUS is configured, 2-bit freqLocation of legacy WUS is used to implicitly indicate the location of the WUS resource(s) for Rel-16 WUS and 3 bits are used to select the WUS resource ID for Rel-16 WUS as one of the following entries

- WUS resource ID 0

- WUS resource ID 1

  - FFS using TDM/FDM with WUS resource ID 0

- WUS resource ID 0, 1

  - FFS using TDM/FDM

- WUS resource ID 1, 2

  - FFS using TDM/FDM

- WUS resource ID 0, 1, 2 using up to 2-FDM and up to 2-TDM

- WUS resource ID 1, 2, 3 using up to 2-FDM and up to 2-TDM

- WUS resource ID 0, 1, 2, 3 using up to 2-FDM and up to 2-TDM

- FFS WUS resource ID 0, 1, 2 using FDM

Proposal 1b: If Rel-15 WUS is not configured, [1bit or 2bits] is/are used to indicate the location of WUS resource(s) for Rel-16 and 2 bits are used to select the WUS resource ID for Rel-16 WUS as one of the following entries

- WUS resource ID 0

- WUS resource ID 0, 1

  - FFS using TDM/FDM

- WUS resource ID 0, 1, 2 using up to 2-FDM and up to 2-TDM

- WUS resource ID 0, 1, 2, 3 using up to 2-FDM and up to 2-TDM

Proposal 2a: If Rel-15 WUS is configured, predefine the time/freq location for up to 4 WUS resources with 2-FDM and 2-TDM based on the 2-bit freqLocations of WUS resource 0 as

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations if Rel-15 WUS is configured | f; 0. t. | Alt1: f. 0. t. Alt2: f. 0. t. Down-select: - Alt1 only - Alt2 only - Alt1 if NB is below center carrier; otherwise Alt2 | f. 0; t. |

If 1-bit for WUS resource location is agreed in Proposal 1b, we need to define the details:

Proposal 2b: If Rel-15 WUS is not configured, predefine the time/freq location for up to 4 WUS resources with 2-FDM and 2-TDM based on 1-bit freqLocation of WUS resource 0 as n0 or n2

Proposal 3: For WUS resources with 2-FDM and 2-TDM, down select the alternatives for WUS resource ID mapping order.

- Alt1: WUS resource ID 0, 1 in same time location and 0, 2 in same freq location

- Alt2: WUS resource ID 0, 1 in same time location and 0, 3 in same freq location

- Alt3: WUS resource ID 0, 1 in same freq location and 0, 2 in same time location

For WUS UE group hopping:

Proposal 4: When legacy WUS is configured as common WUS and the UE groups are same service type, alternate minimum UE groups and keep same number of WUS groups per WUS resource; otherwise, alternate all UE groups per WUS resource together.

Proposal 5a: Alternating all UE groups per WUS resource is based on

m=m0+divSFN+1024H_SFNTmod M

- M number of WUS resources for group WUS with M>1

- m0 with m0=0,…M-1 is based on initial WUS resource index as m0=NIDresource if NIDresource=0 is used for group WUS or m0=NIDresource-1, otherwise.

- T is the cell-specific DRX cycle measured in radio frames

Proposal 5b: Alternating minimum UE groups is based on

g=g0+ Gmin·divSFN+1024H_SFNTmod Gtotal

- g0={0,…, Gtotal-1} is the initial UE group index

- Gm is the number of UE groups of the m-th WUS resource

- Gtotal=m=0M-1Gm  and Gmin=minm=0,…M-1Gm.

- T is the cell-specific DRX cycle measured in radio frames

Proposal 6: 1bit is used to enable/disable alternating UE group among more than one WUS resources for group WUS.

For WUS sequence:

Proposal 7: The scrambling initialization c_init for Rel-16 WUS is

- cinit_WUS=(NIDNcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDNcell+NIDResource·229

Proposal 8: In the second PRB pair, the MWUS sequence w(m) is mapped from the highest subcarrier in decreasing order to the lowest subcarrier such that the complete sequence per symbol is [w(0), w(1), …, w(11), w(11), …, w(1), w(0)], assuming the NB-IoT WUS sequence is [w(0), w(1), …, w(11)].

Other issues related with RAN2:

Proposal 9: RAN2 to determine the mapping between higher layer grouping and grouping used to determine UE group per resource.

# References

| R1-1911830 | Ericsson | Observation 1 Configuring WUS with multiple time resources will disqualify some DRX and short eDRX gap combinations. Observation 2 Explicit enabling of resource alternation may be preferable to an ill-suited implicit alternation rule. Observation 3 Alternating individual UE groups instead of all groups in a WUS resource will affect performance of service-based grouping. Observation 4 Two-fold repetition of a signal in the frequency domain increases PAPR of that signal up to 6 dB. Observation 5 Using different sequences in different paging narrowbands will substantially reduce PAPR. Observation 6 The effect from flipping the signal in one of the PRBs is that higher power boosting levels are possible without increased PAPR compared to a signal repeated over two PRBs. Proposal 1 Four WUS constellations are defined according to: a. A single group WUS resource, frequency multiplexed with legacy WUS, b. Two frequency multiplexed group WUS resources, in turn frequency multiplexed with legacy WUS, c. A down-selection between the following two alternatives: i. Two time multiplexed group WUS resources, in turn frequency multiplexed with legacy WUS, or ii. One group WUS time multiplexed with legacy WUS, d. Three group WUS resources, where each resource is either frequency multiplexed, time multiplexed or both frequency and time multiplexed with legacy WUS. Proposal 2 Within the six possible group WUS resources, resource indexation is performed according to Table 1. If legacy WUS is not configured, a Rel-16 freqLocation information element is used to indicate NIDresource = 0. Proposal 3 Explicitly indicate in SI if alternation is enabled or not. Proposal 4 UE groups in a resource share alternation pattern. Proposal 5 WUS resource alternation is performed according to: NIDresource(HSFN, SFN)=NIDresource+1024×HSFN+SFNTmod (Nmaxresource) Proposal 6 The scrambling initialization is determined from the following equation: cinit_WUS=NIDcell+NIDresourcePNB mod 512+110nf_start_PO+nstart_PO2+NIDresourcemod 2048+129+NIDcell+NIDresource229 Proposal 7 In the second PRB pair, the MWUS sequence w(m) is mapped from the highest subcarrier in decreasing order to the lowest subcarrier such that the complete sequence per symbol is [w(0), w(1), …, w(11), w(11), …, w(1), w(0)], assuming the NB-IoT WUS sequence is [w(0), w(1), …, w(11)]. Proposal 8 For common WUS, g = 0 is used unless for a resource where legacy WUS is configured, and the common WUS is configured to not be the legacy WUS, in which case g = 126 is the common WUS. |
| --- | --- | --- |
| R1-1911917 | Huawei, HiSilicon | Proposal 1: The UE group ID NIDGroup is jointly numbered across the configured WUS resources. Proposal 2: The UE group alternating method shall consider the case where the number of UE groups for each WUS resource is different since it is a common case, i.e., 98% of all combinations. Proposal 3: The UE group changing among WUS resources is supported via a 2-step mechanism as follows: - Step 1: Calculate a temporary variable x based on x=NIDGroup+ MminGroups·SFN + 1024*HyperSFNTmod MtotalGroups - M0Groups, M1Groups, …, MK-1Groups: configured number of UE groups at each WUS resource assuming K orthogonal WUS resources are configured - MminGroups= min⁡(M0Groups, M1Groups…, MK-1Groups): the minimum of configured number of UE groups of each WUS resource - MtotalGroups=M0Groups+M1Groups+…+MK-1Groups: the sum of configured number of UE groups of each WUS resource - NIDGroup∈{0,1,2,…,MtotalGroups-1}: UE group ID - SFN, HyperSFN: System Frame Number and Hyper-SFN of the current PO - T: cell-specific DRX cycle - Step 2: Obtain the WUS resource ID based on x and the “x-to WUS resource” mapping table in Table 2 Observation 1: UE group alternating among WUS resources aims to solve two unfairness: different false alarm probability, and different wake up time. Observation 2: Under the proposed 2-step mechanism (i.e., Proposal 3), all the UEs, no matter they are configured with cell or UE specific DRX cycle, will still have the same probability of being assigned to each WUS resource, thus achieving fairness among UE groups. |
| R1-1912409 | ZTE | Proposal 1: Up to 2 orthogonal resources including legacy MWUS resource may be configured in frequency domain. Proposal 2: 8 UE-group MWUS resource configurations should be supported. Proposal 3: UE group weighting is not supported. Proposal 4: The actual UE-group MWUS resource is determined by the following formula: actual UE-group MWUS resource index = mod(configured UE-group MWUS resource index + offset, number of configured UE-group MWUS resource) - offset = Mn+floor((SFN+1024*H_SFN)/256) - The value of Mn is shown as following table and n = mod(floor((SFN+1024*H_SFN)/32),8) Number of UE-group MWUS resources; M. 4; 0 2 1 3 2 1 3 0. 3; 0 2 1 0 2 1 0 2. 2; 0 1 1 0 1 0 0 1. Proposal 5: UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs. Proposal 6: The details of UE_group_index for determining UE-group MWUS sequence shall be determined in RAN2 |
| R1-1911980 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 studies whether the maximum power boost in each of the WUS resources should be limited to less than 4.8 dB when multiple orthogonal WUS resources are configured in the frequency domain. Proposal 2: Eight WUS resource patterns are supported, as illustrated in Figures 1 to 8, where each pattern defines the WUS resource locations both when the Rel-15 WUS is configured and when it is not configured, which may be different. Proposal 3: WUS resources excluding the legacy WUS are sequentially assigned IDs first in frequency (low to high) and then in time (starting in the WUS time period closest to the paging occasion and going backwards). Proposal 4: If the legacy WUS is configured in one of the resources, sharing of this resource with the Rel-16 WUS is configured through assignment of Rel-16 UE groups to WUS resources. Proposal 5: The network can separately enable or disable operation of Rel-15 WUS and Rel-16 UE-group WUS in each cell. Proposal 6: Support fallback to Rel-15 WUS in a cell if it is enabled and if Rel-16 WUS is disabled in the cell. Proposal 7: The Rel-16 WUS sequence assigned to a group depends on its group ID. Proposal 8: The number of UE groups associated with each resource is maintained when rotating each UE group among different WUS resources. Proposal 9: Use the highest value among the cell-specific and all the UE-specific DRX cycle lengths in the predefined method for WUS resource rotation. |
| R1-1912374 | LGE | Observation 1: UE in a normal coverage might be woken up more frequently due to the other UE paging of larger coverage. Observation 2: The Enhanced Coverage Restricted UE camps on a cell with normal coverage. So the Enhanced Coverage Restricted UE may require shorter WUS duration than maximum WUS duration. Observation 3: Information on the Enhanced Coverage Restricted is available for both UE and eNB. Observation 4: Unnecessary wake up due to the other UE paging might be occurred more frequently if a UE which shares same WUS resource and sequence have lower paging probability. Observation 5: Power consumption of UE may be affected by a UE with high paging probability which share same WUS resource and sequence. Proposal 1: To take advantage of parameter(s) configured by MME (e.g. the Enhanced Coverage Restricted and/or Paging probability information), consider following options: - Support different starting subframe and actual WUS duration candidates - Monitor WUS in the last [X] WUS subframes within the maximum WUS duration. - Here, X is equal to or less than the maximum number of WUS subframes in the WUS duration - Configure Different WUS resource(s) and/or sequence(s) - Turn off common WUS monitoring (Low paging probability information case only) Proposal 2: Relative location in terms of the frequency domain of the non-legacy WUS resource to the legacy WUS resource is fixed in any WUS resource allocation pattern. - The non-legacy WUS resource is in the top 2 PRBs if the legacy WUS is in the center 2 PRBs - The non-legacy WUS resource is in the center 2 PRBs if the legacy WUS is in the bottom 2 PRBs Proposal 3: Support 8 patterns in Figure 2 for UE group WUS resource allocation in MTC - Only the WUS resource 0 can be shared by the legacy WUS and the non-legacy WUS. Proposal 4: If WUS resource hopping is enabled, UE group alternates WUS resources in time domain first and then frequency domain second. Proposal 5: The resource ID of non-legacy WUS resources are determined based on its relative time/frequency location to the legacy WUS resource and it is same for all WUS resource allocation pattern. |
| R1-1912186 | Intel Corporation | Proposal 1: Support FDM of 3 WUS in the frequency domain if low-PAPR of WUS can be provided. Proposal 2: UE-group WUS sequence in the 2nd PRB of WUS resource is the same as in the 1st PRB with the reversed sequence mapping order onto subcarriers in the frequency domain. If changing order of the WUS sequence mapping is not supported, the WUS configuration pattern with 3 WUS resources multiplexed in the frequency domain should be excluded. |
| R1-1912332 | Sony | Observation1: The WUS grouping assignment to WUS resources should target to support fairness in paging performance such that blocking is avoided, reduced false wake-up, and low UE power consumption. Observation 2: Dynamic mapping to change WUS resource of a UE group from time-to-time can result in achieving fairness of power consumption. Observation 3: Dynamic mapping to change WUS resource of a UE group from time-to-time should supports mobility in WUS operation, particularly when the UE moves from one cell to another. Proposal 1: 4 Define the WUS resource reference numbering to facilitate the signalling of WUS resource pattern to the UE. Proposal 2: The legacy WUS freqLocation is used as the WUS resource reference (index #0) Proposal 3: 3 bits WUS pattern is used to represent the number of WUS resources and the used WUS resource(s) by an eNB. Proposal 4: The dynamic mapping of a WUS resource for a WUS group should be based on the initial WUS resource index (reference) and time reference number (e.g. SFN, DRX cycle). |
| R1-1913262 | Qualcomm Incorporated | Proposal 1: 3bits are used for Rel-16 WUS resource configuration: - 2bits for WUS resource number M for Rel-16 WUS - If Rel-15 WUS is enabled, - Legacy WUS freqLocation to indicate WUS resource location Pattern 1, 2 or 3 in Table 1. - 1bit to indicate whether NIDresource=0 is used for Rel-16 WUS - If Yes, NIDresource={m, m=0,…M-1} with M=1, 2, 3 or 4; - Otherwise, NIDresource=m+1, m=0,…M-1 with M=1, 2, or 3. - Otherwise, NIDresource={m, m=0,…M-1} with M=1, 2, 3 or 4 - 1 bit to select WUS resource location Pattern 1 or 2 in Table 1. - Proposal 2: Alternating WUS groups can be enabled when there are more than one WUS resource for Rel-16 WUS. - Proposal 3: At least when legacy WUS is not configured as common WUS, all UE groups per WUS resource are alternated together. Proposal 4: Alternating all UE groups per WUS resource together among M>1 WUS resources for Rel-16 WUS is based on - m=m0+divSFN+1024H_SFNTmin+divSFN+1024HSFNTminLCMM,TcellTminmod M - m0={0,…, M-1} is based on initial WUS resource ID of a UE group as m0=NIDresource if NIDresource=0 is used for group WUS or m0=NIDresource-1, otherwise. - SFN is the radio frame index, H_SFN is the hyper-SFN - Tcell is cell-specific DRX cycle measured in radio frames - Tmin is min UE-specific DRX cycle measured in radio frames For group WUS sequence: Proposal 5: The group WUS base sequence is initialized according to cinit_WUS=(NIDcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDcell+NIDResource∙229 - where NIDResource is WUS resource ID. |
| R1-1912865 | NTT DOCOMO, INC. | Proposal 1: 3-bit indication to configure WUS resource pattern configuration should be supported - At least, pattern 1, 2a, 2b, 3a or 3b, 3c, and 4a should be supported for WUS resource pattern configuration Proposal 2: If both legacy and Rel-16 WUS is configured, time and frequency position of Rel-16 WUS resource among all the WUS resource should be fixed in spec. Proposal 3: UE group ID is numbered within a WUS resource - WUS resource index in which UE group ID belongs needs to be indicated separately Proposal 4: The weighting factor of UE distribution should not be supported in Release 16. |

# Appendix: Agreements in previous meetings

## RAN1#98

Agreement

Both options can be configured simultaneously to have up to 4 orthogonal WUS resources including legacy WUS resource

- Up to 2 orthogonal resources including legacy WUS resource may be configured in time domain

- Up to 2 orthogonal resources may be configured in frequency domain

Note: The two orthogonal resources do not necessarily include the legacy WUS resource

Agreement

The maximum number of UE groups per WUS resource is 8.

Agreement

Different WUS sequences are used in same/different WUS resources.

Working Assumption

To differentiate WUS sequences in different WUS resources, 2-bit MSB of scrambling initialization c_init is supported

Agreement

The following working assumption is confirmed with the modification and under the condition that the eNB can set the power offset between Rel-15 and Rel-16 sequences (UE does not need to know the power offset)

- UE assumes may assume the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Agreement

A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs.

Agreement

The specification supports configurability to enable UE group to be changed among WUS resources.

- FFS: Details including implicit/explict signalling

## RAN1#97

Agreement

Support the following options for WUS resource configuration

- Up to 2 orthogonal resources may be configured in time domain.

- Up to 2 orthogonal resources may be configured in frequency domain

- FFS: both options can be configured simultaneously to have up to 4 orthogonal WUS resources

Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource

For further discussion

Different WUS sequences associated with the same PO are used for UE groups regardless of whether the UE groups are in the same or different WUS resources.

Working Assumption

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Agreement

Each UE monitors up to X WUS sequences.

- Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

## RAN1#96bis

Agreement

For evaluation purpose and down-selection on multiplexing schemes in RAN1#97:

- UE assumes the same WUS max duration and same transmit power for Rel-15 WUS and Rel-16 WUS

Agreement

Down-select one of the following options until RAN1#97 based on evaluation results including power saving gain, usage of resources, etc

- Up to 2 orthogonal WUS resources may be configured in time domain

- Up to 2 orthogonal WUS resources may be configured in frequency domain

- Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

- Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s).

Agreement

Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI

- FFS explicitly or implicitly

- Same WUS parameters are assumed for both Rel-15 and Rel-16 WUS in case both are on the same legacy WUS resource

Agreement

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

## RAN1#96:

Agreement

UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs.

Agreement

- A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location.

Agreement

- Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource.

- Select one among the following:

  - Up to 2 orthogonal WUS resources may be configured in time domain

  - Up to 2 orthogonal WUS resources may be configured in frequency domain

  - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

  - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

- FFS: whether one of the WUS resources is configured to be shared with legacy WUS

Agreement

If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

## RAN1#95:

Agreement

For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- TDM

- FDM

- single-seq CDM

- single-seq CDM+TDM

- single-seq CDM+FDM

- FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

Agreement

For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- single-seq CDM

- FDM

- single-seq CDM+TDM

- single-seq CDM+FDM

Note: At least the maximum number of UE groups should be considered.

Agreement

The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO

Agreement

UE group ID is used as a parameter to generate WUS UE group sequence(s).

Agreement

One group WUS is designed as a single sequence

- Agreement

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

Agreement

Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM.

- No new gap higher layer signaling will be introduced for TDM

In RAN1 #94, we have following agreements:

Agreement

UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing

Agreement

- UE-group MWUS is supported based on eNB’s and UE’s capability.

- Whether the network supports UE-group MWUS is done by higher layer signalling.

- FFS: The number of UE groups is configured by SIB.

- Note that the UE-group MWUS is UE optional

Agreement

Rel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

Agreement

Study the RAN1 consequence of UE-grouping on the following basis:

- UE ID

- Coverage

- DRX/eDRX

- Gap configuration

- Services

## RAN1#94:

Agreement

UE grouping is based on at least UE ID or some function of UE ID

Agreement

The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled

- Performance impact on legacy WUS should be carefully considered

Agreement

Group WUS is based on at least legacy WUS and UE-group ID.

Agreement

Configuration of group WUS is at least signaled in SI

Agreement

A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS